3GPP TSG RAN WG1 Meeting #92bis	 R1-1804445

Sanya, China, April 16th –20th, 2018

Agenda Item:	7.1.2.2.6

Source:	Huawei, HiSilicon

Title:	Further enhancements on CSI reporting and codebook design

Document for:	Discussion and decision

Introduction

At the previous meetings [1-3], it is agreed that:

Agreements:

FFS to support a common design of W2 for Cat. 1, Cat. 2 and Cat. 3

  - FFS for Cat. 3, W2 only feedback is allowed

    - FFS amplitude feedback for W2 (e.g., wideband, subband, etc.)

  - Note: this does not mean NR supports all three categories

FFS whether or not to merge Category 1 and Category 3 using a unified codebook formulation

Agreements:

Slides 4 to 24 in R1-1709232 are agreed

  - For slide 20, FFS whether or not support frequency-dependent parameterization and if so, the details

  - FFS whether or not to further enhance analog beamforming related operations especially for >1 layers

Agreement:

At most RI=2 is allowed to be reported for a CSI report setting configured with Type II codebook.

In this contribution, we discuss the potential enhancement of the DL codebook design in NR. This contribution is a revision of R1-1802074.

Guidelines for codebook design

The following codebooks had been agreed in NR:

- Type I single panel codebook

- Type I multiple panel codebook

- Type II single panel codebook

- Beamformed codebook for type II

With the principle of beam selection, the feedback overhead for type I codebook is considerable low and is comparable with Class A codebook in LTE. However, the performance of type I codebook is still far away from that with ideal feedback. With the principle of beam combination, the performance of type II codebook has a considerable large gain over that of type I codebook, while the feedback overhead increases significantly. Actually, the essential principle of codebook design can be concluded as aiming to guarantee the performance while reducing the feedback overhead as much as possible, or guarantee the feedback overhead while increasing the performance as much as possible. In other words, we should maximize the ratio of performance to overhead. Looking at the performance and feedback overhead of type I and type II codebook, it seems that both type I and type II codebook have some drawbacks.

In eMBB scenario of NR, the performance of MU-MIMO should be guaranteed. At most RI=2 is allowed to be reported for a CSI report setting configured with type II codebook in Rel-15, which means type I codebook and type II codebook cannot be combined for a CSI report with RI>2. Even with type II codebook for RI=1 or 2, the system performance is still far away from that with ideal feedback.

Therefore, in our view, there are some tasks required to be addressed for codebook design:

- Codebook enhancement for type II with rank>2.

- Codebook design to achieve a well trade-off between the performance and feedback overhead

- Codebook design to approach the performance with ideal feedback.

Codebook enhancement for type II

Actually, it is highly necessary to support of high rank codebooks in NR. 32 antenna ports at gNB side has been agreed to support and sufficient space isolation is provided to support MU users with more transmission layers. Meanwhile 4Rx UEs are becoming more and more popular. Simulation results shows that it is of high possibility that UE report rank 3/4.

One straightforward method to design high rank codebook is to separately quantize and feedback combining coefficients in W2 for each layer. However, the CSI reporting overhead for high rank will be very large. Therefore, in our view, the overhead for high rank reporting should be comparable, or at least without significant increase over that for low ranks. On the other hand, the performance should also be guaranteed if the scheduled transmission falls back to rank 1/2.

As discussed above, both type I and type II codebook have some drawbacks, in which the feedback overhead and performance cannot be guaranteed at same time. Beam selection codebook has a considerable low feedback overhead while the performance is limited. Beam combination codebook can achieve a considerable large gain over beam selection codebook while the feedback overhead increases significantly.

Figure 1. Performance comparison of hybrid codebook and type-II-like codebook in UMa.

Figure 2. Performance comparison of hybrid codebook and type-II-like codebook in UMi.

Proposal 1: NR shall extend type II codebook design for rank 3 and 4 by taking into account trade-off between the performance and feedback overhead.

Therefore, it is straightforward to consider a hybrid codebook design with both beam selection and beam combination. Take rank 4 as an example, beam combination codebook is utilized for the first one or two layers and beam selection is utilized for the other layers. For the layers with beam combination, the beams for combination are selected from an orthogonal 2D-DFT beam group. As type II codebook in NR, wideband amplitude, subband amplitude and subband phase are feedback to represent the coefficients for linear combination. For each layer with beam selection, the beam can be selected from the same orthogonal 2D-DFT beam group and may be same or different with the selected beams for the layers with beam combination. To reduce the inter-layer interference, the orthogonality of different layers can be jointly considered in the selection of beams and corresponding co-phasing. For example, a simple way to guarantee the orthogonality is to make sure that no beam is reused in both beam selection layers and beam combination layers.

Preliminary results of system-level simulation for UMa and UMi scenarios are illustrated in Fig. 1 and Fig. 2. The performance gain of type-II-like codebook and the proposed hybrid codebook over type I codebook is shown. The baseline uses type I codebook for rank 1-4. For type-II-like codebook, the agreed type II codebook is reused for rank 1/2, and extends type II for rank 3/4 with each layer quantized independently and same linear combination codebook for each layer. The hybrid codebook also uses the agreed type II for rank 1/2, and hybrid codebook for rank 3/4. It can be seen that the designed hybrid codebook has only around 5% performance loss compared with type-II-like codebook. However, the overhead with hybrid codebook is only around 1/3 of the overhead with type-II-like codebook for rank 3/4 case.

Frequency domain compression

Codebook for frequency domain

The overhead of type II codebook increases approximately linearly with the number of subbands. The overhead would be too large and should be reduced when the number of subbands is considerably large. Since there is correlation between channels of different subbands, the property of the correlation may also be exploited by compression in frequency domain to further reduce the feedback overhead.

Exploiting the property of high channel correlation in frequency domain, channel or coefficients for linear combination in type II from all the subbands can be compressed and jointly quantized.

Figure 3. Illustration of space-frequency matrix.

For each specified layer, a space-frequency matrix can be obtained by concatenating the precoding vectors of different subbands. The dimension of the space-frequency matrix is X×N, where X denotes the number of TXRU ports and N denotes the number of subbands. An illustration of space-frequency matrix is shown in Fig. 3. The space-frequency matrix can be compressed to reduce the feedback overhead.

Due to the channel correlation of different subbands, there may exist some basic patterns along the frequency dimension for rows of space-frequency matrix. All the basic patterns make up a frequency basic codebook. The rows of space-frequency matrix can be approximately represented as linear combinations of several items within the frequency basic codebook, similar to the existing approach in the spatial domain.

In existing feedback approaches, e.g. type II codebook, spatial domain compression would be achieved to reduce the overhead. However, correlation remains existing between coefficients for adjacent subbands after spatial compression. The idea of frequency domain compression above can also be applied to the compressed coefficients after spatial domain compression (e.g. W2 in type II) of different subbands, in which case the space-frequency matrix consists of spatial combination coefficients from different subbands. Then frequency domain compression can be used along with spatial domain compression, which may further compress channel and reduce the feedback overhead.

The space-frequency matrix can also be compressed by joint compression of spatial and frequency domains, in which case two basic codebooks are introduced for both domains, respectively.

A previous approach utilizes the frequency correlation of the beam combining coefficients to reduce the PMI feedback overhead [4], in which FFT and IFFT operations are involved. However, it is difficult to define FFT or IFFT operations in spec because implementation algorithms of gNB and UE may be involved. By introducing codebook for frequency domain, the FFT and IFFT operations can be implemented as a form of frequency codebook consisting of DFT basis.

Proposal 2: Codebook for frequency domain compression should be further studied to reduce feedback overhead in NR.

Potential form of space-frequency matrix with two codebooks

It is natural to consider the precoding matrix relatively independent for each layer, as the existing type II approach. However, by introducing a frequency domain codebook, the spatial precoding vectors of all the subbands in type II can be concatenated and form a space-frequency matrix. Considering a single layer, the type II precoding vector of subband n can be written as

wn=Wspacew2n

where wn is a column vector with dimension of X×1, and Wspace consists of the wideband spatial 2D-DFT beams with dimension of X×2L, and w2n is a vector of combination coefficients with dimension of 2L×1, where X denotes the number of TXRU ports and L denotes the number of spatial beams for each polarization. By concatenating the precoding vectors of different subbands, a space-frequency matrix is obtained as W=w1…wN, and then

W=w1…wN=WspaceW2

where W2=w2(1)…w2(N), the dimension of W2 is 2L×N and N is the number of subbands. The matrix W2 is a concatenating matrix of the spatial combination coefficients. Each row of W2 can be represented as a linear combination of items in frequency domain codebook, with a mathematical form of

W2=WWfreqH

where Wfreq is a matrix of N×K consisting of K selected items in the frequency codebook and W is a combination coefficients matrix whose dimension is 2L×K.

Therefore, the original space-frequency matrix W can be represented by the following form

W=WspaceWWfreqH

where Wspace and Wfreq are the spatial codebook and frequency codebook, respectively. The dimension of the coefficients matrix W is 2L×K, with L and K depend on the feedback precision. With this space-frequency codebook form involving two sets of basic codebooks, the compression of space-frequency domain is achieved.

Proposal 3: Representation of space-frequency matrix with two sets of basic codebooks can be further studied in NR.

Simulation results

Preliminary results of system-level simulation for UMa and UMi scenarios are illustrated in Fig. 4 and Fig. 5. The proposed space-frequency compression codebook follows the form in section 4.2. The spatial basis reuses the oversampled 2D-DFT beam, while the frequency basis use a DFT beam. The coefficients matrix W, whose size is 2L×K is quantized with 3 bits for amplitude and 3 bits for phase. Detailed parameters are shown in the appendix.

Figure 4. Performance comparison of space-frequency compression codebook and type-II codebook in UMa.

Figure 5. Performance comparison of space-frequency compression codebook and type-II codebook in UMi.

According to the simulation result, it can be seen that under the given parameter setting the proposed space-frequency codebook has almost the same performance, while the cell edge performance of the proposed codebook increases by 19%. The overhead of the proposed codebook is only 42% of type II codebook. Compared with Type II codebook, the proposed space-frequency codebook can reduce the overhead significantly with a similar performance.

Actually, the basis for spatial domain and frequency domain can be optimized to further improve the system throughput as well as the quantization method of coefficients for linear combination.

Conclusions

The contribution discusses the codebook design or enhancement for Rel-16, based on which the following proposals are made.

Proposal 1: NR shall extend type II codebook design for rank 3 and 4 by taking into account trade-off between the performance and feedback overhead.

Proposal 2: Codebook for frequency domain compression should be further studied to reduce feedback overhead in NR.

Proposal 3: Representation of space-frequency matrix with two sets of basic codebooks can be further studied in NR.

- References

“RAN1 Chairman’s Notes”, Spokane, USA, April 3-7, 2017.

“RAN1 Chairman’s Notes”, Hangzhou, China, May 15-19, 2017.

“RAN1 Chairman’s Notes”, Prague, Czech Republic, October 9-13, 2017.

Ericsson, “Frequency parameterization for Type II CSI feedback”, R1-1705927, Spokane, USA, April 3-7, 2017.

- Appendix

| Parameters | UMa | UMi |
| --- | --- | --- |
| Carrier frequency | 4GHz |  |
| Subcarrier spacing | 15kHz |  |
| System Bandwidth | 10MHz |  |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites, 570 UEs |  |
| ISD | 500m | 200m |
| Minimum distance | 35m | 10m |
| TP antenna height | 25m | 10m |
| TP Tx power | 46dBm/10MHz | 41dBm/10MHz |
| TP antenna configuration | X-pol (+/-45), (dH, dV) = (0.5, 0.8) λ, (M, N, P, Mg, Ng) = (4,4,2,1,1) |  |
| UE antenna configuration | X-Pol (M, N, P, Mg, Ng) = (1,2,2,1,1) |  |
| UE antenna pattern | Omnidirectional |  |
| UE antenna height | 1.5m |  |
| UE dropping | indoor UE 80%, outdoor UE 20% |  |
| UE speed | indoor UE 3km/h, outdoor UE 30km/h |  |
| UE receiver | MMSE-IRC |  |
| Scheduler | PF |  |
| Traffic model | Full buffer |  |