3GPP TSG RAN WG1 Meeting #92bis                       R1-1804540

Sanya, China, April 16th – 20th, 2018

Agenda Item:	7.1.1.5.1

Source: 	LG Electronics

Title: 	Remaining details on L3 measurement and mobility management

Document for:	Discussion and decision

- Introduction

For stabilization of the specification, lots of issues related to RRM measurement had been handled and the agreements were made during last meetings. However, some issues remain unresolved, and we discuss about remaining issues listed in below.

- Resource configuration for RSSI measurement

  - RSSI measurement bandwidth

- CSI-RS resource configuration in C-DRX mode

- Rate matching in SMTC window without Measurement Gap

- Resources for RSSI measurement with SS/PBCH block

It was agreed in RAN1 #91 that the time-domain measurement resource for RSSI in SSB-based RRM measurement was finalized. On the other hand, we did not get any agreement about RSSI measurement resources in frequency domain. In this section, we discuss about the RSSI measurement resource in frequency domain.

RSSI measurement bandwidth

Wide bandwidth for RSSI measurement is desirable for reflecting real traffic load as well as accurate measurement if measurement bandwidth does not exceed the system bandwidth. Therefore, if it is not indicated that the bandwidth of active BWP (measurement bandwidth) exceeds the system bandwidth of neighbour cells in the frequency carrier, the bandwidth of active BWP should be configured as the bandwidth for RSSI measurement in case of intra-frequency measurement. However, since the bandwidth part is not defined for inter-frequency or inter-RAT measurement, the bandwidth of RSSI measurement should be same with the RSRP measurement bandwidth in those cases.

Proposal 1. The bandwidth for RSSI measurement is configured with the bandwidth of active BWP if the bandwidth of active BWP does not exceeds the system bandwidth of neighbour cells in case of intra-frequency measurement. Otherwise, the bandwidth for RSSI measurement should be same with the bandwidth of RSRP measurement.

- Resource configuration in C-DRX mode

In DRX mode, UE tries to operate (e.g. PDCCH reception, tracking operation, RRM measurement including RLM and neighbour cell measurement, etc) for as short duration as possible in order to minimize UE power consumption. For this operation in C-DRX mode, CSI-RS resource configuration and UE measurement operation in C-DRX mode was discussed in last meeting, and it was agreed that UE is not required to measures CSI-RS outside the active time. However, we did not reach any conclusion on CSI-RS resources configuration and UE operation for other time region.

As mentioned in above, UE generally tries to stay awake for as short time as possible in C-DRX mode. However, if it is determined that measurement results for serving cell after waking up is not good or beam tracking for neighbour cell is needed (which is a choice according to UE implementation), it might be beneficial that UE does not enter sleep mode and needs to keep measuring CSI-RS while changing receiver beam. Therefore, for stable connection, network needs to configure same CSI-RS periodicity with non-C-DRX mode and UE performs RRM measurement outside active time whenever necessary. In conclusion, we propose that same periodicity of CSI-RS resources is kept in C-DRX mode and it is up to UE implementation that UE measure CSI-RS resources of neighbour cell even outside active time.

Proposal 2. UE assumes that the periodicity of CSI-RS resources configured before entering C-DRX mode is kept during C-DRX mode, and it is up to UE implementation that UE measures CSI-RS resources of neighbour cell even outside active time.

- Rate matching in SMTC window without Measurement Gap

According to RAN4 agreement (R4-1801088, shown in below text box), if SMTC window is not overlapped with measurement gap, network could transmit PDCCH and/or PDSCH over certain OFDM symbols within a SMTC window, except for some OFDM symbols (e.g. SSB symbols to be measured, X symbol(s) before each consecutive SSB symbols and X symbol(s) after each consecutive SSB symbols). However, depending on the number of X, the number of available symbols within in a slot might not be enough for PDSCH transmission if SS/PBCH block is transmitted in the slot. In addition, if some OFDM symbols in CORESET is overlapped with the non-available symbols, stable reception of PDCCH might not be guaranteed by UE or redundant CORESET configuration is needed for PDCCH transmission in SMTC window. Therefore, it might be desirable that PDCCH/PDSCH is not scheduled, at least to eMBB UE, in a slot containing SS/PBCH block(s) within a SMTC window in FR2.

Within a SMTC window which is not overlapped with measurement gap in non-CA case:

- In FR1 intra-frequency SS-RSRP/RSRQ/SINR measurement:

  - For UE not supporting mixed numerology scenarios, UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on SSB symbols to be measured, [1] symbol before each consecutive SSB symbols and [1] symbol after each consecutive SSB symbols within SMTC window duration if useServingCellTimingForSync is enabled

  - For UE not supporting mixed numerology scenarios, UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on all symbols within SMTC window duration if useServingCellTimingForSync is not enabled

- In FR2 intra-frequency SS-RSRP/SINR measurement:

  - UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on SSB symbols to be measured, X symbol(s) before each consecutive SSB symbols and X symbol(s) after each consecutive SSB symbols within SMTC window duration (assuming that  useServingCellTimingForSync is always enabled for FR2)

- In FR2 intra-frequency SS-RSRQ measurement:

  - UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on SSB symbols to be measured , RSSI measurement symbols, X symbol(s) before each consecutive SSB/RSSI symbols and X symbol(s) after each consecutive SSB/RSSI symbols within SMTC window duration (assuming that  useServingCellTimingForSync is always enabled for FR2)

- Note: whether or not to define specific behavior for rate matching around above symbols is up to RAN1

- Note: RAN4 needs to check if cell synchronization accuracy complies with cell phase synchronization requirement in TS 38.133 when useServingCellTimingForSync is enabled

- Regarding the value of “X”,

  - Option 1: X = [1]

  - Option 2: Companies are encouraged to investigate the value of X. If it is identified that X needs to be large value e.g., more than 4, following alternative solutions should be considered

    - SMTC-based solution, i.e., all symbols within SMTC window are not available for data transmission/reception

    - SSB-slot-based solution, i.e., all symbols within slot(s) containing SSB are not available for data transmission/reception

- Conclusion

In this contribution, we discussed about remaining items for RRM measurement configuration, and our proposals are given as follows:

Proposal 1. The bandwidth for RSSI measurement is configured with the bandwidth of active BWP if the bandwidth of active BWP does not exceeds the system bandwidth of neighbour cells in case of intra-frequency measurement. Otherwise, the bandwidth for RSSI measurement should be same with the bandwidth of RSRP measurement.

Proposal 2. UE assumes that the periodicity of CSI-RS resources configured before entering C-DRX mode is kept during C-DRX mode, and it is up to UE implementation that UE measures CSI-RS resources of neighbour cell even outside active time.

- Reference

RAN1 #92 Chairman’s notes