TSG-RAN WG1 #92bis	R1-1805663

Sanya, China, April 16 – 20, 2018

Source:	Ericsson

Title:	Summary of offline discussion on 7.1.3.3.1 (resource allocation) – part II

Agenda Item:	7.1.3.3.1

Document for:	Discussion and Decision

# Discussion

## Default tables for system information delivery

Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 1

| DL-DMRS-typeA-pos | I | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| (0) 2 | 0 | Type A | 0 | 2 | 12 |
| (1) 2 | 1 | Type A | 0 | 2 | 10 |
| (2) 2 | 2 | Type A | 0 | 2 | 9 |
| (new) 2 | 4 | Type A | 0 | 2 | 5 |
| (6) 2 | 5 | Type B | 0 | 9 | 4 |
| (5) 2 | 6 | Type B | 0 | 4 | 4 |
| (7) 3 | 0 | Type A | 0 | 3 | 11 |
| (8) 3 | 1 | Type A | 0 | 3 | 9 |
| (9) 3 | 2 | Type A | 0 | 3 | 8 |
| (11) 3 | 4 | Type A | 0 | 3 | 4 |
| (12) 3 | 5 | Type B | 0 | 10 | 4 |
| (16) 3 | 6 | Type B | 0 | 6 | 4 |
| (4) x | 7 | Type B | 0 | 5 | 7 |
| (13) x | 8 | Type B | 0 | 5 | 2 |
| (14) x | 9 | Type B | 0 | 9 | 2 |
| (15) x | 10 | Type B | 0 | 12 | 2 |
| Include | 11 | Type A | 0 | 1 | 13 |
| Include | 12 | Type A | 0 | 1 | 6 |
| Include 2 | 3 | Type A | 0 | 2 | 7 |
| Include 3 | 3 | Type A | 0 | 3 | 6 |
| Include | 13 | Type A | 0 | 2 | 4 |
| Include | 14 | Type B | 0 | 4 | 7 |
| Include | 15 | Type B | 0 | 8 | 4 |

Same table sorted differently for clarity (sorting done by feature lead after the session):

| L | DL-DMRS-typeA-pos | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |
| 1 | 2 | Type A | 0 | 2 | 10 |
|  | 3 | Type A | 0 | 3 | 9 |
| 2 | 2 | Type A | 0 | 2 | 9 |
|  | 3 | Type A | 0 | 3 | 8 |
| 3 | 2 | Type A | 0 | 2 | 7 |
|  | 3 | Type A | 0 | 3 | 6 |
| 4 | 2 | Type A | 0 | 2 | 5 |
|  | 3 | Type A | 0 | 3 | 4 |
| 5 | 2 | Type B | 0 | 9 | 4 |
|  | 3 | Type B | 0 | 10 | 4 |
| 6 | 2 | Type B | 0 | 4 | 4 |
|  | 3 | Type B | 0 | 6 | 4 |
| 7 | x | Type B | 0 | 5 | 7 |
| 8 | x | Type B | 0 | 5 | 2 |
| 9 | x | Type B | 0 | 9 | 2 |
| 10 | x | Type B | 0 | 12 | 2 |
| 11 | x | Type A | 0 | 1 | 13 |
| 12 | x | Type A | 0 | 1 | 6 |
| 13 | x | Type A | 0 | 2 | 4 |
| 14 | x | Type B | 0 | 4 | 7 |
| 15 | x | Type B | 0 | 8 | 4 |

Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 2

| I | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type B | 0 | 2 | 2 |
| 1 | Type B | 0 | 4 | 2 |
| 2 | Type B | 0 | 6 | 2 |
| 3 | Type B | 0 | 8 | 2 |
| 4 | Type B | 0 | 10 | 2 |
| 5 | Type B | 1 | 2 | 2 |
| 6 | Type B | 1 | 4 | 2 |

Note: From a table size perspective, it is possible to add 7 more rows with same K0 and S as above but with L=4 as well as 2 more rows S=0 and L=4.

Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 3

| i | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- |
| 0 | Type B | 0 | 4 | 2 |
| 1 | Type B | 0 | 6 | 2 |
| 2 | Type B | 0 | 8 | 2 |
| 3 | Type B | 0 | 10 | 2 |

Note: From a table size perspective, additional entries are possible.

## Need for SSB rate matching indication in DCI

No agreement on whether it is needed or not.

## Allocation for OSI, RA-RNTI, etc

If SIB1 configures a table with values for time-domain allocation, use this table for

- at least for CSS configured by MIB/RMSI: RA-RNTI and TC-RNTI (msg4)

- FFS: OSI, P-RNTI

If SIB1 does not provide a table, use the RMSI table above.

- FFS: which pattern (Same pattern as used for SIB1? Always pattern 1? New pattern? …)