3GPP TSG RAN WG1 Meeting #92bis                                                                 R1-1803786

Sanya, China, April 16th – 20th, 2018

Source:	CATT

Title:	Initial self evaluation on peak data rate and peak spectrum efficiency

Agenda Item:	7.8.2

Document for:	Discussion and Decision

- Introduction

According to Report ITU-R M.2410 [2], the peak data rate and peak spectral efficiency is defined as in subsection 4.1 and 4.2:

Peak data rate is related to the peak spectral efficiency in a specific band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the user peak data rate Rp is given by:

Rp = W × SEp	                                         (1)

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is

R=i=1Q  Wi × SEpi 	(2)

where Wi and SEpi (i = 1,…Q) are the component bandwidths and spectral efficiencies respectively.

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz)。

The minimum requirements for peak data rate and peak spectral efficiency are provide in Report ITU-R M.2410 [2] and summarized in  Table 1-1.

Table 1-1  the requirements of peak data rate and peak spectral efficency

| Performance Measure | DL | UL |
| --- | --- | --- |
| Peak data rate | 20 Gb/s | 10 Gb/s |
| Peak spectral efficiency | 30 bps/Hz | 15 bps/Hz |

Also in Report ITU-R M.2412[3], more detailed evaluation methodologies are defined, where it is noted that proponents should demonstrate that the peak spectral efficiency requirement can be met for, at least, one of the carrier frequencies assumed in the test environments under the eMBB usage scenario, and proponents should demonstrate that the peak data rate requirement can be met for, at least, one carrier frequency or a set of aggregated carrier frequencies (where it is the case), assumed in the test environments under the eMBB usage scenario.

In this document, the peak spectrum efficiency and peak data rate evaluation for NR are considered and updated. Considering NR works on two frequency ranges, sub 6 GHz and above 24 GHz, this document presents the considerations for both frequency ranges.

- Discussions on methodology

Besides the definitions and evaluation methodologies in [2][3], there was also discussion in RAN1[1], which summarized the approximate data rate for a given number of aggregated carriers in a band or band combination as follows for potential peak data rate calculation when considering UE category. That is also helpful and can be referred to investigate peak data rate facing to self-evaluation, which is aligned with the definition in ITU [3].

(3)

Wherein：

- J is the number of aggregated component carriers in a band or band combination

- is the highest coding rate

- For the j-th CC:

- is the maximum number of layers

- is the maximum modulation order

- is the scaling factor

- The scaling factor can at least take the values 1 and 0.75.

- is signalled per band and per band per band combination as per UE capability   signalling

- is the numerology (as defined in TS38.211)

- is the average OFDM symbol duration in a subframe for numerology , i.e. .   Note that normal cyclic prefix is assumed.

- is the maximum RB allocation in bandwidth  with numerology , as given     in  TR 38.817-01 section 4.5.1 (to be eventually defined in TS 38.101), where  is the UE supported maximum bandwidth in the given band or band combination

- is the overhead

From (1)  , peak spectral efficiency for a specific component carrier can be derived by the peak data rate and the available bandwidth of the specific component carrier  :

(4)

Wherein:

- is the peak spectral efficiency of the jth component carrier

- is the bandwidth of the jth component carrier

The above formulas are applied to the following sessions for calculating peak data rate and peak spectral efficiency.

- Initial evaluation results

  - DL assumption

The values of the key parameters are needed and their assumed values for DL peak data rate are provided as below:

Table 3-1   Parameters for DL peak data rate

| Parameters | Values | Remarks |
| --- | --- | --- |
|  | 8 | NR supports up to 8 layers for a single user for DL |
|  | 8 | NR supports up to 256QAM for DL |
| f | 1 | The value of 1 is chosen as scaling factor for DL peak spectral efficiency evaluation. |
|  | 0.9258 | NR supports highest coding rate as =948/1024. |

The maximum transmission bandwidth configuration NRB for each BS channel bandwidth and subcarrier spacing is specified in table 5.3.2.-1 for FR1 and FR2 in Section 5.3.2 of TS38.104[4] . Considerating the requirement  of peak data rate ,the following bandwidths of a component carrier are firstly investigated.

Table 3-2  DL maximum spectrum utilization (in PRB) for various bandwidth/subcarrier spacing

|  | FR1 |  |  |  |  |  |  |  | FR2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 MHz |  |  | 50 MHz |  |  | 100 MHz |  | 50 MHz |  | 100 MHz |  | 200 MHz |  | 400 MHz |
| SCS(Khz) | 15 | 30 | 60 | 15 | 30 | 60 | 30 | 60 | 60 | 120 | 60 | 120 | 60 | 120 | 120 |
|  | 106 | 51 | 24 | 270 | 133 | 65 | 273 | 135 | 66 | 32 | 132 | 66 | 264 | 132 | 264 |

For the assessment of DL peak data rate , overhead due to control channel and reference signal must be considered. SSB, TRS, PDCCH, DM-RS, CSI-RS ,CSI-IM and PTRS assumed for FR1 and FR2 are summarized in Table 3-3. It is noted that overhead assumed in this table is only for reasonable peak data rate calculation, but not meant it also applies to self evaluation for e.g. spectral efficiency.

Table 3-3 DL evaluation assumption for peak data rate and spectral efficiency

| Parameter | Setting |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| SSB | 2 SSB per 20 ms | 16 SSB per 20 ms |
| TRS | 50 PRB wide, occurs every 80 ms | 50 PRB wide, occurs every 10 ms |
| PDCCH | 4 CCE in every slot | 4 CCE in every slot |
| DM-RS | DMRS config2 16RE/slot/RB | DMRS config1 24RE/slot/RB |
| CSI-RS | 8 CSI-RS ports with 1 RE/RB/port, with periodicity of 20ms | 8 CSI-RS ports with 1 RE/RB/port,with periodicity of 2.5ms |
| CSI-IM | 4 RE/RB, with periodicity of 20 ms | 4 RE/RB, with periodicity of 2.5 ms |
| PTRS | Null | 2 ports PTRS, Frequency density is 4 RB, and time domain density is 1 symbol |

  - DL evaluation results

Based on the above assumptions and considerations, the initial evaluation results of peak spectral efficiency for FR1 and FR2 are provided in Table 3-4 and Table 3-5. Also, the initial evaluation results of peak data rate for FR1 and FR2 are provided in Table 3-6.

Table 3-4  Initial evaluation results of DL peak spectral effiency and overhead for FR1 under a certain component carrier

| Requirement (bps/Hz)[2] | FR1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | BW(MHz) of a component carrier | 20 |  |  | 50 |  |  | 100 |  |
|  | SCS(Khz) | 15 | 30 | 60 | 15 | 30 | 60 | 30 | 60 |
|  | overhead% | 12.08 | 13.67 | 17.40 | 10.74 | 11.22 | 12.49 | 10.44 | 11.00 |
|  | SE-peak(bps/Hz) | 46.59 | 44.02 | 39.64 | 48.20 | 47.23 | 45.50 | 48.89 | 48.06 |

Table 3-5 Initial evaluation results of DL peak spectral effiency and overhead for FR2 under a certain component carrier

| Requirement (bps/Hz)[2] | FR2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | BW(MHz) of a component carrier | 50 |  | 100 |  | 200 |  | 400 |
|  | SCS(Khz) | 60 | 120 | 60 | 120 | 60 | 120 | 120 |
|  | overhead% | 23.04 | 25.50 | 20.80 | 21.75 | 19.69 | 19.98 | 19.10 |
|  | SE-peak(bps/Hz) | 40.63 | 38.14 | 41.81 | 41.32 | 42.40 | 42.25 | 42.71 |

Table 3-6 Initial evaluation results of DL peak data rate

| Requirement (Gbit/s)[2] |  | BW(MHz) of a component bandwidth | SCS(KHz) | Total CA BW (MHz) | Peak data rate(Gbit/s) |
| --- | --- | --- | --- | --- | --- |
| 20 | FR1 | 50 | 15 | 450 | 21.68 |
|  |  |  | 30 |  | 21.25 |
|  |  |  | 60 |  | 20.48 |
|  |  | 100, 20 | 30 | 420 | 20.44 |
|  |  |  | 60 |  | 20.02 |
|  | FR2 | 50 | 60 | 550 | 22.35 |
|  |  |  | 120 |  | 20.98 |
|  |  | 100 | 60 | 500 | 20.90 |
|  |  |  | 120 |  | 20.65 |
|  |  | 200, 100 | 60 | 500 | 21.14 |
|  |  |  | 120 |  | 21.03 |
|  |  | 400, 100 | 120 | 500 | 21.22 |

Note: when different types of component carrier are applied to peak data rate calculation, e.g. 100MHz+20MHz, spectral efficiency for each component carrier is Table 3-4 and 3-5 is used respectively.

Observation 1:  NR can fulfill the peak spectral efficiency requirements of DL for both below and above 6GHz.

Observation 2: NR can fulfill the peak data rate requirements of DL for both below and above 6GHz.

Observation 3: Since DL peak spectral efficiency of a certain component carrier below 6GHz is higher than that of above 6GHz, for some of cases, more component carriers are needed to meet the requirement of peak data rate.

  - UL assumption

The values of the key parameters are the same compared to DL in Table 3-1 except the maxim number of layer  is 4.

The maximum transmission bandwidth configurations NRB are the same as those for DL in Table 3-2.

For the assessment of UL peak data rate, overhead due to control channel and reference signal must be considered. SRS, PUCCH, DM-RS and PTRS assumed for FR1 and FR2 are summarized in Table 3-7. It is noted that overhead assumed in this table is only for reasonable peak data rate calculation, but it does not meant that it also applies to self evaluation for e.g. spectral efficiency.

Table 3-7 UL evaluation assumption for peak data rate and spectral efficiency

| Parameter | Setting |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| SRS | Resources ensuring the whole bandwidth measured within 20ms | Resources ensuring the whole bandwidth measured within 2.5ms |
| PUCCH | Short PUCCH with 1 PRB over 1 symbol every slot | Short PUCCH with 1 PRB over 1 symbol every slot |
| DM-RS | DMRS config2 8RE/slot/RB | 1 complete symbol per slot (config.1) |
| PTRS | Null | 2 ports PTRS, Frequency density is 4 RB, and time domain density is 1 symbol |

  - UL evaluation results

Based on the above assumptions and considerations, the initial evaluation results of peak spectral efficiency for FR1 and FR2 are provided in Table 3-8 and Table 3-9. Also, the initial evaluation results of peak data rate for FR1 and FR2 are provided in Table 3-10.

Table 3-8 Initial evaluation results of UL peak spectral effiency and overhead for FR1 under a certain component carrier

| Requirement (bps/Hz)[2] | FR1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | BW(MHz) of a component carrier | 20 |  |  | 50 |  |  | 100 |  |
|  | SCS(Khz) | 15 | 30 | 60 | 15 | 30 | 60 | 30 | 60 |
|  | overhead% | 5.19 | 5.08 | 5.15 | 5.15 | 4.99 | 4.96 | 4.97 | 4.90 |
|  | SE-peak(bps/Hz) | 25.12 | 24.20 | 22.76 | 25.61 | 25.27 | 24.71 | 25.94 | 25.67 |

Table 3-9 Initial evaluation results of UL peak spectral effiency and overhead for FR2 under a certain component carrier

| Requirement (bps/Hz)[2] | FR2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | BW(MHz) of a component carrier | 50 |  | 100 |  | 200 |  | 400 |
|  | SCS(Khz) | 60 | 120 | 60 | 120 | 60 | 120 | 120 |
|  | overhead% | 11.54 | 11.29 | 11.48 | 11.18 | 11.46 | 11.13 | 11.10 |
|  | SE-peak(bps/Hz) | 23.35 | 22.71 | 23.37 | 23.45 | 23.37 | 23.46 | 23.47 |

Table 3-10 Initial evaluation results of UL peak data rate

| Requirement (Gbit/s)[2] |  | BW(MHz) of a component bandwidth | SCS(KHz) | Total CA BW (MHz) | Peak datarate(Gbit/s) |
| --- | --- | --- | --- | --- | --- |
| 10 | FR1 | 50,20 | 15 | 420 | 10.75 |
|  |  |  | 30 |  | 10.61 |
|  |  |  | 60 |  | 10.37 |
|  |  | 100 | 30 | 400 | 10.38 |
|  |  |  | 60 |  | 10.27 |
|  | FR2 | 50 | 60 | 450 | 10.51 |
|  |  |  | 120 |  | 10.21 |
|  |  | 100,50 | 60 | 450 | 10.52 |
|  |  |  | 120 |  | 10.55 |
|  |  | 200,50 | 60 | 450 | 10.52 |
|  |  |  | 120 |  | 10.55 |
|  |  | 400,50 | 120 | 450 | 10.56 |

Note: when different types of component carrier are applied to peak data rate calculation, e.g. 50MHz+20MHz, spectral efficiency for each component carrier is Table 3-8 and 3-9 is used respectively.

Observation 4:  NR can fulfill the peak spectral efficiency requirements of UL for both below and above 6GHz.

Observation 5: NR can fulfill the peak data rate requirements of UL for both below and above 6GHz.

Observation 6: Since UL peak spectral efficiency of a certain component carrier below 6GHz is higher than that of above 6GHz, for some of cases, more component carriers are needed to meet the requirement of peak data rate.

- Conclusion

In this document, the peak spectral efficiency and peak data rate of NR for the two frequency ranges are analysed. The following observations are made.

Observation 1:  NR can fulfill the peak spectral efficiency requirements of DL for both below and above 6GHz.

Observation 2: NR can fulfill the peak data rate requirements of DL for both below and above 6GHz.

Observation 3: Since DL peak spectral efficiency of a certain component carrier below 6GHz is higher than that of above 6GHz, for some of cases, more component carriers are needed to meet the requirement of peak data rate.

Observation 4:  NR can fulfill the peak spectral efficiency requirements of UL for both below and above 6GHz.

Observation 5: NR can fulfill the peak data rate requirements of UL for both below and above 6GHz.

Observation 6: Since UL peak spectral efficiency of a certain component carrier below 6GHz is higher than that of above 6GHz, for some of cases, more component carriers are needed to meet the requirement of peak data rate.

- References

  - RP-172172, “Reply to LS on NR UE Category”, Ericsson,Intel

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - TS38.104 V15.0.0.0, “Base Station (BS) radio transmission and reception”,Dec,2017