3GPP TSG RAN WG1 Meeting #93				                          R1-1805845

Busan, Korea, May 21st – May 25th, 2018

Source:                ZTE

Title:                    Link-level simulation results for NOMA

Agenda item:      7.4.5

Document for:    Discussion and Decision

# Background

The link-level performance metrics and simulation assumptions for NoMA study were agreed in RAN1#92 and RAN1#92bis meeting [1][2].

In this contribution, we present some preliminary link-level evaluation results for the QAM-sequence based symbol-level spreading scheme (MUSA) based on the agreed LLS assumptions.

# Preliminary link-level evaluation results for MUSA

## Simulation parameters

Considering the simulation efforts, not all the parameters with multiple options are evaluated. The parameters selected for this initial submission can be found as below and the full set of the assumptions can be found in the Appendix.

Table 1 Parameters selected for this initial submission

| Parameter with "or" condition | For the initial submission in this contribution |
| --- | --- |
| Carrier frequency for URLLC: 700MHz or 4GHz | 4GHz |
| Numerology for URLLC: 60 or 30kHz | 60kHz with #OS=7 |
| BS antenna for 700MHz: 2 or 4 | 2 |
| BS antenna for 4GHz: 4 or 8 | 4 |

The spreading sequences used for the simulation are listed in the Appendix 1. For each set of TB size and number of UEs, the spreading factor, and modulation order are adjusted in order to achieve better performance in some of the simulation cases. In addition, as analyzed in [3], multi-layer transmission per UE could be adopted to further enhance the performance for very high per UE spectral efficiency.

Linear MMSE with hard-SIC receiver is adopted where the details can be found in [4]. For realistic channel estimation, NR DMRS design with 1/7 overhead is adopted as starting point. For mMTC and eMBB with 14 OFDM symbols, NR DMRS can support up to 12 ports in the case of DMRS type 2. For the case of more than 12 UEs, we simply double the number of FDM combs to increase the orthogonal ports to e.g. 24. Consequently, the accuracy of channel estimation would suffer due to the less number of estimation samples in frequency domain.

## Preliminary simulation results

- mMTC, fixed MA signature allocation

- Ideal channel estimation                                   (b) Realistic channel estimation

Figure 1 Performance evaluation of symbol-level linear spreading for mMTC with 20 Byte and equal avg. SNR.

The results for mMTC with 20-Byte TBS under different number of UEs are shown in Figure 1. Compared with OMA single user which is the upper bound of per UE performance, 20 UEs can be multiplexed with less than 2dB SNR loss for both ideal and realistic channel estimation. The achievable sum throughput corresponding to 20 UEs with 0.1 BLER is 2.67 bps/Hz where the per UE SE = (20*8bits)/(6*12*15kHz)/1ms.

Figure 1 shows the results with the same assumptions but at realistic channel estimation. It can be found that for TDL-A model, there will be about 0.5 dB loss for realistic channel estimation (RCE) compared with ideal channel estimation (ICE) based on NR DMRS for up to 12 UEs, and about 1dB loss for RCE based on NR DMRS with double number of ports , e.g.,  up to 24 UEs.

- Ideal channel estimation                                   (b) Realistic channel estimation

Figure 2 Performance evaluation of symbol-level linear spreading for mMTC with 40 Byte and unequal avg. SNR.

For lager TBS, the overloading capability of NOMA is expected to be reduced since the total spectral efficiency the system can achieve is limited. For example as shown in Figure 2, 12 UEs can be multiplexed with less than 3 dB SNR loss compared to the OMA single user case. The achievable sum throughput corresponding to 12 UEs with 0.1 BLER is 3.2 bps/Hz where the per UE SE = (40*8bits)/(6*12*15kHz)/1ms.

- mMTC, random MA signature allocation

Data-only based symbol-level spreading scheme is considered for the evaluation in the case of random MA signature allocation. Random collision of MA signatures (i.e. spreading sequences and DMRS/preamble) is allowed so that NOMA can operate in “true” grant-free settings. For data-only solution, there is no dedicated DMRS involved in the transmitter side processing. The user identification and channel estimation are based on the data signal itself with the only prior information being modulation order. As shown in our companion contribution [4], the receiver implementation for the NOMA transmission operates in more “blind” fashion, without the aid of DMRS/preamble detection.

| (a) TBS = 10bytes, TDL-A 30ns | (b) TBS = 10bytes, TDL-C 300ns |
| --- | --- |
| (c) TBS = 20bytes, TDL-A 30ns | (d) TBS = 20bytes, TDL-C 300ns |

Figure 3 Performance evaluation of mMTC with random MA signature allocation, equal avg. SNR.

Figure 3 shows some preliminary simulation results on BLER vs SNR for data-only based NOMAs scheme with symbol-level linear spreading. From these simulation results we can observe that high loading of users can be achieved with limited per UE performance loss respected to single user upper bound. More results of data-only solution can be found in the Appendix 2.

- URLLC & eMBB, fixed MA signature allocation

Due to more number of Rx antennas assumed for URLLC and eMBB use cases, the overloading capability of symbol-level linear spreading can be further enhances. For example for URLLC case with 60byte TBS as shown in Figure 4, there is no performance loss for number of UEs less than 8 with ideal channel estimation. However, significant performance degradation can be observed for realistic channel estimation, which means the current NR design of DMRS may need enhancement for the NOMA transmission under mini-slot based assumptions.

- Ideal channel estimation                                   (b) Realistic channel estimation

Figure 4 Performance evaluation for URLLC with 4Rx and 60bytes TBS.

For eMBB case as shown in Figure 5, 24 UEs (with 3.2 bps/Hz sum throughput) and 20 UEs (with 5.33 bps/Hz sum throughput) can be achieved for TBS with 40 Bytes and 80 Bytes respectively, with less than 3dB per UE SNR loss compared with the OMA single-user reference.

Figure 5 Performance evaluation for eMBB with equal avg. SNR and Realistic CE.

Observation 1: NOMA with QAM-sequence based symbol-level linear spreading can achieve excellent overloading capability with very limited per UE performance loss in all the interested use scenarios.

Proposal 1: Support QAM-sequence based symbol-level linear spreading for NOMA.

# Conclusion

In this contribution, we provided some link level simulations and preliminary performance comparisons among different NOMA schemes from various aspects. Some proposals and observations were made as follows.

Observation 1: NOMA with QAM-sequence based symbol-level linear spreading can achieve excellent overloading capability with very limited per UE performance loss in all the interested use scenarios.

Proposal 1: Support QAM-sequence based symbol-level linear spreading for NOMA.

# Reference

- 3GPP Chairman’s Notes RAN1#92, Feb. 2018.

- 3GPP Chairman’s Notes RAN1#92bis, April. 2018.

- 3GPP R1-1805840, Key processing modules at transmitter side for NOMA, ZTE

- 3GPP R1-1805841, Multi-user receivers for NOMA, ZTE

# Appendix 1

Table A1 Simulation assumptions used for preliminary evaluations

| Parameters | mMTC | URLLC |  |  | eMBB |  |  | Further specified values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier Frequency | 700 MHz | 4 GHz |  |  | 4 GHz |  |  |  |
| Waveform (data part) | CP-OFDM and DFT-s-OFDM | CP-OFDM |  |  | CP-OFDM |  |  |  |
| Channel coding | NR LDPC |  |  |  |  |  |  | MaxLogMAP decoder, 25 decoding iterations. |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 | SCS = 60 kHz, #OS = 7 |  |  | SCS = 15 kHz #OS = 14 |  |  |  |
| Allocated bandwidth | 6 | 12 |  |  | 12 |  |  |  |
| TBS per UE | [10, 20, 40, 60, 75] bytes | [10, 20, 40, 60, 75] bytes. |  |  | [20, 40, 80, 120, 150] bytes. |  |  | DMRS overhead is 1/7 |
| Target BLER for one transmission | 10% | 0.1% |  |  | 10% |  |  |  |
| Number of UEs multiplexed in the same allocated bandwidth | Indicated in the figures |  |  |  |  |  |  |  |
| BS antenna configuration | 2 Rx |  |  | 4Rx |  |  | 4Rx |  |
| UE antenna configuration | 1Tx |  |  |  |  |  |  |  |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h |  |  |  |  |  |  |  |
| Max number of HARQ transmission | 1 |  |  |  |  |  |  |  |
| Channel estimation | Both ideal and realistic |  |  |  |  |  |  |  |
| MA signature allocation (for data and DMRS) | Fixed and Random with collision |  | Fixed |  |  | Fixed |  |  |
| Distribution of avg. SNR | Both equal and unequal |  | Equal |  |  | Both equal and unequal |  | Random uniform discrete values for unequal case with range [x - 3, x + 3] (dB) with 1 dB step, where x is the average SNR among UEs |
| Timing offset | 0 |  |  |  |  |  |  |  |
| Frequency error | 0 |  |  |  |  |  |  |  |
| Traffic model for link level | Full buffer |  |  |  |  |  |  |  |
| For link level calibration purpose only | OMA single user whose spectral efficiency is the same as per UE SE in NOMA |  |  |  |  |  |  |  |

Table A2 Sequence pool (before normalization) used for the symbol-level linear spreading scheme.

| Linear spreading codebook for SF=2 (the table is repeated for the simulation with more than 6 UEs) |  |
| --- | --- |
| Linear spreading codebook for SF=3 (the table is repeated for the simulation with more than 9 UEs) |  |
| Linear spreading codebook for SF=4 | See Table A3 |
| Linear spreading codebook for SF=6 | See Table A4 |

Table A3 Example of QPSK sequence with SF = 4, pool size = 64 (before normalization).

| No. | c1 | c2 | c3 | c4 | No. | c1 | c2 | c3 | c4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 33 | 1 | 1 | 1 | -j |
| 2 | 1 | 1 | -1 | -1 | 34 | 1 | 1 | -1 | j |
| 3 | 1 | -1 | 1 | -1 | 35 | 1 | -1 | 1 | j |
| 4 | 1 | -1 | -1 | 1 | 36 | 1 | -1 | -1 | -j |
| 5 | 1 | 1 | -j | j | 37 | 1 | 1 | -j | 1 |
| 6 | 1 | 1 | j | -j | 38 | 1 | 1 | j | -1 |
| 7 | 1 | -1 | -j | -j | 39 | 1 | -1 | -j | -1 |
| 8 | 1 | -1 | j | j | 40 | 1 | -1 | j | 1 |
| 9 | 1 | -j | 1 | j | 41 | 1 | -j | 1 | 1 |
| 10 | 1 | -j | -1 | -j | 42 | 1 | -j | -1 | -1 |
| 11 | 1 | j | 1 | -j | 43 | 1 | j | 1 | -1 |
| 12 | 1 | j | -1 | j | 44 | 1 | j | -1 | 1 |
| 13 | 1 | -j | -j | -1 | 45 | 1 | -j | -j | j |
| 14 | 1 | -j | j | 1 | 46 | 1 | -j | j | -j |
| 15 | 1 | j | -j | 1 | 47 | 1 | j | -j | -j |
| 16 | 1 | j | j | -1 | 48 | 1 | j | j | j |
| 17 | 1 | 1 | 1 | -1 | 49 | 1 | 1 | 1 | j |
| 18 | 1 | 1 | -1 | 1 | 50 | 1 | 1 | -1 | -j |
| 19 | 1 | -1 | 1 | 1 | 51 | 1 | -1 | 1 | -j |
| 20 | 1 | -1 | -1 | -1 | 52 | 1 | -1 | -1 | j |
| 21 | 1 | 1 | -j | -j | 53 | 1 | 1 | -j | -1 |
| 22 | 1 | 1 | j | j | 54 | 1 | 1 | j | 1 |
| 23 | 1 | -1 | -j | j | 55 | 1 | -1 | -j | 1 |
| 24 | 1 | -1 | j | -j | 56 | 1 | -1 | j | -1 |
| 25 | 1 | -j | 1 | -j | 57 | 1 | -j | 1 | -1 |
| 26 | 1 | -j | -1 | j | 58 | 1 | -j | -1 | 1 |
| 27 | 1 | j | 1 | j | 59 | 1 | j | 1 | 1 |
| 28 | 1 | j | -1 | -j | 60 | 1 | j | -1 | -1 |
| 29 | 1 | -j | -j | 1 | 61 | 1 | -j | -j | -j |
| 30 | 1 | -j | j | -1 | 62 | 1 | -j | j | j |
| 31 | 1 | j | -j | -1 | 63 | 1 | j | -j | j |
| 32 | 1 | j | j | 1 | 64 | 1 | j | j | -j |

Table A4 Example of BPSK or {+1/-1} sequence with SF = 6, pool size = 16 (before normalization).

| No. | c1 | c2 | c3 | c4 | c5 | c6 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 1 | 1 | 1 | 1 | -1 | -1 |
| 3 | 1 | 1 | 1 | -1 | 1 | -1 |
| 4 | 1 | 1 | 1 | -1 | -1 | 1 |
| 5 | 1 | 1 | -1 | 1 | 1 | -1 |
| 6 | 1 | 1 | -1 | 1 | -1 | 1 |
| 7 | 1 | 1 | -1 | -1 | 1 | 1 |
| 8 | 1 | 1 | -1 | -1 | -1 | -1 |
| 9 | 1 | -1 | 1 | 1 | 1 | -1 |
| 10 | 1 | -1 | 1 | 1 | -1 | 1 |
| 11 | 1 | -1 | 1 | -1 | 1 | 1 |
| 12 | 1 | -1 | 1 | -1 | -1 | -1 |
| 13 | 1 | -1 | -1 | 1 | 1 | 1 |
| 14 | 1 | -1 | -1 | 1 | -1 | -1 |
| 15 | 1 | -1 | -1 | -1 | 1 | -1 |
| 16 | 1 | -1 | -1 | -1 | -1 | 1 |

# Appendix 2 Evaluation results

- mMTC scenario, fixed MA signature

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-1. BLER Performance for mMTC (Equal SNR, TDL-A, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-2. BLER Performance for mMTC (Equal SNR, TDL-C, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-3. BLER Performance for mMTC (Equal SNR, TDL-A, CP-OFDM, Realistic CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-4. BLER Performance for mMTC (Equal SNR, TDL-C, CP-OFDM, Realistic CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-5. BLER Performance for mMTC (Unequal SNR, TDL-A, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-6. BLER Performance for mMTC (Unequal SNR, TDL-C, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-7. BLER Performance for mMTC (Unequal SNR, TDL-A, CP-OFDM, Realistic CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-8. BLER Performance for mMTC (Unequal SNR, TDL-C, CP-OFDM, Realistic CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-9. BLER Performance for mMTC (Equal SNR, TDL-A, DFT-s-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-10. BLER Performance for mMTC (Equal SNR, TDL-C, DFT-s-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-11. BLER Performance for mMTC (Unequal SNR, TDL-A, DFT-s-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.a-12. BLER Performance for mMTC (Unequal SNR, TDL-C, DFT-s-OFDM, Ideal CE)

- mMTC scenario, random MA signature

| (a). TBS=10 | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |

Figure A2.b-1. BLER Performance for mMTC (Equal SNR, TDL-A, CP-OFDM, Realistic CE)

| (a). TBS=10 | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |

Figure A2.b-2. BLER Performance for mMTC (Equal SNR, TDL-C, CP-OFDM, Realistic CE)

- URLLC scenario

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.c-1. BLER Performance for URLLC (Equal SNR, TDL-A, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=10 | (c). TBS=20 |
| --- | --- | --- |
| (d). TBS=40 | (e). TBS=60 | (f). TBS=75 |

Figure A2.c-2. BLER Performance for URLLC (Equal SNR, TDL-A, CP-OFDM, Realistic CE)

- eMBB scenario, fixed MA signature

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.d-1. BLER Performance for eMBB (Equal SNR, TDL-A, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.d-2. BLER Performance for eMBB (Equal SNR, TDL-C, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.d-3. BLER Performance for eMBB (Equal SNR, TDL-A, CP-OFDM, Realistic CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.d-4. BLER Performance for eMBB (Equal SNR, TDL-C, CP-OFDM, Realistic CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.d-5. BLER Performance for eMBB (Unequal SNR, TDL-A, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.d-6. BLER Performance for eMBB (Unequal SNR, TDL-C, CP-OFDM, Ideal CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.d-7. BLER Performance for eMBB (Unequal SNR, TDL-A, CP-OFDM, Realistic CE)

| (a). OMA reference | (b). TBS=20 | (c). TBS=40 |
| --- | --- | --- |
| (d). TBS=80 | (e). TBS=120 | (f). TBS=150 |

Figure A2.d-8. BLER Performance for eMBB (Unequal SNR, TDL-C, CP-OFDM, Realistic CE)