3GPP TSG RAN WG1 Meeting #93	R1-1805931

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.8.2

Source:	Huawei, HiSilicon

Title:	Considerations and evaluation results for IMT-2020 for peak spectral efficiency and peak data rate

Document for:	Discussion and decision

# Introduction

In this contribution, the consideration on self evaluation for peak spectral efficiency and peak data rate is provided.

In previous contribution [1], the calculation method on peak spectral efficiency and peak data rate were provided. In RAN1#91 meeting, the data rate calculation method was adopted in NR UE category discussion, as shown in [2]. In this contribution, the peak spectral efficiency and peak data rate evaluation for NR are considered. In addition, the two frequency ranges, i.e., frequency range 1 (FR1) for sub 6 GHz, and frequency range 2 (FR2) for above 24 GHz, are taken into account.

# Peak spectral efficiency

According to Report ITU-R M.2410 (see [3]), the peak spectral efficiency is defined as in subsection 3.1. Based on the definition, the DL and UL peak spectral efficiency of NR is analysed.

## Calculation of peak spectral efficiency

From the definition and [2] we can derive the generic formula for peak spectral efficiency for FDD and TDD for a specific component carrier (say j-th CC) as below.

(1)

wherein

- Rmax = 948/1024

- For the j-th CC,

  - is the maximum number of layers

  - is the maximum modulation order

  - is the scaling factor

      - The scaling factor can at least take the values 1 and 0.75.

      - is signalled per band and per band per band combination as per UE capability signalling

  - is the numerology (as defined in TS38.211)

  - is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed.

  - is the maximum RB allocation in bandwidth  with numerology , as given in TR 38.817-01 section 4.5.1, where  is the UE supported maximum bandwidth in the given band or band combination.

  - is the overhead  calculated as the average ratio of the number of REs occupied by L1/L2 control, Synchronization Signal, PBCH and reference signals etc. with respect to the total number of REs in effective bandwidth  time product.

j is the normalized scalar considering the downlink/uplink ratio; for FDD j=1 for DL and UL; and for TDD and other duplexing j for DL and UL is calculated based on the frame structure.

## DL peak spectral efficiency

Based on the formula provided in Eq. (1), the DL peak spectral efficiency for NR is given in this sub-section.

### FDD DL peak spectral efficiency

The DL peak spectral efficiency for NR FDD for different bandwidth and sub-carrier spacing (SCS) parameters is given in Table 1. The results are according to Eq. (1) and the detailed parameters as listed in Appendix. It is noted that if the channel bandwidth is less than the bandwidth of SSB, then SSB is not transmitted and the overhead of SSB is zero. If the channel bandwidth is less than TRS bandwidth given in Table A-1, the TRS bandwidth is assumed to be equal to the channel bandwidth. The above assumptions are also used for TDD peak spectral efficiency.

Table 1 NR FDD DL peak spectral efficiency (bit/s/Hz)

| SCS [kHz] |  | 5 MHz | 10 MHz | 15 MHz | 30 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 15 | 41.8 | 45.0 | 46.1 | [47.2] | 47.0 | 47.1 | 48.4 | 48.4 | - | - | - | - | - | 30 |
|  | 30 | 36.4 | 40.6 | 43.9 | [46.1] | 44.7 | 46.1 | 47.0 | 47.1 | 48.4 | [48.4] | 48.6 | [48.8] | 48.9 | 30 |
|  | 60 | - | 36.4 | 41.1 | [44.4] | 41.1 | 43.0 | 44.7 | 46.1 | 46.7 | [47.1] | 47.4 | [48.2] | 48.4 | 30 |

NOTE: The square brackets are due to the tentative value of maximum number of RBs for the related bandwidths.

### TDD DL peak spectral efficiency

The DL peak spectral efficiency for NR TDD for different bandwidth and SCS parameters is shown in Table 2. The results are according to Eq. (1) and the detailed parameters as listed in Appendix. In this evaluation, the DL dominant frame structure “DDDSU” is selected.

Table 2 NR TDD DL peak spectral efficiency (bit/s/Hz) with DL dominant frame structure (“DDDSU”)

| SCS [kHz] |  | 5 MHz | 10 MHz | 15 MHz | 30 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 15 | 40.3 | 44.0 | 45.1 | [46.2] | 45.4 | 46.1 | 46.8 | 46.8 | - | - | - | - | - | - | - | 30 |
|  | 30 | 35.0 | 39.7 | 42.9 | [44.5] | 43.7 | 44.5 | 45.9 | 46.1 | 46.8 | [46.8] | 47.5 | [47.7] | 47.8 | - | - | 30 |
|  | 60 | - | 35.5 | 39.7 | [42.9] | 40.1 | 42.0 | 43.7 | 44.5 | 45.6 | [46.0] | 46.3 | [46.6] | 46.8 | - | - | 30 |
| FR2 | 60 | - | - | - | - | - | - | - | 39.9 | - | - | - | - | 41.0 | 41.5 | - | 30 |
|  | 120 | - | - | - | - | - | - | - | 37.2 | - | - | - | - | 40.5 | 41.5 | 41.5 | 30 |

NOTE: The square brackets are due to the tentative value of maximum number of RBs for the related bandwidths.

## UL peak spectral efficiency

Similarly, based on the formula provided in Eq. (1), the UL peak spectral efficiency for NR is given in this sub-section.

### FDD UL peak spectral efficiency

The UL peak spectral efficiency for NR FDD for different bandwidth and SCS parameters is shown in Table 3. The results are according to Eq. (1) and the detailed parameters as listed in Appendix.

Table 3 NR FDD UL peak spectral efficiency (bit/s/Hz)

| SCS [kHz] |  | 5 MHz | 10 MHz | 15 MHz | 30 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 15 | 22.9 | 23.8 | 24.4 | [24.7] | 24.5 | 24.6 | 25.0 | 25.0 | - | - | - | - | - | 15 |
|  | 30 | 20.1 | 22.0 | 23.5 | [24.1] | 23.6 | 24.1 | 24.5 | 24.6 | 25.0 | [25.0] | 25.1 | [25.2] | 25.3 | 15 |
|  | 60 | - | 20.1 | 22.2 | [23.5] | 22.2 | 23.0 | 23.6 | 24.1 | 24.4 | [24.6] | 24.8 | [24.9] | 25.0 | 15 |

NOTE: The square brackets are due to the tentative value of maximum number of RBs for the related bandwidths.

### TDD UL peak spectral efficiency

The TDD UL peak spectral efficiency for NR TDD for different bandwidth and SCS parameters is shown in Table 4 and Table 5, respectively. In this evaluation, the DL dominant frame structure “DDDSU” and the UL dominant  frame structure (DL:UL=1:4) are selected.

Table 4 TDD UL peak spectral efficiency with DL dominant frame structure (“DDDSU”)

| SCS [kHz] |  | 5 MHz | 10 MHz | 15 MHz | 30 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 15 | 20.2 | 21.0 | 21.2 | [21.5] | 21.4 | 21.5 | 21.5 | 21.8 | - | - | - | - | - | - | - | 15 |
|  | 30 | 17.7 | 19.4 | 20.4 | [30.0] | 20.6 | 21.0 | 21.4 | 21.5 | 21.8 | [21.8] | 21.9 | [22.0] | 22.0 | - | - | 15 |
|  | 60 | - | 17.7 | 19.4 | [20.4] | 19.4 | 20.0 | 20.6 | 21.0 | 21.2 | [21.4] | 21.6 | [21.7] | 21.8 | - | - | 15 |
| FR2 | 60 | - | - | - | - | - | - | - | 20.2 | - | - | - | - | 20.5 | 20.5 | - | 15 |
|  | 120 | - | - | - | - | - | - | - | 19.6 | - | - | - | - | 20.5 | 20.5 | 20.5 | 15 |

NOTE: The square brackets are due to the tentative value of maximum number of RBs for the related bandwidths.

Table 5 NR TDD UL peak spectral efficiency  with UL dominant frame structure (DL:UL=1:4)

| SCS [kHz] |  | 5 MHz | 10 MHz | 15 MHz | 30 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Req. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 15 | 22.9 | 23.8 | 24.1 | [24.7] | 24.5 | 24.6 | 25.0 | 25.0 | - | - | - | - | - | - | - | 15 |
|  | 30 | 20.1 | 22.0 | 23.2 | [24.1] | 23.6 | 24.1 | 24.5 | 24.6 | 25.0 | [25.0] | 25.1 | [25.2] | 25.3 | - | - | 15 |
|  | 60 | - | 20.1 | 22.0 | [23.5] | 22.2 | 23.0 | 23.6 | 24.1 | 24.4 | [24.6] | 24.8 | [24.9] | 25.0 | - | - | 15 |
| FR2 | 60 | - | - | - | - | - | - | - | 23.4 | - | - | - | - | 23.4 | 23.4 | - | 15 |
|  | 120 | - | - | - | - | - | - | - | 22.7 | - | - | - | - | 23.4 | 23.4 | 23.4 | 15 |

NOTE: The square brackets are due to the tentative value of maximum number of RBs for the related bandwidths.Based on the above evaluation results, it is observed that,

Observation 1: NR can fulfill DL and UL peak spectral efficiency requirements for all SCS and bandwidth, for both below and above 6GHz.

# Peak data rate

According to Report ITU-R M.2410, the peak data rate is defined as in subsection 4.1. Based on the definition, the DL and UL peak data rate of NR is analyzed.

## Calculation of peak data rate

The DL/UL peak data rate for FDD and TDD can be calculated as below

(3)

where the scaling factor (j) is the value for DL or UL on band j, respectively.

It is noted that in the above equation, Wj is the effective bandwidth that takes into account the uplink/downlink ratio as defined in Report ITU-R M.2410.

## DL peak data rate

The DL peak data rate for NR is shown in Table 6.

Table 6 NR DL peak data rate

| Duplexing | SCS [kHz] |  | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Number of CC | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | FR1 | 15 | 50 | 2.42 | 10 | 500 | 24.19 | 20 |
|  |  | 30 | 100 | 4.89 | 5 | 500 | 24.46 |  |
|  |  | 60 | 100 | 4.84 | 5 | 500 | 24.19 |  |
| TDD (DL:UL=4:1) | FR1 | 15 | 50 | 1.89 | 11 | 550 | 20.81 |  |
|  |  | 30 | 100 | 3.83 | 6 | 600 | 22.96 |  |
|  |  | 60 | 100 | 3.78 | 6 | 600 | 22.70 |  |
|  | FR2 | 60 | 200 | 6.73 | 3 | 600 | 20.18 |  |
|  |  | 120 | 400 | 13.46 | 2 | 800 | 26.91 |  |

## UL peak data rate

The UL peak data rate for NR is shown in Table 7.

Table 7 NR UL peak data rate

| Duplexing | SCS [kHz] |  | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Number of CC | Total bandwidth (MHz) | Aggregated peak data rate (Gbit/s) | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | FR1 | 15 | 50 | 1.25 | 10 | 500 | 12.50 | 10 |
|  |  | 30 | 100 | 2.53 | 5 | 500 | 12.63 |  |
|  |  | 60 | 100 | 2.50 | 5 | 500 | 12.50 |  |
| TDD (DL:UL=1:4) | FR1 | 15 | 50 | 1.0 | 10 | 500 | 10.00 |  |
|  |  | 30 | 100 | 2.02 | 5 | 500 | 10.11 |  |
|  |  | 60 | 100 | 2.0 | 5 | 500 | 10.00 |  |
|  | FR2 | 60 | 200 | 3.74 | 3 | 600 | 11.22 |  |
|  |  | 120 | 400 | 7.49 | 2 | 800 | 14.97 |  |

Based on the above evaluation results, it is observed that,

Observation 2: NR can fulfill DL and UL peak data rate requirements for both below and above 6GHz with carrier aggregation.

Observation 3: The frequency range of below 6 GHz can achieve similar peak data rate performance to above 6GHz by using less bandwidth.

# Conclusions

In this document, the peak spectral efficiency and peak data rate of NR for the two frequency ranges are evaluated. The following observations and proposal are made.

Observation 1: NR can fulfill DL and UL peak spectral efficiency requirements for all SCS and bandwidth, for both below and above 6GHz.

Observation 2: NR can fulfill DL and UL peak data rate requirements for both below and above 6GHz with carrier aggregation.

Observation 3: The frequency range of below 6 GHz can achieve similar peak data rate performance to above 6GHz by using less bandwidth.

Proposal: Capture the evaluation results in Section 2 and 3 into TR37.910.

# References

R1-1719419, “Consideration on self evaluation of peak spectral efficiency and peak data rate for IMT-2020”, Huawei, HiSilicon, November 2017.

R1-1721732, “Reply to LS on NR UE Category”, RAN1, November 2017.

Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

# Appendix       Detailed evaluation assumptions

Table A-1 Parameter assumptions of NR FDD DL peak spectral efficiency

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
|  | 8 | NR supports up to 8 layers for a single user for DL |
|  | 8 | NR supports up to 256QAM for DL |
|  | 1 | The value of 1 is chosen as scaling factor for DL peak spectral efficiency evaluation. |
| Rmax | 948/1024 = 0.9258 | NR supports highest coding rate as Rmax=948/1024. |
|  | 0, 1, 2, 3 | SCS, i.e., - 15kHz SCS for =0 - 30kHz SCS for  =1 - 60kHz SCS for  =2 - 120kHz SCS for  =3 |
|  | For FR1: - 270 for 50MHz with 15kHz SCS - 273 for 100MHz with 30kHz SCS - 135 for 100MHz with 60kHz SCS | See Section 5.3.2 of TS38.104 v0.5.0 |
|  |  | SCS, i.e., - 15kHz SCS for =0 - 30kHz SCS for  =1 - 60kHz SCS for  =2 - 120kHz SCS for  =3 |
|  | For FR1: - 0.10 for 50MHz with 15kHz SCS - 0.10 for 100MHz with 30kHz SCS - 0.10 for 100MHz with 60kHz SCS | For FR1: - 1 single symbol CORESET of 12 RB (2 CCE) in every slot - 12 RE/RB/slot - TRS burst of 2 slots with periodicity of 20ms and occupies 24RB - 6 RE/RB/slot - DMRS: 16 RE/RB/slot in every slot and RB - CSI-RS: 8 CSI-RS ports with 8 RE/RB/slot with periodicity of 20ms in every RB - 2 SS/PBCH blocks per 20ms; one SS/PBCH block occupies 960REs = 4 OFDM symbols × 20 RB × 12 REs/RB NOTE1: if the channel bandwidth is less than the bandwidth of SS/PBCH block, then SS/PBCH block is not transmitted and the overhead of SS/PBCH block is zero. NOTE2: If the channel bandwidth is less than TRS bandwidth, the TRS bandwidth is assumed to be equal to the channel bandwidth. |
|  | See Section 2.2. | See Section 5.3.2 of TS38.104 |

Table A-2 Parameter assumptions of NR TDD DL peak spectral efficiency 
for DL dominant frame structure (DDDSU)

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
|  | 0.7714 | This value corresponds to DL:UL=4:1, where 3 DL slots, 1 S slot mixing DL/UL symbols, and 1 UL slot are configured in every 5 slots; S slot includes 11 DL symbols , one symbol for GP, and two UL symbols. |
|  | For FR1: - 0.12 for 50MHz with 15kHz SCS - 0.12 for 100MHz with 30kHz SCS - 0.12 for 100MHz with 60kHz SCS For FR2: - 0.20 for 200MHz with 60kHz SCS - 0.20 for 400MHz with 120kHz SCS | GP overhead is included in DL slot. For FR1: - 1 single symbol CORESET of 12 RB (2 CCE) in every slot - 12 RE/RB/slot - TRS burst of 2 slots with periodicity of 20ms and occupies 24RB - 6 RE/RB/slot - DMRS: 16 RE/RB/slot in every slot and RB - CSI-RS: 8 CSI-RS ports with 8 RE/RB/slot with periodicity of 20ms in every RB - 2 SS/PBCH blocks per 20ms; one SS/PBCH block occupies 960REs = 4 OFDM symbols × 20 RB × 12 REs/RB NOTE1: if the channel bandwidth is less than the bandwidth of SS/PBCH block, then SS/PBCH block is not transmitted and the overhead of SS/PBCH block is zero. NOTE2: If the channel bandwidth is less than TRS bandwidth, the TRS bandwidth is assumed to be equal to the channel bandwidth. For FR2: - 1 single symbol CORESET of 24 RB (4 CCE) in every slot - 12 RE/RB/slot - TRS burst of 2 slots with periodicity of 10ms and occupies 24RB - 6 RE/RB/slot - DMRS: 24 RE/RB/slot in every slot and RB - PTRS: 5 RE/RB/slot (2 port PT-RS, with 25% frequency domain density) in every slot and RB - CSI-RS: 8 CSI-RS ports with 8 RE/RB/slot with periodicity of 10ms in every RB - CSI-RS for BM: 1 CSI-RS port with 2 RE/RB/slot with periodicity of 10ms in every RB - 8 SS/PBCH blocks per 20ms; one SS/PBCH block occupies 960REs = 4 OFDM symbols × 20 RB × 12 REs/RB |
| NOTE: Other parameters are the same as NR FDD DL (Table A-1). |  |  |

Table A-3 Parameter assumptions of NR FDD UL peak spectral efficiency

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
|  | 4 | NR supports up to 4 layers for a single user for UL |
|  | 8 | NR supports up to 256QAM for UL |
|  | 1 | The value of 1 is chosen as scaling factor for UL peak spectral efficiency evaluation. |
| Rmax | 948/1024 = 0.9258 | NR supports highest coding rate as Rmax=948/1024. |
|  | 0, 1, 2, 3 | SCS, i.e., - 15kHz SCS for =0 - 30kHz SCS for  =1 - 60kHz SCS for  =2 - 120kHz SCS for  =3 |
|  | For FR1: - 270 for 50MHz with 15kHz SCS - 273 for 100MHz with 30kHz SCS - 135 for 100MHz with 60kHz SCS For FR2: - 264 for 200MHz with 60kHz SCS - 264 for 400MHz with 120kHz SCS | See Section 5.3.2 of TS38.104 |
|  |  | SCS, i.e., - 15kHz SCS for =0 - 30kHz SCS for  =1 - 60kHz SCS for  =2 - 120kHz SCS for  =3 |
|  | For FR1: - 0.07 for 50MHz with 15kHz SCS - 0.07 for 100MHz with 30kHz SCS - 0.07 for 100MHz with 60kHz SCS | For FR1: - DM-RS: 1 symbol per slot - SRS: 2 symbols per slot with periodicity of 5ms and SRS BW of 8 PRB |
|  | See Section 2.3. | See Section 5.3.2 of TS38.104 |

Table A-4 Parameter assumptions of NR TDD UL peak spectral efficiency 
for UL dominant DL/UL configuration

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
|  | 0.8 | This value corresponds to DL:UL=1:4, where 1 DL slot and 4 UL slots are configured in every 5 slots; one symbol for GP is assumed in DL slot when the slot switches from DL to UL. |
|  | For FR1: - 0.07 for 50MHz with 15kHz SCS - 0.07 for 100MHz with 30kHz SCS - 0.07 for 100MHz with 60kHz SCS For FR2: - 0.11for 200MHz with 60kHz SCS - 0.11 for 400MHz with 120kHz SCS | GP overhead is included in DL. For FR1: - DM-RS: 1 symbol per slot - SRS: 2 symbols per slot with periodicity of 5ms and SRS BW of 8 PRB For FR2: - DM-RS: 1 symbol per slot - SRS: 4 symbols per slot with periodicity of 5ms and SRS BW of 16 PRB - PTRS: 6 RE/RB/slot (2 port PT-RS, with 25% frequency domain density) in every slot and RB |
| NOTE: Other parameters are the same as NR FDD UL (Table A-3). |  |  |

Table A-5 Parameter assumptions of NR TDD UL peak spectral efficiency 
for DL dominant DL/UL configuration (“DDDSU”)

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
|  | 0.2286 | This value corresponds to DL:UL=4:1, where 3 DL slots, 1 S slot mixing DL/UL symbols, and 1 UL slot are configured in every 5 slots; S slot includes 11 DL symbols , one symbol for GP, and two UL symbols. |
|  | For FR1: - 0.08 for 50MHz with 15kHz SCS - 0.07 for 100MHz with 30kHz SCS - 0.07 for 100MHz with 60kHz SCS For FR2: - 0.11 for 200MHz with 60kHz SCS - 0.11 for 400MHz with 120kHz SCS | GP overhead is included in DL. For FR1: - DM-RS: 1 symbol per slot - SRS: 2 symbols per slot with periodicity of 5ms and SRS BW of 8 PRB For FR2: - DM-RS: 1 symbol per slot - SRS: 4 symbols per slot with periodicity of 5ms and SRS BW of 16 PRB - PTRS: 6 RE/RB/slot (2 port PT-RS, with 25% frequency domain density) in every slot and RB |
| NOTE: Other parameters are the same as NR FDD UL (Table A-3). |  |  |