3GPP TSG RAN WG1 Meeting #93                          R1-1806134

Busan, Korean, May 21st – 25th, 2018

Source:					ZTE

Title:						Remaining issues for data resource allocation

Agenda item:		7.1.3.3.1

Document for:		Discussion and Decision

# Introduction

In this contribution, the remaining issues on the data resource allocation have been discussed.

# Time-domain resource allocation

In this section, detailed discussion on the mapping between SLIV value and actual configuration of resource allocation as well as the remaining issues about slot-aggregation have been conducted based following agreements [1][2]:

Agreements:

For PDSCH:

- Supported combinations for PDSCH mapping type A:

  - Starting symbol can be symbol index #0, 1, 2, 3 in a slot.

  - Length of the PDSCH is at least X symbols, up to 14 symbols within a slot, such that slot boundary is not crossed

    - FFS the value of X

- Supported combinations for PDSCH mapping type B:

  - Length of the PDSCH can be 2, 4, or 7 symbols.

  - Starting symbol can be any position within a slot, such that slot boundary is not crossed.

For PUSCH

- PUSCH mapping type A:

  - Starting symbol is symbol index #0 in a slot.

  - Length of the PUSCH is at least Y symbols, up to 14 symbols

    - FFS the value of Y

- PUSCH mapping type B (All 105 combinations)

  - Length of the PUSCH can be 2 through 14 symbols, and with 1 symbol as a working assumption

Starting symbol can be any position within a slot, such that slot boundary is not crossed.

Agreements:

- Regarding “Length of the PDSCH is at least X symbols” (PDSCH mapping type A), X=3

- Regarding “Length of the PUSCH is at least Y symbols” (PUSCH mapping type A), Y=4

## The indication of SLIV values

In the existing spec, the Time domain resource assignment field of the DCI provides a row index of a higher layer configured table for UL/DLwith up to 16 rows. Moreover, the following content is included in each row configured by RRC:

For DL table:

- K0 using 2 bits (for DL table)

- an index (6-bit) into a table/equation in RAN1 specs capturing valid combinations of start symbol and length (jointly encoded)

- PDSCH mapping type A or B.(1-bit)

For UL table:

- K2 using 3 bits (for UL table)

- an index (7-bit) into a table/equation in RAN1 specs capturing valid combinations of start symbol and length (jointly encoded)

- PUSCH mapping type A or B. (1-bit)

For UL table, 7 bits are used to indicate the combinations of each start symbol and its length. In total, there are 105 different SLIV values in UL, i.e. integers from 1 to 105. 7 bits can represent at most 128 different integers, which are enough to express all the SLIV values for both NCP and ECP.Thus, for UL indication, the 7-bit index can be directly interpreted as SLIV value.

Proposal 1: For UL time-domain resource allocation, the 7-bit index can be directly interpreted as SLIV value.

Based on the agreements above, the applicable combinations of start symbol and length of normal CP and extended CP are showed in Table 2.1-1.

Table 2.1-1: Valid S and L combinations

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3} | {3,…,14} | {3,…,14} | {0,1,2,3} | {3,…,12} | {3,…,12} |
| Type B | {0,....,12} | {2,4,7} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |

However, for DL table, only 6 bits are used to indicate the combinations of each start symbol and its length. 6 bits are not enough if they are directly interpreted as SLIV values. For example, the SLIV value equals 87 where S=3 and L=10, it is obvious that 6-bit index can not be directly interpreted as 87.

Therefor, the mapping tables of 6-bit index to all possible SLIV values are needed, which can represent all possible combinations of start symbol and length for NCP and ECP.

Due to the different combinations of PDSCH mapping type A/B and NCP/ECP, 4 predefined mapping tables are proposed, i.e., PDSCH mapping type A with NCP (Table 2.1-2), PDSCH mapping type A with ECP (Table 2.1-3), PDSCH mapping type B with NCP (Table 2.1-4) and PDSCH mapping type B with ECP (Table 2.1-5).

Table 2.1-2 Mapping table for PDSCH mapping type A with NCP

| # Index | # SLIV | # Start | # Length |
| --- | --- | --- | --- |
| # 0 | # 27 | # 0 | # 14 |
| # 1~4 | # 28,29,30,31 | # 0,1,2,3 | # 3 |
| # 5~6 | # 40,41 | # 0,1 | # 13 |
| # 7~10 | # 42,43,44,45 | # 0,1,2,3 | # 4 |
| # 11~13 | # 53,54,55 | # 0,1,2 | # 12 |
| # 14~17 | # 56,57,58,59 | # 0,1,2,3 | # 5 |
| # 18~21 | # 66,67,68,69 | # 0,1,2,3 | # 11 |
| # 22~25 | # 70,71,72,73 | # 0,1,2,3 | # 6 |
| # 26~29 | # 79,80,81,82 | # 0,1,2,3 | # 10 |
| # 30~33 | # 84,85,86,87 | # 0,1,2,3 | # 7 |
| # 34~37 | # 92,93,94,95 | # 0,1,2,3 | # 9 |
| # 38~41 | # 98,99,100,101 | # 0,1,2,3 | # 8 |

Table 2.1-3 Mapping table for PDSCH mapping type B with NCP

| # Index | # SLIV | # Start | # Length |
| --- | --- | --- | --- |
| # 0~12 | # 14~26 | # 0~12 | # 2 |
| # 13~23 | # 42~52 | # 0~10 | # 4 |
| # 24~31 | # 84~91 | # 0~7 | # 7 |

Table 2.1-4 Mapping table for PDSCH mapping type A with ECP

| # Index | # SLIV | # Start | # Length |
| --- | --- | --- | --- |
| # 0 | # 23 | # 0 | # 12 |
| # 1~4 | # 24~27 | # 0~3 | # 3 |
| # 5~6 | # 34~35 | # 0,1 | # 11 |
| # 7~10 | # 36~39 | # 0~3 | # 4 |
| # 11~13 | # 45~47 | # 0,1,2 | # 10 |
| # 14~17 | # 48~51 | # 0~3 | # 5 |
| # 18~21 | # 56~59 | # 0~3 | # 9 |
| # 22~25 | # 60~63 | # 0~3 | # 6 |
| # 26~29 | # 68~71 | # 0~3 | # 8 |
| # 30~33 | # 72~75 | # 0~3 | # 7 |

Table 2.1-5 Mapping table for PDSCH mapping type B with ECP

| # Index | # SLIV | # Start | # Length |
| --- | --- | --- | --- |
| # 0~10 | # 12~22 | # 0~10 | # 2 |
| # 11~19 | # 36~44 | # 0~8 | # 4 |
| # 20~25 | # 60~65 | # 0~5 | # 6 |

Proposal 2: For DL time-domain resource allocation, four separate tables 2.1-2 to 2.1-5 for the mapping between 6-bit index and SLIV value should be considered.

## Resource allocation for slot-aggregation

For UL/DL slot-aggregation, the same symbol allocation is used across slots in UL/DL. And the aggregation factor configured by RRC indicate the number of slots used for aggregation. As shown below, in the existing 38.214 [10], the restriction ‘consecutive slots’ is added in the description of aggregation slots for both UL and DL.

5.1.2.1 Resource allocation in time domain When the UE is configured with aggregationFactorDL > 1, the UE may expect that the TB is repeated within each symbol allocation among each of the aggregationFactorDL consecutive slots and the PDSCH is limited to a single transmission layer. #### 6.1.2.1 Resource allocation in time domain When the UE is configured with aggregationFactorUL > 1, the same symbol allocation is applied across the aggregationFactorUL consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the aggregationFactorUL consecutive slots applying the same symbol allocation in each slot.

However, the aforementioned restriction on consecutive slots may not always suitable for the slot-aggregation transmission due to the semi-static DL/UL assignment configuration of slots and some other reasons. Take UL slot aggregation for an example, as shown in the figure 2.2-1 below, the frame structure is DDXU ( 2ms, 30KHz ) with aggregationFactorUL = 4. In this case, not all the four consecutive slots can be used for UL transmission. The DL slot should not be counted as a UL aggregation slot although it is consecutive or there will be only two slots used for the UL aggregation in fact. In other words, UE uses less UL slots for slot aggregation transmission than the aggregation factor configured by RRC, this may cause performance decline and transmission fail. For other frame structure, the same issue will also happen in the DL/UL slot-aggregation case with the number of consecutive DL/UL slots less than the aggregation factor configured by RRC.

Figure 2.2-1 Slot-aggregation based on N consecutive slots

Therefore, two slot chosen criteria should be supported for slot-aggregation to guarantee the performance.

The first criterion is explained as follows. For UL slot-aggregation, only the “U” and “X” slot can be chosen as the transmission slot; for DL slot-aggregation, only the “D” and “X” slot can be chosen as the transmission slot. For the reason that the same symbol allocation is used across aggregationFactorDL/aggregationFactorUL slots, the chosen slot must have enough DL/UL time domain resource to support the time resource allocation scheme in the first aggregation slot. More specifically, the SLIV value of the first aggregation slot corresponding to the combination of start symbol and length should be guaranteed for the following chosen aggregation slots..

As the example mentioned above, there is no UL time domain resource in the two DL slots (“D” slots), i.e., the two DL slots cannot be chosen as UL aggregation slots. As shown in the figure 2.2-2, if following “X” slot and “U” slot satisfy the first criterion, they should be counted as aggregation slot.

Figure 2.2-2 Slot-aggregation based on N consecutive DL/UL slots

Considering the BWP switching within slot aggregation, another criterion should be supported. If UE receives a BWP switching DCI in a DL slot, then the following slots (after the transition time) will be handled in the new BWP pair. As shown in the figure 2.2-3, the bandwidth of the new UL BWP could be different from the original one before BWP switching. When the bandwidth of the new BWP is smaller than the BWP before BWP switching, the UL slots in the new BWP can not be used for the slot aggregation if there is not enough frequency resource to support the transmission. Thus, the second criterion is that the chosen slot must have enough frequency resource to support traffic transmitted in the first aggregation slot.

Figure 2.2-3 Slot-aggregation with BWP switching

Proposal 3: For slot-aggregation, the following two criteria should be support to choose the slots used for slot-aggregation.

- The first criterion: For UL slot-aggregation, only the “U” and “X” slot can be chosen as the transmission slot; for DL slot-aggregation, only the “D” and “X” slot can be chosen as the transmission slot. The SLIV value of the first aggregation slot corresponding to the combination of start symbol and length can be used in the following chosen slots.

- The second criteria: For UL slot-aggregation or DL slot-aggregation, only the slots that with the frequency resource larger than or equal to the frequency resource of the first aggregation slot.

Besides, the transition time of BWP switching should also be considered. Based on the agreement achieved in RAN1#92,

Agreements:

- A UE is not expected to receive DL signals or transmit UL signals during the transition time of active DL or UL BWP switch

  - For DCI-based active BWP switch, from RAN1 perspective, the transition time of active DL or UL BWP switch is the time duration from the end of last OFDM symbol of the PDCCH carrying the active BWP switch DCI till the beginning of a slot indicated by K0 in the active DL BWP switch DCI or K2 in the active UL BWP switch DCI

As shown in figure 2.2-4, the slots that are in the transition time of BWP switching should not be counted in the slot-aggregation.

Figure 2.2-4 Transition time of BWP switch

Proposal 4: The slots in the transition time of BWP switching should not be counted in the slot-aggregation.

Proposal 5: The text in 38.214 should be changed as:

5.1.2.1 Resource allocation in time domain When the UE is configured with aggregationFactorDL > 1, the UE may expect that the TB is repeated within each symbol allocation among each of the aggregationFactorDL consecutive slots with the following restriction and the PDSCH is limited to a single transmission layer. Restriction 1. In time domain, the slot only can be DL slot or X slot, and the slots are not in the transition time of a BWP switching. Restriction 2. In frequency domain, the frequency resource of the slots must larger than or equal to the frequency resource of the first aggregation slot. #### 6.1.2.1 Resource allocation in time domain When the UE is configured with aggregationFactorUL > 1, the same symbol allocation is applied across the aggregationFactorUL consecutive slots with the following restriction and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the aggregationFactorUL consecutive slots applying the same symbol allocation in each slot. Restriction 1. In time domain, the slot only can be UL slot or X slot, and the slot are not in the transition time of a BWP switching. Restriction 2. In frequency domain, the frequency resource of the slot must larger than or equal to the frequency resource of the first aggregation slot.

## Slot aggregation with BWP switching

As the case mentioned above, when the BWP switching occurs among the aggregation slots, the following slots (after the transition time) will be handled in the new BWP. Since any slot among aggregation slots transmits the same TB and each slot configured with the same symbol allocation, the number of allocated RBs in each slot should be the same. Even though the BWP changes, the number of allocated RBs should not change.

For supporting the smooth transition of BWP in slot-aggregation as well as satisfying the two criteria mentioned before, RBs mapping between the two BWPs should be further discussed. The discussion can be divided into two aspects according to whether the allocated frequency resources in the two BWPs are overlapped or not.

- If the frequency-domain allocated resources of two BWPs are totally overlapped, the PRBs allocated in current BWP are directly mapped to the new BWP as shown in figure 2.3-1(a)

- If the frequency-domain allocated resources of two BWPs are not totally overlapped, the PRBs allocated in current BWP are the mapped to the corresponding place of the new BWP with the respective reference location, i.e., the lowest PRB of respective BWP. Take figure 2.3-1(b) for an example, the third PRB in current BWP is mapped to the third PRB in the new BWP.

(a)                               (b)

Figure 2.3-1 RB association between two BWPs

Proposal 6: The RB mapping between two BWPs should be considered for BWP switching in slot-aggregation.

# RMSI PDSCH time domain resource allocation for pattern 1

In RAN1 #92bis meeting, RMSI time domain resource allocation was discussed. The following table for multiplexing pattern 1 was agreed.

Agreements: - Adopt the tables below for RMSI time domain resource allocation - The previously agreed 4-bit indicator in DCI is not changed Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 1 L; DL-DMRS-typeA-pos; PDSCH mapping type; K0; S; L. 0; 2; Type A; 0; 2; 12. 3; Type A; 0; 3; 11. 1; 2; Type A; 0; 2; 10. 3; Type A; 0; 3; 9. 2; 2; Type A; 0; 2; 9. 3; Type A; 0; 3; 8. 3; 2; Type A; 0; 2; 7. 3; Type A; 0; 3; 6. 4; 2; Type A; 0; 2; 5. 3; Type A; 0; 3; 4. 5; 2; Type B; 0; 9; 4. 3; Type B; 0; 10; 4. 6; 2; Type B; 0; 4; 4. 3; Type B; 0; 6; 4. 7; x; Type B; 0; 5; 7. 8; x; Type B; 0; 5; 2. 9; x; Type B; 0; 9; 2. 10; x; Type B; 0; 12; 2. 11; x; Type A; 0; 1; 13. 12; x; Type A; 0; 1; 6. 13; x; Type A; 0; 2; 4. 14; x; Type B; 0; 4; 7. 15; x; Type B; 0; 8; 4.

In this section, we give our further consideration and corresponding text proposal for RMSI time domain resource allocation.

## Analysis

Some analysis below show that the minimum SS/PBCH block periodicity can be supported for the case that there is no collision between RMSI PDCCH monitoring window and SS/PBCH block slots.

Figure 3.1-1 RMSI PDCCH monitoring windows for pattern1 with N=1, M=1/2

Figure 3.1-2 RMSI PDCCH monitoring windows for pattern1 with N=1, M=2

For the above monitoring window configuration, the shortest duration of RMSI transmission can be found in Fig3.1-1, i.e. (L/2+1) slots, where L is the number of SSBs. For other configurations, the duration will be longer, i.e. (L+1) slots for N=1, M=1, and 2L slots for N=1, M=2(as shown in Fig3.1-2). N corresponds to the number of search space sets per slot as shown in tables 13-11 and 13-12 in 38.213.

The following tables give the lowest SSB periodicity in non-overlapping manner, i.e. the minimum SS/PBCH block periodicity can be supported for the case that there is no collision between RMSI PDCCH monitoring window and SS/PBCH block slots.

Table 3.1-1 The minimum SSB periodicity for the case of N=2 and M=1/2

| Frequency range Number of SSB | SCS (SSB, RMSI) kHz | Duration of SSB | The duration of RMSI (L/2+1) slots | The minimum SSB periodicity in non-overlapping manner |
| --- | --- | --- | --- | --- |
| 0~3GHz L=4 | (15, 15) | 2ms | 3ms | 5ms |
|  | (15, 30) | 2ms | 1.5ms | 5ms |
|  | (30, 15) | 1ms | 3ms | 5ms |
|  | (30, 30) | 1ms | 1.5ms | 5ms |
| 3~6GHz L=8 | (15, 15) | 4ms | 5ms | 10ms |
|  | (15, 30) | 4ms | 2.5ms | 10ms |
|  | (30, 15) | 2ms | 5ms | 10ms |
|  | (30, 30) | 2ms | 2.5ms | 5ms |
| Above 6GHz L=64 | (120, 60) | ~5ms | 8.25ms | 20ms |
|  | (120, 120) | ~5ms | 4.125ms | 20ms |
|  | (240, 60) | ~2.5ms | 8.25ms | 20ms |
|  | (120, 120) | ~2.5ms | 4.125ms | 10ms |

Table 3.1-2 The minimum SSB periodicity for the case of N=1 and M=2

| Frequency range Number of SSB | SCS (SSB, RMSI) kHz | Duration of SSB | The duration of RMSI 2L slots | The minimum SSB periodicity in non-overlapping manner |
| --- | --- | --- | --- | --- |
| 0~3GHz L=4 | (15, 15) | 2ms | 8ms | 10ms |
|  | (15, 30) | 2ms | 4ms | 10ms |
|  | (30, 15) | 1ms | 8ms | 10ms |
|  | (30, 30) | 1ms | 4ms | 5ms |
| 3~6GHz L=8 | (15, 15) | 4ms | 16ms | 20ms |
|  | (15, 30) | 4ms | 8ms | 20ms |
|  | (30, 15) | 2ms | 16ms | 20ms |
|  | (30, 30) | 2ms | 8ms | 10ms |
| Above 6GHz L=64 | (120, 60) | ~5ms | 32ms | 40ms |
|  | (120, 120) | ~5ms | 16ms | 40ms |
|  | (240, 60) | ~2.5ms | 32ms | 40ms |
|  | (120, 120) | ~2.5ms | 16ms | 20ms |

According to the above tables, the minimum SSB periodicity of 5ms can not be supported in most SCS combinations. Further more, even the periodicity of 20ms can not be supported in some cases, which means the normal initial access will be influenced.

Observation 1: The Collision between RMSI PDSCH and SS/PBCH block should be considered for multiplexing pattern 1.

According to the following description in 38.214[10], there is no assumption on rate matching around SS/PBCH block for RMSI PDSCH reception.

5.1 UE procedure for receiving the physical downlink shared channel[10] When receiving the PDSCH conveying [SystemInformationBlockType1], a UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.

Observation 2: For RMSI PDSCH, time domain resource allocation without overlapping with SS/PBCH block symbols should be supported.

Compared with multiplexing pattern2 and 3, pattern 1 aims to provide better coverage for the reason that more symbols can be used for carrying RMSI PDSCH.

However, in pattern 1, time domain resource for RMSI PDSCH will be interrupted considering the potential mapping of SSBs within RMSI PDSCH slot. This will result in fewer continuous symbols that can be used for RMSI PDSCH in pattern 1, which may even fewer than that in pattern 2 or pattern 3(i.e. RA within SSB symbols).

Taking Table 3.1-3 as an example, 2 symbols will be occupied by one SSB under the SCS combination of {SSB, RMSI}={30kHz, 15kHz}. Then there will be only 2 symbols for carrying RMSI PDSCH in pattern2, and even fewer in pattern 3. For pattern 1, according to current default table for pattern1, there are also only 2 symbols (i.e. symbol 12, 13) used for carrying RMSI PDSCH. And there will be 4 symbols (i.e. symbol 8, 9, 12, 13) that can be used for PDSCH transmission if discontinuous time domain resource allocation is supported.

Table 3.1-3 Example of discontinuous time domain resource allocation for RMSI PDSCH

| SCS combination of (SSB, RMSI) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (30kHz, 15kHz) |  |  | SSB0 |  | SSB1 |  |  |  | SSB2 |  | SSB3 |  |  |  |
|  | Dc | Dc | Dc | Dc | Dd | Dd | Dd | Dd | Dd | Dd | SSB3 |  | Dd | Dd |

More cases with similar issue can be found in Table 3.1-4.

Table 3.1-4 Cases for PDSCH transmission with discontinuous time domain resource allocation

| SCS combination of (SSB, RMSI) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 8 | 9 | 10 |  | 11 | 12 |  | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (15kHz,30kHz) |  |  |  |  | SSB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dc | Dc | Dd | Dd | SSB |  |  |  |  |  |  |  |  |  | Dd |  | Dd |
| (15kHz, 15kHz) |  |  | SSB0 |  |  |  |  |  |  | SSB1 |  |  |  |  |  |  |  |
|  | Dc | Dc | Dd | Dd | Dd | Dd | Dd | Dd |  | SSB1 |  |  |  |  | Dd |  | Dd |
|  | Dc | Dc | SSB0 |  |  |  | Dd | Dd |  | SSB1 |  |  |  |  | Dd |  | Dd |
| (30kHz, 15kHz) |  |  | SSB0 |  | SSB1 |  |  |  |  | SSB2 |  | SSB3 |  |  |  |  |  |
|  | Dc | Dc | Dc | Dc | Dd | Dd | Dd | Dd |  | Dd | Dd | SSB3 |  |  | Dd |  | Dd |
|  |  |  | SSB0 |  |  | SSB1 |  |  |  | SSB2 |  |  | SSB3 |  |  |  |  |
|  | Dc | Dc | Dc | Dc | Dd | Dd | Dd | Dd |  | Dd | Dd | Dd | SSB3 |  |  |  | Dd |
| (30kHz, 30kHz) |  |  |  |  | SSB0 |  |  |  |  | SSB1 |  |  |  |  |  |  |  |
|  | Dc | Dc | Dd | Dd | SSB0 |  |  |  |  | Dd | Dd | Dd | Dd |  |  | Dd | Dd |
|  | Dc | Dc | Dd | Dd | Dd | Dd | Dd | Dd |  | SSB1 |  |  |  |  |  | Dd | Dd |
| (120kHz, 60kHz) |  |  | SSB0 |  | SSB1 |  |  |  |  | SSB2 |  | SSB3 |  |  |  |  |  |
|  | Dc | Dd | SSB0 |  | Dd | Dd | Dd | Dc |  | Dd | Dd | Dd | Dd |  |  |  |  |
|  | Dc | Dd | Dd | Dd | SSB1 |  | Dd | Dc |  | Dd | Dd | Dd | Dd |  |  |  |  |
|  | Dc | Dc | Dd | Dd | SSB1 |  | Dd | Dc |  | Dc | Dd | Dd | Dd |  | Dd |  | Dd |
| (120kHz, 120kHz) |  |  | SSB0 |  |  |  | SSB1 |  |  |  |  |  |  |  |  |  |  |
|  | Dc | Dd | SSB0 |  |  |  | Dd | Dc |  | Dd | Dd | Dd | Dd |  | Dd |  | Dd |
| (240kHz, 120kHz) |  |  |  |  | SSB0 |  | SSB1 |  |  | SSB2 |  | SSB3 |  |  |  |  |  |
|  |  |  | SSB4 |  | SSB5 |  | SSB6 |  |  | SSB7 |  |  |  |  |  |  |  |
|  | Dc | Dd | SSB4 |  | Dd | Dd | Dd | Dc |  | Dd | Dd | Dd | Dd |  |  |  |  |
|  | Dc | Dc | SSB4 |  | Dd | Dd | Dd | Dc |  | Dc | Dd | Dd | Dd |  | Dd |  | Dd |
|  | Dc | Dd | Dd | Dd | SSB0/5 |  | Dd | Dc |  | Dd | Dd | Dd | Dd |  |  |  |  |
|  | Dc | Dc | Dd | Dd | SSB0/5 |  | Dd | Dc |  | Dc | Dd | Dd | Dd |  | Dd |  | Dd |
|  | Dc | Dc | Dc | Dc | Dd | Dd | SSB1/6 |  |  | Dd | Dd | Dd | Dd |  | Dd |  | Dd |
|  | Dc | Dc | Dc | Dc | Dd | Dd | Dd |  | Dd | Dd | Dd | SSB3 |  |  | Dd |  | Dd |

Proposal 7: NR should support discontinuous time domain RA for RMSI PDSCH with consideration of potential collision between RMSI PDSCH and SS/PBCH block for pattern 1.

## Proposed change for the issue

Without introducing additional bit and reserving all RA patterns agreed in the last meeting, the following consideration have been made for indicate more resource allocation type:

- Considering that resource allocation index 0~2 are used in the case of only one search space set per slot, some resource allocation under the case of two search space sets per slot can be further indicated.

- The following parameters can also be introduced to further identify different RA patterns: SCS combination of SSB and RMSI, search space index within slot, number of symbols within CORESET.

- Considering that resource allocation index 10 is applied to the cases with =1, some resource allocation under the case with =1 can be further indicated.

- For supporting discontinuous time domain resource allocation, RA pattern instead of ‘S + L’ can be used for indicating the symbols allocated.

Based on the above analysis, we propose the following text proposal on top of Table 5.1.2.1-x agreed in last meeting.

--------------------------------------------Text Proposal for Section 13 in TS38.213 [9]-------------------------------------

Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 1 L; DL-DMRS-typeA-pos; Number of search space sets per slot; SCS combination, search space index within slot,; PDSCH mapping type; K0; RA pattern. 0; 2; 1; -; Type A; 0; 2, 3, 4, ..., 13. 3; 1; -; Type A; 0; 3, 4, 5, ..., 13. x; 2; (15kHz, 15kHz), for the first search space; Type A; 0; 2, 3, 4, 5. (15kHz, 15kHz), for the second search space; Type B*; 0; 6, 7, 12, 13. (30kHz, 15kHz); Type B*; 0; 8, 9, 12, 13. (30kHz, 30kHz); Type B*; 0; 2, 3, 8, 9. (120kHz, 60kHz); Type B*; 0; 1, 4, 5, 6. (120kHz, 120kHz); Type B*; 0; 1, 6, 8, 9. (240kHz, 120kHz), =1; Type B*; 0; 1, 4, 5, 6. (240kHz, 120kHz), =2; Type B*; 0; 4, 5, 6, 9. 1; 2; 1; -; Type A; 0; 2, 3, 4, ..., 11. 3; 1; -; Type A; 0; 3, 4, 5, ..., 11. x; 2; (30kHz, 15kHz); Type B*; 0; 8, 9, 10, 13. (30kHz, 30kHz), for the first search space; Type A; 0; 2, 3, 4, 5. (30kHz, 30kHz), for the second search space; Type B*; 0; 6, 7, 12, 13. (120kHz, 60kHz); Type B*; 0; 1, 2, 3, 6. (240kHz, 120kHz), =1; Type B*; 0; 1, 2, 3, 6. (240kHz, 120kHz), =2; Type B*; 0; 2, 3, 6, 9. 2; 2; 1; -; Type A; 0; 2, 3, 4, ..., 10. 3; 1; -; Type A; 0; 3, 4, 5, ..., 10. x; 2; (120kHz, 60kHz); Type B*; 0; 2, 3, 6, 9. (240kHz, 120kHz), for the first search space; Type B*; 0; 4, 5, 8, 9. (240kHz, 120kHz), for the second search space; Type B*; 0; 8, 9, 12, 13. 3; 2; 1; -; Type A; 0; 2, 3, 4, ..., 8. 3; 1; -; Type A; 0; 3, 4, 5, ..., 8. 4; 2; 1; -; Type A; 0; 2, 3, 4, 5, 6. 3; 1; -; Type A; 0; 3, 4, 5, 6. 5; 2; 1; -; Type B; 0; 9, 10, 11, 12. 3; 1; -; Type B; 0; 10, 11, 12, 13. 6; 2; 1; -; Type B; 0; 4, 5, 6, 7. 3; 1; -; Type B; 0; 6, 7, 8, 9. 7; x; 1; -; Type B; 0; 5, 6, 7, ..., 11. 8; x; 1; -; Type B; 0; 5, 6. 9; x; 1; -; Type B; 0; 9, 10. 10; x; 1; -; Type B; 0; 12, 13. 11; x; 1; =1; Type A; 0; 1, 2, 3, ..., 13. 1; (15kHz,30kHz), =2; Type B*; 0; 2, 3, 12, 13. 1; (15kHz,15kHz), =2; Type B*; 0; 6, 7, 12, 13. 12; x; 1; -; Type A; 0; 1, 2, 3, 4, 5, 6. 13; x; 1; -; Type A; 0; 2, 3, 4, 5. 14; x; 1; -; Type B; 0; 4, 5, 6, ..., 10. 15; x; 1; -; Type B; 0; 8, 9, 10, 11. Note: Type B* represents discontinuous time domain RA. ...

# Discussion on Simultaneous PDSCH receptions for UE in IDLE mode

In the RAN1 #92BIS meeting, the following proposals on DL/UL scheduling and HARQ management were reached [2].

Proposals:

- UE behavior in RRC_IDLE: If any two PDSCH among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH are overlapped with at least one symbol for a given UE from the primary cell, the UE is not required to decode both of PDSCH simultaneously

  - FFS: the prioritization of the PDSCH to be decoded

According to the discussion in RAN1 #92BIS meeting[2], UE behavior in RRC_IDLE should be limited for more efficient hardware implementation at the lowest possible cost.

UE in RRC_IDLE may receive any two PDSCH among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH that are overlapped with at least one symbol. Limited by the UE capability, UE is not required to decode both PDSCH simultaneously. That’s to say, UE may decode the PDSCH with higher priority and skip the other one or skip both of them. The decoding prioritization of PDSCH with different RNTIs needs further discussion.

If one UE receives RA-RNTI/TC-RNTI PDSCH, it means this UE has already started RACH procedures. Skipping the RA-RNTI/TC-RNTI PDSCH will result in RACH failure and then UE has to restart RACH procedures after a certain backoff time interval. This will certainly increase uplink overhead and control plane latency, which may increase UE’s risk of not meeting the system requirements specified in TR 38.913. Thus, RA-RNTI/TC-RNTI PDSCH is supposed to give the highest decoding priority.

In addition, the priority of P-RNTI PDSCH should be higher than SI-RNTI PDSCH because the CMAS and ETWS may be carried by paging message.

Based on the discussion above, the following proposal is made:

Proposal 8: UE behavior in RRC_IDLE: If any two PDSCH among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH are overlapped with at least one symbol for a given UE from the primary cell,the decoding prioritization of the PDSCH needs to be followed by RA-RNTI/TC-RNTI PDSCH > P-RNTI PDSCH > SI-RNTI PDSCH.

# Consideration on TBS determination

## Consideration on Channel Coding Session

After RAN1#92b meeting, the issue on VoIP support of TBS determination procedure in 38.214 has been discussed in Channel coding session in [2].

Discussed VoIP support of TBS determination procedure in 38.214. There is no consensus on modifying the TBS table. Further discussion in Scheduling/HARQ session when DOCOMO’s contribution is discussed in that agenda item.

For the channel coding session, most companies didn’t agree to modify the current TBS table or the current TBS determination procedure in 38.214. For example, it observed in [5] that 1)  the overhead is small and acceptable when scheduling one of the missing packet sizes using the existing TBS table, 2)  the number of ways of scheduling a new entry reduces the number of ways of scheduling the next TBS by the same number when adding entries to the TBS table, and 3)  the existing entries in Table 5.1.3.2-2 in TS 38.214 are sufficient when considering the need of supporting VoIP in an efficient manner, so there is no need to add or change any entries. Moreover, [7] proposed that it is unnecessary to add the missing VoIP sizes to the current TBS table because there are no benefits for the performance of MAC overhead ratio, scheduling flexibility and BLER after adding missing VoIP sizes in the current TBS table.

Observation 3: For channel coding session, most companies proposed that it is unnecessary to add VoIP sizes into current TBS table because there are no performance benefits and an inferior scheduling flexibility after adding missing VoIP sizes in the current TBS table.

In this section, we further investigate the performance of current TBS table and compared with a proposal in [6] for adding another table or adding VoIP TBS sizes into current TBS table for VoIP transmission, and present our views on TBS determination in NR in Scheduling/HARQ session.

## Consideration on Scheduling/HARQ session

### MAC overhead ratio

As we know, for TBS table in [3] as shown in Table 8-1 in Appendix, the overhead ratio(r) for TBS k is defined as

,

where  is the required number of padding bits and k is the TBS index, and the overhead ratio for each TBS is less than 5% (as shown in Figure 1 in [7]). In order to limit the actual MAC overhead ratio of some special TBSs which are not included in TBS table to a reasonable extent (namely, (TBSTBS_table - TBSspecial) /TBSTBS_table < 5%), we also evaluate the actual overhead ratio of some VoIP sizes(that are not included in current TBS table proposed by [6,8]) determined by the steps in [3] and the results is shown in Table 8-2 in Appendix. From the simulation results, we can see that the overhead ratio of special TBSs for VoIP are all less than 4% with current TBS Table. At least from overhead perspective, it may be unnecessary to add another table or add VoIP sizes proposed in [6] into the current TBS table.

Observation 4: The overhead ratio of special TBSs for VoIP which aren’t included in current TBS table are less than 4% with zero padding.

### Scheduling flexibility

We evaluate the number of MCS indices supported by each TBS determined by the steps in [3] based on the current TBS table and the new TBS table (as shown in Table 8-3 in the Appendix by adding TBSs not included in current TBS table, namely 328, 344, 360, 560, 568, 584, 592, 656, 680, 688, 696 and 712) respectively. Simulation parameters are shown in Table 8-4 in Appendix. Comparison of the simulation results of the two TBS tables are shown as Figure 8-1 and Figure 8-2 in Appendix.

From Figure 8-1 and Figure 8-2, we can see that the number of MCS indices supported by every VoIP sizes added in the new TBS table are obviously smaller than that of nearby entries in current TBS table for both initial and re-transmission. Furthermore, the number of MCS indices supported by those added sizes is rather limited for re-transmission which in turn will lead to inferior scheduling flexibility.

Observation 5: The number of MCS indices supported by those added sizes of 328, 344, 360, 560, 568, 584, 592, 656, 680, 688, 696 and 712 are obviously smaller than that of nearby entries in current TBS table for both initial and re-transmission.

### BLER vs. SNR

In order to verify that the BLER performance for VoIP packet sizes determined by current look-up table are not poorer than that determined by the new look-up table, we simulate and compare the BLER vs SNR link curves for VoIP packet size {328, 344, 360, 560, 568, 584, 592, 656, 680, 688, 696, 712} determined by new TBS table and the corresponding TBSs {336, 352, 368, 576, 576, 608, 608, 672, 704, 704, 704, 736} determined by current TBS table with the same allocated resource parameters based on simulation parameters in Table 8-5 in the Appendix. Note that 8 or 16 or 24 bits are zero padded for current TBS table. The compared simulation results are shown in Figure 8-3 in Appendix.

From Figure 8-3, we can see that the BLER vs SNR curves for every two TBSs determined by Table 8-1 and Table 8-3 respectively are almost identical. For all the cases evaluated, the difference of SNR between each two TBSs at the same BLER is almost 0.1 dB because of such small difference of the actual code rate between the two TBSs. Therefore, it is unnecessary to adjust the resulting TBS (e.g. plus/minus 8) after TBS determination procedure in current specification.

Observation 6: For VoIP packet sizes of 328, 344, 360, 560, 568, 584, 592, 656, 680, 688, 696 and 712 proposed in [6,8] to be added into current TBS table, there is no obvious difference of BLER performance between two TBSs determined by current TBS table and new TBS table respectively.

In summary, we have the following proposal based on observations of MAC overhead, scheduling flexibility and BLER performance.

Proposal 9: For Scheduling/HARQ session, from MAC overhead, scheduling flexibility and BLER performance perspective, it is unnecessary to add missing VoIP packet sizes of 328, 344, 360, 560, 568, 584, 592, 656, 680, 688, 696 and 712 into current TBS table.

Moreover, after RAN2#101b meeting, there is no conclusion by consensus on transport block size for NR VoIP in [4]. Namely, RAN1 did not receive a liaison sent by RAN2 about the result of NR VoIP. Therefore, if RAN1 dose not receive a LS from RAN2 about the result of NR VoIP sizes, we could not consider the issue on VoIP support of current TBS determination procedure.

Observation 7: After RAN2#101b meeting, there is no consensus on NR VoIP sizes and RAN2 did not send a LS to RAN1 about the result of NR VoIP.

Proposal 10: For Scheduling/HARQ session, we propose that it is unnecessary to discuss the issue on VoIP support of current TBS determination procedure before RAN2 send RAN1 a LS of the result of NR VoIP sizes.

# Conclusion

Proposal 1: For UL time-domain resource allocation, the 7-bit index can be directly interpreted as SLIV value.

Proposal 2: For DL time-domain resource allocation, four separate tables 2.1-2 to 2.1-5 for the mapping between 6-bit index and SLIV value should be considered.

Proposal 3: For slot-aggregation, the following two criteria should be support to choose the slots used for slot-aggregation.

1. The first criterion: For UL slot-aggregation, only the “U” and “X” slot can be chosen as the transmission slot; for DL slot-aggregation, only the “D” and “X” slot can be chosen as the transmission slot. The SLIV value of the first aggregation slot corresponding to the combination of start symbol and length can be used in the following chosen slots.

2. The second criteria: the slot must have enough frequency resource to support the traffic transmitted in the first aggregation slot.

Proposal 4: The slots in the transition time of BWP switching should not be counted in the slot-aggregation.

Proposal 5: The text in 38.214 should be changed as:

5.1.2.1 Resource allocation in time domain When the UE is configured with aggregationFactorDL > 1, the UE may expect that the TB is repeated within each symbol allocation among each of the aggregationFactorDL consecutive slots with the following restriction and the PDSCH is limited to a single transmission layer. Restriction 1. In time domain, the slot only can be DL slot or X slot, and the slots are not in the transition time of a BWP switching. Restriction 2. In frequency domain, the frequency resource of the slots must larger than or equal to the frequency resource of the first aggregation slot. #### 6.1.2.1 Resource allocation in time domain When the UE is configured with aggregationFactorUL > 1, the same symbol allocation is applied across the aggregationFactorUL consecutive slots with the following restriction and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the aggregationFactorUL consecutive slots applying the same symbol allocation in each slot. Restriction 1. In time domain, the slot only can be UL slot or X slot, and the slot are not in the transition time of a BWP switching. Restriction 2. In frequency domain, the frequency resource of the slot must larger than or equal to the frequency resource of the first aggregation slot.

Proposal 6: The RB mapping between two BWPs should be considered for BWP switching in slot-aggregation.

Observation 1: The Collision between RMSI PDSCH and SS/PBCH block should be considered for multiplexing pattern 1.

Observation 2: For RMSI PDSCH, time domain resource allocation without overlapping with SS/PBCH block symbols should be supported. .

Proposal 7: NR should support discontinuous time domain RA for RMSI PDSCH with consideration of potential collision between RMSI PDSCH and SS/PBCH block for pattern 1.

Proposal 8: UE behavior in RRC_IDLE: If any two PDSCH among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH are overlapped with at least one symbol for a given UE from the primary cell,the decoding prioritization of the PDSCH needs to be followed by RA-RNTI/TC-RNTI PDSCH > P-RNTI PDSCH > SI-RNTI PDSCH.

Observation 3: For channel coding session, most companies proposed that it is unnecessary to add VoIP sizes into current TBS table because there are no performance benefits and an inferior scheduling flexibility after adding missing VoIP sizes in the current TBS table.

Observation 4: The overhead ratio of special TBSs for VoIP which aren’t included in current TBS table are less than 4% with zero padding.

Observation 5: The number of MCS indices supported by those added sizes of 328, 344, 360, 560, 568, 584, 592, 656, 680, 688, 696 and 712 are obviously smaller than that of nearby entries in current TBS table for both initial and re-transmission.

Observation 6: For VoIP packet sizes of 328, 344, 360, 560, 568, 584, 592, 656, 680, 688, 696 and 712 proposed in [6,8] to be added into current TBS table, there is no obvious difference of BLER performance between two TBSs determined by current TBS table and new TBS table respectively.

Observation 7: After RAN2#101b meeting, there is no consensus on NR VoIP sizes and RAN2 did not send a LS to RAN1 about the result of NR VoIP.

Proposal 9: For Scheduling/HARQ session, from MAC overhead, scheduling flexibility and BLER performance perspective, it is unnecessary to add missing VoIP packet sizes of 328, 344, 360, 560, 568, 584, 592, 656, 680, 688, 696 and 712 into current TBS table.

Proposal 10: For Scheduling/HARQ session, we propose that it is unnecessary to discuss the issue on VoIP support of current TBS determination procedure before RAN2 send RAN1 a LS of the result of NR VoIP sizes.

# References

- Chairman's Notes RAN1 #92

- Chairman's Notes RAN1 #92bis

- R1-1805xxx, “draftCR to 38.214 capturing the RAN1#92bis meeting agreements”, 3GPP TSG-RAN1 Meeting #92bis

- Chairman's Notes RAN2 #101bis

- R1-1803937, “Remaining Issues of Channel Coding”, Ericsson, #92bis, April 16th – 20th, 2018

- R1-1805055, “Resource allocation and TBS determination”, NTT DOCOMO, #92bis, April 16th – 20th, 2018

- R1-1804489, “Consideration on TBS determination”, ZTE, Sanechips, #92bis, April 16th – 20th, 2018

- R2-1806095, “Transport Block size for NR VoIP”, Sharp, NTT DOCOMO, INC., KDDI, #101bis, 16th – 20th April, 2018

- 3GPP TS 38.213 v15.1.0: "Physical layer procedures for control",  2018-03.

- 3GPP TS 38.214 v15.1.0: "Physical layer procedures for data", 2018-03.

# Appendix

## Figure 8-1 the total number of MCS indices supported by each TBS when Ninfo ≤ 3824 for initial transmission

## Figure 8-2 the total number of MCS indices supported by each TBS when Ninfo ≤ 3824 for re-transmission

- 328 and 336

(c)  360 and 368

(d)  560 and 576

(e)  568 and 576

(f)  584 and 608

(g)  592 and 608

(h)  656 and 672

(i)  680 and 704

(j)  688 and 704

(k)  696 and 704

(l)  712 and 736

## Figure 8-3 BLER vs SNR performance for TBSs determined by Table 8-1 and Table 8-3 respectively

## Table 8-1: current look-up table in TS 38.214

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

## Table 8-2 the overhead ratio of some special TBSs for VoIP proposed in [5]

| TBS for VoIP packet (Ninfo/N'info) | TBS determined by TBS table in [2] | overhead ratio |
| --- | --- | --- |
| 328 | 336 | 2.40% |
| 344 | 352 | 2.30% |
| 360 | 368 | 2.20% |
| 560 | 576 | 2.80% |
| 568 | 576 | 1.40% |
| 584 | 608 | 3.90% |
| 592 | 608 | 2.60% |
| 656 | 672 | 2.40% |
| 680 | 704 | 3.40% |
| 688 | 704 | 2.30% |
| 696 | 704 | 1.10% |
| 712 | 736 | 3.30% |

## Table 8-3: new look-up table by adding TBSs in blue region

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 328 | 61 | 808 | 91 | 2472 |
| 2 | 32 | 32 | 336 | 62 | 848 | 92 | 2536 |
| 3 | 40 | 33 | 344 | 63 | 888 | 93 | 2600 |
| 4 | 48 | 34 | 352 | 64 | 928 | 94 | 2664 |
| 5 | 56 | 35 | 360 | 65 | 984 | 95 | 2728 |
| 6 | 64 | 36 | 368 | 66 | 1032 | 96 | 2792 |
| 7 | 72 | 37 | 384 | 67 | 1064 | 97 | 2856 |
| 8 | 80 | 38 | 408 | 68 | 1128 | 98 | 2976 |
| 9 | 88 | 39 | 432 | 69 | 1160 | 99 | 3104 |
| 10 | 96 | 40 | 456 | 70 | 1192 | 100 | 3240 |
| 11 | 104 | 41 | 480 | 71 | 1224 | 101 | 3368 |
| 12 | 112 | 42 | 504 | 72 | 1256 | 102 | 3496 |
| 13 | 120 | 43 | 528 | 73 | 1288 | 103 | 3624 |
| 14 | 128 | 44 | 552 | 74 | 1320 | 104 | 3752 |
| 15 | 136 | 45 | 560 | 75 | 1352 | 105 | 3824 |
| 16 | 144 | 46 | 568 | 76 | 1416 |  |  |
| 17 | 152 | 47 | 576 | 77 | 1480 |  |  |
| 18 | 160 | 48 | 584 | 78 | 1544 |  |  |
| 19 | 168 | 49 | 592 | 79 | 1608 |  |  |
| 20 | 176 | 50 | 608 | 80 | 1672 |  |  |
| 21 | 184 | 51 | 640 | 81 | 1736 |  |  |
| 22 | 192 | 52 | 656 | 82 | 1800 |  |  |
| 23 | 208 | 53 | 672 | 83 | 1864 |  |  |
| 24 | 224 | 54 | 680 | 84 | 1928 |  |  |
| 25 | 240 | 55 | 688 | 85 | 2024 |  |  |
| 26 | 256 | 56 | 696 | 86 | 2088 |  |  |
| 27 | 272 | 57 | 704 | 87 | 2152 |  |  |
| 28 | 288 | 58 | 712 | 88 | 2216 |  |  |
| 29 | 304 | 59 | 736 | 89 | 2280 |  |  |
| 30 | 320 | 60 | 768 | 90 | 2408 |  |  |

## Table 8-4: Simulation parameters for PDSCH

| Parameters | Values |
| --- | --- |
| The number of PRB(nPRB) | 1~273 in NR. |
| The number of mapping layers(v) | 1~4 |
| The number of symbols within a slot() | 3~14 |
| The overhead configured by Xoh-PDSCH() | {0,6,12,18} |
| The number of REs within a PRB(N’RE) | {6,8,12,16,18,20,24,28,30,32,36,40,42,44,48,52,54,56,60,64,66,68,70,72,76,78,80,82,84,88,90,92,94,96,100,102,104,106,108,112,114,116,118,120,124,126,128,130,132,136,138,140,144,148,150,152,156} in NR; |
| MCS Tables | Table 5.1.3.1-1: MCS index table 1 for PDSCH in [2]; Table 5.1.3.1-2: MCS index table 2 for PDSCH in [2]. |
| The CRC bits of transport block(LTB_CRC) | if TBS> 3824, LTB_CRC =16bits; else LTB_CRC = 24bits in NR; |

## Table 8-5 Link Level Simulation Parameters

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Channel model | AWGN |  |
| Channel estimation | Ideal |  |
| Modulation | QPSK | 16QAM |
| Code rate (*1024) | 120,193,308,449,602 | 378 |
| Information length(wo CRC) | 328 bits, 336bits, 344bits, 352bits, 360bits, 368bits, 560bits, 568bits, 576bits, 584bits, 592bits, 608bits, 656bits, 672bits, 680bits, 688bits, 696bits, 704bits, 712bits, 736bits |  |
| Coded block size | Information size (w/o CRC)/code rate |  |
| Target BLER | 0.1, 0.01, 0.001 |  |
| Code construction | LDPC with agreed BG1 and BG2 |  |
| CRC length | 16 bits |  |
| Decoding algorithm | Min-Sum decoding algorithm with alpha=0.75 |  |
| Maximum number of iterations | 25 |  |