3GPP TSGRAN WG1 Meeting #93	R1-1806209

Busan, Korea, 21st- 25th  May, 2018

Source:	Ericsson

Title:	Corrections for UL Antenna Port Identification

Agenda Item:	7.1.2.1

Document for:	Discussion and Decision

# Introduction

In the January RAN1 NR adhoc, it was decided to correct the order of UL DMRS precoding and resource mapping, as well as to update the DMRS port definition:

Conclusion

Specification should be revised to place the UL DMRS precoding after physical resource mapping and update UL DMRS port definition

Relevant tdoc# R1-1801242, R1-1801035, and R1-1801093

As a result of these discussions, a new set of antenna ports starting with 1000 roughly corresponding to the UE’s physical antennas was defined in addition to a set of ports for PUSCH and associated DMRS starting with 0.   The ports starting with 1000 were indicated to carry SRS, but this is an incomplete definition, since physical antennas carry PUSCH and associated DMRS after precoding as well as SRS.  In this contributions, we discuss the precoding for and port mapping for PUSCH, PUSCH DMRS, and SRS, and suggest how to clarify the content of antenna ports starting with 1000.

# Antenna Port Mapping & Port Identification

Figure 1 below illustrates precoding and layer mapping operations for PUSCH layers, PUSCH DM-RS, and SRS.  The processing flow is shown on the left.  How the processing steps for DMRS and SRS are represented in the current version of 38.211 is shown in the middle and right of the figure, respectively.

Figure 1

Starting with the precoding step, DM-RS associated with antenna port pν are precoded to form precoded DM-RS associated with antenna ports pρ.  Ports prior to precoding have a tilde while ports after precoding does not have a tilde notation.

PUSCH is precoded in the same manner as its associated DM-RS, hence PUSCH layers associated with antenna port p̃ν are precoded to form precoded PUSCH associated with antenna ports pρ.

One can visualize the latter antenna port (without tilde) as a physical antenna (although it is not mandated by specifications as it is part of UE implementation) and the UE can use any implementation mapping between these antenna ports and the physical antennas. Note also that this antenna port is not defined by a reference signal, thereby extending the definition of an antenna port somewhat (although this extension was also used in LTE uplink for the P “uplink antenna ports”). Note also that the precoding matrix W is the identity matrix in case non-codebook based precoding is used: as the layers rpj are already precoded transparently to gNB.

The precoded PUSCH and thus also the embedded DM-RS are next mapped to respective physical resources ak,lpj,μ, associated with one antenna port pj.  SRS reference signals do not have a specified precoding step and are directly associated with antenna ports pi.  Consequently, the SRS sequence r(pi) only needs to be mapped to the physical resource ak,lpi and hence, the SRS implementation mapping to physical antennas is always specification transparent.

Given the mapping above, it may be observed that the antenna port p is associated with both SRS and with precoded PUSCH in the current version of 38.211.  This is essential if SRS is to be useful: if SRS were transmitted on antenna ports unrelated to PUSCH, the gNB would have no way to transform its measurements of SRS to CSI that can be used to determine W, or SRI, etc.  Furthermore, SRS is transmitted at different times than PUSCH, and is not always configured (for example, prior to RRC configuration during radio link setup) and likewise, antenna ports must exists prior to RRC configuration in order to be able to define a PUSCH transmission.

Since antenna port p exists independently of SRS and carries either or both of PUSCH and SRS, it is not properly called an ‘SRS port’.  The LTE terminology of ‘antenna port’ is more accurate, but perhaps more vague.  An alternative to this is to refer to ‘precoded PUSCH ports’ to differentiate it from PUSCH layers that are defined prior to precoding.

When there is more than one SRS resource for codebook based precoding, the UE applies transparent precoding / spatial filtering to each SRS resource, and gNB can determine a corresponding W for ports within each SRS resource.  Note that since all these SRS resources are mapped to the same antenna ports, it is the implementation specific step that is different between these SRS resource transmission, i.e. the mapping of the defined antenna ports to the physical antennas. SRI identifies which SRS resource that should be used by the UE to precode / spatial filter the PUSCH and the UE knows for that SRS resource which mapping of these ports to physical antennas was used when performing the SRS resource transmission previously.  Given this use of SRI, port numbering within each SRS resource is sufficient (as is currently specified in 38.211).

Observations:

- It is necessary for SRS to occupy the set of antenna ports also occupied by PUSCH (after precoding in case of codebook based UL) for SRS to be useful for PUSCH CSI.

- Antenna ports for PUSCH transmission must exist independently of SRS

  - SRS is not transmitted simultaneously with PUSCH, nor always configured

As can be seen in the text from 38.211 section 6.2 below, antenna ports starting with 1000 are presently only identified with SRS.  This should be corrected such that both SRS and PUSCH after precoding can be carried on these ports.

Proposal:

- Correct what is carried on antenna ports pi (that start with index 1000) in 38.211 from being only SRS to SRS, precoded PUSCH, and precoded PUSCH demodulation reference signals.

For 38.211 section 6.2:

>>>>>>>>>>>> Start text proposal 1 >>>>>>>>>>>>

## 6.2	Physical resources

The frame structure and physical resources the UE shall use when transmitting in the uplink transmissions are defined in Clause 4.

The following antenna ports are defined for the uplink:

-	Antenna ports starting with 0 for PUSCH and associated demodulation reference signals

-	Antenna ports starting with 1000 for SRS, precoded PUSCH, and precoded PUSCH demodulation reference signals

-	Antenna ports starting with 2000 for PUCCH

-	Antenna port 4000 for PRACH

>>>>>>>>>>>> End text proposal 1 >>>>>>>>>>>>>

# Conclusion

In this contribution, we have clarified that both SRS, precoded PUSCH, and precoded DMRS are all carried on antenna ports starting with 1000, rather than just SRS, making the following observation and proposal:

Observations:

- It is necessary for SRS to occupy the set of antenna ports also occupied by PUSCH (after precoding in case of codebook based UL) for SRS to be useful for PUSCH CSI.

- Antenna ports for PUSCH transmission must exist independently of SRS

  - SRS is not transmitted simultaneously with PUSCH, nor always configured

Proposal:

- Correct what is carried on antenna ports pi (that start with index 1000) in 38.211 from being only SRS to SRS, precoded PUSCH, and precoded PUSCH demodulation reference signals.

Text is provided for 38.211 to implement the above proposal.