3GPP TSG RAN WG1 Meeting #93	R1-1806231

Busan, Korea, May 21st – 25th, 2018

Source:	Ericsson

Title:	On correlation of DMRS Gold sequences

Agenda Item:	7.1.2.3.8

Document for:	Discussion and Decision

# Introduction

In this contribution we present evaluation results on correlation between PN reference signal sequences when they are subsampled as following by the subcarrier-specific DM-RS mapping proposed in [1].

# Discussion

## Mapping generated PN sequence to subcarriers for DMRS

Resource-specific DM-RS mapping is captured in [1] as follows

(UL)             (DL).

The corresponding frequency-domain mapping is illustrated in Figure 1 from which we observe that the DM-RS sequence in one CDM group is copied into the other CDM group(s). Such mapping may have CM/PAPR issues when DM-RS ports from different CDM groups are spatially multiplexed such that ports are sharing same PA’s [1].

Figure 1. Currently captured DM-RS mapping to CDM groups.

Instead of mapping the same sequence into all CDM groups as shown in Figure 1,  a subcarrier-specific DM-RS mapping as the one illustrated in Figure 2 has the advantage of resulting in a time domain signal with CM/PAPR that will be on par with PDSCH/PUSCH [1]. We note that this subcarrier-specific mapping results in the effectively used sequence being a subsampling of rk. In the following we investigate how such subsampling affect correlation properties of DMRS sequences.

Figure 2. Subcarrier specific DM-RS mapping.

## Evaluation of correlation

Reference signal sequences used for the CP-OFDM waveform refer to QPSK modulated symbols derived from pseudo-random binary sequences. In the evaluations below, a reference signal sequence of 24 samples - corresponding to the mapped sequence values of a single DMRS port in a 4 RB allocation with DMRS type 1 was generated using two methods:

- Subsampled: A sequence rm of length 4x12 = 48 was generated and r(k) was created by selecting elements from  r(m) corresponding to the mapping in  Figure 2.

- Non-subsampled: A sequence rm of length 24 was formed and rk=rk

Pairs of nID, {n1,n2}, was selected and correlation between the two sequences generated from n1 and n2, respectively was computed for the 140-time instances corresponding to l=0…13 and ns,f=0…9. CDFs of correlation across time for a small set of random pairs {n1,n2} is given in Figure 3.

Figure 4	CDF across time for sequences generated using random pairs of NIDs. Sequences generated either from a long sequence by subsampling, or directly to desired length.

For each {n1,n2} pair, the 95th percentile of the correlation across time was used as a figure of merit. A histogram of this figure of merit across 500 000 randomly picked pairs was created and is shown in Figure 4. Clearly the correlation properties do not deteriorate when the sequence is subsampled.

Figure 5	Histogram of 95th percentile of correlation for the two methods.

# Conclusions

Based on the evaluations in this paper we conclude that the correlation properties of the Gold sequences for DMRS (CP-OFDM case) are as good for the subsampled sequences effectively used according to a subcarrier-specific DM-RS mapping as for non-subsampled sequences of the same length.

# References

R1-1806231, “Benefits with resource-specific DMRS mapping”, Ericsson