3GPP TSG RAN WG1 Meeting #93		                                                                  R1-1806276

Busan, Korea, May 21st – 25th, 2018

Source:		CATT

Title:			Remaining issues on codebook based UL transmission

Agenda Item:		7.1.2.1.2

Document for:	Discussion and Decision

- Introduction

In RAN1 #92 meeting, the following agreement on the default transmission scheme of UL transmission was achieved[1]:

# Agreement

- If txConfig  is not configured, the default transmission is 1 PUSCH port based

  - The PUSCH is triggered by DCI format 0_0

  - Note: there is no TPMI, no SRI indication in format 0_0

  - Note: this does not imply new uplink transmission scheme shall be defined

The following agreement on SRS resource configuration was reached[1]:

# Agreement

- gNB should configure at least 1 SRS resource when txConfig is codebook based UL transmission. No restriction on gNB’s configuration of SRS for UL scheduling by DCI format 0_0.

  - If and when the SRS is triggered is up to gNB implementation

In this contribution we discuss remaining issues on codebook based UL transmission, including clarifications on antenna ports and precoding determination, details on the UL transmission procedure with configured grant, as well as power control for UL MIMO.

- Discussion

  - Antenna port definition and determination

In subclause 6.3.1.5 of TS38.211 for PUSCH precoding, it is mentioned that “The set of antenna ports  shall be determined according to the procedure in [6, TS 38.214]”. However, the definition of  is missing, and the SRS antenna port determination procedure is not captured in TS38.214. Therefore, the definition and determination procedure of SRS antenna ports should be captured in TS38.214.

During the email discussion of draft CR of TS38.214 after RAN1 #92bis meeting, it was proposed that the precoder that a UE applies to PUSCH and DMRS should be determined by the selected SRS resource/beam in addition of TPMI. Therefore, the following text was endorsed in the CR of TS38.214 with the text highlighted in yellow captures the relationship of the PUSCH precoder and SRS resources/beams.

where the TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} that correspond to the SRS resource.

There is no specification on how the precoder corresponds to the SRS resource in NR, so “that corresponds to the SRS resource selected by the SRI” and “that correspond to the SRS resource” are quite confusing. In subclause 6.3.1.5 of TS38.211 for PUSCH precoding, the relationship of antenna ports {0…ν-1} and antenna ports are defined, where  should be the SRS antenna ports of the SRS resource selected by the SRI when multiple SRS resources are configured or be the SRS antenna ports of the SRS resource if a single SRS resource is configured. Therefore, the definition of  can be captured in TS38.214 and the description that the precoder corresponds to the SRS resource selected by SRI in TS38.214 is not necessary.

Proposal 1: Capture the definition and determination procedure of  for codebook based UL transmission in TS38.214 as “the set of antenna ports in subclause 6.3.1.5 of [4, TS 38.211] are the SRS ports in the SRS resource selected by the SRI with an increasing order” for the case when multiple SRS resources are configured and “the set of antenna ports  in subclause 6.3.1.5 of [4, TS 38.211] are the SRS ports in the SRS resource” for the case when a single SRS resource is configured.

When only one antenna port is configured for the SRS resource(s) for codebook based UL transmission, the bitwidth of Precoding information and number of layers field in DCI is 0 bit. When there is no DCI field of Precoding information and number of layers, the transmission rank is 1, and the antenna port  in subclause 6.3.1.5 of TS 38.211 is the SRS port in the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured the antenna port  in subclause 6.3.1.5 of TS 38.211 is the SRS port in the SRS resource.

Proposal 2: Clarify in TS 38.214 that when only one antenna port is configured for SRS resource(s) for codebook based UL transmission, the transmission rank is 1, and the antenna port  in subclause 6.3.1.5 of TS 38.211 is the SRS port in the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured the antenna port  in subclause 6.3.1.5 of TS 38.211 is the SRS port in the SRS resource.

  - Transmission schemes for UL transmission with configured grant

For codebook based PUSCH transmission with dynamic grant and Type 2 PUSCH transmission with configured grant, the antenna port, TPMI/TRI and SRI are indicated by DCI. For codebook based Type 1 PUSCH transmission with configured grant, the antenna port, precoding TPMI/TRI and SRI are RRC configured. Therefore the determination procedure of the transmission parameters for codebook based Type 1 PUSCH transmission with configured grant would be different from PUSCH transmission with dynamic grant. The UL transmission procedure for Type 1 PUSCH with configured grant should be captured in TS 38.214.

For PUSCH transmission with configured grant, the higher layer parameter txConfig is expected to be always configured. When txConfig is configured, similar transmission schemes as Type 1 PUSCH transmission with dynamic grant can be used for PUSCH transmission with configured grant:

For codebook based UL transmission, the UE determines the precoder and transmission rank based on high layer parameters srs-ResourceIndicator, precodingAndNumberOfLayers in rrc-ConfiguredUplinkGrant, and determines the DMRS ports based on high layer parameters precodingAndNumberOfLayers  and antennaPort in rrc-ConfiguredUplinkGrant.

Proposal 3: Capture the procedure of codebook based Type 1 PUSCH transmission with configured grant in subclause 6.1.1.1 of TS38.214.

According to proposal 1-3, the TP for 38.214 is provided as follows:

-------------------------------------------------------Start of TP for 38.214--------------------------------------------------------------

#### 6.1.1.1	Codebook based UL transmission

For codebook based transmission, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank, where the SRI, TPMI and the transmission layer are given by DCI fields of SRS resource indicator and Precoding information and number of layers in subclause 7.3.1.1.2 of [TS 38.212], or the SRI, TPMI and the transmission rank are given by high layer parameters srs-ResourceIndicator and precodingAndNumberOfLayers in rrc-ConfiguredUplinkGrant in configuredGrantConfig for Type 1 codebook based transmission with a configured grant,  the TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} and the set of antenna ports  in subclause 6.3.1.5 of [4, TS 38.211] are the SRS ports in the SRS resource selected by the SRI with an increasing order when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} and the set of antenna ports  in subclause 6.3.1.5 of [4, TS 38.211] are the SRS ports in the SRS resource. When only one antenna port is configured for the SRS resource(s) for codebook based transmission, the transmission rank is 1, and the antenna port  in subclause 6.3.1.5 of [4, TS 38.211] is the SRS port in the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured the antenna port  in subclause 6.3.1.5 of [4, TS 38.211] is the SRS port in the SRS resource.  The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Subclause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n.

<Unrelated part omitted>

-----------------------------------------------------------------End of TP ----------------------------------------------------------------

  - Power control of UL MIMO

In TS38.213, the power scaling rule for PUSCH is specified as follows:

For PUSCH, a UE first scales a linear value of the transmit power on UL BWP , as described in Subclause 12, of carrier of serving cell , with parameters as defined in Subclause 7.1.1, by the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme. The resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted.

For codebook based UL transmission, the “number of configured antenna ports for the transmission scheme” should be the number of antenna ports of one SRS resource for codebook based UL transmission. For non-codebook based UL transmission, the “number of configured antenna ports for the transmission scheme” should be the number of SRS resources that is configured for non-codebook based UL transmission.

In NR, the following 3 levels of UE coherent transmission capability for UL MIMO transmission are supported:

- Full coherence

All ports can be transmitted coherently

- Partial coherence

Port pairs can be transmitted coherently

- Non-coherence

No port pairs can be transmitted coherently

Considering the coherent transmission capability of UEs, UL codebook subset restriction based on high layer parameter codebookSubset is introduced. Based on the power scaling rule for PUSCH in TS38.213 and the codebook for UL MIMO, the followings can be observed:

Observation 1:

- When codebookSubset is set to be nonCoherent, the total transmit power corresponding to codewords with rank N is N/M of , where M is the number of configured antenna ports.

- When codebookSubset is set to be partialAndNonCoherent, the total transmit power corresponding to codewords with rank 1 is 1/M or 2/M of , where M is the number of configured antenna ports.

An UE at low SNR region is expected to transmit with low rank (rank 1) and the maximum transmit power. However, a UE can’t transmit with the maximum transmit power for low rank transmission when UL MIMO is configured and codebookSubset is set to be nonCoherent or partialAndNonCoherent based on the power scaling rule for PUSCH in TS38.213. In order to make sure a UE with non-coherent transmission capability or with partial coherent transmission capability is able to achieve the maximum transmit power, one solution is not to scale by the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme. However, without such scaling, a UE with partial coherent transmission capability is supposed that any antenna pair can achieve the maximum output power, a UE with non-coherent transmission capability is supposed that any antenna port can achieve the maximum output power.  If a UE with partial coherent transmission capability is not capable of transmitting from any antenna pair with the maximum output power, or a UE with non-coherent transmission capability is not capable of transmitting from any antenna port with the maximum output power, the transmit power maybe lower than what gNB expects, then the UL transmission performance will be degraded. In LTE, only one antenna connector is required to achieve the maximum output power, not all antenna connectors are required to meet the maximum output power requirement. It is likely that similar requirement will be specified in NR. Therefore, the scaling of is needed in NR.

In order to let a UE with non-coherent or with partial coherent transmission capability to be able to transmit with the maximum transmit power with rank 1, it can be predefined that the PUSCH transmit power of one of the rank 1 codewords is not scaled with the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports, then the corresponding transmit power of this codeword is able to achieve the maximum transmit power.

Proposal 4: For codebook based UL transmission, the scaled power equals to the transmit power when Precoding information and number of layers field in DCI is mapped to index 0, otherwise, the scaled power  is the transmit power  scaled by the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme.

The analysis for power control for non-codebook based UL transmission is provided in subclause 2.4 of [2].According to the discussions in this subclause and the analysis of subclause 2.4 of [2],  the following TP for power control of UL transmission is provided:

------------------------------------------------------Start of TP for 38.213---------------------------------------------------------------

7.1	Physical uplink shared channel

For PUSCH, a UE first scales a linear value  of the transmit power  on UL BWP , as described in Subclause 12, of carrier  of serving cell , with parameters as defined in Subclause 7.1.1, by the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme except that:

- When the number of configure antenna port for the transmission scheme is 1, the scaled power  equals to the transmit power ;

- When the DCI field of Precoding information and number of layers in Subclause 7.3.1.1.2 of [TS 38.212] is mapped to index 0 for codebook based transmission, the scaled power  equals to the transmit power ;

- When a UE with pusch-TransCoherence set to “nonCoherent”or “partialNonCoherent” and DCI field of SRI in Subclause 7.3.1.1.2 of [TS 38.212] is mapped to index 0 for non-codebook based transmission, the scaled power  equals to the transmit power ;

- When a UE with pusch-TransCoherence set to “partialNonCoherent” and DCI field of SRI in Subclause 7.3.1.1.2 of [TS 38.212] is mapped to indexes that indicating one SRS resource other than index 0 for non-codebook based transmission, , the scaled power  is the transmit power  scaled by ratio 1/2;

- When a UE with pusch-TransCoherence set to “fullCoherent” and the transmission scheme is non-codebook based UL transmission, the scaled power  equals to the transmit power ;

The resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted. The UL BWP  is the active UL BWP.

----------------------------------------------------------------End of TP -------------------------------------------------------------------

Proposal 5: Adopt the TP in this subclause for PUSCH transmission power control.

- Conclusions

In this contribution, we discussed the remaining issues on codebook based UL transmission, including clarifications on antenna ports and precoding determination, details on the UL transmission procedure with configured grant, as well as power control for UL MIMO. We propose that:

Proposal 1: Capture the definition and determination procedure of  for codebook based UL transmission in TS38.214 as “the set of antenna ports in subclause 6.3.1.5 of [4, TS 38.211] are the SRS ports in the SRS resource selected by the SRI with an increasing order” for the case when multiple SRS resources are configured and “the set of antenna ports  in subclause 6.3.1.5 of [4, TS 38.211] are the SRS ports in the SRS resource” for the case when a single SRS resource is configured.

Proposal 2: Clarify in TS 38.214 that when only one antenna port is configured for SRS resource(s) for codebook based UL transmission, the transmission rank is 1, and the antenna port  in subclause 6.3.1.5 of TS 38.211 is the SRS port in the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured the antenna port  in subclause 6.3.1.5 of TS 38.211 is the SRS port in the SRS resource.

Proposal 3: Capture the procedure of codebook based Type 1 PUSCH transmission with configured grant in subclause 6.1.1.1 of TS38.214.

According to proposal 1-3, the TP for 38.214 is provided as follows:

-------------------------------------------------------Start of TP for 38.214--------------------------------------------------------------

#### 6.1.1.1	Codebook based UL transmission

For codebook based transmission, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank, where the SRI, TPMI and the transmission layer are given by DCI fields of SRS resource indicator and Precoding information and number of layers in subclause 7.3.1.1.2 of [TS 38.212], or the SRI, TPMI and the transmission rank are given by high layer parameters srs-ResourceIndicator and precodingAndNumberOfLayers in rrc-ConfiguredUplinkGrant in configuredGrantConfig for Type 1 codebook based transmission with a configured grant,  the TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} and the set of antenna ports  in subclause 6.3.1.5 of [4, TS 38.211] are the SRS ports in the SRS resource selected by the SRI with an increasing order when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} and the set of antenna ports  in subclause 6.3.1.5 of [4, TS 38.211] are the SRS ports in the SRS resource. When only one antenna port is configured for the SRS resource(s) for codebook based transmission, the transmission rank is 1, and the antenna port  in subclause 6.3.1.5 of [4, TS 38.211] is the SRS port in the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured the antenna port  in subclause 6.3.1.5 of [4, TS 38.211] is the SRS port in the SRS resource.  The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Subclause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n.

<Unrelated part omitted>

-----------------------------------------------------------------End of TP ----------------------------------------------------------------

Proposal 4: For codebook based UL transmission, the scaled power equals to the transmit power when Precoding information and number of layers field in DCI is mapped to index 0, otherwise, the scaled power  is the transmit power  scaled by the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme.

According to proposal 4, the TP for 38.213 is provided as follows:

------------------------------------------------------Start of TP for 38.213---------------------------------------------------------------

7.1	Physical uplink shared channel

For PUSCH, a UE first scales a linear value  of the transmit power  on UL BWP , as described in Subclause 12, of carrier  of serving cell , with parameters as defined in Subclause 7.1.1, by the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme except that:

- When the number of configure antenna port for the transmission scheme is 1, the scaled power  equals to the transmit power ;

- When the DCI field of Precoding information and number of layers in Subclause 7.3.1.1.2 of [TS 38.212] is mapped to index 0 for codebook based transmission, the scaled power  equals to the transmit power ;

- When a UE with pusch-TransCoherence set to “nonCoherent”or “partialNonCoherent” and DCI field of SRI in Subclause 7.3.1.1.2 of [TS 38.212] is mapped to index 0 for non-codebook based transmission, the scaled power  equals to the transmit power ;

- When a UE with pusch-TransCoherence set to “partialNonCoherent” and DCI field of SRI in Subclause 7.3.1.1.2 of [TS 38.212] is mapped to indexes that indicating one SRS resource other than index 0 for non-codebook based transmission, , the scaled power  is the transmit power  scaled by ratio 1/2;

- When a UE with pusch-TransCoherence set to “fullCoherent” and the transmission scheme is non-codebook based UL transmission, the scaled power  equals to the transmit power ;

The resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted. The UL BWP  is the active UL BWP.

----------------------------------------------------------------End of TP -------------------------------------------------------------------

Proposal 5: Adopt the TP in this subclause for PUSCH transmission power control.

- References

- RAN1#92 Chairman’s Notes

- R1-1806277, Remaining issues on non-codebook based UL transmission, CATT

- R1-1805793, CR to 38.211 capturing the RAN1#92bis meeting agreements , Ericsson

- R1-1805796, CR to 38.214 capturing the RAN1#92bis meeting agreements , Nokia