3GPP TSG RAN WG1 Meeting #93			R1-1806290

Busan, Korea, April 21st – 25th, 2018

Source:	CATT

Title:	On UE procedures for SFI reception

Agenda Item:	7.1.3.1.3

Document for:	Discussion and Decision

# Introduction

The few remaining issues on UE behavior when SFI is received in DCI format 2_0 were discussed at the RAN1 #92bis meeting [1]. Some issues were agreed while others are unresolved and are the focus of this contribution.

# Discussion on open issues

Valid lengths for the slot format combination

A first open issue is the UE assumption when the number of slots configured in a slot format combination, denoted here as N, is less than the SFI monitoring period, TSFI. One mentioned use case is when the transmission direction is relatively fixed and the gNB would rather the UE follow the transmission direction indicated either by RRC configuration or based on dynamic scheduling. Some possible solutions for UE behavior mentioned at the last meeting include:

- Alt 1: When gNB configures a UE-specific SFI table, each entry of the table covers at least the number of slots equals to the configured SFI monitoring period

- Alt 2: If an entry of UE-specific SFI table has length shorter than the SFI monitoring period, the UE will assume slot format 255 for the remaining slots within the period.

- Alt 3: If an entry of UE-specific SFI table has length shorter than the SFI monitoring period, the UE will assume the same behaviour for the remaining slots before the next SFI monitoring occasion as if the SFI is not detected.

The UE behavior corresponding to Alt. 3 would be to cancel RRC configured measurements and transmissions and only monitor the PDCCH in those slots. If the gNB intended such an operation it could very well have dynamically indicated it. Alt. 2 is based on the RAN1 #92bis agreement that,

Agreements

- In the slot format table in TS38.211, the entry 255 is defined such that when a slot format for a slot is indicated as 255, the UE does not use this information in deciding the cancellation of UE-specific RRC configured DL receptions or UE-specific RRC configured UL transmissions

In other words entry #255 can be seen as a “blank” slot format informing the UE to follow the transmission direction provided by RRC configuration, if any. This is in direct contrast to Alt. 3 which cancels RRC configured reception/transmission. Alt. 1 on the other hand can be seen to be equivalent to Alt. 2 if the length of the slot format combination is set to TSFI and the last TSFI – N slot formats are set to 255. Therefore, Alt. 1 can be adopted

Proposal 1: the number of slots configured in a slot format combination is equal to or greater than the SFI monitoring period TSFI. By implementation the network may configure the number L of non-reserved slot format entries in the slot format combination to be less than TSFI by configuring the remaining TSFI – L entries with the value 255.

SFI for Extended CP

It was agreed to reuse the set of candidate SFI values specified for normal CP in Table 11.1.1-1 of 38.213 for extended CP deployment:

Agreements:

- On slot format indication for ECP, reuse the slot format indication for NCP, and determine the direction of an ECP symbol using the indicated direction of (partially) overlapping NCP symbols

  - If all (partially) overlapping NCP symbols are D, the ECP symbol is D

  - If all (partially) overlapping NCP symbols are U, the ECP symbol is U

  - If one of (partially) overlapping NCP symbols is X, the ECP symbol is X

This is fine for all but one entry of the table. The outlier is the asymmetric entry with index #55 in Table 11.1.1-1 which is still a working assumption from the RAN1 AH-1801 meeting,

Agreements:

- Support XXXXXXXDDDDDDD in the slot format table

- (working assumption) Support DDXXXUUUDDDDDD in the slot format table

It can be seen in Figure 1 that symbol #6 in extended CP overlaps with an UL and DL symbol in normal CP. One reason is that this entry is not in line with the RAN1 #90bis agreement that for two D/U switching points in a slot the first 7 symbols start with zero or more DL symbols and ends with at least one UL symbol at symbol #6, whereas the second 7 symbols starts with one or more DL symbols and ends with zero or more UL symbols.

Figure 1 Indication of SFI in ECP for entry #55 of the normal CP table

The overlapping symbol in ECP should be either D or U but not X as this would violate the agreed design principle that X comes between D and U in that order and not the other way round. For this case shown in Figure 1, we propose to set symbol #6 in ECP to U because this somewhat maintains the same proportion of possible DL and UL symbols compared to the NCP case.

Proposal 2: if the working assumption for entry #55 in Table 11.1.1-1 of TS 38.213 is confirmed, the corresponding ECP entry is {D, X, X, X, X, U, U, D, D, D, D, D}.

# Corrections to TS 38.213

The description of the slot format #255 in Section 11.1.1 and Table 11.1.1-1 of TS 38.213 do not accurately reflect the agreement. A TP is provided below.

-------------------------------------- Begin TP for Sec. 11.1.1 of TS 38.213 --------------------------------------------

<Text omitted>

If a PDCCH monitoring periodicity for DCI format 2_0, provided to a UE for the search space set  by higher layer parameter monitoringSlotPeriodicityAndOffset, is smaller than a duration of a slot format combination the UE obtains at a PDCCH monitoring occasion for DCI format 2_0 by a corresponding SFI-index field value, the UE expects a same format for each slot of the slot format combination that occurs at or after the next PDCCH monitoring occasions for DCI format 2_0 to be indicated by corresponding SFI-index field values.

<Text omitted>

Table 11.1.1-1: Slot formats for normal cyclic prefix

| Format | Symbol number in a slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 0 | D | D | D | D | D | D | D | D | D | D | D | D | D | D |
| <Entries omitted> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 55 | D | D | X | X | X | U | U | U | D | D | D | D | D | D |
| 56 – 254 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 255 | Equivalent to the case where a UE is not configured to monitor a DCI format 2_0 for this cell |  |  |  |  |  |  |  |  |  |  |  |  |  |

----------------------------------------- End TP for Sec. 11.1.1 of TS 38.213 --------------------------------------------

# Conclusion

This contribution discussed several remaining issues on UE behavior for SFI reception. Text proposals on corrections to TS 38.213 are described and a set of proposals is also presented as follows:

- Proposal 1: the number of slots configured in a slot format combination is equal to or greater than the SFI monitoring period TSFI. By implementation the network may configure the number L of non-reserved slot format entries in the slot format combination to be less than TSFI by configuring the remaining TSFI – L entries with the value 255.

- Proposal 2: if the working assumption for entry #55 in Table 11.1.1-1 of TS 38.213 is confirmed, the corresponding ECP entry is {D, X, X, X, X, U, U, D, D, D, D, D}.

# References

  - R1-1805530, “Offline  discussion summary on GC-PDCCH carrying SFI”, Qualcomm, RAN1 #92bis