3GPP TSG RAN WG1 Meeting #93							R1- 1806302

Busan, South Korea, 21st  – 25th May 2018

# Source:              CATT

Title:                  Dynamic Power Sharing for NE-DC

Agenda Item:     7.1.5

Document for:   Discussion and Decision

Introduction

In RAN1 NR AH#1801, dynamic power sharing mechanism between NR and LTE dual connectivity was discussed with the following agreements,

Agreement:

- P_LTE and P_NR are configured separately via UE specific RRC (i.e., as dBm numbers with similar value range as p-Max in LTE)

  - P_LTE and P_NR are UE-specific

- P_cmax for LTE and P_cmax for NR are derived based on P_LTE and/or P_NR (details to be decided by RAN4)

- RAN4 to define maximum total LTE and NR power in FR1 (X_total) that the UE should never exceed.

- When dynamic power sharing is used,

  - If total power for LTE and NR in FR1 exceeds X_total, UE reduces NR transmission power or drops NR transmission so that total power does not exceed X_total

    - Note: As per previous agreement LTE power control procedure is not changed

In RAN#78, the UE capability on power scaling for EN dual connectivity was endorsed with the following proposals,

- Proposal 1

  - Agree to introduce Rel-15 capability signaling to indicate whether the UE supports dynamic EN-DC power sharing

    - Dynamic power sharing means that the UE can operate with P_LTE + P_NR > P_powerclass configuration

  - Agree that the intent is to make dynamic EN-DC power sharing mandatory at a future time

  - Check any possible updates on status above in March

- Proposal 2

  - For UEs without dynamic LTE-NR power sharing capability, the support of single UL operation (Operation A with Case 1 in Slide 5) is mandatory with capability signalling

  - Single UL operation is optional for dynamic power sharing capable UEs

In RAN1 NR AH#2, the power sharing between NR and LTE dual connectivity was agreed as follows,

Agreements:

- Regarding power sharing for LTE-NR dual connectivity, support at least semi-static power sharing between LTE and NR

  - FFS details

  - Discuss further whether or not to support dynamic power sharing between LTE and NR

  - Discuss further impacts due to other factors, e.g., different TTI lengths, channel/service types, synchronous vs. asynchronous, different processing latency for LTE vs. NR, assumption regarding communication between NR vs. LTE at UE, specification impact to LTE (if any) and/or NR, etc.

In RAN1#90, dynamic power sharing mechanism between NR and LTE dual connectivity based on power overbooking principle was discussed with the following agreements:

Agreements:

- At least for LTE-NR NSA operation

  - Maximum allowed power values for LTE (P_LTE) and NR (P_NR) are set separately

    - i.e., when UE is configured for NR, P_LTE can be configured up to P_cmax and  P_NR can be configured up to P_cmax.

    - e.g. P_LTE + P_NR > P_cmax or P_LTE + P_NR = P_cmax

  - Signaling details for P_LTE, P_NR are left to RAN2, RAN4.

  - Note: ‘P_cmax’ is a limit that is similar to ‘The configured maximum UE output power’ that was specified for LTE.

  - Note: The network will still have flexibility to prioritize or reserve certain NR transmission power depending on network implementation

  - All UEs are mandated to handle P_LTE + P_NR = P_cmax while handling of P_LTE + P_NR > P_cmax depends on UE capability

  - At least, when DL/UL LTE sTTI/reduced UE processing time based operation is not configured for the UE, if total transmit power exceeds P_cmax when there is simultaneous NR and LTE UL tx,

    - For NR, UE scales down/drops NR transmission and NR power scaling details are left to UE implementation (note: it is not intended to have RAN4 test from RAN1 perspective)

      - If there are two or more UL carriers, the power scaling or tx dropping can be performed for each of the UL carriers separately or jointly up to UE implementation

    - For LTE, no change in power control procedure

  - FFS the case when DL/UL LTE sTTI/reduced UE processing time based operation is configured for the UE

  - The following is FFS

    - The case when P_NR is configured such that P_NR < P_cmax, and UE can use power up to P_cmax in NR when it knows that there will be no UL transmission in LTE by semi-static configuration (e.g., measurement gap, DL/UL configuration)

In this contribution, we discuss the power sharing mechanism for NE-DC (NR and LTE dual connectivity for architecture option 4).

Dynamic Power Sharing in NE-DC

Power sharing mechanism for LTE and NR dual connectivity is to define the UE power allocation strategy for NR and LTE UL transmission.  In EN-DC, the configured maximum power P_LTE and P_NR could be the same as the maximum power of the power class, e.g,, 23 dBm, or overbooked with the total power P_LTE + P_NR greater than the power class.   All UEs need to support the sum of the total power would not exceed the maximum power of the power class.   If the sum of the total power of LTE and NR PUSCH/PUCCH/SRS transmission is over the maximum power limit of the power class, UE is required to scale down the total transmit power or drop the NR PUSCH transmission based on UE implementation in order to protect the LTE PUSCH transmission based on the request from Operators.  It was agreed in RAN#78 that capability signaling is introduced to indicate whether the UE supports dynamic EN-DC power sharing, which the total Tx power from LTE and NR could exceed the maximum power of the power class, in Rel-15.   It was intended to make the UE power scaling capability for dynamic EN-DC power sharing as mandatory at a future time.    If UE indicates not supporting dynamic EN-DC power sharing, it is the network responsibility to set the proper maximum power of NR and LTE and the scheduled UL transmission to ensure the total transmit power would not exceed the maximum power of the power class at any time.

For NE-DC with NR as the PCell, NR supports different numerologies comparing to that of LTE.   The UL maximum power PCMAX,c configured for NR component carrier for NE-DC might have slot length equal or shorter the LTE subframe length.   Even NR has the same numerology of 15 kHz SCS as that of LTE,  the NR component carrier might have different slot format, which the number of UL symbols within a slot could be different even NR and LTE carriers have same numerology as shown in Figure 1.  Thus, the total Tx power could vary among symbols within a slot when multiple PUSCH are transmitted with same numerology for NE-DC.

Figure 1: CCs with same numerology but different slot formats

For NR and LTE carriers with different numerologies, the NR slot length would be different to LTE subframe length as shown in Figure 2.  The UL total Tx power could vary when component carriers with different numerologies are transmitted in the same time.  Since the Tx power control is independent configured the PCMAX,c for each CC, the total Tx power might exceed the total output power of the UE power class in the some symbols for NE-DC.  UE has to scale down the Tx power when the total output power over the power class of the specific band.

Figure 2: CCs with differnt numerologies

Dynamic power sharing between NR and LTE component carriers are the baseline assumptions with the configured maximum power PCMAX,c for each component carrier based on overbooking principle.  The maximum Tx power PCMAX,c for each component carrier can be individually configured up to the maximum power for the UE power class of the specific band  for dynamic power sharing among NR and LTE carriers.    Each component carrier would have the Tx power controlled independently for NR and LTE component maximum power PCMAX,c if the sum of Tx power from all NR and LTE carriers is not over the total maximum power.  Due to different slot formats and numerologies between NR and LTE  as shown in Figure 1 and Figure 2, the UE output power might be contributed from Tx power of one carrier at some OFDM symbols and more than one carrier at other OFDM symbols.  The total power in some OFDM symbols might be over the maximum power of the UE power class due to the sum of Tx power from NR and LTE component carriers.  UE needs to scale down the Tx power of those OFDM symbols to meet the requirements and prevent the signal distortion from Tx power amplifier.

For NE-DC, the NR PUSCH transmission power would be more robust with shorter interval comparing to that of LTE due to different slot formats or numerology.    The power scaling or power dropping on LTE SCell to protect the NR PCell would be difficult since the LTE PUSCH transmission interval is longer than that of NR PUSCH and over the maximum power might happen in the middle of LTE subframe.  Since there is no request from operator to protect the NR PCell, it would be UE implementation choice of scaling the transmit power If the UE output power at some OFDM symbols exceeds the maximum power of the UE power class for NE-DC.

Proposal 1:  It would be UE implementation solution of scaling the transmit power for dynamic power sharing among NR and LTE in NE-DC If the UE output power at some OFDM symbols exceeds the maximum power of the UE power class when the total power P_LTE + P_NR is configured greater than the power class

Conclusion

In this contribution, we discuss dynamic power sharing mechanism for NE-DC (NR and LTE dual connectivity for architecture option 4).   With all the analysis, we propose the following,

- Proposal 1:  It would be UE implementation solution of scaling the transmit power for dynamic power sharing among NR and LTE in NE-DC If the UE output power at some OFDM symbols exceeds the maximum power of the UE power class when the total power P_LTE + P_NR is configured greater than the power class

References

- R1-1804811	, “Remaining aspects of NR-LTE coexistence”, Qualcomm Incorporated

- R1-1805143, “Power control for EN-DC option 4”, Nokia, Nokia Shanghai Bell