3GPP TSG RAN WG1 Meeting # 93                                                                               R1-1806305

Busan, Korea, May 21st –25th, 2018

Source:	CATT

Title:	NOMA transmitter side signal processing

Agenda Item:	7.4.1

Document for:	Discussion and Decision

Introduction

As suggested in NR Non-orthogonal Multiple Access (NOMA) SID [1], following objectives are defined for NOMA scheme in transmitting:

- Transmitter side signal processing schemes for non-orthogonal multiple access [RAN1]:

  - Modulation and symbol level processing, including spreading, repetition, interleaving, new constellation mapping, etc.

  - Coded bit level processing including interleaving and/or scrambling, etc.

  - Symbol to resource element mapping, sparse or not, etc.

  - Demodulation reference signal. Other signal is not excluded.

In [2], transmitter side processing for pattern division multiple access (PDMA) was discussed. In this contribution, we provide further explanation about PDMA scheme and also give some LLS results considering the effect of different PDMA pattern matrix. This is a resubmission of R1-1803770.

PDMA scheme

PDMA is proposed as a novel NOMA scheme with the idea of joint design between transmitter and receiver [3]. To solve the error propagation problems from conventional serial interference cancellation receiver, unequal diversities are introduced for different users, with the former detected user having higher diversities than the latter detected user to improve detection accuracy of the former detected user. For the latter detected user, since much less interference are suffered after interference cancellation, detection accuracy can also be achieved. That’s to say, different users can achieve similar diversity after detection. The character of unequal diversity can bring further sparsity character in signal spreading. That’s, for every user, not all allocated resources are mapped by their data, but for some users only part of the allocated resources are mapped with data. Assuming six users are multiplexed on four resource elements (REs), Figure 1 shows an example of the sparse resource mapping with unequal diversity. User1’s data are mapped to all four REs in the group, user2’s data are mapped to the first three REs, user3’s data are mapped to the first and third REs, user4’s data are mapped to the second and fourth REs, user5’s data are mapped to the third RE, user6’s data are mapped to the fourth RE. The order of transmission diversity of the six users is 4, 3, 2, 2, 1, and 1 respectively.

Figure 1: PDMA pattern for 6 users on 4 REs.

Since sparsity brings the advantages of lower complexity in detection, more high performance detection algorithms can be used to approaching the optimal capacity, such as Belief Propagation (BP) algorithm and also its iterative detection and decoding algorithm (BP-IDD).

LLS results of PDMA

PDMA targets for 5G typical scenarios including eMBB, URLLC and mMTC. To support both grant-free and grant-based transmissions, PDMA pattern design may be a key factor which should be considered, so here we mainly evaluate the LLS performance of PDMA with different PDMA pattern matrix.

As shown in the annex, given the updated simulation conditions agreed in RAN#92, LLS performance of different PDMA pattern matrix are compared. For those varying parameters for simulation, usually TBS per UE and overload factor (OF) are regarded as key factors affecting patterns which should be given full consideration. While other factors like application scenario and channel type can be evaluated with representatives, to avoid a redundancy huge amount of simulation. Here, mMTC and eMBB are selected as representatives of the scenarios, and TDL-A channel is selected as a representative of the wireless channel.

As suggested in the simulation condition shown in the annex, totally three kinds of overload factors are involved in the LLS simulation, including 150%, 200%, 300%. For each overload, two types of PDMA pattern matrix are defined as examples, as shown in Table 1.

Table 1: Examples of different PDMA pattern matrix

| overload | type | PDMA pattern matrix |
| --- | --- | --- |
| 150% | Type 1 |  |
|  | Type 2 |  |
| 200% | Type 1 |  |
|  | Type 2 |  |
| 300% | Type 1 |  |
|  | Type 2 |  |

mMTC scenario

- TBS=10 bytes, OF=150%

Figure 2: mMTC scenario, TDL-A channel, TBS=10 bytes, OF=150%

- TBS=10 bytes, OF=200%

Figure 3: mMTC scenario, TDL-A channel, TBS=10 bytes, OF=200%

- TBS=10 bytes, OF=300%

Figure 4: mMTC scenario, TDL-A channel, TBS=10 bytes, OF=300%

- TBS=40 bytes, OF=150%

Figure 5: mMTC scenario, TDL-A channel, TBS=40 bytes, OF=150%

eMBB  scenario

- TBS=20 bytes, OF=150%

Figure 6: eMBB scenario, TDL-A channel, TBS=20 bytes, OF=150%

- TBS=20 bytes, OF=200%

Figure 7: eMBB scenario, TDL-A channel, TBS=20 bytes, OF=200%

- TBS=20 bytes, OF=300%

Figure 8: eMBB scenario, TDL-A channel, TBS=20 bytes, OF=300%

- TBS=80 bytes, OF=150%

Figure 9: eMBB scenario, TDL-A channel, TBS=80 bytes, OF=150%

Based on the above simulation results, we have the following observations.

Observation 1: Negligible performance difference is observed for different PDMA pattern matrixes.

Observation 2: PDMA is robust to codebook collision.

Conclusion

In this contribution, we explained further about PDMA scheme and also evaluate LLS performance of PDMA with different pattern matrixes with following observations.

Observation 1: Negligible performance difference is observed for different PDMA pattern matrixes.

Observation 2: PDMA is robust to codebook collision.

References

- RP-170829, New Study Item proposal: Study on Non-orthogonal Multiple Access for NR, ZTE, CATT, Intel, Samsung, RAN#75

- R1-1801753, “NOMA transmitter side signal processing”, CATT, 3GPP RAN1#92

- Chen S., Ren B., Gao Q., Kang S., Sun S., Niu K., Pattern Division Multiple Access (PDMA) - A Novel Non-orthogonal Multiple Access for 5G Radio Networks [J]. IEEE trans. on vehicular technology, 2016, 66(4): 3185-3196.

- Annex: LLS simulation conditions agreed in RAN1#92

| Parameters | mMTC | URLLC | eMBB |
| --- | --- | --- | --- |
| Carrier Frequency | 700 MHz | 700 MHz or 4 GHz | 4 GHz, 700 MHz as optional |
| Waveform (data part) | CP-OFDM and DFT-s-OFDM | CP-OFDM as starting point | CP-OFDM as starting point |
| Channel coding | URLLC: NR LDPC eMBB: NR LDPC mMTC: NR LDPC |  |  |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 | Case 1: SCS = 60 kHz, #OS = 7 (normal CP), optionally 6 (ECP) Case 2: SCS = 30 kHz, #OS = 4 | SCS = 15 kHz #OS = 14 |
| Allocated bandwidth | 6 as the starting point | 12 as the starting point | 12 as the starting point |
| TBS per UE | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. Lower than 0.1 bits/RE is optional | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. | At least five TBS that are [20, 40, 80, 120, 150] bytes. Other values higher than 20 bytes are not precluded. |
| Target BLER for one transmission | 10% | 0.1% | 10% |
| Number of UEs multiplexed in the same allocated bandwidth | To be reported by companies. |  |  |
| BS antenna configuration | 2 Rx or 4 Rx for 700MHz, 4Rx or 8 Rx for 4 GHz 8Rx as optional |  |  |
| UE antenna configuration | 1Tx |  |  |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h, CDL optional |  |  |
| Max number of HARQ transmission | 1 as starting point. | 1 as starting point. More values, 2 for URLLC can be used. | 1 as starting point. |
| Channel estimation | Ideal channel estimation |  |  |
| MA signature allocation (for data and DMRS) | Fixed/Random |  |  |
| Distribution of avg. SNR | Both equal and unequal | Equal | Both equal and unequal |
| Timing offset | 0 as starting point. |  |  |
| Frequency error | 0 as starting point. |  |  |
| Traffic model for link level | Full buffer as starting point. |  |  |