3GPP TSG RAN WG1 Meeting #93                                      	 R1- 1806338

Busan, South Korea, 21st – 25th May, 2018

Agenda Item:	7.1.2.1.2

Source:	Lenovo, Motorola Mobility

Title:	Remaining Issues on Codebook Based Transmission for UL

Document for:	Discussion

Introduction

This contribution is a resubmission of R1-1804202.

In this contribution, we discuss on the PTRS configurations for non-coherent transmission with 4 antenna ports.

Up to 2-port UL PTRS is supported in Rel-15 as following:

Agreement in RAN1#90bis

For CP-OFDM, antenna port (AP) configuration

  - Same as DL, support at least up to 2 UL PTRS ports in Rel-15.

Non-coherent transmission is supported for codebook based UL transmission in Rel-15:

Agreement in RAN1#90bis:

NR supports 3 levels of UE capability for UL MIMO transmission

Full coherence

All ports can be transmitted coherently

Partial coherence

Port pairs can be transmitted coherently

Non-coherence

No port pairs can be transmitted coherently

For 1 SRS resource

Full coherence

All ports corresponding to ports in an SRS resource can be transmitted coherently

Non-coherence

All ports corresponding to ports in an SRS resource are not transmitted coherently

Partial coherence

Ports pairs corresponding to ports in an SRS resource can be transmitted coherently

The association between SRS resources and PTRS ports or between SRS ports and PTRS ports is defined as the following agreements:

Agreement in RAN1#91

If UE has reported capability of supporting full-coherent UL transmission, the UE may expect the number of UL PT-RS ports to be configured as one.

For partial-coherent and non-coherent codebook based UL transmission, the higher layer parameter maxNrofPorts indicates the maximum number of PTRS ports.

  - The actual number of UL PTRS port(s) to transmit is determined based on TPMI and/or TRI.

For partial-coherent and non-coherent codebook based UL transmission, if the higher -layer parameter UL-PTRS-ports is 2, the actual UL PTRS port(s) and the associated transmission layer(s) are derived from indicated TPMI with the following rule:

  - SRS port 0 and 2 in indicated TPMI share PTRS port 0,

  - SRS port 1 and 3 in indicated TPMI share PTRS port 1.

  - UL PTRS port 0 is associated with the UL layer x of layers which are transmitted with SRS port 0 and SRS port 2 in indicated TPMI;

  - UL PTRS port 1 is associated with the UL layer y of layers which are transmitted with SRS port 1 and SRS port 3 in indicated TPMI;

  - Up to 2-bit indicator is used in uplink grant where bit 1 is used for x and bit 2 is used for y.

Discussion

For non-coherent transmission with three or four layers, the corresponding number of non-coherent transmission ports or TX chains are also three or four. Examples include TPMI 0 in Table 6.3.1.5-6 and Table 6.3.1.5-7 in TS38.211 v15.0.1. Although some of those antenna ports or TX chains may share the same crystal oscillator and phase-locked loop, we cannot exclude some special terminal configurations where each TX chain or antenna port has independent crystal oscillator and PLL. For this case, the phase noises of these ports are independent and need to be estimated separately using three or four PT-RS ports. However, in the current version of TS38.211 and TS38.214, only PT-RS with up to two ports are supported, where SRS port 0 and 2 share PT-RS port 0, SRS port 1 and 3 share PT-RS port 1 for partial and non-coherent transmission. This cannot support the three or four layer non-coherent transmission for the special case mentioned above. It should believe that is necessary to support 4 PTRS ports. It is not simply a nice-to-have new feature, and it should be addressed in Rel-15.

Based on the above analysis, we have the following proposal:

Proposal 1: Up to 4 PT-RS ports should be supported for UL codebook based transmission with non-coherent ports.

If a UE has reported the capability of supporting non-coherent UL transmission with four antenna ports, the UE may expect the number of UL PT-RS ports to be configured as four by higher layer parameter maxNrofPorts with the value of n4.

Proposal 2: Four-port PTRS should be configured by higher layer parameters if a UE reporting its capability of ‘NonCoherent’ transmission with four antenna ports.

The actual number of PT-RS ports transmitted by UE is min{v,4}, where v is the rank for the current UL transmission.

For example, if the UE receive a TPMI indicate to the precoder of W=12100100001000, it will transmit with 3 PT-RS ports. Each PT-RS port is associated with a transmission layer transmitted with a corresponded SRS port, i.e. PT-RS port 0 is associated with the transmission layer transmitted with SRS port 0, PT-RS port 1 is associated with the transmission layer transmitted with SRS port 1, PT-RS port 2 is associated with the transmission layer transmitted with SRS port 2.

Proposal 3: For non-coherent UL transmission, each PT-RS port is associated with a transmission layer transmitted with a corresponding SRS port.

Conclusion

Proposal 1: Up to 4 PT-RS ports should be supported for UL codebook based transmission with non-coherent ports.

Proposal 2: Four-port PTRS should be configured by higher layer parameters if a UE reporting its capability of ‘NonCoherent’ transmission with four antenna ports.

Proposal 3: For non-coherent UL transmission, each PT-RS port is associated with a transmission layer transmitted with a corresponding SRS port.

- References

3GPP TS38.214 v15.0.0.

Chairman’s notes, RAN1#90bis.

Chairman’s notes, RAN1#91.