3GPP TSG-RAN WG1 Meeting #93	 R1-1806433

Busan, Korea, 21st – 25th May, 2018	(Resubmission of R1-1805228)

Agenda Item:	7.8.2

Source:	Ericsson

Title:	IMT2020 self evaluations: On eMBB spectral efficiency with NR

Document for:	Discussion

# Introduction

It has been agreed within 3GPP that according to the New Study Item on Self Evaluation towards IMT-2020 Submission [1] self evaluation results should be provided against technical performance requirements for extreme mobile broadband (eMBB) as defined in report ITU-R M.[IMT-2020. TECH PERF REQ], including spectral efficiency [2].

The key minimum technical performance requirements defined by the ITU-R are intended for assuring that IMT-2020 technologies are able to fulfil the objectives of IMT-2020 and to set a specific level of performance that each of the proposed radio interface technologies needs to achieve in order to be considered by ITU-R for IMT-2020.

This contribution presents the preliminary results on spectral efficiency for eMBB and shows that the corresponding IMT-2020 requirements can be met.

# Requirements on the spectral efficiency

Among other technical requirements there are requirements on spectral efficiency (SE), which are set to be a three-times improvement in comparison to those of IMT-Advanced. SE is evaluated in terms of its average and 5th percentile values.

## Average spectral efficiency

Average SE is defined as the aggregate throughput of all users (the number of correctly received bits, i.e., the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bps/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio. Formally, it is given by

,

where Ri (T) is the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs; W denotes the channel bandwidth and T is the time over which the data bits are received.

The conditions for evaluation including carrier frequency and antenna configuration are described in [3] for of the specified each test environments [4], namely Indoor Hotspot (InH), Dense urban (DU) and Rural.

## 5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput, i.e., the number of correctly received bits (contained in the SDUs delivered to Layer 3), over a certain period of time, divided by the channel bandwidth and is measured in bps/Hz. The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Formally, the (normalized) user throughput of user i, ri, is defined according to

,

with Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth.

The conditions for evaluation including carrier frequency and antenna configuration are described in [3] for each test environment.

# Test environments and system setup

The evaluations for SE are performed by means of simulations in both downlink and uplink. An NR-like 5G prototype multi-user MIMO system has been simulated with explicit impaired CSI. The configurations are summarized in Tables 1 and 2 below.

Table 1 Selected configurations in downlink.

| CSI periodicity | 5 ms |
| --- | --- |
| CSI delay | 5 ms |
| CSI mode | Explicit CSI feedback |
| UE noise figure | 9 dB |
| Scheduling | Proportional fair, MU-MIMO |
| CSI-RS | Channel estimation error modeled |
| Max MU rank | 8 |
| Max SU rank | 8 |
| Interference estimation impairment model | Non-ideal estimation |
| Modulation | OFDM |
| Outer loop LA | Yes, 10% BLER target |
| Traffic model | Full buffer, 10 UE/cell |
| Handover margin | 3 dB |
| UE Antenna | 1x4 cross poles, 0.5λ separation, random orientation in xy-plane |
| BS Antenna | 4x32 cross poles, 0.8λ in vertical (0.5 λ for 5G InH), 0.5λ in horizontal |
| Modulation | OFDM (also in UL) |

Table 2 Selected configurations in uplink.

| Scheduling to transmission delay | 4 ms |
| --- | --- |
| ACK NACK processing delay | 2 ms |
| CSI mode | Ideal SRS |
| eNB noise figure | 5 dB |
| Modulation | OFDM |
| Max MU rank | 16 |
| Max SU rank | 2 |
| ULPC SNR target | 7 dB |
| ULPC M0 (# subcarriers for which target is reached) | 144 subcarriers |
| eNB receiver | IRC |
| Outer loop LA | Yes, 10% BLER target |
| Traffic model | Full buffer, 10 UE/cell |
| Handover margin | 3 dB |
| UE Antenna | 4x2 cross poles, 0.5λ separation, random orientation in xy-plane |
| BS Antenna | 4x32 cross poles, 0.8λ in vertical (0.5 λ for 5G InH), 0.5λ in horizontal |
| Modulation | OFDM (also in UL) |

Three test environments are considered are DU, Rural and InH scenarios. The simulations performed herein utilize similar scenarios to the ones defined in [4] with some slight modifications, according to Table 3. For instance, DU and Rural scenarios are simulated as layouts with wrap around of 13 base stations serving 3 sectors each, that is, 39 cells in total. The InH scenario is simulated with 8 cells with 20 m inter-site distance (ISD) corresponding to a moderate-sized venue.

Table 3 Scenario-specific parameters.

| 5G Dense Urban (macro level only) |  |
| --- | --- |
| Channel model | Urban Macro [4] |
| ISD | 200 m |
| UE speed | 3 km/h |
| Carrier frequency | 4 GHz |
| Wrapping | Radio distance based |
| Number of cells | 39 |
| Indoor outdoor | 80% indoor |
| Bandwidth | 20 MHz |
| 5G Rural |  |
| Channel model | ITU Rural Macro [5] |
| ISD | 1732 m |
| UE speed | 120 km/h |
| Carrier frequency | 4 GHz |
| Wrapping | Radio distance based |
| Number of cells | 39 |
| UE antenna gain | 0 dBi |
| Bandwidth | 20 MHz |
| 5G Indoor Hotspot |  |
| Channel model | Indoor Office [4] |
| ISD | 20 m |
| UE speed | 0.5 km/h |
| Carrier frequency | 30 GHz |
| Number of cells | 12 |
| UE antenna gain | 5 dBi |
| Bandwidth | 80 MHz |

# Simulation results

The results of system-level simulations for the given test environments are presented in Table 4 below.

Table 4 Simulation results vs. requirements.

| Scenario | Setup | Average SE [bps/Hz/TRxP] |  | 5 percentile SE [bps/Hz/TRxP] |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Simulated | Requirement | Simulated | Requirement |
| 5G Dense Urban | Downlink | 12.99 | 7.8 | 0.315 | 0.225 |
|  | Uplink | 7.53 | 5.4 | 0.169 | 0.15 |
| 5G Rural | Downlink | 18.5 | 3.3 | 0.453 | 0.12 |
|  | Uplink | 11.1 | 1.5 | 0.076 | 0.045 |
| 5G Indoor Hotspot | Downlink | 14.8 | 9 | 0.37 | 0.3 |
|  | Uplink | 10.54 | 6.75 | 0.323 | 0.2 |

As it can be seen from the table, for all the three scenarios the IMT-2020 requirements are fulfilled in terms of SE for both uplink and downlink. This holds for both average and 5th percentile SE.

IMT-2020 requirements on spectral efficiency can be fulfilled for the specified test environments for both uplink and downlink.

# Conclusion

In this contribution, we have discussed the IMT-2020 requirements on spectral efficiency, as well as provided our preliminary input to the Study Item on Self Evaluation towards IMT-2020 Submission. It has been shown that for both downlink and uplink the IMT-2020 requirements can be fulfilled in terms of both average and 5th percentile spectral efficiency for the test environments specified by ITU-R.

The following observations have been made:

Observation 1	IMT-2020 requirements on spectral efficiency can be fulfilled for the specified test environments for both uplink and downlink.

# References

- RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission,” RAN#75, Ericsson et al.

Report ITU-R M.2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s),” 2017.

Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020,” 2017.

TR 38.901 V14.3.0, “Study on channel model for frequencies from 0.5 to 100 GHz,” 2018.

Technical report ITU-R M.2135-1, “Guidelines for evaluation of radio interface technologies for IMT-Advanced,” ITU 2009.