3GPP TSG RAN WG1 Meeting #93                                    	    R1-1806459

Busan, Korea, May 21st – 25th, 2018

Title:                     Discussion on frame structure for NR-U

Source: 	ZTE

Agenda Item:	7.6.2

Document for:	Discussion and Decision

Introduction

The SID on NR-based Access to Unlicensed Spectrum [1] aims to identify and evaluate solutions and techniques for next generation wireless systems operating on unlicensed bands e.g. 5GHz, 60GHz. Besides, coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements should also be studied. The SID includes the following objectives:

SID RP-170828:

- Study NR-based operation in unlicensed spectrum (RAN1, RAN2, RAN4) including

  - Physical channels inheriting the choices of duplex mode, waveform, carrier bandwidth, subcarrier spacing, frame structure, and physical layer design made as part of the NR study and avoiding unnecessary divergence with decisions made in the NR WI

    - Consider unlicensed bands both below and above 6GHz, up to 52.6GHz

    - Consider unlicensed bands above 52.6GHz to the extent that waveform design principles remain unchanged with respect to below 52.6GHz bands

    - Consider similar forward compatibility principles made in the NR WI

  - Initial access, channel access. Scheduling/HARQ, and mobility including connected/inactive/idle mode operation and radio-link monitoring/failure

  - Coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements in e.g., 5GHz, 37GHz, 60GHz bands

    - Coexistence methods already defined for 5GHz band in LTE-based LAA context should be assumed as the baseline for 5GHz operation. Enhancements in 5GHz over these methods should not be precluded. NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier;

In 92bis meeting, the following agreement about frame structure and physical layer procedures for NR-U (NR-based access to Unlicensed Spectrum) has been reached.

Agreement:

- NR-U supports both Type-A and Type-B mapping already supported in NR

  - Additional starting positions and durations are not precluded

- For sub-7 GHz, NR-U study the SCSs, 15/30/60KHz

  - Study performance difference between different SCS

  - Study if changes to UL design are needed to meet the PSD and OCB requirements

  - Study if an SS block design/RMSI/OSI with 60KHz SCS is needed

  - Impact on MIB and SIB1 content

  - Need for use of ECP for 60KHz

  - RACH design with 60KHz SCS in addition to options currently part of NR

  - Other considerations are not precluded.

  - Impact on support of different BWs with different SCS

- Study supporting more than one switching points within a TxOP

  - FFS the LBT requirement for each DL/UL data/control burst in the TxOP

Agreement:

- Study possible enhancements for HARQ operation

- Study changes needed for Configured Grant support in NR-U

- Baseline for study: If absence of Wi-Fi cannot be guaranteed (e.g. by regulation) in the band (sub-7 GHz) where NR-U is operating, the NR-U operating bandwidth is an integer  multiple of 20MHz

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

- Study whether or not the following techniques enhance performance beyond the baseline LBT mechanisms

  - Techniques to cope with directional antennas/transmissions

  - Receiver assisted LBT : RTS/CTS type mechanism

    - On-demand receiver assisted LBT: For example receiver assisted LBT enabled only when needed

  - Techniques to enhance spatial reuse

  - Preamble detection

  - Enhancements to baseline LBT mechanisms above 7 GHz

- Note: LTE-LAA LBT mechanism are assumed as baseline for evaluations for 5GHz.

- Note: Other aspects are not precluded from being included

In this contribution, we discuss frame structure aspects for NR-U (NR-based access to Unlicensed Spectrum), such as supported different SCS, frame structure configuration, multiple slot durations and so on.

Frame structure

Discussion on SCS for NR-U

In licensed band, multiple subcarrier spacing (SCSs) can be selected to support NR transmission. For example, for NR sub-6G licensed band, 15/30/60 KHz SCS are supported, and for above-6G licensed bands, 60KHz and 120KHz can be selected for data transmission.

For sub-6G unlicensed bands, e.g., 5GHz, the 15KHz SCS with normal CP length is supported in LTE, and the same SCS could also be supported in NR unlicensed access to achieve similar coverage and performance as LTE. Meanwhile, if a larger SCS is applied, the symbol duration will be shorter and some benefits could be achieved for operation on unlicensed bands. For example, a larger SCS can facilitate the single wideband carrier operation and the shortened symbol duration is beneficial to enhance unlicensed spectrum utilization and reduce transmission latency. Hence, multiple SCSs as in licensed band should be supported on unlicensed bands besides 15KHz. Furthermore, for NR on 6-7 GHz unlicensed bands, similar SCSs as sub-6G can be supported.

For above-7G unlicensed bands, e.g., 60GHz, the available bandwidth is very large. The WiFi systems use 2.16 GHz per channel. Accordingly, for coexistence with such system, SCSs larger than those for licensed bands below 52.6GHz, e.g., 240/480/960 KHz or even larger SCS may be considered to extend the bandwidth of single carrier. However, further study should be considered together with other aspects such as LBT schemes.

For NR, the SCS for each channel or signal is configured through BWP IE configuration, and the subcarrier spacing in the BWP IE is to be used for all channels and reference signals unless explicitly configured elsewhere. And the reference SCS of the slot structure configuration is included in SIB1. Besides, the SCS for SIB1, Msg.2/4 for initial access and broadcast SI-messages is included in the MIB. For NR-U, these SCS configuration schemes can be reused directly or supported.

Observation 1:  There is no impact on MIB and SIB1 content if 15/30/60KHz SCS is supported. And the SCS configuration for each channel and signal can reuse the existing mechanics for NR-U.

MCOT frame structure/slot configuration

In licensed band, NR supports dynamic and semi-statically configured UL/DL configuration or slot configuration and the configuration period can be {1, 2, 4, 5, 10, 20} ms.  The semi-static DL/UL assignment is done via cell-specific and UE-specific RRC configuration. With the cell-specific configuration, the UE is allocated a DL-unknown-UL pattern. The “unknown” resources are flexible resources that can be assigned DL or UL direction using UE-specific configuration or dynamic signaling. The UE-specific RRC signaling or dynamic signaling includes the indication as per slot basis that can override the “unknown” allocation in the cell-specific configuration. Resources in the slot configuration that are without DL/UL indication remain flexible resources.

For NR-U, dynamic and semi-statically configured UL/DL configuration as in licensed band can also be considered. For example, some semi-static UL/DL assignments can be used to indicate DL slots for UE to receive SSB, and some other slots can be assigned for UE to transmit PRACH or configured grant transmission. If gNB performs LBT successfully on the semi-statically configured DL slot, gNB can override the unknown resource through DCI format 2_0 dynamically.

Besides, the frame structure design in NR-U should be flexible and efficient enough to make use of the MCOT. For example, more than one bi-directional slot or DL-to-UL switch points should be included in the MCOT to reduce the scheduling and feedback delay.  However, more DL-to-UL switch points means more gap to perform LBT upon every change of transmission direction and the resource may be wasted or the channel may be lost during this time. Therefore the trade-off between fast direction switching and overhead should be further studied in possible use cases. Furthermore, as the MCOT duration is different for different channel access priority and area regulation, we can define different maximum number of DL-to-UL switch point for different MCOT duration.

Proposal 1:

- NR-U should support both dynamic and semi-static DL/UL configurations.

- The maximum number of DL-to-UL switch points within the MCOT could be limited and can be different for different MCOT duration.

Multiple slot durations in an MCOT

In Rel-13 LAA, the eNB can only start transmission at slot boundaries (symbol 0/7) with 15KHz subcarrier spacing. The reservation signal is transmitted before the DL transmission start position, i.e., the slot boundary, to reserve the carrier and prevent other devices from occupying the spectrum, which is the overhead of LAA system. Consequently, the overhead of reservation signal should be reduced to a minimum.

Different NR-U slot types including slot and mini-slot (non-slot-based scheduling) can be utilized to minimize the reservation signal length. The NR gNB or UE should be able to send DL/UL TX burst as soon as possible whenever it finishes LBT/CCA, resulting in different slot duration. As illustrated in Figure 1, the gNB can choose the type of first slot i.e. mini-slot based on LBT/CCA success location and slot boundary, and then select the type of following slots with a larger size e.g. slot. The type of last slot(s) is/are chosen according to the remaining time within the MCOT. In Figure 1, the rest of the maximum COT is not enough for a normal slot, but enough for two whole mini-slots.

Figure 1: multiple slot durations in an MCOT

For DL, as CORESET can be configured to start from any symbol, gNB can start downlink transmission from the nearest symbol after a successful LBT so that reservation signal overhead can be minimized. However, it will introduce blind detection complexity to UE. Therefore, NR-U should design an efficient mechanism for a UE to detect the beginning of a downlink transmission. For UL, NR support non-slot based PUSCH scheduling and the start symbol is dynamically DCI indicated. Therefore, the UE can transmit PUSCH starting from the indicated symbol. However, this position may not well matched with the nearest symbol where LBT succeeds. So more reservation signals may be needed for UL. As NR already supports multiple start positions and durations, it can provide enough transmission flexibility for NR-U as well. Therefore, we think additional start points and durations are not necessary. Further enhancements can be considered to better adapt to the characteristics of NR-U.

Proposal 2: Multiple slot durations within a MCOT can be considered to minimize overhead of reservation signal and improve spectrum efficiency.

Proposal 3: Existing start positions and durations for NR can provide enough transmission flexibility for NR-U. Further enhancements can be considered to better adapt to the characteristics of NR-U.

Flexible BWP transmission

In licensed band, NR support transmissions with wide bandwidth, such as 100 MHz or 200 MHz. As we know, BWP aspect was introduced in NR and resource allocation is based on BWP. And in the 92bis meeting, it has agreed that NR-U operating bandwidth is an integer multiple of 20MHz and LBT can be performed in units of 20 MHz. For DL, gNB can perform LBT on multiple of 20MHz in a carrier and determine the activated BWP index based on its LBT outcomes.  For UL, it is also beneficial for NR-U to support UL transmissions with flexible BWP. For example, gNB can configure or allocate multiple candidate BWPs for UL transmission, and the UEs can decide their transmission BWP based on LBT outcomes.

Proposal 4: The activated BWP index can be selected based on LBT results.

Conclusion

In this contribution, we discuss the frame structure aspects for NR operation in unlicensed spectrum, and have the following proposals:

Observation 1:  There is no impact on MIB and SIB1 content if 15/30/60KHz SCS is supported. And the SCS configuration for each channel and signal can reuse the existing mechanics for NR-U.

Proposal 1:

- NR-U should support both dynamic and semi-static DL/UL configurations.

- The number of DL-to-UL switch points within the MCOT could be limited and can be different from MCOT duration.

Proposal 2: Multiple slot durations within a MCOT can be considered to minimize overhead of reservation signal and improve spectrum efficiency.

Proposal 3: Existing start positions and durations for NR can provide enough transmission flexibility for NR-U. Further enhancements can be considered to better adapt to the characteristics of NR-U.

Proposal 4: The activated BWP index can be selected based on LBT results.

Reference

- RP-170828, New SID on NR-based Access to Unlicensed Spectrum, Qualcomm, March 6 - 9, 2017, RAN#78

- R1-1803952, Design of Channel access mechanism for NR-U, ZTE, Sanechips