3GPP TSG RAN1 Meeting #93                                                                    R1-1806473

Busan, Korea, 21st May – 25th May 2018

Source:	Thales

Title:	NR-NTN: Proposed recommendations

TDOC Type:	pCR

Agenda Item:	7.3.2

Document for:	Discussion

Release:	Rel-15

Specification:	38.811 Study on NR to support non-terrestrial networks

# Abstract

This document proposes a draft text for the clause “recommendations on the way forward” of the 3GPP TR 38.811.

# Proposed Text

We suggest adding section 8, with the text below, in order to briefly describe the complexity and the constraints:

* * * Start of changes * * * * (new text)

# 8	Recommendations on the way forward

## 8.1	General outcomes

This report has

- Provided detailed descriptions of some deployment scenarios for non-terrestrial networks and the related system parameters such as altitude, orbit etc.

- Defined a way to generate Non-Terrestrial network channel models leveraging on the models defined in TR 38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 14)”

- Identified potential key impact areas on the NR to support Non-Terrestrial networks

In line with 3GPP TR 38.901, the study considered a possible deployment of NR in Non-Terrestrial networks operating in any frequency bands between 0.5 and 100 GHz.

Specific frequency bands (S and Ka band) have nevertheless been considered for the channel modelling and the identification of potential key impact areas on the NR.

## 8.2	Reference deployment scenarios

A number of scenarios is considered in clause 5.1 “Scenarios overview” including GEO, Non GEO as well as HAPS based access networks. The intent is assess relatively worst case scenarios with respect to Doppler shift, Doppler variation rate, Delay and Delay variation rate. For concerns  non-GEO deployment scenarios, the has been set to 600 km , which is pessimistic with respect to the Doppler, and Doppler rate, related effects.

Non-Terrestrial access networks can be used to serve directly User Equipment or indirectly via a relay node.

Other deployment scenarios based on space or aerial vehicles at different altitude, orbit may be considered in subsequent studies on NR supporting Non-Terrestrial networks.

## 8.3	Non-Terrestrial network channel modelling

Leveraging on the models defined in TR 38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 14)”, complementary features and specific parameters have been identified and proposed in chapter 6 of this report to generate Non-Terrestrial network channel models for all possible deployment scenarios characterised by altitude/orbit of space/aerial vehicles, user environments (rural, sub-urban, urban and dense urban), targeted user equipment type (omni or directive antennas, LOS/NLOS conditions) as well as frequency bands.

## 8.3	NR impacts to support Non-Terrestrial networks

8.3.1 Type of potential NR impacts

The potential NR impacts are characterised according to the classification in the table below:

Table 8.3-1: Type of NR impacts to support Non-Terrestrial networks

| Type of NR impacts | Modifications of 3GPP specifications |
| --- | --- |
| Higher layers impact | Required in Layer 2/3 protocols/architecture |
| Physical layer impact | Required in physical layer key parameters |
| No impact | Not needed |
| Performance enhancement | Preferable |

8.3.2 Assessment of potential NR impacts to support Non-Terrestrial networks

The table below identifies the potential key impact areas on the NR. The intent is to identify the impacts mandatory to enable operation of NR in Non-Terrestrial networks. Other impacts which could lead to performance improvement and could be addressed in Release 17 and beyond.

Table 8.3-2: Evaluation of NR impacts to support Non-Terrestrial networks

| Non-Terrestrial network specifics | Effects | Impacted NR features | Type of impact | Comment | Impacted WGs |
| --- | --- | --- | --- | --- | --- |
| Motion of the space/aerial vehicles (especially for Non GEO based access network) | Moving cell pattern | Hand-over/paging | Higher layers impact | Paging and Hand-over procedures should be adapted to take into account the relative motion of the cell pattern with respect to the tracking area. Further analysis on tracking area design may need to be carried out. Mobility management in case of relay is also to be adapted (Note 2) | RAN2/3 |
|  | Delay variation | TA adjustment | No impact | However the frequency of the TA adjustment command may need to be increased | RAN1 |
|  | Doppler | Init synchro downlink | No impact | The preferred SCS values for Non-Terrestrial Networks are respectively 60 KHz for frequency bands lower than 6 GHz and 240 KHz for frequency bands above 6 GHz. However it can also operate with lower SCS value | RAN1 |
|  |  | DMRS time | No impact | The prefered DM-RS configuration is type 1 to cope with Doppler variation rate | RAN1 |
| Altitude | Long latency | HARQ | No impact | It is assumed that HARQ can be deactivated when operating the radio link at a sufficiently low BLER | RAN1 |
|  |  | Control loops (ACM, power control…), timers | A priori no impact (For Further Study) | The open/closed operation/configuration of Adaptive power and coding/modulation control loop protocols may have to be adapted. | RAN1 |
|  |  |  | Higher layer impact | Timers limit of MAC/RLC loop protocols may have to be extended | RAN2 |
| Cell size | Differential delay | TA in Random access response message | A priori no impact (For Further Study) | It is assumed that a Doppler/Delay compensation technique (Note 1) is implemented and its accuracy is sufficient. This needs confirmation with further analysis/simulations using the channel model defined. | RAN1 |
|  |  | RACH | A priori no impact (For Further Study) |  | RAN1 |
| Propagation channel | Impairments | DMRS frequency | No impact | Non terrestrial network propagation channel features a frequency selective at most comparable with cellular channel | RAN1 |
|  | Impairments | Cyclic prefix | No impact | Non terrestrial network propagation channel features a worst delay spread at most comparable to cellular channels. | RAN1 |
| Spectrum | Regulatory constraints | Access scheme (TDD/FDD) | Higher layers impact | FDD is prefered especially for most spectrum allocation to satellite systems. This may require possible adaptations of the Frequency band definition, channel numbering and spacing aspects depending on the regulatory regime applying to the targeted frequency bands | RAN4 |
| Satellite or aerial Payload performance | phase noise impairment | PT-RS | No impact | Radio links are typically operated with relatively low order modulation scheme, in most of the cases up to 16QAM resulting a negligeable degradation on the link budget | RAN1 |
|  | Back-off | PAPR | No impact (not mandatory in the short term) | It is recommended to use the DFT-s modulation on the uplink. Low PAPR modulation scheme can be defined in future releases especially on the downlink to improve performance | RAN1 |
| New network type | Specific Network identity | Network discovery | Higher layers impact | The Network discovery procedure may need to be modified to take into account Non-Terrestrial network identities | RAN3 |

Note 1: Doppler/Delay compensation techniques can be implemented especially for Non GEO satellites:

- GNSS based techniques: The User Equipment equipped with a GNSS receiver determines its position and the universal time. Thanks to pre-loaded / updated ephemeris of the satellite constellation which can be theoretical or actual, the UE is able to compute the position and motion of the possible serving satellites enabling to determine the Doppler shift and variation rate as well as the absolute Delay and Delay variation.

- Non GNSS based techniques can also be envisaged and implemented. They are for further study

Note 2: Some of the considered deployment scenarios assume indirect access based on fixed or even mobile relay which are yet to be defined in 3GPP for NR. This needs to be studied starting from the appropriate conclusions of the “Integrated Access and Backhaul” Study Item.

For NGSO-based deployment scenarios NTN and/or relays mounted on a moving platform such as a train, mobility management also needs further studies.

## 8.4	Next steps

Subsequent studies are necessary to further characterise some potential impacts on NR and define solutions to address the remaining issues.

The study may take place over consecutive phases during which different sets of deployment scenarios may be considered.

Table 8.4-1: Subsequent RAN studies on NR support Non-Terrestrial networks

| Targeted deployment scenarios | - LEO satellite based access - HAPS based access - GEO based access |
| --- | --- |
| RAN1 – physical layer features | Mitigating Doppler/Delay, potential areas of impact (to be confirmed through analysis): - Physical layer control procedures (e.g. CSI feedback, power control) - Uplink Timing advance/RACH procedure including PRACH sequence/format/message - deactivation of HARQ Improving performance, potential areas of impact: - MCS and TBS configuration and enhancements for low SNR and/or enhancements ensuring low PAPR - Extended HARQ operation Performance assessment of NR in the selected deployment scenarios |
| RAN2 features | Mitigating Doppler/Delay, potential areas of impact: - Timers of MAC/RLC control loops - random access response, TA parameters, Scheduling request, buffer status reporting, Power Headroom reporting, DRX Coping with moving cells, potential areas of impact: - Paging/hand-over procedure for UE or relay Improving performance, potential areas of impact: - 2 steps RACH |
| RAN3 features | Coping with moving cells, potential areas of impact: - RAN architecture (e.g. handling of network identities, Mobility management) Improving performance, potential areas of impact: - Network slicing in RAN incl. satellite Satellite/cellular Link aggregation |
| RAN4 features | Supporting selected satellite frequency bands, impact: - Frequency band definition, channel numbering and spacing aspects |

* * * End of changes * * * *