3GPP TSG RAN WG1 Meeting #93  	R1-1806741

# Busan, Korea, May 21st – 25th, 2018

Agenda item:	7.1.4

Source:	Samsung

Title: 				Remaining Issues on Channel Coding

Document for:		Discussion and Decision

1. Introduction

NR channel coding schemes have been almost finalized, but there are still some remaining issues. In this contribution, we consider two remaining issues on small block length coding (repetition, simplex, and RM) and polar coding. First, we identify that there are ambiguities in determining the small block length channel coding schemes, and a method for avoiding the ambiguity is proposed. Second, we show that there is a circular dependency between the rate-matching output size and the number of CRC bits, and we propose to fix the number of CRC bits to 11 in order to remove the circular dependency.

2. Small Block Length Channel Coding for PUCCH

In [1], small block lengths channel coding for PUCCH is not clear even though it can be happened that information length is less than 2 bits for PUCCH. For example, for 2 CSI-RS ports, if wide-band (WB) PMI and sub-band (SB) CQI are configured, then the CSI-part2 is 2 bits (for rank 1) or 1 bit (for rank 2). First, channel coding of small block lengths is defined in 5.3.3.1 as below, but the placeholders “x” and “y” is defined only for Subclause 6.3.1.1 of [4, TS 38.211], which is for PUSCH.

5.3.3.1 Encoding of 1-bit information For , the code block is encoded according to Table 5.3.3.1-1, where and is the modulation order for the code block. Table 5.3.3.1-1: Encoding of 1-bit information Encoded bits. 1. 2. 4. 6. 8. The "x" and "y" in Table 5.3.3.1-1 are placeholders for Subclause 6.3.1.1 of [4, TS 38.211] to scramble the information bits in a way that maximizes the Euclidean distance of the modulation symbols carrying the information bits.

Second, the small block length channel coding schemes for PUCCH is defined only for the case that information bit length is larger than or equal to three bits since they are made for PUCCH format 2/3/4.

6.3.1 Uplink control information on PUCCH The procedure in this subclause applies to PUCCH formats 2/3/4. … 6.3.1.3.2 UCI encoded by channel coding of small block lengths Information bits are delivered to the channel coding block. They are denoted by , where is the number of bits. The information bits are encoded according to Subclause 5.3.3.

Therefore, we propose the following alternatives in order to solve this issue.

- Alt 1: The number of bits of CSI part 2 should be larger than 3.

- Alt 2: If the number of bits in a CSI part 2 is less than 3 bits, zeros shall be appended to the CSI part 2 sequence until the payload size equals 3.

It has not been discussed in MIMO session to consider sub-band CSI in partial reporting, so it is natural to exclude the case of 1-bit and 2-bit payload in CSI-part2. Thus, Alt 1 is more reasonable and preferable.

Proposal 1: The number of bits of CSI-part2 should be larger than 3.

3. UCI Size and Polar Code Segmentation

In [1], the following agreement on the polar code segmentation for large UCI is captured.

Agreement (RAN1 #91): Segmentation is applied when K >= 360 and M >= 1088 where - K is UCI payload size without CRC - M is the total number of coded bits for the UCI payload Agreement (RAN1 #92bis): Polar code segmentation is applied if ( and ) or A >= 1013 - Editor to capture this in Section 6.3.1.2.1 of TS 38.212

The rate-matching output size for UCI is determined based on Table 1 [1].

Table 1  Rate matching output sequence length EUCI described in Table 6.3.1.4.1-1 [1]

| UCI(s) for transmission on a PUCCH | UCI for encoding | Value of |
| --- | --- | --- |
| HARQ-ACK | HARQ-ACK |  |
| HARQ-ACK, SR | HARQ-ACK, SR |  |
| CSI (CSI not of two parts) | CSI |  |
| HARQ-ACK, CSI (CSI not of two parts) | HARQ-ACK, CSI |  |
| HARQ-ACK, SR, CSI (CSI not of two parts) | HARQ-ACK, SR, CSI |  |
| CSI (CSI of two parts) | CSI part 1 |  |
|  | CSI part 2 |  |
| HARQ-ACK, CSI (CSI of two parts) | HARQ-ACK, CSI part 1 |  |
|  | CSI part 2 |  |
| HARQ-ACK, SR, CSI (CSI of two parts) | HARQ-ACK, SR, CSI part 1 |  |
|  | CSI part 2 |  |

As pointed out in [2], there is a circular dependency in determining the rate-matching output size EUCI and the number of CRC bits L. As shown in Table 1, EUCI is determined based on L. However, EUCI is also exploited to determine the code block segmentation and L. If (A≥360 and EUCI≥1088) or A≥1013, segmentation into two segments is applied and L=22. Otherwise, a single code block is used and L=11.

A simple approach to handle this issue of the circular dependency is to fix L regardless of segmentation. We have two options as follows:

- Alt1: L is always assumed to be 11 if CA-polar codes are used (A≥20).

- Alt2: L is always assumed to be 22 if CA-polar codes are used (A≥20).

Figure 1  Desired and practical code rates when L is fixed in determination of EUCI

Fig. 1 shows desired code rates and resultant code rates when we fix L in determination of EUCI. The maximum code rate is 0.8, and QPSK modulation is assumed. If we fixed L=11, then the maximum difference between desired and resultant code rates is about 0.01, while the maximum gap is larger than 0.2 if we fix L=22. The impact caused by correcting errors should be minimized since Release-15 is almost finalized, it is more reasonable to fix L to 11.

Proposal 1: In determining the rate matching output size EUCI, the number of CRC bits L should be assumed to be 11 regardless of segmentation if payload size (HARQ-ACK, SR, CSI-part1) is greater than or equal to 20.

4. Summary

In this contribution, we proposed solutions to avoid ambiguity in NR channel coding. Proposals in this contribution are summarized as follows:

Proposal 1: The number of bits of CSI-part2 should be larger than 3.

Proposal 2: In determining the rate matching output size EUCI, the number of CRC bits L should be assumed to be 11 regardless of segmentation if payload size (HARQ-ACK, SR, CSI-part1) is greater than or equal to 20.

References

- 3GPP TS 38.212 V15.1.1 (2018-04), 3GPP TSG RAN NR multiplexing and channel coding (release 15).

- R1-1804882, Sharp, “Discussion on RB selection for PUCCH resource,” 3GPP TSG RAN WG1 #92bis, Sanya, China, 16-20, April, 2018.