3GPP TSG RAN WG1 Meeting #93                                              R1-1806771

Busan, Korea, May 21st – 25th, 2018

Agenda item:	7.1.1.4.2

Source: 	MediaTek Inc.

Title: 	On remaining issues of RACH procedure

Document for:	Discussion and decision

# Introduction

In this paper, we discuss some remaining issues in the RACH procedure. We first discuss when is the proper timing to adjust TA timing when UE receives a TA adjustment command. In the following section, we discuss whether a BWP index should be included in a PDCCH order. Then we share our views on the RACH search space. Finally the QCL assumption in RACH procedure is discussed especially for RRC connected UEs.

# TA adjustment timing

Working assumption: (RAN1 #92bis) For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where - N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 - N2 also refers to the value determined in control session with UE capability #1 - L2 refers to the MAC processing latency and is equal to 0.5 msec. - TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS - Note: Spec captures the value k for different SCS - UE is not expected that UL transmissions in the same slot shall apply different TA values - If this happens, it is up to UE’s implementation. - FFS: the reference SCS of slot n+k and the time duration of (k-1) slots when UL and DL have different SCSs.

In the above working assumption agreed at RAN1 #92bis meeting, there are two issues to be further clarified. The first issue is the reference SCS(s) when UL and DL have different sub-carrier spacings. And the other issue is that the definition of TA_max when UL carriers belonging to the same TAG have different SCSs.

According to the agreements in the control session, when downlink and uplink have different SCSs, the N1 and N2 values are determined by the minimum SCS of DL and UL SCSs. In a similar way, we think that the reference SCS of N1 shall be determined by the minimum SCS of the subcarrier spacing of the downlink carrier with which the TA command is transmitted and the subcarrier spacing of the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted. As to the reference SCS for N2, TA_max and slot_duration, in order to apply the updated TA value to all uplink carriers in a TAG at the same time, we propose that the reference SCS is determined by the minimum SCS of the uplink carriers in the TAG.

Based on the above analysis and for the sake of simplicity, we propose to confirm the working assumption with the some modification highlighted in red color as follows.

Proposal 1: For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

- N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 and is determined by the reference SCS

- N2 also refers to the value determined in control session with UE capability #1 and is determined by the reference SCS

- L2 refers to the MAC processing latency and is equal to 0.5 msec.

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS  the reference SCS

- Note: Spec captures the value k for different reference SCS

- UE is not expected that UL transmissions in the same slot shall apply different TA values

  - If this happens, it is up to UE’s implementation.

- The reference SCS is the minimum SCS of the subcarrier spacing of the downlink carrier with which the TA command is transmitted and the subcarrier spacings of the uplink carriers belonging to the same TA group (TAG).

  - Slot n, slot n+k, the time duration of (k-1) slots, and slot_duration are based on the reference SCS.

# DCI design for PDCCH order

At the RAN1 AH1801 meeting, RAN1 received an LS from RAN2 asking about what fields to be included in a PDCCH order to trigger a random access procedure [1]. At RAN1 #92bis meeting, RAN1 reached an agreement to include the following fields in a PDCCH order. Whether to include a BWP index becomes a remaining issue.

Agreements: (RAN1 #92bis) - PDCCH order for RACH procedure includes the following fields: - Random Access Preamble index – 6 bits. Indicating which Random access preamble to use in case of contention-free random access procedure, or the value 000000 in case of contention-based random access procedure - For CFRA only: - FFS BWP index. Indicating which BWP to transmit the Random access preamble on - SUL indicator – 1 bit. Indicating whether to transmit the Random access preamble on SUL or normal uplink carrier - SSB index - 6 bits are used to indicate an SSB index, which, in turn, identifies a group of RACH occasions - RACH occasion index - 3 bits are used to indicate the relative RACH occasion index that corresponds to the indicated SSB index identified group of RACH occasions

Section 15: Bandwidth part operation in TS38.321 [2] Upon initiation of the Random Access procedure, the MAC entity shall: 1> if PRACH resources are configured for the active UL BWP: 2> perform the Random Access procedure on the active DL BWP and UL BWP. 1> else (i.e. PRACH resources are not configured for the active UL BWP): 2> switch to initial DL BWP and UL BWP; 2> perform the Random Access procedure on the initial DL BWP and UL BWP.

Agreements (RAN2 #101) => For FDD and CBRA, PRACH configuration/resources are linked with DL BWPs (implicitly or explicitly). The UE only monitors RAR on DL BWPs that are linked to the used PRACH resources => Working assumption: UL BWP k is linked with DL BWP k. If the UE intends to transmit preamble on UL BWP k, then the active DL BWP has to be DL BWP k. ASN.1 signalling supports this

Based on the description of BWP operation in TS 38.321, it is clear that UE can switch to the initial active UL and DL BWPs for a RACH procedure if there is no RACH resources in its current active UL BWP. Therefore, we do not think it is necessary to include the BWP index in the PDCCH order. However, if the BWP index is indeed to be included, we think it should be used only for contention-free random access. For CBRA in FDD, RAN2 has agreed there is a linkage between PRACH resources and DL BWPs. If the BWP index is included in the PDCCH order, it implies in FDD UE has to change only UL BWP for CFRA while it has to change both UL and DL BWPs for CBRA. It is preferred if the UE behaviour after receiving a PDCCH order can be as unified as possible.

Proposal 2: NR selects one of the following proposals:

- Alt. 1: PDCCH order does not include a BWP index.

- Alt. 2: PDCCH order includes a BWP index for CFRA.

  - The number of bits for the BWP index is 2.

# Msg3 coverage

According to RAN2’s LS [3], the Msg3 size being considered in NR is 56 bits and 72 bits for RRC Connection Request and for RRC Connection Resume Request, respectively. However, based on the RAN1’s LS to RAN2 [4] , Msg.3 TB size larger than 56 bits for NR has a risk of reducing the coverage of Msg.3 PUSCH compared to LTE Msg.3 PUSCH. It is important for NR to support the same coverage as LTE especially in the beginning of NR deployment stage. Therefore, we would like discuss some possible solutions to this problem from RAN1’s perspective.

A first solution is to use the existing HARQ mechanism. Similar to other PUSCH transmissions, the HARQ mechanism is applied to Msg3. If Msg3 cannot be detected successfully, the network schedules another uplink grant to the UE for Msg3 retransmission. This solution causes no impact on specification at the cost of latency increase.

Another solution is to introduce the cross-slot aggregation function to Msg3 transmission. In NR, it has been agreed that slot aggregation in uplink can be configured by the UE specific RRC parameter pusch-AggregationFactor (enum{n2, n4, n8}). However, since it is configured by UE specific signaling, it is not applicable to PUSCH transmissions before RRC connection such as Msg3, unless some modification is introduced to the specification. For example, one-bit signaling can be used to indicate whether time-domain repetition is enabled or not for Msg3 transmission. If it is enabled, then the corresponding UE transmits Msg3 for two slots in a row. Otherwise, UE transmits Msg3 in the slot according to the time-domain resource allocation in the UL grant in MAC RAR.  In summary, to resolve the Msg3 coverage issue, we have the following proposal.

Proposal 3: RAN1 selects one of the following to enhance Msg3 coverage and sends LS to RAN2

- Alt. 1: Msg3 coverage is enhanced by the existing HARQ-ARQ mechanism

- Alt. 2: Msg3 coverage is enhanced by cross-slot repetitions

# RRC Connected: QCL assumption

At previous RAN1 meetings, we had made the following agreements on SSB selection principle for Msg1 transmission and quasi co-located (QCL) assumption for RACH.

Agreements: (RAN1 #90) - It is up to UE implementation how to select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy threshold(s) - If UE does not detect a SS block that satisfy threshold(s), it has the flexibility to select any SS block that allows UE to meet the target received power of the RACH preamble with its maximum transmit power - UE has a flexibility to select its RX beam to find the list of SS blocks that satisfy the threshold(s) - FFS: whether threshold(s) for SS block selection is configured or fixed in the spec - Counter of power ramping when UE changes its selected SS-block in message 1 re-transmission is unchanged

Agreements: (RAN1 #90bis) - UE may assume that the DMRS of PDCCH and the DMRS of PDSCH conveying Msg2 are QCL'ed with the SS block that the UE selected for RACH association and transmission - UE may assume that the DMRS of PDCCH conveying Msg3 retransmission grant is QCLed with the SS block that the UE selected for RACH association and transmission - If there is no beam reporting in RACH message 3, UE may assume that the DMRS of PDCCH and the DMRS of PDSCH conveying Msg4 are QCL’ed with the SS block that the UE selected for RACH association and transmission - FFS: Whether to support beam reporting in Msg3 - FFS: If and how beam reporting in RACH message 3 impacts message 4 Tx QCL assumption

However, in RRC_CONNECTED mode, UE can be configured with additional DL BWPs that can support full finalities of common search space for RMSI, OSI, paging and random access.

Agreements: (RAN1 #91) - C-SS in each DL BWP of the PCell/PScell - On C-SS, Yp ,kp= 0. - In Rel.15, - For scheduling RMSI, OSI, Paging, UE monitors common search space in the PCell only - In addition, for random access and fall back, UE monitors common search space in the PCell and PSCell only - Working assumption: The UE is not expected to be configured without C-SS on the PCell (PSCell) in the active DL BWP - NOTE: RAN1 does not expect additional impact on the UE behavior due to not having PRACH resource in the BWP - Working assumption: In Rel.15, - A UE is expected to monitor C-SS (if configured) in the activated BWP - Full functionalities of C-SS (scheduling RMSI, OSI, Paging, random access, etc) are supported by the C-SS configured by UE-specific RRC signaling. - All RRC parameters defined for UE-SS are also defined for C-SS that is configured by UE-specific RRC signaling.

Section 10.1 of TS 38.213 Error! Reference source not found. If a UE is configured for downlink bandwidth part (BWP) operation, the above configurations for the common search spaces apply for the initial active DL BWP. The UE can be additionally configured a control resource set for Type0-PDCCH common search space, Type0A-PDCCH common search space, Type1-PDCCH common search space, or Type2-PDCCH common search space for each configured DL BWP on the primary cell, other than the initial active DL BWP.

According to the above agreements in control session and statements in TS38.213, in RRC_CONNECTED mode, Type0, Type0A, Type1, and Type2-PDCCH C-SS may be all associated with the same CORESET. Hence they are associated with the same TCI-State and have the same QCL assumption. An example is illustrate in Figure 1. With all the information about CORESET and search space configuration for UE in connected, now, we take a second look at the SSB selection principle for Msg1 transmission that was agreed in RAN1 #90 meeting. If the SSB selection principle still applies to Msg1 transmission in connected mode contention-based random access (CBRA), it implies that Type-1 PDCCH C-SS may have different QCL assumption than other C-SSs. Then during the time duration when UE monitors Msg2/Msg3 reTx scheduling/Msg4, what QCL association should UE assume for the CORESET associated with common search space? And how can UE monitors all the different types of PDCCH C-SS at the same time? We hence have the following two proposals to be down selected from.

Figure 1: In connected mode, Type0, Type0A, Type1, and Type2-PDCCH C-SS may be all associated with the same CORESET and the same transmission configuration indication state (TCI-State).

Proposal 4: NR should discuss how UE selects RACH preamble indices/resources for contention-based random access in RRC_CONNECTED mode.

Proposal 5: When UE conducts CBRA in RRC connected mode, UE selects RACH preamble indices/resources that are associated with the downlink reference signal that QCL’ed with the DM-RS antenna port(s) associated with PDCCH reception in common search space(s) where UE is expected to monitor the random access response.

Proposal 6: If the SSB that UE selects for RACH association and Msg1 (re)transmission is not quasi-co-located (QCLed) with the DM-RS antenna port associated with PDCCH reception in the Type0-PDCCH common search space, the Type0A-PDCCH common search space, the Type2-PDCCH common search space, or Type3-PDCCH common search space, UE is not expected to decode PDSCH indicated by a PDCCH scrambled by an RNTI other than RA-RNTI if they are assigned in the same time instance.

# Configurations of RACH RRC parameters

Agreements: (RAN1 NR AH1801) - For the separate configuration of the number of PRACH transmission occasions FDMed in one time instance for initial access, support: - The value range of the 2-bit RRC parameter prach-FDM is {1,2,4,8}

Agreements: (RAN1 #92) - In RA-RNTI equation (RA-RNTI= 1 + s_id + 14*t_id + 14*X*f_id + 14*X*Y*ul_carrier_id)), - Value of X and Y are 80 and 8 respectively.

To make the specification more complete, we propose the following for the RRC parameter prach-FDM.

Proposal 7: NR supports the value range of the 2-bit RRC parameter prach-FDM is {1, 2, 4, 8} for other RACH use cases besides initial access.

## RACH configuration

Agreements: - Support cyclically mapping the actually transmitted SS/PBCH blocks and ROs within a period - FFS the definition of the period - FFS whether or not there a case of ROs that are not enough for a complete association with all the actually transmitted SS/PBCH blocks and if so, how to handle Agreements: - For the cyclic mapping of association between ROs and all the actually transmitted SS/PBCH blocks, if there are leftover ROs after an integer number of cycles within the defined period, - no SS/PBCH blocks are mapped to these leftover ROs Agreements: - NR supports the following number of SSBs per RACH occasion: {1/8, 1/4, 1/2, 1, 2, 4, 8, 16} - Note: This agreement includes the possibility that FDMed RACH occasions are configured to support configurations where all FDMed RACH occasions get mapped to one set of SSBs Agreements: - For SSB-per-rach-occasion {1/8, 1/4, ½, 1, 2}: CB-preambles-per-SSB is selected from the set of values 4*N, with N=1, …, floor(16/max(1, SSB-per-rach-occasion)) - For SSB-per-rach-occasion {4, 8, 16}: CB-preambles-per-SSB is selected from the set of values N, with N=1, …, floor(64/SSB-per-rach-occasion)

The fractional numbers among the supported numbers of SSBs per RACH occasion, i.e. 1/8, 1/4, and 1/2, were agreed with the underlying assumed scenario that all FDMed RACH occasions were associated with one SSB when analog beamforming was applied which is illustrated in Figure 2. However, when we enumerate all possible combinations of SSB-per-RO and prach-FDM, we notice that when the inverse of SSB-per-RO is larger than prach-FDM, each SSB is mapped to RACH occasion at more than one time instance at a time. An example of this type of configuration is illustrated in Figure 3. This type of configurations should not be supported because the same density of RACH occasions for each SSB can be achieved repeating the mapping pattern in Figure 2 more frequently.

Proposal 8: NR does not support configurations such that1SSB-per-RACH-occasion> prach-FDM.

Figure 2: Each SSB is mapped to all FDMed RACH occasion at one time instance at a time when the inverse of SSB-per-RO is same as prach-FDM. This type of configurations can be supported.

Figure 3: Each SSB is mapped to all FDMed RACH occasions at two consecutive time instances when the inverse of SSB-per-RO is larger than prach-FDM. This type of configurations should not be supported.

Another problem with the agreed value ranges of the above two RRC parameter is that the number of actually transmitted SSBs can be smaller than the configured value of SS-per-RO. In that case, how the association should be handled has not been discussed. For example, the number of actually transmitted SSBs could be five but the SSB-per-RO could not be configured to five. It is possibly configured as 8 or 16. If it is configured to 8, we think UE should associated the configured RACH resources with the configured SSB-per-RO instead of with the number of actually transmitted SSBs. But we do not see a reason why it can be configured as 16 when the number of actually transmitted SSBs is only five. We hence have the following proposals.

Proposal 9: When the number of actually transmitted SSBs is smaller than SSB-per-RO, UE associates the RACH resources with the configured SSB-per-RO instead of with the number of actually transmitted SSBs.

Proposal 10: It is regarded as an error case if the configured SSB-per-RO is larger than the number of actually transmitted SSBs while there is another SSB-per-RO value among the agreed values that is larger than the number of actually transmitted SSBs while is smaller than the configured SSB-per-RO. And UE is not expected to handle this kind of error cases.

# Conclusions

The proposals are summarized in the following.

Proposal 1: For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

- N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 and is determined by the reference SCS

- N2 also refers to the value determined in control session with UE capability #1 and is determined by the reference SCS

- L2 refers to the MAC processing latency and is equal to 0.5 msec.

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS  the reference SCS

- Note: Spec captures the value k for different reference SCS

- UE is not expected that UL transmissions in the same slot shall apply different TA values

  - If this happens, it is up to UE’s implementation.

- The reference SCS is the minimum SCS of the subcarrier spacing of the downlink carrier with which the TA command is transmitted and the subcarrier spacings of the uplink carriers belonging to the same TA group (TAG).

  - Slot n, slot n+k, the time duration of (k-1) slots, and slot_duration are based on the reference SCS.

Proposal 2: NR selects one of the following proposals:

- Alt. 1: PDCCH order does not include a BWP index.

- Alt. 2: PDCCH order includes a BWP index for CFRA.

  - The number of bits for the BWP index is 2.

Proposal 3: RAN1 selects one of the following to enhance Msg3 coverage and sends LS to RAN2

- Alt. 1: Msg3 coverage is enhanced by the existing HARQ-ARQ mechanism

- Alt. 2: Msg3 coverage is enhanced by cross-slot repetitions

Proposal 4: NR should discuss how UE selects RACH preamble indices/resources for contention-based random access in RRC_CONNECTED mode.

Proposal 5: When UE conducts CBRA in RRC connected mode, UE selects RACH preamble indices/resources that are associated with the downlink reference signal that QCL’ed with the DM-RS antenna port(s) associated with PDCCH reception in common search space(s) where UE is expected to monitor the random access response.

Proposal 6: If the SSB that UE selects for RACH association and Msg1 (re)transmission is not quasi-co-located (QCLed) with the DM-RS antenna port associated with PDCCH reception in the Type0-PDCCH common search space, the Type0A-PDCCH common search space, the Type2-PDCCH common search space, or Type3-PDCCH common search space, UE is not expected to decode PDSCH indicated by a PDCCH scrambled by an RNTI other than RA-RNTI if they are assigned in the same time instance.

Proposal 7: NR supports the value range of the 2-bit RRC parameter prach-FDM is {1, 2, 4, 8} for other RACH use cases besides initial access.

Proposal 8: NR does not support configurations such that1SSB-per-RACH-occasion> prach-FDM.

Proposal 9: When the number of actually transmitted SSBs is smaller than SSB-per-RO, UE associates the RACH resources with the configured SSB-per-RO instead of with the number of actually transmitted SSBs.

Proposal 10: It is regarded as an error case if the configured SSB-per-RO is larger than the number of actually transmitted SSBs while there is another SSB-per-RO value among the agreed values that is larger than the number of actually transmitted SSBs while is smaller than the configured SSB-per-RO. And UE is not expected to handle this kind of error cases.

# References

- R1-1801073 (R2-1801600), “LS on PDCCH order for initiation of random access,” 3GPP RAN1 AH1801 meeting, Vancouver, Canada, Jan. 2018

- R2-1801546, “General corrections on TS 38.321,” 3GPP RAN2 AH1801 meeting, Vancouver, Canada, Jan. 2018

- R2-1806508, “Reply LS on MSG3 size for NR,” 3GPP RAN1 #92bis meeting, Sanya, China, April 2018

- R1-1805657, “Reply LS on Message 3 size for NR,” 3GPP RAN1 #92bis meeting, Sanya, China, April 2018