3GPP TSG RAN WG1 Meeting #93	R1-1806804

Busan, Korea, 21st –25th May, 2018

Agenda item: 	7.2.1

Source: 	MediaTek Inc.

Title: 	On CQI and MCS Design for URLLC

Document for:	Discussion/Decision

# Introduction

In RAN1#92bis meeting, the following agreements were made [1]:

Agreements:

- The two BLER targets that are configurable for URLLC for CSI reporting are:

  - Option B. (10-1, 10-5)

  - Note: The definition of the test case for the BLER target of 10-5 should take into account channel and interference variations and estimation errors.

Agreements:

- Highest spectral efficiency for CQI based on 10-5 BLER target for URLLC is no more than 772/1024*6

- Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6

  - It doesn’t necessarily mean that the CQI table introduced for eMBB cannot be directly reused for URLLC – it’s still a separate discussion

- Note that

  - Whether or not to have two tables or a single table covering both BLER targets is a separate issue

Agreements:

- In total, there are two CQI tables for URLLC CQI reporting

  - The first table for URLLC CQI reporting is the same as the existing 64QAM CQI table without any change, which is for BLER target 10-1 for URLLC

    - Note: this means the agreement on “Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6” is overturned

  - The new table will have entries corresponding to BLER target 10-5

- For CSI reporting, the CQI field is 4-bit.

Agreements:

- For BLER 10-5,

- Companies are encouraged to perform simulations for the new CQI table for URLLC, including

  - The lowest SE entry

    - E.g., 30~50/1024*2

  - Note that the highest SE entry of no more than 772/1024*6 is already agreed

  - Consider using approximately equally spaced SNR values

    - Other options are not precluded

    - Whether or not some existing CQI entries for BLER 10-1 can be reused

      - Consider existing CQI entries when applicable

  - In total 15 CQI entries (+1 OOR entry)

- In performing the simulations, consider

  - Fading channel (TDL-A, 30ns) & (TDL-C, 300ns)

    - Other options are not precluded

  - Payload of 32 bytes

    - Other payload sizes can also be considered, up to each company

  - SNR at 5% geometry for the lowest SE entry

    - Other options are not precluded

  - For other simulation assumptions, refer to agreements from RAN1#92

- Similar considerations are also applicable to the MCS table evaluations

In this contribution, we share our views regarding the design of the CQI and MCS tables for URLLC.

# CQI Design

Table 1 shows the eMBB 64QAM CQI table, which is also agreed to be URLLC CQI table 1 with BLER target 10-1. The last column is the required SNR (dB) for achieving BLER 10-1 in NR PDSCH. These required SNR values are obtained by simulation with payload size 32 bytes and under AWGN channels. In Fig. 1 (a), SNR gap between two adjacent CQI indices is shown. The definition of SNR gap is the difference between the required SNR’s of CQI (i+1) and CQI (i). Mean of these SNR gaps is 2.03 and standard deviation is 0.17. From the simulation results, it shows the required SNR range of URLLC CQI table1 is much aligned with LTE CQI design targets:

- SNR range under AWGN channel is from 8 dB to 20 dB

- SNR gap is uniform with 2 dB step.

To design a new URLLC CQI table 2 with BLER target 10-5, the same concepts could be applied.

Table 2 shows our proposed URLLC CQI table 2 with BLER target 10-5. The last three columns are the required SNR (dB) for achieving BLER 10-5 in NR PDSCH with AWGN, TDL-C 300 ns, TDL-A 30 ns channels. For URLLC, ITU has defined a test environment Macro Urban - URLLC [2]. In RAN#79 meeting, the calibration results for IMT-2020 self-evaluation were approved [3]. Considering the robustness, we set the SINR target to be the worst reported SINR as -3.1 dB, which is the worst case from all companies’ evaluations. Based on the results provided in the last column of Table 2, CQI with spectral efficiency 30/1024*2 can provide the required SNR as -3.7 dB, which meets the ITU 5th percentile of geometry requirement from different companies’ evaluations. Therefore, we have set the lowest spectral efficiency to 30/1024*2. The highest spectral efficiency can be set to the agreed upper bound value of  772/1024*6 and the spectral efficiencies of CQI indices 3-15 can reuse the entries of the URLLC CQI table with BLER target 10-1.

Since the SNR gap between spectral efficiencies 30/1024*2 and 78/1024*2 is about 4dB, one additional spectral efficiency 50/1024*2 is added to make the SNR gap uniform with ~ 2dB step.

Proposal 1: Two additional CQI indices with spectral efficiencies 30/1024*2 and 50/1024*2 can be added.

In Figs. 1 and 2, SNR gap between two adjacent CQI indices is shown under AWGN and TDL channels. The definition of SNR gap is the difference between the required SNRs of CQI (i+1) and CQI (i). SNR gap shown in Fig. 1 (b) under AWGN channel has mean 2.09 and standard deviation 0.22. It means the SNR gap under AWGN channel with BLER target 10-5 is uniform enough compared with that of eMBB CQI under AWGN with BLER 10-1. In Figs. 2 (a) and (b), SNR gap under multipath channels has larger mean and standard deviation than that under AWGN channels. However, the criteria of uniform SNR gap and 2 dB step are used under AWGN channel. Larger SNR gap mean and deviation can be acceptable for multipath channels. Therefore, the proposed URLLC table 2 can meet the design target as in LTE.

Proposal 2: URLLC CQI table with BLER target 10-5 can be designed as Table 2.

Table 1. URLLC/eMBB CQI Table with BLER target 10-1

| CQI index | modulation | code rate x 1024 | efficiency | required SNR (dB) for BLER 10-1 |
| --- | --- | --- | --- | --- |
| 0 | out of range |  |  | AWGN |
| 1 | QPSK | 78 | 0.1523 | -7.8105 |
| 2 | QPSK | 120 | 0.2344 | -5.9272 |
| 3 | QPSK | 193 | 0.3770 | -3.8867 |
| 4 | QPSK | 308 | 0.6016 | -1.5000 |
| 5 | QPSK | 449 | 0.8770 | 0.6568 |
| 6 | QPSK | 602 | 1.1758 | 2.6037 |
| 7 | 16QAM | 378 | 1.4766 | 4.4136 |
| 8 | 16QAM | 490 | 1.9141 | 6.3486 |
| 9 | 16QAM | 616 | 2.4063 | 8.3206 |
| 10 | 64QAM | 466 | 2.7305 | 10.1663 |
| 11 | 64QAM | 567 | 3.3223 | 12.1521 |
| 12 | 64QAM | 666 | 3.9023 | 14.2660 |
| 13 | 64QAM | 772 | 4.5234 | 16.1879 |
| 14 | 64QAM | 873 | 5.1152 | 18.2700 |
| 15 | 64QAM | 948 | 5.5547 | 20.6282 |

Table 2. Proposed URLLC CQI Table with BLER target 10-5

| CQI index | Modulation | code rate x 1024 | efficiency | required SNR (dB) for BLER 10-5 | required SNR (dB) for BLER 10-5 | required SNR (dB) for BLER 10-5 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | out of range |  |  | AWGN | TDL-C 300ns | TDL-A 30ns |
| 1 | QPSK | 30 | 0.0586 | -10.4331 | -7.0295 | -3.7500 |
| 2 | QPSK | 50 | 0.0977 | -8.1733 | -4.7265 | -1.3216 |
| 3 | QPSK | 78 | 0.1523 | -6.2067 | -2.8109 | 0.4113 |
| 4 | QPSK | 120 | 0.2344 | -4.3236 | -0.8894 | 2.6124 |
| 5 | QPSK | 193 | 0.3770 | -2.3019 | 1.6330 | 5.7500 |
| 6 | QPSK | 308 | 0.6016 | 0.3202 | 4.7500 | 9.4404 |
| 7 | QPSK | 449 | 0.8770 | 2.3702 | 7.6613 | 12.7500 |
| 8 | QPSK | 602 | 1.1758 | 4.3417 | 11.0846 | 15.5748 |
| 9 | 16QAM | 378 | 1.4766 | 6.6330 | 12.7500 | 17.4084 |
| 10 | 16QAM | 490 | 1.9141 | 8.6495 | 15.5000 | 19.8248 |
| 11 | 16QAM | 616 | 2.4063 | 10.3584 | 18.0000 | 22.0000 |
| 12 | 64QAM | 466 | 2.7305 | 12.6060 | 20.0000 | 24.1187 |
| 13 | 64QAM | 567 | 3.3223 | 14.6330 | 22.5527 | 26.2500 |
| 14 | 64QAM | 666 | 3.9023 | 16.6776 | 25.1124 | 28.3248 |
| 15 | 64QAM | 772 | 4.5234 | 18.8584 | 27.5000 | 30.5000 |

Figure 1. SNR gap between two adjacent CQI indices under AWGN channel

Figure 2. SNR gap between two adjacent CQI indices under TDL-C/A channels

# MCS Design

In Section 2, existing URLLC CQI table 1 and proposed URLLC CQI table 2 are shown. URLLC MCS tables can be derived from the CQI tables by interpolating spectral efficiencies. Table 3 shows the proposed URLLC MCS table 1 with BLER target 10-1, which is the same as the existing eMBB 64QAM MCS table. Table 4 shows the proposed URLLC MCS table with BLER target 10-5 by adding 4 MCS indices based on the proposed CQI table 2 and removing the last 4 entries in eMNN 64QAM MCS table. Since the spectral efficiency of lowest CQI is for control channel, we only consider the second lowest spectral efficiency, 50/1024*2, in the MCS table. The spectral efficiencies 64/1024*2 and 99/1024*2 are the average from (50, 78)/1024*2 and (78, 120)/1024*2. Therefore, we have the following proposals.

Proposal 3: URLLC MCS table with BLER target 10-1 can reuse existing eMBB 64QAM MCS table.

Proposal 4: URLLC MCS table with BLER target 10-5 can be designed as Table 4.

Table 3. Proposed URLLC MCS index table 1 with target BLER 10-1

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Table 4. Proposed URLLC MCS index table 2 with target BLER 10-5

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 50 | 0.0977 |
| 1 | 2 | 64 | 0.1250 |
| 2 | 2 | 78 | 0.1523 |
| 3 | 2 | 99 | 0.1934 |
| 4 | 2 | 120 | 0.2344 |
| 5 | 2 | 157 | 0.3066 |
| 6 | 2 | 193 | 0.3770 |
| 7 | 2 | 251 | 0.4902 |
| 8 | 2 | 308 | 0.6016 |
| 9 | 2 | 379 | 0.7402 |
| 10 | 2 | 449 | 0.8770 |
| 11 | 2 | 526 | 1.0273 |
| 12 | 2 | 602 | 1.1758 |
| 13 | 2 | 679 | 1.3262 |
| 14 | 4 | 340 | 1.3281 |
| 15 | 4 | 378 | 1.4766 |
| 16 | 4 | 434 | 1.6953 |
| 17 | 4 | 490 | 1.9141 |
| 18 | 4 | 553 | 2.1602 |
| 19 | 4 | 616 | 2.4063 |
| 20 | 4 | 658 | 2.5703 |
| 21 | 6 | 438 | 2.5664 |
| 22 | 6 | 466 | 2.7305 |
| 23 | 6 | 517 | 3.0293 |
| 24 | 6 | 567 | 3.3223 |
| 25 | 6 | 616 | 3.6094 |
| 26 | 6 | 666 | 3.9023 |
| 27 | 6 | 719 | 4.2129 |
| 28 | 6 | 772 | 4.5234 |
| 29 | 2 | Reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

# Conclusion

The following summarizes proposals in this contribution.

Proposal 1: Two additional CQI indices with spectral efficiencies 30/1024*2 and 50/1024*2 can be added.

Proposal 2: URLLC CQI table with BLER target 10-5 can be designed as Table 2.

Proposal 3: URLLC MCS table with BLER target 10-1 can reuse existing eMBB 64QAM MCS table.

Proposal 4: URLLC CQI table with BLER target 10-5 can be designed as Table 4.

# References

- Chairman’s Notes, RAN1#92bis, Sanya, China, April 2018.

- Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”

- RP-180524, “Summary of calibration results for IMT-2020 self-evaluation”