3GPP TSG RAN WG1 Meeting #93	R1-1806908

Busan, Korea, May 21st – 25th, 2018

Title:	Reply LS on the design of PRACH table

Response to:	R1-1805803(R2-1806227)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Huawei, HiSilicon [RAN WG1]

To:	RAN WG2

Cc:

# Contact Person:

# Name:	                Huang Huang

E-mail Address:	 huanghuang (at) huawei (dot) com

# 1. Overall Description:

RAN1 would like to thank RAN2 for their LS in R1-1805803(R2-1806227). Based on RAN2’s recommendation and another RAN2 LS R1-1801073(R2-1801600), RAN1 agrees that the PDCCH order for RACH procedure includes the following fields

- Random Access Preamble index – 6 bits. Indicating which Random access preamble to use in case of contention-free random access procedure, or the value 000000 in case of contention-based random access procedure

- BWP index. Indicating which BWP to transmit the Random access preamble on

- SUL indicator – 1 bit. Indicating whether to transmit the Random access preamble on SUL or normal uplink carrier

- For CFRA only:

  - SSB index - 6 bits are used to indicate an SSB index, which, in turn, identifies a group of RACH occasions

  - PRACH mask index - 4 bits are used indicates 16 indices in the PRACH table. Within the table: (a) one index for all the RACH-Occasions; (b) 8 indices for 8 RACH-Occasions; (c) 1 index for the even number of RACH-Occasions and 1 index for the odd number of RACH-Occasions

# 2. Actions:

# To RAN WG2

# RAN1 would like ask RAN2 to take the above information into consideration.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting 94	  20 – 24 Aug 2018, Gothenburg, Sweden