3GPP TSG RAN WG1 Meeting #93	R1-1806953

Busan, Korea, 21st-25th May 2018

Source:	Ericsson

Title:	On OFDM symbol level PDSCH power boosting

Agenda Item:	7.1.2.2.6

Document for:	Discussion and Decision

# Introduction

ZP CSI-RS is supported in NR to facilitate PDSCH rate matching around certain REs such as CSI-IM resources. In general, a UE should not make any assumption about what is transmitted or not transmitted in a ZP CSI-RS resource. In most cases, however, nothing is likely transmitted in a ZP CSI-RS resource by the gNB. In those cases, the gNB may boost PDSCH transmit power in the OFDM symbols containing the ZP CSI-RS resource.

In this contribution, we discuss potential enhancement to enable PDSCH power boosting in certain OFDM symbols in NR.  This is a revision of R1-1804980.

# Discussion

## PDSCH power allocation in LTE

In LTE, a PDSCH can have different transmit power levels, or energy per RE (EPRE), in different OFDM symbols in a subframe depending on whether the OFDM symbols contain CRS, DMRS or no RS [1].

The ratio of PDSCH EPRE to CRS EPRE in OFDM symbols not containing CRS or DMRS is denoted by , while the ratio of PDSCH EPRE to CRS EPRE in OFDM symbols containing CRS is denoted by .  A UE determines  based on the RRC parameter, PA,  and determines  based on another RRC parameter, PB, where  PB=.

In OFDM symbols containing DMRS, the ratio of PDSCH EPRE to DMRS EPRE is 0 dB for number of transmission layers less than or equal to two and -3 dB otherwise.

In LTE, PDSCH can have different transmit power levels in different OFDM symbols in a subframe, depending on whether CRS, DMRS or no RS is contained in an OFDM symbol. The different power levels are informed to a UE.

## PDSCH power boosting in NR

Currently, a NR UE determines the ratio of the PDSCH EPRE to DMRS EPRE according to 38.214 [2] as follows:

For downlink DM-RS associated with PDSCH, the UE may assume the ratio of PDSCH EPRE to DM-RS EPRE (1/[dB]) is given by Table 4.1-1 according to the number of DM-RS CDM groups without data as signalled by DCI [5, TS 38.212].

Table 4.1-1: The ratio of PDSCH EPRE to DM-RS EPRE

| Number of DM-RS CDM groups without data | DM-RS configuration type 1 | DM-RS configuration type 2 |
| --- | --- | --- |
| 1 | 0 dB | 0 dB |
| 2 | -3 dB | -3 dB |
| 3 | - | -4.77 dB |

Therefore,  a constant PDSCH EPRE across OFDM symbols in a slot is currently assumed in NR.

In NR, PDSCH EPRE is assumed to be constant across OFDM symbols in a slot.

As discussed in the introduction, when nothing is transmitted in ZP CSI-RS resources, the power in the resources could be used to boost PDSCH power in the same OFDM symbols. One example is shown in Figure 1, where a ZP CSI-RS resource with 4 REs is configured for rate matching around a CSI-IM in OFDM symbol #8.  In this case, the PDSCH in OFDM symbol #8 can be power boosted by about 1.76dB. Such PDSCH power boosting is, however, not currently supported in NR.

Figure 1: An example of PDSCH power boosting, where PDSCH in OFDM symbol #8 can be boosted by 1.76dB.

To support PDSCH power boosting in certain OFDM symbols containing ZP CSI-RS in NR, a UE needs to know the ratio of the PDSCH EPRE to DMRS EPRE in those OFDM symbols. The question is how to inform the ratios to the UE.

One simple way to inform the UE is by adding an indication in each ZP CSI-RS resource on whether the ZP CSI-RS resource can be used or not to boost PDSCH transmit power scheduled in the same OFDM symbols. When the ZP CSI-RS resource is activated or triggered in a slot, the UE can figure out the number of ZP CSI-RS REs available in an OFDM symbol for PDSCH power boosting and thus the ratio of the PDSCH EPRE to DMRS EPRE in each of those OFDM symbols.

Using the previous example, when the ZP CSI-RS is indicated for PDSCH power boosting, a UE can figure out that there are 4 ZP CSI-RS REs per RB and the PDSCH power boosting amount in OFDM symbol #8 can be calculated as

Pboost dB=10*log10NREPDSCH+NREZP CSI-RSNREPDSCH=10log108+48=1.76dB

Whether a ZP CSI-RS resource can be used or not to boost PDSCH transmit power scheduled in the same OFDM symbols can be indicated in the ZP CSI-RS resource configuration.

A UE can figure out the ratio of the PDSCH EPRE to DMRS EPRE based on the number of ZP CSI-RS REs and PDSCH REs in an OFDM symbol.

Given the potential benefits of PDSCH power boosting and the minor spec impact, we propose to consider supporting the feature in NR.

Consider supporting PDSCH power boosting in OFDM symbols containing ZP CSI-RS in NR.

# Conclusions

In this contribution, we made the following observations:

Observation 1	In LTE, PDSCH can have different transmit power levels in different OFDM symbols in a subframe, depending on whether CRS, DMRS or no RS is contained in an OFDM symbol. The different power levels are informed to a UE.

Observation 2	In NR, PDSCH EPRE is assumed to be constant across OFDM symbols in a slot.

Observation 3	Whether a ZP CSI-RS resource can be used or not to boost PDSCH transmit power scheduled in the same OFDM symbols can be indicated in the ZP CSI-RS resource configuration.

Observation 4	A UE can figure out the ratio of the PDSCH EPRE to DMRS EPRE based on the number of ZP CSI-RS REs and PDSCH REs in an OFDM symbol.

Based on the observations, we make the following proposal:

Proposal 1	Consider supporting PDSCH power boosting in OFDM symbols containing ZP CSI-RS in NR.

# References

TS36.213.

TS38.214, v15.1.0.