3GPP TSG RAN WG1 Meeting #93		R1-1806976

Busan, Korea, May 21st – 25th, 2018

## Agenda item:	7.1.2.2.5

Source: 	AT&T

Title: 	Remaining Issues in CQI/MCS

Document for:	Discussion/Decision

# Introduction

In RAN #71, a new study item New Radio (NR) Access Technology was approved.  The technical specifications were agreed during the last RAN meeting. According to the current specification, for MCS table configuration NR uses an adaptive table for DFT-s-OFDM. This is because DFT-s-OFDM uses 5 modulation schemes for data transmission. However, we observed few problems with the existing table configuration.

In this contribution, we describe our views on MCS table configuration for DFT-s-OFDM for NR.

# MCS table Update for DFT-s-OFDM

Since the number of modulation schemes supported for PUSCH is 5 which is different from PUSCH using CP-OFDM, a separate tables was designed. However, since the pi/2 BPSK modulation is an optional feature, it was decided to use the entries for Pi/2 BPSK with QPSK if the UE is not capable of pi/2 BPSK modulation. However, in our view, this solution is not efficient as this indicates that as long as the UE is supporting pi/2 BPSK, network needs to have MCS table entries with pi/2 BPSK. This might poses a coverage problem, if the network didn’t implement pi/2 BPSK receiver. This is because the network can’t use the first two entries in the MCS table. Hence to avoid this problem, we recommend the network should configure these entries and inform to the UE whether pi/2 BPSK is enabled or not. In this way the network and the UE are in sync and gives full flexibility to the network. Hence we propose

Proposal: Introduce a new RRC signaling to indicate to the UE that pi/2 BPSK is enabled or not enabled for PUSCH transmission

# Conclusions

In this contribution we described our views on the MCS table for DFT-s-OFDM

Based on our observations, we have the following proposal:

Proposal:  Introduce a new RRC signaling to indicate to the UE that pi/2 BPSK is enabled or not enabled for PUSCH transmission