3GPP TSG RAN WG1 Meeting 93								R1-1806983 Busan, South Korea, May 21th – 25th, 2018

Agenda Item:	7.1.2.2.3

Source:	AT&T

Title:	Remaining details of beam reporting

Document for:	Discussion/Approval

# Introduction

Much progress has been made to finalize the beam management procedure for NR Rel. 15, in particular beam measurement and reporting. Network-triggered periodic, semi-persistent and aperiodic beam measurement and reporting has been agreed based on CSI-RS. For P1/P3 procedures, to measure the receive beams, beam sweeping is done at the receiver. The following agreement has been reached in RAN 90bis [1] for receive beam measurement and reporting.

Agreement:

Working assumption from RAN1#90 is confirmed:

- For beam management CSI-RS, NR supports higher layer configuration of a set of single-symbol CSI-RS resources where

The set configuration contains an information element (IE) indicating whether repetition is “on/off”

- Note: In this context, repetition “on/off” means:

“On”: The UE may assume that the gNB maintains a fixed Tx beam

“Off”: The UE can not assume that the gNB maintains a fixed Tx beam

- Note: This does NOT necessarily mean that the CSI-RS resources in a set occupy adjacent symbols

Furthermore, the following details are agreed

- CSI-RS resources in the resource set are TDMed if repetition is ON

If repetition is ON, The UE does not expect different values for the following parameters across different CSI-RS resources within a resource set

Transmission periodicity

Number of antenna port subject to RAN4 decision

FFS for other parameters

Agreement:

NR supports the following configurations for beam management where a resource set is formed from multiple beam management CSI-RS resources and is contained within a resource setting:

- Single resource set with repetition = “OFF”

UE reports CSI-RS resource indicator(s) within this resource set for CRI feedback

- Single resource set with repetition = “ON”

UE does not report CRI

- FFS: Further support additional configuration by down selection from the following two alternatives:

(a) Multiple resource sets, all with repetition = “ON”

UE reports CSI-RS resource set indicator(s) for CRI feedback

      - FFS: Whether set ID(s) are local within a resource setting or global across all resource settings

(b) Multiple equal-size resource sets, all with repetition = “OFF”

UE reports distinct local CSI-RS resource indicator(s) within one or more resource sets. The UE can assume that the gNB applies the same Tx beams in the same order for each of the sets

- Note: Not all configurations are applicable for P1/P2/P3

- FFS: Dimensioning of the bit width of the UCI field which can carry either CRI(s) or CSI-RS resource set indicator(s)

In this paper, we discuss the remaining aspects of beam measurement and reporting at the UE for downlink beam management.

# UE Triggered Rx Beam Measurement

Beam management procedure is used to achieve beam alignment between the DL Tx beam at the base station, and the Rx beam at the UE. It is especially important in FR2, to counter the blocking effect that happens due to UE movement, rotation, obstacles around the UE, self-blocking, etc. Receive beam management adjusts the receive beams such that the UE can adapt to the propagation path that is the most favorable to receive the DL transmission.

CSI-RS with repetition on is used to measure the receive beam quality, and identify the receive beam at the UE. CSI-RS resources are higher layer configured with repetition on, and TDMed for the UE to measure the best receive beam. The network does not know how many beams or resources the UE needs to sweep to identify the best receiving beams.

When the UE operates in FR2, it is expected to have two or more antenna panels, to provide spherical coverage, and overcome the adverse effects of blocking. For receiver beam switching, switching antenna panels has the same effect as switching beams within a panel, except that antenna panel switching requires a larger processing time than switching beams within a panel. The larger processing time due to switching panels, can be in excess of 20 slots, and may occur because of powering the antenna panel on/off for energy savings.

Observation: Inter antenna panel beam sweeping at the UE requires a larger processing time than intra panel beam sweeping

The details of the receive beam measurement at the UE are unknown to the network, and antenna panel switching for example, can result in a data transmission interruption, especially if the network triggers receive beam measurement periodically or aperiodically without knowing the status of the receive beam at the UE.

A UE triggered receive beam procedure should therefore be considered.  Since the UE has better knowledge of the receive beam resources, and the antenna blocking, such a UE triggered procedure can mitigate transmission interruption due to long processing time due to antenna panel switching. In addition, such a procedure can reduce the overhead of the network periodically triggering beam measurement procedures at the UE.

Proposal: A UE triggered receive beam measurement procedure should be introduced in NR

# Summary

In this contribution, we discussed receive beam measurement and reporting, and we made the following observation and proposal.

Observation: Inter antenna panel beam sweeping at the UE requires a larger processing time than intra panel beam sweeping

Proposal: A UE triggered receive beam measurement procedure should be introduced in NR

# References

[1] 3GPP TSG RAN WG1 90bis Chairman’s notes, Prague, CZ