3GPP TSG RAN WG1 Meeting #93			R1-1807004

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	7.1.5

Source:	Apple

Title:	Single Tx Switched UL Support for EN-DC in LTE TDD Band

Document for:	Discussion/Decision

# Introduction

In RAN4 #86bis meeting [8], it was agreed to allow Single Tx Switched UL operation for EN-DC in band 41. Band 41 is a TDD band for which Single Tx Switched UL operation has not been discussed in RAN1. This contribution focuses on the solutions that allow semi-static TDM UL resource partition between LTE and NR in LTE TDD band to allow Single Tx Switched UL operation.

A key enabler for early commercialization of 5G NR is the non-stand-alone (NSA) operation, which is expected to be adopted in many of the initial deployment world-wide. The NSA mode, which is based on LTE-NR dual connectivity (EN-DC), requires UEs to support both LTE and NR uplink transmissions.

In EN-DC, based on whether UE is required to transmit LTE UL and NR UL simultaneously, UE can have two different UL operation modes

- Dual UL transmission: in this mode, UE is required to transmit LTE UL and NR UL simultaneously

- Single UL transmission: in this mode, at any given time, UE is only required to transmit uplink in at most one RAT, i.e. either LTE or NR. We also refer to it as Single Tx Switched UL.

Compared to dual UL transmission, Single Tx Switched UL has many performance benefits especially for the UE in power limited scenario (i.e. cell edge) as well as implementation advantages.

- Single Tx Switched UL solves the DL desense problem: as explained in [1, 2, 3], depending on the band combinations and resource allocation, sending two concurrent uplinks can produce undesirable intermodulation products that could fall right into the NR or LTE receiving bands, causing degradation of UE’s receiver sensitivity. This is especially problematic for band combinations for both LTE and NR bands below 6 GHz, the impacted receiver bands due to MSD can be virtually not usable in practice, and it creates extreme challenges in RF implementations.

- Single Tx Switched UL removes the UE additional maximum transmit power backoff requirement i.e. A-MPR: As discussed in [4], simultaneous LTE and NR UL transmission can cause intermodulation products outside the desired transmission bandwidth, which can violate the regulatory emission requirement. To meet the regulatory emission requirement, UE is required to apply additional maximum transmit power backoff, i.e. A-MPR.

- Single Tx Switched UL ensures coverage for UE not capable of dynamic power sharing: For EN-DC, total maximum transmit power budget has to be shared between LTE and NR in case of dual UL transmission. However, the initial NR UE may not support dynamic power sharing between LTE and NR. As results, maximum transmission power budget has to be split semi-statically between LTE and NR. For example, a 23dBm total maximum transmit power limit can be split equally such that both LTE and NR have 20dB maximum transmit power per RAT. Compared to single UL transmission where each RAT can transmit up to 23dBm, this reduces the coverage by 3dB.

- Single Tx Switched UL reduces the impact on the RF frontend architecture: Compared to single UL transmission, supporting of dual UL transmission requires the device to completely change its RF frontend architecture while adds significantly more complexity. As results, mandating dual UL transmission will cause the UE to support much less number of band combinations. The detailed explanation can be found in [5]

In 3GPP RAN#77 [6], it was agreed to introduce the indication of whether UE supports 2 simultaneous UL transmission for certain band combinations.

- Define in RAN4 specs for which band combinations and channel allocations (to the extent specified by RAN4) within that band combination the UE is allowed to indicate that it does not support 2 simultaneous UL tx (this is not implying the granularity of the signalling) - Signalling to be defined to support the 'red text' from RAN4 part of the single tx discussion (i.e. as in RP-172064): - UE capability indicates that the UE does not allow 2 simultaneous UL transmission for the RAN4 specified channel allocations in a given band combination. If the network chooses to operate the UE in a way that is not consistent with this capability indication then the UE behavior is not specified and the UE might not meet the performance criteria.

Single Tx Switched UL operation requires the UL resources to be allocated to NR and LTE in a TDM manner. From RAN1 perspective, the design of mechanism for Single Tx Switched UL needs to consider at least the following aspects

- Minimum or no impact on the DL operation, especially no restrictions on the resources that can be used for DL scheduling. To support DL operation, ACK/NAK needs to be feedback on UL. When UL resources are restricted due to the TDM resource partition between LTE and NR, it is important to avoid potential restriction on the DL HARQ operation.

- Minimum or no impact on the UL HARQ operation. This is especially important for LTE as LTE employs synchronous UL HARQ operation. When UL resources are restricted for LTE, it is important to ensure that UL HARQ operation is not impacted.

- It is desirable to allow the UL resource to be semi-statically allocated between LTE and NR in TDM manner, instead of relying on the dynamic scheduling to coordinate the LTE and NR UL transmission to avoid collision.

NR can already operate with restricted UL resources due to its flexible design. For example, NR supports asynchronous DL and asynchronous UL operation, scheduling and HARQ timing for both DL and UL can be flexibly configured. Furthermore, NR also supports 2, 4 or 7 symbol min-slot operation, in addition to flexible PUCCH configurations, including short PUCCH (1-2 symbols) and long PUCCH (4-14 symbols). Last but not the least, NR supports scalable numerology. Therefore, it is feasible for NR to operate with limited UL resources.

On the other hand, LTE uses fixed HARQ feedback timing, and synchronous UL operation. As results, it is more challenging to design Single Tx Switched UL operation for LTE. Hence, it is important to design the semi-static TDM UL resource partition between LTE and NR such that LTE DL and UL HARQ operations are not restricted.

The Single Tx Switched UL design should cover both the case of LTE TDD PCell and LTE FDD PCell. So far, RAN1 Single Tx Switched UL design mainly focused on the FDD case, i.e. LTE FDD PCell. In 3GPP RAN1#90 in August [7], specific TDM patterns were agreed to enable the single Tx operation for LTE FDD PCell:

Agreements: - When the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), but the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers - For LTE carrier, UE can be configured with - Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell - For scheduling/HARQ timing of LTE FDD carrier, DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell is applied - UE is allowed to transmit NR UL signals at least in the subframe(s) where LTE UL transmission is not allowed according to the DL-reference UL/DL configuration - FFS whether or not a UE-specific subframe offset for the DL-reference UL/DL configuration can be configured considering system resource utilization and potential spec impact - Case 2: Release 15 LTE-FDD HARQ timing - No impact on LTE RAN1 specifications - Note: it doesn’t necessarily imply that UE has to support both cases

Case 1 represents a semi-static TDM UL resources partition between LTE and NR with no restrictions on DL scheduling, compared to Case 2 for LTE in FDD band. The main idea is to use the TDD HARQ timing that requires only a restricted subset of all FDD UL subframes/slot, which is already supported in LTE for LTE TDD-FDD CA.

In the last RAN4 #86bis meeting [8], band 41 (TDD band) was agreed to allow Single Tx Switch UL operation for EN-DC deployment,

- Single Switched UL is allowed - Multiple Options for TDM – RAN4 may have specific requirements for these options. - Option 1: UL TDM pattern as per R4-1805323 - Option 2: UL TDM based on LTE eIMTA framework, to reserve subset of the available UL subframes for LTE. - Option 3: Dynamic UL TDM - Other options are not precluded. - As a per-BC UE capability, UE may signal support for SUO for a given EN-DC band combination. If dual transmission is scheduled instead of single switched uplink, UE behavior is unspecified. - No additional signaling required

However, Single Tx Switched UL operation for EN-DC in LTE TDD band is not discussed or designed so far. The targeted deployment scenario for band41 is intra-band EN-DC. Intra-band EN-DC deployment poses the most design challenges as LTE and NR need to be deployed synchronously in terms of TDD pattern, i.e. using one of the 7 LTE TDD configurations. As results, LTE needs to give up some of its UL resources in time domain in order to allow NR UL operation. In this contribution, we discuss two designs that allow such an operation

- Case 1: Use eIMTA design to semi-statically allocate the UL resource between LTE and NR in TDM manner

- Case 2: Use the GP and UpPTS in special subframes of LTE for NR

We also provide analysis of both DL and UL HARQ operation, focusing on LTE side due to the reason discussed above, to show that both solutions do not impose restriction on the DL and UL HARQ operation. Note that, the above two solutions can be operated jointly to improve latency, etc.

# Case 1: Use eIMTA for semi-static TDM UL resource partition

In TDD LTE, DL ACK/NAK is carried out on the regular UL slots. If some of the UL slots in LTE TDD configurations are taken away for NR use, we need to ensure that LTE HARQ operations is not impacted for both DL and UL. LTE eIMTA design from Rel-12, which allows flexible reconfiguration of UL-DL in LTE, can be used for semi-static TDM UL resource partition between LTE and NR. eIMTA has three TDD UL/DL configurations

- SIB1 configuration, i.e. “subframeAssignment”, provides the TDD configuration for the minimum set of DL subframes. This means the DL and special subframe in this configuration cannot be switched to UL. It is used for the purpose of (1) coexist with legacy UEs that do not support eIMTA (2) define the UL HARQ timing including the PUSCH (re)-transmission scheduling as well as ACK/NAK (PHICH) feedback (3) provide the fixed DL resources for RRM/RLM measurement, etc.

- RRC dedicated signalling, “eimta-HARQ-ReferenceConfig-r12”, provides TDD configuration for the minimum set of UL subframes. This means the UL and special subframe in this configuration cannot be switched to DL. It is used to define the DL HARQ timing of when to feedback ACK/NAK.

- L1 configuration, i.e. DCI, which allows the dynamic DL/UL configuration for flexible subframes, i.e. subframes not marked as DL in SIB1 nor marked as UL in RRC configuration.

The SIB1 signalling and RRC configuration can be used together to partition LTE TDD UL resources in a TDM way. For an existing LTE TDD deployment, the same configuration can be signaled in SIB1 for coexistence consideration. Meanwhile, a TDD configuration with less number of, i.e. a subset of, UL subframes can be configured via RRC in order to operate LTE with reduced number of UL subframes. The unused UL subframes per radio frame can be used by NR for Single Tx Switched UL operation.

We use TDD configuration 2 (2 UL, 6 DL, 2 special subframe) as an example to illustrate and analyze the DL and UL HARQ operation. Figure 1 shows the configuration at both LTE and NR side as well as how NW operates. From LTE side, TDD configuration 2 can be signaled in SIB1, while TDD configuration 5 (1 UL, 8 DL, 1 special subframe) can be signaled via RRC. From NR side, only subframe 7 is used for NR UL transmission. From the NW operation perspective, clearly, 2 UL subframes in TDD configuration 2 are semi-statically split between LTE and NR, i.e. SF2 assigned to LTE and SF7 assigned to NR.

Figure 1 Use eIMTA for semi-static TDM UL resource partition

It is also interesting to note the usage of special subframe. It is still possible to allocate UpPTS in LTE special subframe, or even together with GP, to NR to improve latency or higher UL date rate. We will discuss the resource partition within LTE special subframe in the next section.

To validate the solution, we need to check the HARQ operation from LTE side for both DL and UL to ensure that there are no restrictions. We start with the DL HARQ processing. Since LTE DL is asynchronous operation, we only need to check the ACK/NAK feedback.

Figure 2 illustrates the DL ACK/NAK feedback according to Table 10.1.3.1-1 and Table 10.1.3.1-1A in [9]. Clearly, single UL subframes, i.e. SF2, can be used to carry the DL ACK/NAK feedback of the rest 9 subframe. Different colors (black and red) are used to illustrate the PUCCH resource allocation in order to more efficiently coexist with the legacy UE that does not support eIMTA. From Figure 2, we can conclude that DL ACK/NAK feedback has no issue.

Figure 2 LTE DL ACK/NAK timing for eIMTA

Then, we need to check the UL HARQ timing. Since LTE UL is synchronous operation, we need to check both the PUSCH scheduling and ACK/NAK feedback timing. Figure 3 illustrates the UL HARQ timing according to Table 8-2 and Table 8.3-1 in [9]. Clearly, from Figure 2, we can conclude that UL HARQ timing has no issue. It is also interesting to note that, the HARQ turn-around time, i.e. time between the first PUSCH (re)-transmission and the following PUSCH re-transmission, is 10ms. This means the retransmission of the PUSCH uses the same subframe which allows the UL resources to be semi-statically partitioned without impacting the UL HARQ operation.

Figure 3 LTE UL HARQ timing for eIMTA

Based on the discussion and analysis, we have the following observation

Observation 1: To support Single Tx Switched UL for EN-DC in LTE TDD band, eIMTA frame work can be used to semi-statically allocate UL resources between LTE and NR in TDM manner

# Case 2: Use GP and UpPTS in LTE special subframe for NR

As we mentioned in the previous section, GP and UpPTS in LTE special subframe can also be semi-statically allocated for NR UL. In LTE, UpPTS in special subframe is only used for SRS and short RACH, which does not impact the DL and UL HARQ operation. Therefore, via SRS and RACH configuration, some UpPTS resource can be vacated in LTE to be used for NR.

Using TDD configuration 2 as an example, Figure 4 illustrates the Single Tx Switched UL operation for EN-DC in LTE TDD band with NR using GP and UpPTS in the LTE special subframe. For the regular UL subframe (SF2 and SF7), either LTE uses all of them or eIMTA framework can be used to semi-statically partition them between NR and LTE, as discussed in the previous section. For the special subframe (SF1 and SF6), NR could use all or part of UpPTS, as well as some of GP, as long as LTE SRS and RACH configuration semi-statically vacate those resources.

Figure 4 Single Tx Switched UL for EN-DC in LTE TDD band with NR using GP and UpPTS

Figure 5 provides the detailed illustration of the semi-static resource partition in LTE special subframe to support NR UL using GP and/or UpPTS. We use special subframe configuration 4, 7 and 9 as examples. Note

- Using part of GP for NR UL transmission assumes that the remaining GP is enough to cover RTT and DL/UL switching time, as well as other consideration such as interference avoidance from neighboring NodeB, during the deployment

- To use UpPTS for NR UL, we assume that LTE configurates SRS and PRACH such that part or all of the symbols in UpPTS in some special subframes are unused for LTE

- For special subframe configuration 4, the GP is only 1 symbol of 15kHz SCS. NR can use 30kHz SCS symbol to utilize the second half of the GP.

Figure 5 Semi-Static Resource partition in LTE special subframe for NR UL

Based on the discussion, we have the following observation

Observation 2: To support Single Tx Switched UL for EN-DC in LTE TDD band, GP and UpPTS in LTE special subframe can be semi-statically allocated for NR UL.

# Impact on other WGs to support Single Tx Switched UL for EN-DC in LTE TDD band

In this section, we briefly discuss the potential impact to other WGs in order to support Single Tx Switched UL for EN-DC in LTE TDD band

RAN2: No Impact unless similar signaling is preferred/needed to separate the eIMTA UE operation with Single Tx Switched UL operation

- UE capability signaling of single UL operation is already defined for band combinations that allow Single Tx Switched UL operation

- eIMTA has already introduced the RRC signaling for DL HARQ reference configuration, i.e. “EIMTA-MainConfig”

RAN3: No Impact

- X2/Xn signaling, i.e. “UL Coordination Information”, has been introduced to coordinate between LTE eNB and NR gNB about the UL resource partition between LTE and NR. (TS36.423/TS38.423)

RAN4: No Impact except the potential expansion of difficult band combinations that allows the UE to signal that it does not support dual Tx operation.

- Band 41 is already identified in RAN4 that allows UE to operate in Single Tx Switched UL mode.

# Conclusions

In RAN4 #86bis meeting [8], it was agreed to allow Single Tx Switched UL operation for EN-DC in band 41. Band 41 is a TDD band for which Single Tx Switched UL operation has not been discussed in RAN1. In this contribution, we provide two solutions that can semi-statically allocate UL resources between LTE and NR in LTE TDD band to enable Single Tx Switched UL operation for EN-DC. The two solutions are

- Case 1: Use eIMTA design to semi-statically allocate the UL resource between LTE and NR in TDM manner

- Case 2: Use the GP and UpPTS in special subframes of LTE for NR

We also provide the analysis of both DL and UL HARQ operation at LTE side in order to ensure that the solutions provided in this contribution do not restrict the DL and UL HARQ operation. We have the following observations

Observation 1: To support Single Tx Switched UL for EN-DC in LTE TDD band, eIMTA frame work can be used to semi-statically allocate UL resources between LTE and NR in TDM manner

Observation 2: To support Single Tx Switched UL for EN-DC in LTE TDD band, GP and UpPTS in LTE special subframe can be semi-statically allocated for NR UL.

# References

- R1-1708276, “Uplink sharing in NSA mode”, 3GPP RAN1#89, Hangzhou, China, May 15th-19th, 2017.

- R4-1704759, “Co-existence analysis on LTE-NR band combinations”, 3GPP RAN4 #83, Hangzhou, China, May 15th-19th, 2017.

- R4-1703999, “European LTE-NR band combinations, and coexistence analysis”, RAN4#82bis, Spokane, US, April 3-7, 2017

- R4-1803062, “A-MPR for DC_41A_n41A”, RAN4#86, Athens, Greece, Feb. 26-March 2, 2018

- R4-1805411, “The need of FR1 single-switched UL for EN-DC”, RAN4#86bis, Melbourne, Australia, April 16-20, 2018

- RP-172085, “Signalling for Single/Dual UL Tx”, RAN#77, Sapporo, Japan, Sept 11-15, 2017

- Chairman Note, RAN1#90, Prague, Czech Rep, 9th – 13th October 2017

- Chairman Note, RAN4#86bis, Melbourne, Australia, USA 16 - 20 April, 2018

- 3GPP, “3GPP TS 36.213 Evolved Universal Terrestrial Radio Access (E-UTRA)”, V15.1.0 (2018-03)