3GPP TSG RAN WG1 Meeting #93								   R1-1807006

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	6.2.1.2.4

Source:	InterDigital, Inc.

Title:	Clarification on Xp value determination for sTTI

Document for:	Discussion and Decision

# Introduction

In 36.213, the parameter XP has been defined for HARQ-ACK timing when a UE is configured with higher layer parameter ShortTTI-Length and for PDSCH transmission in a subslot, and it mentioned that XP value is configured by a higher layer parameter. However, there is no description which higher layer parameter determines XP values.

# Text Proposal for 36.213

In 36.331, the following parameter is used to indicate the HARQ-ACK timing:

proc-Timeline-r15                 ENUMERATED {nplus4set1, nplus6set1,

nplus6set2, nplus8set2 }    OPTIONAL,   -- Need ON

Based on the proc-Timeline-r15 configuration, it seems that XP=4 when the configured parameter is nplus4set1, XP=6 when the configured parameter is nplus6set1 or nplus6set2, and XP=8 when the configured parameter value is nplus8set2.

Following text is proposed to clarify how XP value is determined from a configured higher layer signalling.

Modified clause (7.3 of TS 36.213 v15.1.0)

…

[skipped Text]

…

Throughout this section,

-	if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, and otherwise.

-	if the UE is configured with higher layer parameter shortTTI and for PDSCH transmissions in a subslot,  is determined based on higher layer parameter proc-Timeline-r15, where = 4 if proc-Timeline-r15 is set to ‘nplus4set1’, or = 6 if proc-Timeline-r15 is set to ‘nplus6set1’ or ‘nplus6set2’, or = 8 if proc-Timeline-r15 is set to ‘nplus8set2’.

End of modifications