3GPP TSG RAN WG1 Meeting #93								    R1-1807011

Busan, South Korea, May 21st – 25th, 2018

Agenda Item:	7.1.1.4.1

Source:	InterDigital Inc.

Title:	On Remaining Details of NR Physical Random Access Channel

Document for:	Discussion and Decision

# Introduction

In RAN1 #92bis, the following agreements about the invalid RACH occasion collided with SSB and PRACH mapping period have been achieved [1]:

Agreements:

- For both FR1 and FR2, the previous working assumption is replaced with the following working assumption

  - Note: The value with yellow background is not final

- For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot, and it is more than at least N symbols after DL part and/or the last symbol of an SSB.

  - N = 0 for Msg1 SCS = 1.25kHz, 5kHz

  - N = 2 for Msg1 SCS = 15kHz

  - N = 2 for Msg1 SCS = 30kHz

  - N = 2 for Msg1 SCS = 60kHz

  - N = 2 or 3 for Msg1 SCS = 120kHz

  - Note for format B4, N = 0 irrespectively of Msg1 SCS

- UE is not expected to receive DL signals during any valid RACH occasion

- The SSB to RO association pattern period shall repeat at most every 160 ms

  - If there are leftover ROs after an integer number of SSB to RO association period(s) within the SSB to RO association pattern period. Those leftover ROs are invalid.

- Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured

Agreements:

Change the previous working assumption:

- Time period for SSB to RO association is multiple of PRACH configuration periods

  - The number of multiple PRACH configuration periods in the time period is the smallest value in the set {1, 2, 4} such that the number is

≥ Actually transmitted SSBs / SSBs that can be mapped to one PRACH config period

  - The time period starts from frame 0

And replace it with the following:

- Time period for SSB to RO association is multiple of PRACH configuration periods

  - The number of multiple PRACH configuration periods in the time period is the smallest value in the set determined by the RACH configuration period from following table such that the number is

≥ Actually transmitted SSBs / SSBs that can be mapped to one PRACH config period  such that all actually transmitted SSBs can be mapped, at least once, to the ROs within the time period.

  - The time period starts from frame 0

| RACH configuration period (ms) | Mapping period set (# of RACH configuration period ) |
| --- | --- |
| 10 | {1，2，4，8，16} |
| 20 | {1，2，4，8} |
| 40 | {1，2，4} |
| 80 | {1，2} |
| 160 | {1} |

In this contribution, we considered and discussed issues related to invalid RACH occasions, the collision between PRACH and RMSI, and DL/UL switching gap.

# Invalid RACH Occasions

It was discussed in the last meeting that whether the invalid RACH occasion in one RACH configuration period is also invalid in any other PRACH configuration period for UE’s simplicity.

According to the agreement, the RACH occasions in DL part or that collided with SSB are invalide. Since the SSB period and semi-static UL/DL configuration period are not consistent with PRACH configuration period. The number of invalid RACH occasion due to SSB collision can be different in different PRACH configuration period. The period of semi-static UL/DL configuration can be one of {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10}ms. The PRACH configuration period is a integer number of semi-static UL/DL configuration period. For each PRACH configuration period, invalid RO due to the semi-static UL/DL configuration should be the same. Then, the remaining issue would be the different invalid ROs due to the SSB collision in different PRACH configuration period. The SSB period can be one of {5, 10, 20, 40, 80, 160}ms. The duration of SSB is 5ms.

There are following alternatives for invalid ROs in different periods.

Alternative 1: Any PRACH configuration period has the same invalid RO. The ROs invalid in one PRACH configuration period is also invalid in any other PRACH configuration periods.

Alternative 2: Any mapping period has the same invalid RO. The RO invalid in one mapping period is also invalid in any other mapping period.

Alternative 3: There is no need to add any invalid RO based on current agreement. There can be different invalid RO in different RACH period or mapping period.

For example, assume that the PRACH configuration period is 10ms, the SSB period is 40ms. There are 12 RACH occasions in each PRACH configuration period without considering SSB collision. There are 6 RACH occasions in each PRACH configuration considering the SSB collision. There are 8 SSBs. One-to-one mapping between SSB and RO. The invalid ROs for those three alternatives are shown in Figure 1.

Figure 1: invalid ROs for different alternatives.

For alternative 1, the number of RO is reducted in all PRACH configuration periods, i.e., there are 6 RO in all PRACH configuration periods. The number of valid RO is only 50% of the configured RO in most of the periods. For alternative 2, the number of RO is reduceted, but not in all PRACH configuration periods. It is reduced every mapping period, where the number of mapping period is multiple integer of PRACH configuration period, which is larger or equal than the PRACH configuration period. For alternative 3, all configured RO except those collided with SSB are valid. Among three alternatives, alternative 3 is the most efficiency. But for alternative 3, for different mapping period, the valid RO is different, and the location of RO associated with the same SSB is different in different mapping period. Hence, UE has higher complexity. Alternative 2 has more RO than alternative 1 and less RO than alternative 3, while the location of RO associated with the same SSB is the same in different mapping period. Thus, UE has relatively lower complexity. Alternative 1 has the lowest complexity from UE’s perspective, but the number of RO is decreased much.

Proposal 1: NR supports that the invalid ROs are consistent across mapping periods. For simplicity, the invalid ROs can also be consistent across RACH configuration periods.

# Confirm the working assumption of introducing the DL/UL gap

In RAN1 meeting #92bis, working assumption for the gap between DL-UL switching is discussed. The interference from neighbor gNB to the UL transmission of the UE at gNB side should be considered when switching from DL to UL. The channel delay from neighbor cell is larger than the channel delay from UE to the serving gNB, and the delayed signal from neighbor cell may interfere the UL signal of UE in the serving cell.  Moreover, the transmit power of gNB is larger than that of UE. As shown in Figure 5(a), the interfering gNB is the neighbor gNB of gNB1, and UE1 is in the same cell with gNB1. When switching from DL to UL, gNB1 receives the delayed DL signal from interfering gNB as well as the UL signal of UE1 simultanesouly. The UL signal of UE1 is interfered by neighbor gNB’s DL signal. The switching gap is introduced to avoid the interference, as shown in Figure 5(b). The introduced gap can avoid the interference of neighbor cells, or make the interference of neighbor cell week enough to be igonored.

(a): interference from neighbour gNBs                            (b) DL-UL switching gap

Figure 5: DL to UL switching

For long preamble format, introducing gap would make the whole RACH occasion invalid. Thus, no gap is introduced and gNB will handle the interference from neighbor cells. For short preamble with Msg1 SCS 60kHz, 2-symbol gap is introduced, which is 0.0357ms and the corresponding distance is 0.0357×3×105=10km. Considering that the maximum coverage of short sequence is format C2 with coverage of 10km, and the CP of Msg1 can also mitigate part of the delay, 2-symbol gap should be enough for SCS of 60kHz. For SCS of 120kHz, the corresponding distance of 2-symbol gap is 5km. If the coverage of the neighbor cell is 10km, the 2-symbol gap and CP cannot mitigate the interference. In order to reserve sufficient switching gap, 3-symbol gap can be supported if Msg1 SCS = 120kHz.

Proposal 2: Confirm the working assumption with the N=3 for Msg1 SCS = 120kHz

# Conclusions

In this contribution, we considered and discussed issues related to invalid RACH occasions, the collision between PRACH and RMSI, and DL/UL switching gap. We have the following proposal:

Proposal 1: NR supports that the invalid ROs are consistent across mapping periods. For simplicity, the invalid ROs can also be consistent across RACH configuration periods.

Proposal 2: Confirm the working assumption with the N=3 for Msg1 SCS = 120kHz

# References

- Chairman’s Notes, 3GPP TSG-RAN WG1 #92bis, 16th April – 20th April, Sanya, China, 2018