3GPP TSG RAN WG1 #93									  R1-1807012

Busan, South Korea, May 21st – May 25th, 2018

Source: 	InterDigital Inc.

Title:	Remaining Details on Non-CB based UL Transmission

Agenda item:	  7.1.2.1.3

Document for:	Discussion and Decision

# Introduction

In RAN1 WG1 Meeting #92b [1], further details on the operation of non-codebook based UL transmission were reached;

When the UE is configured with AP-SRS and AP-CSI-RS associated with the AP-SRS for non-codebook based UL transmission, - Any of the TCI states in the scheduled CC should not be configured with QCL Type D. - AP-CSI-RS is located in the same slot as the AP-SRS trigger/request. - The presence of AP-CSI-RS is (implicitly) indicated via the SRS request field in the DCI. - The QCL assumption for receiving the associated AP-CSI-RS for non-codebook-based transmission is up to UE implementation - Note: This does not preclude the use of the CSI request field to trigger this AP CSI-RS for DL CSI acquisition

In previous meetings, the signaling of coherence capability for non-codebook based uplink was briefly discussed without any agreement. Another aspect of the discussion is related to UE state of calibration. In a multi-antenna transmission, the actual implementation of antenna panels, their assembly and connection structures to the RX/TX RF chains play an important role in the level of coherency and reciprocity exhibited by the UE. In this contribution, we provide our perspective on calibration for non-codebook operation.

# Non-Codebook UL Operation

In NR, a UE can be configured by the higher layer parameter ulTxConfig to operate in either ‘Codebook’ or ‘NonCodebook’ modes. The selection between the modes would be mainly based on operating conditions, however it is expected that UE perform sufficiently well in either mode.

For non-codebook uplink transmission, following the SRS transmission, and the succeeding gNB indication of SRI, a UE can determine a proper precoder for its PUSCH transmission. The basic assumption for non-code book based operation is to rely on channel reciprocity. In a reciprocity-based CSI framework, since a UE bases its UL transmission on the measurements performed on the downlink channel that is observed through the RF chains and antenna system, it is important that the UE is properly calibrated [2]. As demonstrated in [2], the performance of a reciprocity-based system with a poor calibration can be severely impacted.

Observation 1 – UE calibration is essential for reciprocity-based non-codebook UL transmission.

In a MIMO system, the actual implementation of antenna panels, their assembly and connection structures to the RF feed play an important role in the level of coherency achieved by the UE. A small phase offset and/or amplitude imbalance may lead to an array structure with a limited coherence capability. Such imbalances are usually more prevalent in higher frequencies, and may be resulted from either baseband or RF circuitries in a UE. Therefore, for codebook-based uplink transmission in NR, three levels of coherency are defined, namely, ‘fullAndPartialAndNonCoherent’, or ‘partialAndNonCoherent’ , or ‘Non-Coherent’. As such, upon reporting of its coherence capability, a UE is configured by the higher layer parameter ULCodebookSubset with a proper subset restriction according to its declared coherence capability. Then, the UE can determine the desired precoder based on the indicated TPMI and its configured subset restricted codebook. The main motivation for introduction of codebook subset restriction for codebook-based transmission was to assure that the uplink codebook properly matches the actual coherence capability of a UE. Otherwise, if a UE does not exhibit coherency across its antenna ports, then the indicated PMI will be always mismatched and inaccurate.

Observation 2 – Coherency capability and codebook subset restriction are introduced for codebook-based uplink transmission to assure proper matching between uplink codebook and the actual UE capability.

In non-codebook uplink transmission, a UE can use measurements based on CSI-RS to arrive at an estimate for the downlink channel, and then based on reciprocity principle the UE can determine the best precoder for the uplink transmission. Therefore, for non-codebook uplink transmission, potential impacts and problems associated with RF implementation imperfections are multi-fold as the RF impairments may impact transmission in both transmit and receive directions (Appendix 1). Furthermore, assuming a calibrated gNB, to prevent any distortion between CSI-RS measurements observed by the UE RX chains, and the actual PUSCH transmission through UE TX ports, the conditions expressed in Eq. (A-1) should be met.

Therefore, similar to the notion of coherence capability that is used for codebook-based transmission to make sure that the employed codebook matches UE transmission capability, state of UE calibration should be considered for rank determination and precoder transmission. For example, if a UE is not calibrated then it should not be configured by gNB to operate with a high rank.

Proposal 1 - UE indication of its state of calibration is required to support for reciprocity-based non-codebook UL transmission.

In non-codebook uplink, the coherence capability of a UE could impact the performance of the transmission. However, for the following reasons, there is no need to employ signaling of the coherence capability in the non-codebook mode of operation. Firstly, since the impact of a non-coherent structure is already absorbed and reflected through reciprocity feedback or transmission of the precoded SRS, gNB would not need any additional signaling on coherence capability. Secondly, coherence capability of a UE is based on the hardware implementation rather than the uplink transmission configuration.

Proposal 2 - There is no need to an additional UE capability signaling to indicate UE coherence capability in non-codebook transmission.

# Summary

In this contribution, we provide our perspective on calibration requirement and its requirement for non-codebook based uplink transmission. Based on the presented discussion, following observation and proposal are made:

Observation 1 – UE calibration is essential for reciprocity-based non-codebook UL transmission.

Observation 2 – Coherency capability and codebook subset restriction are introduced for codebook-based uplink transmission to assure proper matching between uplink codebook and the actual UE capability.

Proposal 1 - UE indication of its state of calibration is required to support for reciprocity-based non-codebook UL transmission.

Proposal 2 - There is no need to an additional UE capability signaling to indicate UE coherence capability in non-codebook transmission.

# Appendix 1

Figure 1 shows the basic model of TX/RX transmission chain process in a reciprocity-based MIMO transmission system where  T and R represent the aggregated transmit and receive complex gain blocks, respectively. To exploit the reciprocity principle, it is required that HUL=HDLT, however due to mismatch of the transmitter and the receiver gain elements in the RF chain of eNB and the UE, HUL≠HDLT. The sources of such mismatches are mainly from amplitude and phase variations of the used components in the RF signal chain. In general, the principal requirements for a calibrated multi-antenna system can be expressed as

TgNBRgNB-1= kgNBI

TUERUE-1= kUEI    	 	 		 		 (A-1)

where kgNB and kUE are arbitrary constants. Given the impact of mis-calibration on the validity of the estimated uplink channel, and consequently on the performance of the MIMO transmission [3], a UE should be adequately calibrated for non-codebook based UL MIMO transmission.

Figure 1 - Down link and Uplink TX/RX chain model

# References

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #92b, Sanya, China, April 2018

R1-1705504, “UE Calibration in a Reciprocity-based UL MIMO Transmission”, InterDigital Inc., WG1 Meeting #88bis, Spokane, USA, April 2017

R1-1720658, “Remaining Issues on Non-Codebook Based UL Transmission”, Qualcomm, WG1 Meeting #91, Reno, USA, November 2017