3GPP TSG RAN WG1 Meeting #93		R1-1807101

Busan, Korea, May 21st – 25th, 2018

Agenda item:	6.2.3.1

Source: 	Qualcomm Incorporated

Title: 	Corrections for 1024QAM

Document for:	Discussion/Decision

# Correction on suffixes for RRC parameters

The RRC parameters for altCQI-Table are as follows in TS 36.331:

altCQI-Table-r12           ENUMERATED {

allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}      OPTIONAL       -- Need OP

CQI-ReportConfig-v15x0 ::=      SEQUENCE {

altCQI-Table-1024QAM-r15       ENUMERATED {

allSubframes, csi-SubframeSet1,

csi-SubframeSet2, spare1}      OPTIONAL       -- Need OP

In TS 36.213 the parameter “allSubframes” uses the -rxx suffix, which is not consistent with RRC.

<Text Proposal 1>

If the higher layer parameter altCQI-Table-r12 is configured and is set to allSubframes,

-	the UE shall report CQI according to Table 7.2.3-2.

Else if the higher layer parameter altCQI-Table-1024QAM-r15 is configured and is set to allSubframes

-	the UE shall report CQI according to Table 7.2.3-4.

Else if the higher layer parameter altCQI-Table-r12 is configured and is set to csi-SubframeSet1 or csi-SubframeSet2-,

-	the UE shall report CQI according to Table 7.2.3-2 for the corresponding CSI subframe set configured by altCQI-Table-r12

-	the UE shall report CQI for the other CSI subframe set according to Table 7.2.3-1.

Else if the higher layer parameter altCQI-Table-1024QAM-r15 is configured and is set to csi-SubframeSet1 or csi-SubframeSet2,

-	the UE shall report CQI according to Table 7.2.3-4 for the corresponding CSI subframe set configured by altCQI-Table-1024QAM-r15

-	the UE shall report CQI for the other CSI subframe set according to Table 7.2.3-1.

</Text Proposal 1>

# Correction on alternative TBS

In RAN1#91 the following agreement was reached:

Agreement: Introduce two maximum  entries for 1024QAM, with an RRC parameter selecting between both.

This agreement is captured in the specification in a TM-dependent mode (i.e., even when configured with alternative TBS, the UE will use the larger TBS if configured with e.g. TM4). Note that the maximum TBS may not be decodable even with TM4 if there is additional overhead, e.g. due to more than 1 OFDM symbol for control, or due to presence of e.g. CSI-RS. Thus, we propose to remove the linkage between maximum TBS and TM (or, equivalently, DCI format).

<Text Proposal 2>

else if the higher layer parameter altCQI-Table-1024QAM-r15 is configured, then

-	for DCI format 1A with CRC scrambled by C-RNTI or SPS C-RNTI:

-	for , the UE shall first determine the TBS index () usingand Table 7.1.7.1-1. For a transport block, the TBS is determined by the procedure in Subclause 7.1.7.2.1.

-	for , the TBS is assumed to be as determined from DCI transported in the latest PDCCH/EPDCCH for the same transport block using . If there is no PDCCH/EPDCCH for the same transport block using , and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH/EPDCCH.

-	for DCI format 1/1B/1D/2/2A/2B/2C/2D with CRC scrambled by C-RNTI or SPS C-RNTI;

-	for , the UE shall first determine the TBS index () usingand Table 7.1.7.1-1B except if the transport block is disabled in DCI formats 2, 2A, 2B, 2C and 2D as specified below. When , if the UE is scheduled by DCI formats 2C/2D and is configured with a33 in tbsIndexAlt,  is 33A, or if the UE is scheduled by DCI formats 1/1B/2/2A and is configured with b33 in tbsIndexAlt2,  is 33B; otherwise  is 33. When , if the UE is configured with a37 in tbsIndexAlt3,  is 37A, otherwise  is 37. For a transport block that is not mapped to more than single-layer spatial multiplexing, the TBS is determined by the procedure in Subclause 7.1.7.2.1. For a transport block that is mapped to two-layer spatial multiplexing, the TBS is determined by the procedure in Subclause 7.1.7.2.2. For a transport block that is mapped to three-layer spatial multiplexing, the TBS is determined by the procedure in Subclause 7.1.7.2.4. For a transport block that is mapped to four-layer spatial multiplexing, the TBS is determined by the procedure in Subclause 7.1.7.2.5.

-	for , the TBS is assumed to be as determined from DCI transported in the latest PDCCH/EPDCCH for the same transport block using . If there is no PDCCH/EPDCCH for the same transport block using , and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH/EPDCCH.

-	In DCI formats 2, 2A, 2B, 2C and 2D a transport block is disabled if  and if rvidx = 1 otherwise the transport block is enabled.

</Text Proposal 2>