3GPP TSG RAN WG1 Meeting #93		R1-1807111

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	6.2.7.1.3

Source: 	Qualcomm Incorporated

Title:          Physical Layer Scheduling Request

Document for:	Discussion/Decision

# Introduction

In RAN1 #89, the following agreements were made regarding to physical layer scheduling request (SR) for further NB-IoT enhancements [1]:

Agreements:

- SR should only be used when an NB-IoT UE is in uplink sync in RRC connected mode.

  - TA estimation should not be a design target of SR signal.

- Sending SR with HARQ ACK/NACK can serve as the baseline case for UE with DL data

- Further designs to be considered for dedicated SR signal design are:

  - Based on NPRACH signal;

  - Based on NPUSCH format 2:

    - Non-coherent detection based format is not precluded

- Collision handling for dedicated SR is FFS

In RAN1#90, possible options of piggybacking SR with HARQ-Ack were agreed [2].

Agreements:

- Piggybacked SR with HARQ-ACK is chosen between the following options, with evaluations encouraged at RAN1#90:

  - Option 1: QPSK-based constellation

  - Option 3: Cover code/Orthogonal sequence on ACK/NACK data symbols and/or DM-RS symbols

In this document, we discuss the design of piggybacked and dedicated SR with evaluation results.

In RAN1 92, the following working assumption was made [3]:

Working assumption

Physical-layer SR with and without HARQ-ACK transmission is supported.

- When SR is transmitted with HARQ-ACK:

  - Option 3 is adopted;

  - SR on/off is carried by two orthogonal length-16 cover codes on ACK/NACK data symbols.

    - [1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1] is used to signal SR OFF.

    - [1 -1 1 -1  1 -1  1 -1 1 -1 1 -1  1 -1  1 -1] is used to signal SR ON.

- When SR is transmitted without HARQ-ACK

  - FFS whether it is transmitted in NPUSCH resources or reserved NPRACH resources.

  - FFS whether BSR is conveyed on SR without HARQ-ACK

Note that companies are encouraged to provide evaluation results based on the agreed criterion, which are considered when cofirming the WA.

In RAN1 92bis [4], the following agreements were made:

Agreement:

If the working assumption is confirmed and when SR is transmitted without HARQ-ACK, it is transmitted in NPRACH resource using NPRACH based signal.

Agreement:

- If the working assumption is confirmed, UE transmits the physical layer SR at the first opportunity unless there is a collision with another physical layer transmission/reception:

  - Collision between SR without HARQ-ACK resource and NPDSCH - SR is not transmitted, and remains pending

    - FFS: Collision with NPDCCH search space

# Piggybacked SR with HARQ-ACK/NACK

In [5], it has been shown that using a cover code with element of the form exp(j) has the advantage of adjusting signal distance between HARQ-ACK, HARQ-NACK, and DTX. It has also been shown that the Euclidean distance between NACK and ACK and the distance between DTX and a ACK or NACK can be close, i.e., 8 vs 7. As a result, the performance of piggybacked SR with cover code is mainly limited by the performance of error detection of ACK while NACK is transmitted. When =/2, same performance can be achieved as that with an cover code pair with binary elements. However, if  is changed to be smaller than /2, lower detection rate of a ACK when NACK is transmitted can be achieved.  Hence, we have the following proposal.

Proposal 1: Confirm the WA with the following change on the cover code.

- When SR is transmitted with HARQ-ACK:

  - Option 3 is adopted;

  - SR on/off is carried by two orthogonal length-16 cover codes on ACK/NACK data symbols.

    - [1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1] is used to signal SR OFF.

    - [i i i i i i i i i i i i i i i i ] is used to signal SR ON where i=sqrt(-1).

# NPRACH Based Design

Piggybacked SR with Ack/Nack can only be used when there is DL data. To support SR in other cases, it has been agreed that NPRACH based signal using NPRACH resource is supported.

NPRACH waveform can also be adapted for SR transmission: Each repetition consists of 4 symbol groups and each symbol group consists of 5 symbols and a CP. The use of NPRACH waveform is preferred if SR uses part of the NPRACH resource or FDM with NPRACH.

Since the UL timing of a NPRACH-based SR is known, both symbol group level and symbol level scrambling can be applied to allow UE multiplexing. Depending on further if frequency hopping between symbol groups is applied, there are three options for UE multiplexing:

- Scrambling is applied at symbol level and a length-20 sequence is carrier by a repetition. This allows up to 20 UE to be multiplexed within a tone without frequency hopping between symbol groups.

- Scrambling is applied at symbol group level and a length-4 sequence is carried by a repetition. This allows up to 4 UEs to be multiplexed in a tone without frequency hopping between symbol groups.

Note that longer spreading sequences across multiple repetitions may not be useful as the channel may have varied too much between repetitions and is hence not considered. The impact of inter-symbol interference in options 1 due to the lack of CP could be negligible, thanks to the long symbol duration.  In addition, options 1 could have slightly larger PAPR than option 2 does. This disadvantage of options 1 is, however, outweighed by its higher multiplexing capability and flexibility. In summary, option 1 is preferred if frequency hopping between symbol groups is not applied and option 2 is preferred otherwise.

The repetition level of SR can be derived from the repetition level of NPUSCH format 2. To avoid intercell interference between NPRACH-based SR transmissions, mapping of scrambling sequence to tone index is cell-dependent.

Based on the above discussion, we have the following proposals.

Proposal 2: Down-select from the following NPRACH-based SR waveform design options

- Symbol-level scrambling with a length-20 DFT sequence per repetition.

- Symbol group level scrambling with each repetition scrambled with a length-5 DFT sequence.

Proposal 3: The scrambling sequence applied is a function of cell ID and subcarrier index.

Proposal 4: The number of repetitions of an SR transmission is derived from that of NPUSCH Format 2.

## Resource Allocation

To avoid collision with other NPRACH signals, it is preferred to use part or all of the reserved NPRACH resource [6] for SR. There are two situations with different preferred signal waveform and multiplexing gain. They are

- If both the number of starting subcarriers allocated to SR and the first starting subcarrier of the reserved NPRACH resource are integer multiples of 12, frequency hopping between symbol groups and repetitions are not needed. In this case, option 1 of NPRACH based waveform is preferred to provide multiplexing of up to 20 UEs per starting subcarrier.  NPUSCH format-2 based waveform can also be used.

- Otherwise, frequency hopping needs to be applied to avoid collision with NPRACH. In this case, option 2 of NPRACH based design can be used to allow multiplexing of up to 5 UEs per starting subcarrier.

For each NPRACH starting subcarrier, the N repetitions can be divided into multiple SR resource blocks of smaller number of repetitions. For instance, as shown in the figure below, the N repetitions are divided into three SR resource blocks, two with N/4 repetitions and one with N/2 repetitions.

Figure 3 Dividing an N-repetition NPRACH resource block into multiple SR resource blocks.

Proposal 5: Use part or all of the reserved NPRACH resource for SR to avoid collision with legacy UEs.

# Summary

In this document, we have the following proposal for piggybacked SR.

Proposal 1: Confirm the WA with the following change on the cover code.

- When SR is transmitted with HARQ-ACK:

  - Option 3 is adopted;

  - SR on/off is carried by two orthogonal length-16 cover codes on ACK/NACK data symbols.

    - [1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1] is used to signal SR OFF.

    - [i i i i i i i i i i i i i i i i ] is used to signal SR ON where i=sqrt(-1).

In addition, we have discussed the design of NPRACH-based SR with the following proposals:

Proposal 2: Down-select from the following NPRACH-based SR waveform design options

- Symbol-level scrambling with a length-20 DFT sequence per repetition.

- Symbol group level scrambling with each repetition scrambled with a length-5 DFT sequence.

Proposal 3: The scrambling sequence applied is a function of cell ID and subcarrier index.

Proposal 4: The number of repetitions of an SR transmission is derived from that of NPUSCH Format 2.

Proposal 5: Use part or all of the reserved NPRACH resource for SR to avoid collision with legacy UEs.

# References

- RAN1 Chairman’s notes, 3GPP TSG-RAN WG1#88bis.

- RAN1 Chairman’s notes, 3GPP TSG-RAN WG1#89.

- RAN1 Chairman’s notes, 3GPP TSG-RAN WG1#92.

- RAN1 Chairman’s notes, 3GPP TSG-RAN WG1#92bis.

- R1-1802335, “Physical Layer Scheduling Request”, Qualcomm Inc, 3gpp RAN1 WG1 92, Athens, Greece, February 26th – March 2nd, 2018.

- R2-165830, “Reservation of RA resources in NB-IoT”, Ericsson, NTT Docomo Inc., Qualcomm Incorporated, 3GPP TSG-RAN WG2#95, Gothenburg, Sweden, 22 – 26.