3GPP TSG RAN WG1 Meeting #93                                                                  	R1-1807147

Busan, Korea, May 21st – 25th, 2018

Source:	NTT DOCOMO

Title:	On Type II codebook enhancement

Agenda Item:	7.1.2.2.6

Document for: 	Discussion

- Introduction

In Rel.14, advanced CSI has been introduced which can bring about 10% percent performance gain compared with single beam based codebook [1], while the CSI reporting overhead can still fit to PUCCH format 2 in LTE. On the other hand, for Type II CSI feedback in Rel.15 for NR, codebook enhancement brings performance gain up to 40% compared with Type I CSI feedback, but the overhead is up to 540 bits [2][3].

In this contribution, we discuss the potential Type II codebook enhancement for CSI reporting, and provide initial evaluation results for potential enhancement.

- Current Codebook for Type II CSI

Codebook design for Type II CSI is stable for Rel.15 after RAN1#89 [3], which supports following features:

- Support up to 4 orthogonal beams combination for each layer (free beam selection)

- Support up to Rank 2 codebook design

- Support both WB and SB beam amplitude and phase reporting

We perform system-level evaluations to observe performance gap between current CSI feedback and ideal CSI feedback. The results are summarized in Figure 1. In this evaluation, sum rate is evaluated, where transmission rank is fixed to 1. We adopt WB amplitude feedback with three different feedback schemes, which are Type I CSI, Type II CSI with L=4 (L is the beam number for combination) and ideal CSI feedback. For ideal feedback, the first eigenvector for channel covariance matrix is fed back without quantization, detailed evaluation assumption is presented in the appendix.

Figure 1 Performance comparison between Rel.13, Rel.15 CSI and ideal CSI feedback.

According to the results, we have following observations: Compared with Type I CSI feedback, ideal CSI feedback can bring up to 40% performance gain.

Observation 1: Large performance gain is observed for ideal CSI-T compared with Type II feedback.

- Potential Type II codebook enhancement

Based on the evaluations and the state of the art, Type II CSI feedback can be further enhanced, from quantization resolution point of view, thus higher performance gain can be achieved. Following aspects can be considered to enhance Type II codebook:

Increase combination beam number for codebook design

In Rel. 15, the linear combination of up to 4 beams is supported. When higher CSI feedback resolution is expected, more beams combination can be considered.

We evaluated the system sum rate as a function of beam number. The result is summarized in Figure 2. For the evaluation, rank 1 or 2 transmission is assumed.

Figure 2 Performance comparison between Type II CB with more beams combination and ideal CSI feedback.

From the evaluation results, it can be observed that about 5% performance gain can be obtained comparing L=8 with L=4 case.

Observation 2: 5% performance gain can be obtained by increasing combination beam number from L=4 to L=8.

For L=4~L=8, CSI overhead is calculated as follows. In the evaluation, WB amplitude quantization is assumed, phase quantization is 8-PSK.

Table I: Overhead calculation for codebook enhancement (bits) with beam number increase.

| Overhead | Number of beams |  |  |
| --- | --- | --- | --- |
|  | L=4 | L=6 | L=8 |
| Rank =1 | ~240 | ~370 | ~510 |
| Rank =2 | ~470 | ~730 | ~1000 |

Observation 3: Compared with L=4, the overhead for Type II CSI feedback will be doubled for L=8 case.

Support higher rank codebook design for Type II CSI

In Rel.15, Type II CSI codebook support up to 2 layers. However, Rank 4 transmission is one typical operation not only for SU-MIMO but also for MU-MIMO transmissions. Supported rank can be higher especially for sub-6 operation. Considering that, Type II CSI codebook should be further enhanced to support higher layer MU transmissions.

Observation 4: Type II CSI codebook should be further enhanced to support higher layer transmission.

To support higher rank transmission, possible solutions can be:

Alt.1: Use Type I rank 3-4 codebook for MU transmission

Alt.2: Design Type II rank 3-4 codebook for MU transmission

To verify the effectiveness of Type II codebook for higher rank (rank 3-4) MU transmission, evaluation is conducted and results are summarized in Figures 3 and 4. In the evaluation, we restrict the transmission rank to 3 and 4 only. The maximum number of downlink transmission layers is 12, and maximum MU number is 4. For each UE, the maximum number of downlink transmission layer is 4.

Figure 3 Performance comparison between Type I and Type II CB with rank 3 only.

Figure 4 Performance comparison between Type I and Type II CB with rank 4 only.

From the evaluation results, we can observe that 17%~19% performance gain can be achieved by using Type II codebook for rank 3 transmission, while 24%~30% performance gain can be achieved for rank 4 transmission.

Observation 5: Type II CSI achieves decent performance gain over Type I CSI for rank 3 or rank 4 transmission.

Overhead reduction for Type II CSI

Overhead reduction is another aspect, which should be considered during CSI enhancement. Based on present codebook quantization scheme, the overhead for up to 8 beams combination codebook design, which can support higher ranks, can be calculated and shown in Table II. In the overhead calculation for CSI feedback, both WB and SB amplitude feedback is considered, and unequal bits allocation is considered, both 6 strong beams are assumed for L=4, L=6 and L=8 case.

Table II: Overhead calculation for codebook enhancement (bits).

| Overhead | Number of beams |  |  |
| --- | --- | --- | --- |
|  | L=4 | L=6 | L=8 |
| Rank =1 | ~270 | ~370 | ~460 |
| Rank =2 | ~540 | ~730 | ~910 |
| Rank =4 | ~1070 | ~1440 | ~1800 |

From the overhead calculation in Table II, it can be observed that, if up to 8 beams combination is to be supported, with higher rank, overhead reduction is expected.

Observation 6: Type II CSI codebook enhancement should consider overhead reduction schemes.

- Summary

In this contribution, we discuss the potential enhancement aspects for Type II codebook design. Based on the discussion, we have the following observations.

Observation 1: Large performance gain is observed for ideal CSI-T compared with Type II feedback. Observation 2: 5% performance gain can be obtained by increasing combination beam number from L=4 to L=8.

Observation 3: Compared with L=4, the overhead for Type II CSI feedback will be doubled for L=8 case.

Observation 4: Type II CSI codebook should be further enhanced to support higher layer transmission.

Observation 5: Type II CSI achieves decent performance gain over Type I CSI for rank 3 or rank 4 transmission.

Observation 6: Type II CSI codebook enhancement should consider overhead reduction schemes.

References

[1]	3GPP, R1-1701748, Huawei, “ Details of CSI reporting for advanced CSI”, Feb.,2017

[2]	3GPP, R1-1705899, Erricson,“ Type II CSI feedback”, Apr. 2017

[3]	3GPP, “Draft Report of 3GPP TSG RAN WG1 #89”, May, 2017

Appendix

Table A: Evaluation assumptions.

| Parameter | Values |
| --- | --- |
| Scenarios | NR Urban macro |
| Traffic model | Full buffer/sum rate |
| Carrier frequency | 4 GHz |
| Bandwidth | 10 MHz (Downlink) |
| BS Tx power | 49 dBm |
| BS antenna configurations | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8) (N1,N2)= (4,4) |
| UE antenna configurations | (M, N, P) = (1, 1, 8) |
| UE number per TRP | 10 |
| UE distribution | 80% indoor, 20% outdoor |
| UE attachment | Based on RSRP from CRS BS port 0 |
| UE antenna pattern | Omni-directional |
| UE velocity | 20% Outdoor in cars: 3km/h, 80% Indoor in houses: 3km/h |
| UE receiver | MMSE-IRC |
| Scheduler | Multi-user PF scheduler |
| Maximum MU number | 12 |
| Maximum DL transmission layers | 12 |
| Maximum transmission layer per UE | 4 |
| Oversampling factor | (N1, N2) = (4, 4), (O1, O2) = (4, 4) |