3GPP TSG RAN WG1 Meeting #93	R1-1807173

Busan, Republic of Korea, April 21st – 25th 2018

Agenda item:		7.1.1.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On Remaining Details on Other System Information Delivery

Document for:		Discussion and Decision

# 1	Introduction

In this contribution we discuss about remaining details on Other System Information (OSI) delivery. More specifically we focus on the following topics:

- On RMSI and OSI monitoring

- PDCCH monitoring for OSI

- Initial access DL BWP for RMSI multiplexing pattern 2 and 3

# 2	Discussion

## 2.1	On RMSI and OSI monitoring

In RMSI and SSB multiplexing pattern 1 SIB1 can be scheduled in two slots every 20ms per SSB. For OSI, RAN2 appears to have adopted LTE like approach so that SI windows shall not overlap. As a consequence the SI scheduling needs to be spread over time and thus it would seem difficult to avoid overlap with SIB1 and OSI time windows. Hence, as raised in [2], it would beneficial to be able to separate SIB1 and OSI by having separate RNTIs. Thus it is proposed to inform RAN2 that RMSI and OSI should have different RNTI. Note that same DCI format would still be used.

Proposal 1: Introduce separate RNTI for SIB1 (SIB1-RNTI).

## 2.2	PDCCH monitoring for OSI

Currently a search space definition for OSI is partly undefined, e.g. SI windows are not taken into account for defining slots on which to monitor PDCCH for Type0A-PDCCH common search space. A CORESET configuration and search space for PDCCH monitoring for OSI scheduling (Type0A-PDCCH) are provided currently as follows [3]:

For Type0A-PDCCH common search space or for Type2-PDCCH common search space, the control resource set is same as the control resource set for Type0-PDCCH common search space. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type0A-PDCCH common search space or for Type2-PDCCH common search space are given in Table 10.1-1. If a UE is not provided higher layer parameter searchSpaceOtherSystemInformation for Type0A-PDCCH common search space set, the association between PDCCH monitoring occasions for Type0A-PDCCH common search space set and the SS/PBCH block index are same as the association of PDCCH monitoring occasions for Type0-PDCCH common search space set as described in Subclause 13. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level are given in Table 10.1-1.

For the search space determination by the UE there are thus two cases for IDLE mode UEs:

- UE determines the search space parameters for OSI from four least significant bits of pdcch-ConfigSIB1, included in MasterInformationBlock

- UE determines the search space parameters for OSI from higher layer parameter searchSpaceOtherSystemInformation. However, this IE  is assumed  four bits i.e. correspond to the four LSBs of pdcch-ConfigSIB1 but the network may configure value differently than in MIB

Also for handling of actually transmitted SS/PBCH blocks in OSI monitoring there are two cases:

- For the case the UE is not provided higher layer parameter searchSpaceOtherSystemInformation that information about the actually transmitted SS/PBCH blocks (provided in SIB1) is not taken into in the association between PDCCH monitoring occasions and the SS/PBCH index, while

- when searchSpaceOtherSystemInformation is provided the association can take into account the actually transmitted SS/PBCH blocks.

Furthermore, higher layer parameters tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationCommon2 may also be provided (for TDD). Mapping of PDCCH monitoring occasions upon uplink slots should be avoided as well as overlaps with PRACH occasions in flexible symbols.

For RMSI and SSB multiplexing patterns 2 and 3, if PDCCH monitoring for OSI followed the RMSI monitoring occasions in a SI window, the OSI scheduling and resource allocation flexibility would be limited. In other words, the search space pattern defined for RMSI multiplexing pattern 2 and 3 might not be desirable from data scheduling perspective. Firstly, if RMSI needs to be sent in multiple times within RMSI TTI to enable combining to meet coverage target, it would not be possible to multiplex OSI with SSBs. Secondly, using pattern 3, for example, would imply that OSI would need to be always scheduled middle of slot which might make multiplexing with data more difficult. Thus, for RMSI and SSB multiplexing patterns 2 and 3, the starting symbol(s) for CORESET(s) should be in the beginning of slot, basically following principles of multiplexing pattern 1.

Like noted earlier, it is seen that for defining OSI search space configuration, 4 LSBs of pdcch-ConfigSIB1 could be used also to indicate the search space configuration for OSI. Like also noted above, the interpretation of these bits could be changed at least sligtly. For RMSI multiplexing pattern 2 and 3, for the reason noted above, it would seem to be necessary to consider the placement of the search space in a slot. Hence it, if configuration based only on MIB is used it be would neccesary to have different interpretation of the said bits for Type0A-DPCCH. As for RMSI multiplexing pattern 2 and 3, only one search space configuration is supported for Type0-DPCCH, i.e. in Table 13-13 and Table 13-14 in Section 13 of 38.213 only one entry is used and the rest are reserved. Thus as for pattern 2 and 3, UE always uses only one spesific pattern (for each RMSI multiplexing pattern) for Type0-DPCCH. Therefore the 4 LSBs of pdcch-ConfigSIB1 could be basically reused for Type0A-DPCCH monitoring definition for RMSI pattern 2 and pattern 3. For RMSI pattern 1 this of course would not be possible, as most of the entries in Table 13-11 and Table 13-12 are used. However, as noted above, RSMI multiplexing pattern 1 could be seen as reasonable from from data multiplexing perspective, thus it could be re-used. Basically, the determination of SS inside slot for Type0A-DPCCH, could follow similar principles as defined for RMSI multiplexing pattern 1 for all RMSI patterns. So in addition at least one table (per frequency range) would be needed, giving the PDCCH monitoring occasions in a slot for Type0A-PDCCH common search space.

The current Type0-DPCCH monitoring occasions, define also the slots (in relation to the start of selected SFN) in a fixed manner. For OSI it is seen that the mapping of the monitoring occasions to slots should account UL/DL configuration and the RACH configuration. The slots for in which SS can be pladed would need to be selected based on e.g. the UL/DL configuration. Hence, in difference to RMSI scheduling, all slots within SI window would not neccesarily be valid slots for SI delivery, but would need to account availability based on e.g. UL/DL configuration, so that only DL and flexiple slots could be used. Once the valid slots in SI window are determined, then slots where SS for for a given SSB index is located, could be simply determined in logical (consegutive) order of valid slots (instead of all slots) from the start of the SI window.

To summary the above discussion, we consider the following principles are used to define the OSI search space:

- PDCCH monitoring occasions are associated to SS/PBCH blocks and are mapped to slots within a SI window

- Starting symbol(s) of PDCCH monitoring occasion(s) in slots within a SI window are defined so that they offer good compatibility with data scheduling (follow principles of multiplex pattern 1)

- Slots to which PDCCH occasions for OSI search space are mapped are not uplink slots or slots where there are limited number of downlink symbols taking into account e.g. RACH occasions in flexible slots

- 4 LSBs of pdcch-ConfigSIB1 are used to signal also OSI search space configuration

  - E.g. for multiplexing patterns 2 and 3, first 0-(n-1) indices could refer to the same RMSI search space configuration while for OSI search space the indices could provide n different configurations

Proposal 2: The following principles are used to define the OSI search space:

- PDCCH monitoring occasions are associated to SS/PBCH blocks and are mapped to slots within a SI window

- Starting symbol(s) of PDCCH monitoring occasion(s) in slots within a SI window are defined so that they offer good compatibility with data scheduling (follow principles of multiplex pattern 1)

- Slots to which PDCCH occasions for OSI search space are mapped are not uplink slots or slots where there are limited number of downlink symbols taking into account e.g. RACH occasions in flexible slots

- 4 LSBs of pdcch-ConfigSIB1 are used to signal also OSI search space configuration

  - E.g. for multiplexing patterns 2 and 3, first 0-(n-1) indices could refer to the same RMSI search space configuration while for OSI search space the indices could provide n different configurations

Similar consideration is also presented for paging (Type2-DPCCH) in [4].

# 3	Extension of Initial active DL BWP

The issue of changing the initial active DL BWP for RMSI multiplexing pattern 2 and 3 has been discussed in past meetings. We present the main part of the discussion related to initial active DL BWP extension in [5], including a text proposal to implement the change. Hence, just a brief summary is given here and reader is invited to refer to companion contribution in [5] for further details.

The main merits identified for extending the initial active BWP that have been raised in the past meetings was to enable the utilization of full available bandwidth. This could be used limit the time required for beam sweep by enabling shorter PDSCH allocations via larger number of available RBs in frequency domain. This was not seen to complicate UE design/behaviour , as it could be typically expected that the UE keeps RX open over the combined bandwidth in order to detect SS/PBCH block for time and frequency synchronization tracking together with RMSI reception. Extending the initial active BWP could also be beneficial for mobility and RLM operation, by ensuring that SSB is always inside the active BWP from the beginning of the access procedure.

Proposal 3: Extend the initial active DL BWP to cover both PBCH defined CORESET and SSB for RMSI multiplexing pattern 2 and 3.

# 4	Conclusions

This contribution discussed about remaining issues on OSI delivery in NR. Based on the discussion the following observations and proposals are made:

On RMSI and OSI monitoring:

Proposal 1: Introduce separate RNTI for SIB1 (SIB1-RNTI).

PDCCH monitoring for OSI:

Proposal 2: The following principles are used to define the OSI search space:

- PDCCH monitoring occasions are associated to SS/PBCH blocks and are mapped to slots within a SI window

- Starting symbol(s) of PDCCH monitoring occasion(s) in slots within a SI window are defined so that they offer good compatibility with data scheduling (follow principles of multiplex pattern 1)

- Slots to which PDCCH occasions for OSI search space are mapped are not uplink slots or slots where there are limited number of downlink symbols taking into account e.g. RACH occasions in flexible slots

- 4 LSBs of pdcch-ConfigSIB1 are used to signal also OSI search space configuration

  - E.g. for multiplexing patterns 2 and 3, first 0-(n-1) indices could refer to the same RMSI search space configuration while for OSI search space the indices could provide n different configurations

On the definition of initial active DL BWP for RMSI multiplexing pattern 2 and 3:

Just a brief summary is given and reader is invited to refer to companion contribution in [5] for further details.

Proposal 3: Extend the initial active DL BWP to cover both PBCH defined CORESET and SSB for RMSI multiplexing pattern 2 and 3.

# References

- RP-170847, “New WID on New Radio Access Technology”, NTT DoCoMo

- R1-1805128, “On other system information delivery”, Ericsson

- R1-1805795, “CR to TS 38.213 capturing the RAN1#92bis meeting agreements”, Samsung

- R1-1807174, “On open issues on paging message delivery”, Nokia, Nokia Shanghai Bell

- [5]R1-1807177, ‘Initial active BWP definition for RMSI multiplexing pattern 2 and 3’, Nokia, Nokia Shanghai Bell