3GPP TSG RAN WG1 Meeting #93				R1-1807203

Fusan, Korea, May 21th – May 25th, 2018

Source 	: CATR

Title 	: Further link level simulation results of non-orthogonal multiple-access schemes

Agenda Item	: 7.4.4

Document for	: Discussion and Decision

Introduction

A study item of non-orthogonal multiple access for NR is agreed in RAN#76[1]. The study will further progress on the evaluation of non-orthogonal multiple access schemes focusing on uplink, and provide recommendation on the non-orthogonal multiple access scheme(s) to be specified later. Agreements, observations and evaluation assumption in Rel-14 study shall be the starting point. Link and system level performance evaluation or analysis for non-orthogonal multiple access will be further present with the new evaluation assumptions.

In RAN1#92, the new assumptions of link level performance evaluation are agreed and further clarified in #92b. In this contribution, we present our further link level simulation results of some non-orthogonal multiple access schemes, taking SCMA[2], PDMA[3], MUSA[4], and WSMA[5] as example. Observations are given based on the simulation results.

Simulation Setup

Assumptions of simulation parameters have been agreed in RAN1#92, see in Appendix for details. According to the assumptions, there can be a large number of configuration combinations of evaluation parameters. In this contribution, we continue our former simulation work [6] and also choose the two scenarios, mMTC and eMBB for performance evaluation. The configuration of evaluation parameters is specified as follows. Note that compared with our former simulation work, in this contribution we focus on the evaluation under the configurations with large TBsize and unequal SNR.

Table 1: LLS of Evaluation Parameters for NOMA UL

| Parameters | mMTC | eMBB |
| --- | --- | --- |
| Carrier Frequency | 700 MHz | 4 GHz |
| Waveform (data part) | CP-OFDM | CP-OFDM |
| Channel coding | NR LDPC |  |
| Numerology (data part) | SCS = 15 kHz #OS = 14 | SCS = 15 kHz #OS = 14 |
| Allocated bandwidth(number of PRBs) | 6 | 12 |
| TBS per UE | [10, 40, 75]bytes | [20, 80, 150]bytes |
| Target BLER for one transmission | 10% | 10% |
| Number of UEs multiplexed in the same allocated bandwidth | 6 |  |
| BS antenna configuration | 2Rx for 700MHz, 4Rx for 4 GHz |  |
| UE antenna configuration | 1Tx |  |
| Propagation channel & UE velocity | TDL-C 300ns in TR38.901, 3km/h |  |
| Max number of HARQ transmission | 1 |  |
| Channel estimation | Ideal channel estimation |  |
| MA signature allocation (for data and DMRS) | Fixed |  |
| Distribution of avg. SNR | Equal / Unequal |  |
| Timing offset | 0 |  |
| Frequency error | 0 |  |
| Traffic model for link level | Full buffer |  |
| For link level calibration purpose only | OMA single user whose spectral efficiency is the same as per UE SE in NOMA. |  |
| Receiver | SCMA: MPA( with inner iterations=3 and outer iterations=2) and EPA-SIC PDMA: MPA (with inner iterations=3 and outer iterations=2), EPA-SIC and MMSE-SIC MUSA: MMSE-SIC WSMA: MMSE-SIC |  |
| Modulation | For cases of mMTC@TBS=[10 40]bytes and eMBB@TBS=[20 80]bytes SCMA with 4(3)-point constellation: See in Appendix PDMA:QPSK MUSA:QPSK WSMA:QPSK For cases of mMTC@TBS=75bytes and eMBB@TBS=150bytes SCMA with 16-point constellation: See in Appendix PDMA:16QAM MUSA:16QAM WSMA:16QAM |  |
| Pool of spreading codebook | See in Appendix |  |

Moreover, the coding rate is set as the r=TBsize+CRCbitsNREs×Qm,where NREs denotes the number of available resource elements (REs) for data transmission and Qm is the number of bits carried on each REs.

Evaluation Results and Analyses

The performances of block error rate (BLER) and spectrum efficiency (SE) over the SNR per UE are evaluated. The SNR of a NOMA user is defined as the ratio of average power of the symbol carried on each RE to the noise power in the distribution of white Gaussian. For a fair comparison, all NOMA schemes are evaluated at the same coding rate, length of spreading factor and the total power consumption.

mMTC scenario

In the mMTC scenario, the performances of BLER and SE under the cases with TB size of 10 bytes, 40 bytes and 75bytes are shown in Fig.1(a)- (b), Fig.2(a)- (b), Fig.3(a)- (b) and Fig.4(a)- (b), respectively. Note that in the former contribution, we have present the performance of TB size of 10bytes and 40bytes with equal SNR configuration. Therefore, for TB size of 10bytes and 40bytes, just the performance with unequal SNR configuration is present. The coding rate in cases @TBsize=10bytes, 40bytes and 75bytes for all NOMA schemes is set as 2/9, 21/27 and 43/72, respectively. The spreading factors of NOMA schemes evaluated in all cases are all equal to 4. As shown in the Fig.1-Fig.2, like the performance with equal SNR configuration, each NOMA scheme, i.e., SCMA, PDMA, MUSA, and WSMA, performs similar using the assumed spreading codebook under the unequal SNR configuration in cases @TBsize=10bytes and 40bytes, with equal coding rate, spreading factor and number of users. Compared with the performances under the equal SNR configuration shown in the former contribution, the performances of NOMA schemes under the unequal SNR configuration get a little worse, while not affecting the performance difference among NOMA schemes.

For cases@TBsize=75bytes, SCMA adopts the novel constellation of 16points for the joint modulation and RE mapping process, see in Appendix. While PDMA, MUSA and WSMA adopts 16QAM for modulation module according to the TS 38.211. According to the performances shown in Fig.3-Fig.4, we can see that SCMA performs better than other NOMA schemes for big data transmission with high modulation level, benefiting from the joint design of modulation and RE mapping, which can further separate the symbols of users.

| Fig. 1(a) | Fig. 1(b) |
| --- | --- |

| Fig. 2(a) | Fig. 2(b) |
| --- | --- |

| Fig. 3(a) | Fig. 4(b) |
| --- | --- |
| Fig. 4(a) | Fig. 4(b) |

Observation 1:.Under the unequal SNR configuration, each NOMA scheme, i.e., SCMA, PDMA, MUSA, and WSMA, performs similar for small data transmission with low modulation level, by using the assumed spreading codebook and with equal coding rate, spreading factor and number of users.

Observation 2: The performances of NOMA schemes under the unequal SNR configuration get a little worse than that of equal SNR configuration, while not affecting the performance difference among NOMA schemes.

Observation 3: SCMA performs better than other NOMA schemes (i.e., PDMA, MUSA, and WSMA) for big data transmission with high modulation level, benefiting from the joint design of modulation and RE mapping, which can further separate the symbols of users.

eMBB scenario

In the eMBB scenario, the performances of BLER and SE under the cases with TB size of 20 bytes, 80 bytes and 150bytes are shown in Fig.5(a)- (b), Fig.6(a)- (b), Fig.7(a)- (b) and Fig.8(a)- (b), respectively. Note that in the former contribution, we have present the performance of TB size of 20bytes and 80bytes with equal SNR configuration. Therefore, for TB size of 20bytes and 80bytes, just the performance with unequal SNR configuration is present. The coding rate in cases @TBsize=20bytes, 80bytes and 150bytes for all NOMA schemes is set as 11/54, 41/54 and 19/27, respectively. The spreading factors of NOMA schemes evaluated in all cases are all equal to 4. Since the coding rate setting, modulation level and NOMA scheme configuration in the eMBB scenario are similar to that of mMTC scenario, we can get the same observations from Fig.5-Fig.8 as from Fig.1-Fig.4

| Fig.5(a) | Fig. 5(b) |
| --- | --- |

| Fig.6(a) | Fig.6(b) |
| --- | --- |

| Fig.7(a) | Fig.8(b) |
| --- | --- |
| Fig.8(a) | Fig.8(a) |

Conclusion

In this contribution, we present further link level simulation results of non-orthogonal multiple access schemes including SCMA, PDMA, MUSA, and WSMA. Observations based on the simulation results include:

Observation 1:.Under the unequal SNR configuration, each NOMA scheme, i.e., SCMA, PDMA, MUSA, and WSMA, performs similar for small data transmission with low modulation level, by using the assumed spreading codebook and with equal coding rate, spreading factor and number of users.

Observation 2: The performances of NOMA schemes under the unequal SNR configuration get a little worse than that of equal SNR configuration, while not affecting the performance difference among NOMA schemes.

Observation 3: SCMA performs better than other NOMA schemes (i.e., PDMA, MUSA, and WSMA) for big data transmission with high modulation level, benefiting from the joint design of modulation and RE mapping, which can further separate the symbols of users.

References

- RP-171043  Revision of Study on 5G Non-orthogonal Multiple Access, ZTE, RAN#76 meeting, June 2017.

- R1-162155  Sparse Code Multiple Access (SCMA) for 5G Radio Transmission Huawei, HiSilicon, April 2016.

- R1-163383  Candidate Solution for New Multiple Access, CATT, April 2016.

- R1-1800123 Receiver algorithms of linear-spreading based NOMA, ZTE, Jan. 2018.

- R1-1802767 Signature Design for NoMA, Ericsson, Feb. 2018.

- R1-1805256 Further link level simulation results of non-orthogonal multiple-access schemes, Apr. 2018

- Appendix

- Assumptions for link level evaluation

Table 3 Evaluation parameters – LLS for UL NOMA agreed in RAN1#92

| Parameters | mMTC | URLLC |  | eMBB |  | Further specified values |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier Frequency | 700 MHz | 700 MHz or 4 GHz |  | 4 GHz, 700 MHz as optional |  |  |
| Waveform (data part) | CP-OFDM and DFT-s-OFDM | CP-OFDM as starting point |  | CP-OFDM as starting point |  |  |
| Channel coding | URLLC: NR LDPC eMBB: NR LDPC mMTC: NR LDPC |  |  |  |  | The choice of channel coding here is only for the performance evaluation purpose for NOMA study |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 | Case 1: SCS = 60 kHz, #OS = 7 (normal CP), optionally 6 (ECP) Case 2: SCS = 30 kHz, #OS = 4 |  | SCS = 15 kHz #OS = 14 |  |  |
| Allocated bandwidth | 6 as the starting point | 12 as the starting point |  | 12 as the starting point |  | For high payload such as 75 bytes, larger number of RBs can be considered. |
| TBS per UE | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. Lower than 0.1 bits/RE is optional | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. |  | At least five TBS that are [20, 40, 80, 120, 150] bytes. Other values higher than 20 bytes are not precluded. |  | #bits per RE calculation does not include DMRS overhead (e.g., REs of one every 7 symbols for DMRS would not be used to carry the data) |
| Target BLER for one transmission | 10% | 0.1% |  | 10% |  |  |
| Number of UEs multiplexed in the same allocated bandwidth | To be reported by companies. |  |  |  |  | Companies are encouraged to perform evaulations with various number of UEs Note: refined set of numbers of UEs should be further discussed in the next meeting. |
| BS antenna configuration | 2 Rx or 4 Rx for 700MHz, 4Rx or 8 Rx for 4 GHz 8Rx as optional |  |  |  |  | CDL model in 38.901 should be considered for 8Rx |
| UE antenna configuration | 1Tx |  |  |  |  |  |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h, CDL optional |  |  |  |  |  |
| Max number of HARQ transmission | 1 as starting point. |  | 1 as starting point. More values, 2 for URLLC can be used. |  | 1 as starting point. |  |
| Channel estimation | Ideal channel estimation results should be reported for calibration Realistic channel estimation |  |  |  |  |  |
| MA signature allocation (for data and DMRS) | Fixed/Random |  |  |  |  | Proponents report the details of random MA signature allocation (whether without or with collision) |
| Distribution of avg. SNR | Both equal and unequal | Equal |  | Both equal and unequal |  | Uniform discrete values for unequal case,, range [x - a, x + a] (dB) with 1 dB step, where x is the average SNR among UEs, and the deviation [a=3] |
| Timing offset | 0 as starting point. For grant-free without perfect TA, value is TBD |  |  |  |  |  |
| Frequency error | 0 as starting point. The value(s) is TBD. |  |  |  |  |  |
| Traffic model for link level | Full buffer as starting point. Non-full-buffer model (like Poisson arrival of fixed packet size) is optional. |  |  |  |  |  |
| For link level calibration purpose only | OMA single user whose spectral efficiency is the same as per UE SE in NOMA. AWGN curves can be provided also. |  |  |  |  |  |

- Pool of spreading codebook of NOMA schemes for comparison

Table 4 Pool of spreading codebook of NOMA schemes for comparison

| NOMA scheme | Pool of Spreading Codebook |  |
| --- | --- | --- |
| PDMA | 1 0 1 0 0 01 0 0 1 0 00 1 0 0 1 00 1 0 0 0 1 |  |
| SCMA with 4(3)-point constellation | Spreading codebook:0 1 1 0 1 01 0 1 0 0 10 1 0 1 0 11 0 0 1 1 0 3-point constellation: |  |
| SCMA with 16-point constellation | Spreading codebook:0 1 1 0 1 01 0 1 0 0 10 1 0 1 0 11 0 0 1 1 0 16-point constellation: |  |
| WSMA | -0.4080+0.2696i 0.2666+0.3345i 0.4160-0.1771i -0.3691+0.1399i 0.4899+0.1677i -0.1418-0.6554i 0.3590+0.2963i -0.2979+0.5233i -0.5307+0.0636i -0.5195-0.0881i -0.2114+0.3707i -0.4228-0.0072i-0.4167-0.3349i -0.2269+0.4271i 0.3441-0.4199i -0.0500+0.4607i -0.5140-0.0787i -0.2543+0.3660i-0.4857+0.1499i 0.2242+0.4126i -0.0552+0.4606i 0.0932-0.5858i 0.0369-0.5273i -0.2892+0.2988i |  |
| MUSA SF=4 | Note that when the number of UE is n, choose the first n rows of spreading codebooks. |