TSG-RAN WG1 #93												R1-1807264

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.1.5

Source: 	Ericsson

Title:	On remaining issues of NR-LTE co-existence

Document for:	Discussion and Decision

# Introduction

In this contribution we follow up on the remaining issues related to NR-LTE coexistence and provide a high-level framework for power sharing between NR and LTE for architecture Option 4/4A. Section 2.2 is a slightly revised version of the discussion from [1].

# Discussion

## 2.1 Remaining issues for Case1 HARQ timing

### 2.1.1 PHICH handling

In RAN1#92, it was agreed that for a UE operating in EN-DC when configured with Case 1 HARQ timing on an FDD PCell – “UE does not use PHICH”. This was captured in 36.213 sub clause 8.3 as follows

“For a UE configured with EN-DC and serving cell frame structure type 1, if the UE is configured with subframe-Assignment-r15 for the serving cell, the UE is not expected to receive PHICH corresponding to a transport block on the serving cell.”

The above text is ambiguous and can lead to uncertain UE behavior. The expected UE behavior is that the UE should assume PHICH resources are still present in the subframe as per the PHICH group configuration (e.g. for determining resources for PDCCH), but the PHICH resource itself should not be used to determine whether a non-adaptive retransmission is needed. Instead, the UE should wait until the next UL grant for the same HARQ process (i.e., in this case until subframe n+10 if initial transmission in subframe n+4 is scheduled via a grant in subframe n) and use the NDI bit in the UL grant to determine whether to retransmit same TB or transmit a new TB. This behavior is similar to receiving an ACK on PHICH for LTE (i.e., UE does not flush the HARQ buffer in response to ACK reception, but simply waits for the next grant associated with same HARQ process). Considering the above aspects, we propose to clarify the RAN1#92 agreement as follows

Proposal

------------- start TP to 36.213 sub clause 8.3--------------------------

...

For a UE configured with EN-DC and serving cell frame structure type 1, if the UE is configured with subframe-Assignment-r15 for the serving cell, the UE is not expected to receive PHICH corresponding to a transport block on the serving cell and ACK for that transport block shall be delivered to the higher layers.

….

--------------- end TP--------------------------------------------------------------

### 2.1.2 Switching time between LTE and NR

When Case1 HARQ timing is used in conjunction with ‘single UL operation’ for the difficult band combinations identified in RAN4, the switching times required by the UE hardware to transition between LTE and NR and vice versa are unclear. In [2], RAN1 received information regarding BWP switching times from RAN4. The scenarios described in the LS (e.g. Scenarios 1-3) have similarities with switching between LTE and NR, but exact switching time requirements would depend on UE hardware assumptions for ‘single UL operation’. Considering this, we propose that RAN1 sends an LS to RAN4 requesting information about the switching times.

Proposal

- Send LS to RAN4 requesting information about the switching times required by UE hardware to transition between LTE and NR and vice versa when Case1 HARQ timing is used in conjunction with ‘single UL operation’

## 2.2 Power sharing for NR architecture option 4/4A

In RAN 79, the following was agreement was made regarding NR architecture option 4/4A [3].

With architecture option 4/4A, NR will be configured as MCG while LTE is configured as SCG as shown below.

Figure 1 – NR architecture options 4 and 4A

Below we provide our view for Option 4/4A power sharing framework taking into account RAN guidance to minimize specification impact.

- P_LTE and P_NR signaling

  - P_LTE and P_NR are maximum allowed power values configured on LTE and NR side respectively.

  - These are already specified for EN-DC and can be reused without changes for Option 4/4A

- Dynamic power sharing

  - For EN-DC, a UE supporting ‘dynamic power sharing’ capability can scale down/drop NR (SCG) transmissions when power limited. Details of scaling down/dropping are left to UE implementation.

  - Similar capability should be supported for option 4/4A. In this case, the UE should be able to scale down/drop LTE (SCG) transmissions when power limited, and the details of scaling/dropping can be left to UE implementation.

- Semi-static partitioning of LTE and NR UL transmissions

  - For EN-DC, Case1-OperationA [4] behavior is supported, where network can restrict LTE UL transmissions to a subset of subframes via RRC signaling on LTE side (i.e., by configuring Case1 reference TDD UL/DL configuration for LTE including the LTE FDD Pcell). When such a reference configuration is indicated, and when the UE does not support dynamic power sharing, the UE is not expected to make NR transmissions in slots corresponding to UL subframes indicated by the RRC signaling on LTE side (i.e., the UL subframes of Case1 reference TDD UL/DL configuration).

  - Similar framework can be supported for Option4/4A, for UEs not supporting dynamic power sharing, by replicating the changes agreed for LTE Pcell UE behavior to LTE PScell UE behavior.

In summary, we propose the following high-level framework for Option 4/4A power sharing between NR and LTE

Proposal

- P_LTE and P_NR signaling defined for EN-DC should be reused for Option 4/4A

- Dynamic power sharing capability should be supported for Option 4/4A

  - UE should scale down/drop LTE (SCG) transmissions when power limited, and the details of scaling/dropping can be left to UE implementation.

- For support of semi-static partitioning of LTE and NR UL transmissions similar to Case1-OperationA agreed for EN-DC (for UEs not supporting dynamic power sharing), the changes agreed so far for LTE Pcell UE behavior can be replicated for LTE PScell

# Conclusion

We propose the following on remaining aspects related to NR-LTE coexistence:

Proposal 1

------------- start TP to 36.213 sub clause 8.3--------------------------

...

For a UE configured with EN-DC and serving cell frame structure type 1, if the UE is configured with subframe-Assignment-r15 for the serving cell, the UE is not expected to receive PHICH corresponding to a transport block on the serving cell and ACK for that transport block shall be delivered to the higher layers.

….

--------------- end TP--------------------------------------------------------------

Proposal 2

- Send LS to RAN4 requesting information about the switching times required by the UE hardware to transition between LTE and NR and vice versa when Case1 HARQ timing is used in conjunction with ‘single UL operation’.

Proposal 3

- P_LTE and P_NR signaling defined for EN-DC should be reused for Option 4/4A

- Dynamic power sharing capability should be supported for Option 4/4A

  - UE should scale down/drop LTE (SCG) transmissions when power limited, and the details of scaling/dropping can be left to UE implementation.

- For support of semi-static partitioning of LTE and NR UL transmissions similar to Case1-OperationA agreed for EN-DC (for UEs not supporting dynamic power sharing), the changes agreed so far for LTE Pcell UE behavior can be replicated for LTE PScell

# References

[1] R1-1804455 – “On NR-LTE coex design”, Ericsson, RAN1#92bis, Sanya, P.R. China, April 2018.

[2] R1-1803602 – “LS on BWP switching delay”, RAN4RAN1 LS, RAN1#92bis, Sanya, P.R. China, April 2018.

[3] RP-180554 – “Plan for finalizing all NR architecture options”, TSG-RAN Chair, RAN WG3 Chair, RAN WG2 Chair, RAN WG1 Chair, RAN WG4 Chair, RAN#79, Chennai, India, March 2018.

[4] 3GPP RAN1#91 Chairman notes, Reno, USA, November 2017.

# Annex A – Relevant RAN1 Agreements

In RAN1 NR AH#2, the following was agreed

Agreements:

- Regarding power sharing for LTE-NR dual connectivity, support at least semi-static power sharing between LTE and NR

  - FFS details

  - Discuss further whether or not to support dynamic power sharing between LTE and NR

  - Discuss further impacts due to other factors, e.g., different TTI lengths, channel/service types, synchronous vs. asynchronous, different processing latency for LTE vs. NR, assumption regarding communication between NR vs. LTE at UE, specification impact to LTE (if any) and/or NR, etc.

In RAN1#90, the following was agreed

Agreements:

- At least for LTE-NR NSA operation

  - Maximum allowed power values for LTE (P_LTE) and NR (P_NR) are set separately

    - i.e., when UE is configured for NR, P_LTE can be configured up to P_cmax and  P_NR can be configured up to P_cmax.

    - e.g. P_LTE + P_NR > P_cmax or P_LTE + P_NR = P_cmax

  - Signaling details for P_LTE, P_NR are left to RAN2, RAN4.

  - Note: ‘P_cmax’ is a limit that is similar to ‘The configured maximum UE output power’ that was specified for LTE.

  - Note: The network will still have flexibility to prioritize or reserve certain NR transmission power depending on network implementation

  - All UEs are mandated to handle P_LTE + P_NR = P_cmax while handling of P_LTE + P_NR > P_cmax depends on UE capability

  - At least, when DL/UL LTE sTTI/reduced UE processing time based operation is not configured for the UE, if total transmit power exceeds P_cmax when there is simultaneous NR and LTE UL tx,

    - For NR, UE scales down/drops NR transmission and NR power scaling details are left to UE implementation (note: it is not intended to have RAN4 test from RAN1 perspective)

      - If there are two or more UL carriers, the power scaling or tx dropping can be performed for each of the UL carriers separately or jointly up to UE implementation

    - For LTE, no change in power control procedure

  - FFS the case when DL/UL LTE sTTI/reduced UE processing time based operation is configured for the UE

  - The following is FFS

    - The case when P_NR is configured such that P_NR < P_cmax, and UE can use power up to P_cmax in NR when it knows that there will be no UL transmission in LTE by semi-static configuration (e.g., measurement gap, DL/UL configuration)

In RAN1#91, the following was agreed

Agreement:
For LTE/NR NSA operation,

- If this UE supports dual UL operation and also supports single UL operation with Case 1 HARQ timing, RRC signaling can configure a UE to operate in one of the following modes:

Dual UL operation

Single UL operation with Case 1 HARQ timing

Single UL operation with Case 2 HARQ timing

For UE supporting single UL operation and with Case 1 HARQ timing if UE does not support power scaling for LTE-NR DC with P_LTE+P_NR>Pcmax, UE shall support the following two operations:

Operation A with Case1: P_LTE + P_NR > Pcmax, in which case the UE assumes that no NR UL transmission takes place in an UL subframe/slot that is designated as LTE UL in the Case 1 reference TDD configuration

Operation B with Case1: P_LTE + P_NR <= Pcmax, in which case NR UL can be scheduled in any UL subframe/slot (while the UE behaviour in case of being simultaneously scheduled on LTE and NR uplinks is not specified)

The operation A vs operation B configuration is implicitly determined based on P_LTE and P_NR

Note that the above agreement does not affect the current status on the optional/mandatory support of power scaling for LTE-NR DC with P_LTE + P_NR > Pcmax

Note that the above agreement can become obsolete if power scaling for LTE-NR DC is mandated to all UEs

In RAN#78 the following was endorsed

- Proposal 1

  - Agree to introduce Rel-15 capability signaling to indicate whether the UE supports dynamic EN-DC power sharing

    - Dynamic power sharing means that the UE can operate with P_LTE + P_NR > P_powerclass configuration

  - Agree that the intent is to make dynamic EN-DC power sharing mandatory at a future time

  - Check any possible updates on status above in March

- Proposal 2

  - For UEs without dynamic LTE-NR power sharing capability, the support of single UL operation (Operation A with Case 1 in Slide 5) is mandatory with capability signalling

- Single UL operation is optional for dynamic power sharing capable UEs

In RAN1 NR AH1801 the following was agreed

Agreement:

- P_LTE and P_NR are configured separately via UE specific RRC (i.e., as dBm numbers with similar value range as p-Max in LTE)

  - P_LTE and P_NR are UE-specific

- P_cmax for LTE and P_cmax for NR are derived based on P_LTE and/or P_NR (details to be decided by RAN4)

- RAN4 to define maximum total LTE and NR power in FR1 (X_total) that the UE should never exceed.

- When dynamic power sharing is used,

  - If total power for LTE and NR in FR1 exceeds X_total, UE reduces NR transmission power or drops NR transmission so that total power does not exceed X_total

    - Note: As per previous agreement LTE power control procedure is not changed

In RAN1#92 the following was agreed

Agreement:

For a UE operating in EN-DC when configured with Case 1 HARQ timing on an FDD PCell

- No changes to the current DCI format size for DCI formats 0 and 1A in the CSS

- UE does not use PHICH

- The UL scheduling/HARQ timing is as follows:

  - PUSCH HARQ RTT is 10ms

  - UL grant in subframe n scheduled PUSCH in subframe n+4, and the UL grant for the same UL HARQ process occurs in subframe n+10

  - Note: This supersedes previous agreements on the scheduling/HARQ timing for the UL

- The DAI field and the HARQ process number field in DCI formats in the USS follows the design of FDD SCell with TDD PCell

- Support PUCCH format 3/4/5 procedures for HARQ-ACK feedback as in LTE FDD SCell with a TDD PCell, with the following exceptions

  - The PUCCH format 1a/1b resource corresponding to a PDSCH scheduled with a DCI is derived based on the resource determination procedure for FDD

  - A UE is not expected to receive both unicast PDSCH scheduled by CSS and unicast PDSCH scheduled by USS within a HARQ-ACK bundling window

  - If a unicast PDSCH is scheduled in CSS, UE assumes DAI=1

- Note: The above does not change the agreement that LTE UL transmissions only occur in the UL subframes (not including special subframes) of the reference UL/DL configuration

- Note: Per previous agreement, the PDSCH-to-HARQ feedback timing follows the reference UL/DL configuration, for DL assignments in both CSS and USS.

In RAN1#92bis the following was agreed

Agreement:

In Rel-15, Case 1 HARQ timing is supported for the LTE serving cells only for the following cases

- A single FDD cell

- DL CA with FDD-FDD CA and FDD-TDD CA, where the LTE PCell is FDD and there is only one UL carrier

  - The UE is not configured for cross-carrier scheduling

Agreement:

For a UE operating in EN-DC when configured with Case 1 HARQ timing on an FDD PCell, the following is supported for the LTE serving cells

- For an FDD SCell, the same DL HARQ timing as the PCell is applied for the FDD SCell,

- For a TDD SCell, the same DL HARQ timing as the PCell is applied restricted to the set of DL/special subframes on the TDD SCell

- The UL HARQ timing for the FDD PCell is the same for the CA and non-CA cases