3GPP TSG RAN WG1 Meeting #93	R1-1807322

Busan, Korea, May 21-25, 2018

Agenda Item:	6.2.1.2.3

Source:	Huawei, HiSilicon, Qualcomm

Title:	Correction on SPUCCH coverage

Document for:	Discussion and decision

Introduction

Regarding UE capability and PUCCH transmission for sTTI, the following agreements have been achieved:

Agreement:

- Define separate UE capability per sTTI length combination {DL,UL}:

  - {2,2},{2,7},{7,7},{7,7}+{2,7},{2,2}+{2,7},{7,7}+{2,2}

    - NOTE: The last two combinations only apply if different UL sTTI lengths in different PUCCH groups are supported.

  - For each combination above, define separate UE capability on the maximum number of DL CCs and UL CCs for sTTI operation

Agreement:

When a UE is to transmit UCI in a 2/3-symbol sPUCCH, sPUCCH format selection between sequence-based sPUCCH and PF4-based sPUCCH is based on the following rules.

- For SR (if any) and up to 2 bits HARQ-ACK, the UE shall use sequence-based sPUCCH.

- For SR (if any) and more than 2 bits HARQ-ACK, the UE shall use PF4-based sPUCCH.

Agreement:

The channel coding scheme for sPUCCH format carrying more than 2 bits ACK/NACK and SR (if any) is defined as:

- Up to 11 bits: Single RM coding

- 12-22: Dual RM

- More than 22 bits: TBCC+8bit-CRC

In this contribution, we discussed the coverage of sTTI control channels and proposed to correct the coverage restriction for {2, 2} combination.

sTTI coverage

For a UE configured with {2,2} sTTI length, the required SNRs for downlink and uplink control channels are shown in Table 1 and the simulation assumptions are listed in appendix A.1.

Table 1. Required SNR

| Channel | Required SNR |
| --- | --- |
| Subslot sPDCCH with AL=4 (to achieve 1% BLER) | -2.8dB |
| Subslot sPDCCH with AL=8 (to achieve 1% BLER) | -5.8dB |
| subslot sPUCCH carrying 1 bit (by format 1a) (to achieve 1% ACK miss) | 1.1 dB |
| subslot sPUCCH carrying 2 bit (by format 1b) (to achieve 1% ACK miss) | 2.5 dB |

Comparing with the SINR distribution of the physical channels in Appendix A.2, which are evaluated based on simulation assumptions in Appendix A.1, the coverage of the downlink and uplink control channels can be summarized in Table 2.

Table 2. Channel coverage

| Channel | Ratio of UE in coverage |
| --- | --- |
| Subslot sPDCCH with AL=4 (to achieve 1% BLER) | 95% |
| Subslot sPDCCH with AL=8 (to achieve 1% BLER) | >98% |
| subslot sPUCCH carrying 1 bit (by format 1a) (to achieve 1% ACK miss) | 72% |
| subslot sPUCCH carrying 2 bit (by format 1b) (to achieve 1% ACK miss) | 56% |

It can be shown that while subslot sPDCCH with aggregation level 8 can cover more than 98% of the UEs, subslot sPUCCH carrying 1 bit can cover only 72% UE, which is a significant restriction on coverage.

Observation 1: The subslot sPUCCH carrying 1 bit HARQ-ACK can cover 72% UE, while subslot sPDCCH with AL=8 can cover more than 98% UE.

To correct the coverage restriction, there can be two options:

- Configure the UE to transmit slot-SPUCCH.

- Increase the subslot sPUCCH coverage by repetition.

Although slot-SPUCCH can be supported in spec now, there are some key drawbacks:

From latency point of view, slot-SPUCCH results in larger latency for both downlink and uplink, as if SPUCCH is configured as slot then sPUSCH has to be configured slot also. In addition, compared with 3-time sPUCCH repetition, slot-SPUCCH has larger latency as the HARQ-ACK needs to wait for next slot, thus the waiting time of HARQ-ACK is larger than sPUCCH repetition. There is also larger waiting time for sPUSCH in the same reason.

From scheduling flexibility point of view, with sPUCCH repetition, the repetition number can be configured according to the UE’s channel. As subslot-SPUCCH with 2 repetitions can cover most of UEs, the latency and SPUCCH overhead can be reduced compared with slot-SPUCCH.

From configuration point of view, configuring one carrier to slot-SPUCCH means all carriers in one PUCCH group have to be configured to slot duration for uplink transmission also. With SPUCCH repetition, the carriers of both Pcell and Scell can still use subslot-sPUSCH for less latency.

From UE capability point of view, {2, 2}-capable UEs may not be able to support {2, 7}.

Therefore, based on the above analysis, we proposed that:

Proposal 1: Subslot-PUCCH repetition should be supported to correct the coverage restriction of {2, 2} combination.

sPUCCH repetition

Text proposal

In legacy LTE, HARQ-ACK repetition is configured by a UE specific RRC signaling. Following the same principle, as shown in Figure 1, a UE configured with NANRep times subslot-SPUCCH repetition will transmit its SPUCCH in subslot #n ~ #(n+ NANRep -1) if  it receives an SPDSCH in subslot #n-Xp , where Xp is timing between sPDSCH and HARQ-ACK feedback.

Figure 1. Illustration of subslot-PUCCH repetition

Following the same principle, subslot-SPUCCH repetition can re-use the legacy LTE PUCCH repetition principles. Therefore, we have following text proposal:

Proposal 2: Endorse the following text proposal to 36.213:

------------------------------------------------------Text proposal to 36.213--------------------------------------------

- 10.1.4	HARQ-ACK Repetition procedure

For a non-BL/CE UE, HARQ-ACK repetition is enabled or disabled by a UE specific parameter ackNackRepetition configured by higher layers for subframe PUCCH, or by a UE specific parameter ackNackRepetitionSTTI configured by higher layers for subslot PUCCH. Once enabled, the UE shall repeat any HARQ-ACK transmission with a repetition factor , where  is provided by higher layers and includes the initial HARQ-ACK transmission, until HARQ-ACK repetition is disabled by higher layers. For a PDSCH transmission without a corresponding PDCCH/EPDCCH/SPDCCH detected, the UE shall transmit the corresponding HARQ-ACK response  times using PUCCH resource  configured by higher layers if HARQ-ACK response is transmitted on subframe-based PUCCH, and using PUCCH resource derived in Subclause 10.1.2.1 if HARQ-ACK response is transmitted on subslot-based PUCCH. For a PDSCH transmission with a corresponding PDCCH/EPDCCH/SPDCCH detected, or for a PDCCH/EPDCCH/SPDCCH indicating downlink SPS release, the UE shall first transmit the corresponding HARQ-ACK response once using PUCCH resource derived from the corresponding PDCCH CCE index or EPDCCH ECCE index (as described in Subclauses 10.1.2 and 10.1.3) if HARQ-ACK response is transmitted on subframe-based PUCCH, and using PUCCH resource derived in Subclause 10.1.2.1 if HARQ-ACK response is transmitted on subslot-based PUCCH, and repeat the transmission of the corresponding HARQ-ACK response  times always using PUCCH resource , where is configured by higher layers.

HARQ-ACK repetition is only applicable for UEs configured with one serving cell for FDD and TDD. For TDD, HARQ-ACK repetition is only applicable for HARQ-ACK bundling.

HARQ-ACK repetition can be enabled with PUCCH format 1a/1b on two antenna ports. For a UE configured for two antenna port transmission for HARQ-ACK repetition with PUCCH format 1a/1b, a PUCCH resource value  maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port .

------------------------------------------------------End of text proposal to 36.213------------------------------------

HARQ-ACK bundling

Even configured with {2, 7}, the uplink control channel coverage is reduced compared to 1ms PUCCH as the number of symbol is reduced. To further correct the coverage imbalance between downlink and uplink control channels, we propose that HARQ-ACK bundling between DL sPDSCH should be supported.

Proposal 3: HARQ-ACK bundling across DL sPDSCH is supported when configured with {2,7} sTTI operation.

Proposal 4: Endorse the following text proposal to 36.212 and 36.213.

------------------------------------------------------Text proposal to 36.212--------------------------------------------

5.2.3.1A	Channel coding for UCI HARQ-ACK on SPUCCH

<unchanged parts are omitted>

For FDD with at least one cell configured with higher layer parameter dl-TTI-Length or for the aggregation of more than one DL cell including a primary cell using FDD and at least one secondary cell using TDD and at least one DL cell configured with higher layer parameter dl-TTI-Length, define the bit sequence  corresponding to PDSCH with slot/subslot duration as the result of the concatenation of HARQ-ACK bits for one or multiple DL cells configured with higher layer parameter dl-TTI-Length,  as the number of cells configured with dl-TTI-Length for the UE and  as the number of slot(s) or subslot(s) for which the UE needs to feed back HARQ-ACK bits in UL slot or subslot n for the c-th serving cell. For a cell using FDD and configured with dl-TTI-Length=subslot and ul-TTI-Length=subslot, or for a cell using FDD and configured with dl-TTI-Length=slot and ul-TTI-Length=slot, . For a cell using FDD and configured with dl-TTI-Length=subslot and ul-TTI-Length=slot, . For a cell using TDD, if slot n-4 in the cell is in a DL subframe, or slot n-4 is the first slot in a special subframe with special subframe configurations 1/2/3/4/6/7/8/9/10 and normal downlink CP, or slot n-4 is the second slot in a special subframe with special subframe configurations 3/4/8 and normal downlink CP, and  otherwise.

The bit sequence  is performed according to the following pseudo-code:

Set c = 0 – cell index: lower indices correspond to lower RRC indices of corresponding cell configured with dl-TTI-Length

Set j = 0 – HARQ-ACK bit index

while c <

set l = 0;

if configured with dl-TTI-Length=subslot and ul-TTI-Length=slot

binary AND operation of the HARQ-ACK bits corresponding to the subslot PDSCH of this cell that are to be feedback in this slot

j = j + 1

else

while l <

HARQ-ACK bit of this cell

j = j + 1

l=l+1

end while

end if

c = c + 1

end while

------------------------------------------------------End of text proposal to 36.212------------------------------------

------------------------------------------------------Text proposal to 36.213--------------------------------------------

- 7.3.1	FDD HARQ-ACK reporting procedure

<unchanged parts are omitted>

If a UE is configured with higher layer parameter codebooksizeDetermination-r13 = dai and if the UE transmits HARQ-ACK using PUCCH format 3 or PUCCH format 4 or PUCCH format 5 in subframe/slot n, the UE shall determine the  according to the following pseudo-code:

Set c = 0 – cell index: lower indices correspond to lower RRC indices of corresponding cell

Set j = 0

Set s = 0

Set

Set

Set  to the number of cells configured by higher layers for the UE

while s < 6

while c <

if s = 0 and the UE is not configured with higher layer parameter dl-TTI-Length for serving cell c and if there is a PDSCH on serving cell c associated with PDCCH/EPDCCH or there is a PDCCH/EPDCCH indicating downlink SPS release on serving cell c for which HARQ-ACK is transmitted in subframe n, or

if the UE is configured with higher layer parameter dl-TTI-Length='subslot' and ul-TTI-Length='slot' for serving cell c and if there is a PDSCH on subslot s of serving cell c associated with PDCCH/SPDCCH or there is a PDCCH/SPDCCH indicating downlink SPS release on subslot s of serving cell c with corresponding HARQ-ACK is transmitted in slot n,

if

end if

if

j = j+1

end if

if the higher layer parameter spatialBundlingPUCCH is set FALSE and the UE is configured with a transmission mode supporting two transport blocks in at least one configured serving cell,

= HARQ-ACK bit corresponding to the first codeword of this cell

= HARQ-ACK bit corresponding to the second codeword of this cell

elseif the higher layer parameter spatialBundlingPUCCH is set TRUE and the UE is configured with a transmission mode supporting two transport blocks in at least one configured serving cell,

= binary AND operation of the HARQ-ACK bits corresponding to the first and second codewords of this cell

else

= HARQ-ACK bit of this cell if the UE is not configured with higher layer parameter dl-TTI-Length for the cell, or

= binary AND operation of the HARQ-ACK bit for subslot s of this cell if the UE is configured with higher layer parameter dl-TTI-Length='subslot' and ul-TTI-Length='slot'.

end if

end if

c = c + 1

end while

s = s + 1

end while

if

j = j+1

end if

<unchanged parts are omitted>

- 10.1.2	FDD HARQ-ACK feedback procedures

For FDD and for a UE not configured with PUCCH format 4/5 and transmitting HARQ-ACK using PUCCH format 1b with channel selection or PUCCH format 3, the UE shall determine the number of HARQ-ACK bits, , based on the number of configured serving cells and the downlink transmission modes configured for each serving cell. The UE shall use two HARQ-ACK bits for a serving cell configured with a downlink transmission mode that support up to two transport blocks; and one HARQ-ACK bit otherwise.

If a UE is configured with higher layer parameters ShortTTI-Length, and codebooksizeDetermination= cc, and codebooksizeDeterminationsSTTI= cc for a PUCCH group and transmitting slot/subslot-based PUCCH, the UE shall determine the number of HARQ-ACK bits, , based on the number of configured serving cells for subframe-based PDSCH, number of configured serving cells for slot/subslot-based PDSCH and the downlink transmission modes configured for each serving cell. If a UE is configured with dl-TTI-Length=subslot and ul-TTI-Length=slot for a serving cell, HARQ-ACK bundling is performed per PDSCH across three subslots associated with a single uplink slot by a logical AND operation of all the individual PDSCH transmission (with and without corresponding PDCCH/SPDCCH) HARQ-ACKs and ACK in response to PDCCH/SPDCCH indicating downlink SPS release.

------------------------------------------------------End of text proposal to 36.212------------------------------------

Conclusion

In this contribution, we discussed the coverage of sTTI and have following observation and proposals:

Observation 1: The subslot sPUCCH carrying 1 bit HARQ-ACK can cover 72% UE, while subslot sPDCCH with AL=8 can cover more than 98% UE.

Proposal 1: Subslot-PUCCH repetition should be supported to correct the coverage restriction of {2, 2} combination.

Proposal 2: Endorse the text proposal to 36.213 in section 3.1.

Proposal 3: HARQ-ACK bundling across DL sPDSCH is supported when configured with {2,7} sTTI operation.

Proposal 4: Endorse the text proposal to 36.212 and 36.213 in section 4.

- Appendix A: simulation assumptions

- A.1 Simulation Assumption

Table 3. Link-level simulation assumptions for sPUCCH

| Carrier frequency | 2 GHz |
| --- | --- |
| System bandwidth | 10MHz |
| TTI length | 2 symbols |
| Channel model | EPA 3km/h, ETU 60km/h |
| Antenna configuration | 1Tx (UE), 2Rx (eNB) |
| CP length | Normal |
| Transmission mode | TM1 |
| Number of Multiplexing UE | 1 |
| Receiver type | MMSE |
| Channel estimation | Practical |
| Link adaptation | Disabled |
| HARQ retransmission | Disabled |
| Performance metrics | BLER |

Table 4. System-level simulation assumptions

| Parameters | Configuration |
| --- | --- |
| Carrier frequency for evaluation | 700 MHz |
| BS antenna height | 25 m |
| Total transmit power per TRxP | 49 dBm for 20 MHz bandwidth 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm |
| min UE power | -40 dBm (see 3GPP TS 36.101) |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) |
| Inter-site distance | 500 m |
| Number of antenna elements per TRxP | 8 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 2 TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| Number of UE antenna elements | 2 Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) 0°, 90° polarization |
| Number of TXRU per UE | 2TXRU (1-to-1 mapping) |
| Device deployment | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | 3 km/h for indoor and 30 km/h for outdoor |
| Inter-site interference modeling | Explicitly modelled |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS antenna element gain | 8 dBi |
| BS antenna element pattern | See Table 1 in Section 3.6 |
| UE antenna element gain | 0 dBi |
| UE antenna element pattern | Omni-directional |
| Thermal noise level | -174 dBm/Hz |
| Traffic model | Full buffer |
| Simulation bandwidth | 20 MHz |
| UE density | 10 UEs per TRxP |
| UE antenna height | 1.5 m |
| Channel model variant | Channel model B |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 98° in LCS |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) |
| TRxP boresight | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m |
| Traffic model | Full buffer (Note: it is for SINR CDF distribution derivation) |
| UL PUSCH power control parameters | α=1.0, P0,PUSCH=-106 (suggested value for UL SINR CDF distribution derivation and calibration) Other values are not precluded. If other values are used, it shall be reported. |
| UL PUCCH power control parameters | P0,subframe-PUCCH=-116 P0,slot-SPUCCH=-113 P0,subslot-SPUCCH=-108 (suggested value for UL SINR CDF distribution derivation and calibration) |
| Bandwidth allocation | * PUSCH: For system level simulations, the system bandwidth on the UL is equally split between the number of UEs simulated. Each UE in each TTI/sTTI will be allocated 10 RBs (assuming 10 UE per sector and 100 RB system bandwidth) in a round-robin fashion. • Note: This does not impact the RB allocations assumed for the link level simulations • Working Assumption taken at RAN1#91 * PUCCH: 1 RB (To get a full load SINR for PUCCH, the same mutual interferers as for PUSCH are assumed but on a bandwidth of 1 RB) |

- A.2 SINR distribution of different physical channels

Figure 3 SINR distribution for subslot sPDCCH

Figure 4 SINR distribution for subslot-sPUCCH