3GPP TSG-RAN WG1 Meeting RAN1 93	R1-1807333

Busan, Korea

21st May – 25th May 2018.

Agenda item:	7.1.1.4.2

Source: 	Qualcomm Incorporated

Title: 	Remaining details on RACH procedure

Document for:	Discussion/Decision

# 1. Introduction

This contribution focuses on several aspects of RACH procedure, including search space of RACH procedure, PRACH configurations for CFRA, etc.

# 2. RACH Procedure and TA adjustment Timeline

Proposal 1:

Confirm the working assumption of the following agreement.

- For PDCCH ordered CFRA, the minimum timing gap between PDCCH order reception and Msg1 transmission is

  - N2+BWPSwitching+ Delay,

    - If BWP switching is not required, BWPSwitching=0; otherwise, BWPSwitching is up to RAN4

    - N2 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

    - N2 is dependent on the SCS of Msg1

      - Working assumption: For Msg1 with 1.25 kHz or 5 kHz SCS, calculation of N2 uses 15 kHz SCS.

    - Delay includes at least MAC layer delay in initializing PRACH

      - Value of Delay is FFS (to be decided in RAN1) Delay = 250 us for FR2, Delay = 500 us for FR1

Proposal 2:

Confirm the following working assumption.

For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

- N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1

- N2 also refers to the value determined in control session with UE capability #1

- L2 refers to the MAC processing latency and is equal to 0.5 msec.

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS

- Note: Spec captures the value k for different SCS

- UE is not expected that UL transmissions in the same slot shall apply different TA values.

- If this happens, it is up to UE’s implementation.

•	FFS: the reference SCS of slot n+k and the time duration of (k-1) slots when UL and DL have different SCSs.

# 3. Default search space during RACH procedure

RAN1 made the following offline conclusion in the last meeting.

38.213 describes the default search space of Msg 2/3/4 in the following way: “If a UE is not provided higher layer parameter ra-SearchSpace for Type1-PDCCH common search space, the association between monitoring occasions for Type1-PDCCH common search space and the SS/PBCH block index is the same as the association of monitoring occasions for Type0-PDCCH common search space as described in Subclause 13.”

However, using RMSI search space exactly in RACH search space in practically infeasible. When SSB and RMSI are time division multiplexed, UE assumes the periodicity of RMSI to be 20 ms. That means, that the search space of RMSI corresponding to an SSB repeats after every 20 ms. The duration of RAR window, on the other hand, can be at most 10 ms. RAN1 has already agreed that the DMRS of Msg2 is QCLed with the SSB that selected during Msg1 transmission. Hence, the search space of RMSI corresponding to the SSB that UE selected may not be located within the RMSI window.

Observation 1: When SSB and RMSI are time division multiplexed, UE assumes the periodicity of RMSI to be 20 ms. The duration of RAR window, on the other hand, can be at most 10 ms. Hence, the search space of RMSI corresponding to the SSB that UE selected may not be located within the RAR window.

In order to tackle this issue, RAN1 made the following offline conclusion in the last meeting.

- Following alternatives are considered while re-designing default search space of Msg 2/3/4: - Alt A: Use symbol 0 : (X-1) in every slot within the window - X is determined based on duration of Msg 2/3/4 CORESET - Alt B: Use first symbol index symbol locations within a slot from RMSI search space, configured in MIB, but apply it in every slot - In case there are multiple search spaces sets in a slot, (corresponding to different SSBs), a UE monitors only one is monitored - FFS: Which one - Alt 1: The search space set associated with SSB used for corresponding Msg1 transmission - Alt 2: The search space with the smallest first symbol index within a slot - Alt 3: search space depends on RA-RNTI - Other alternatives are not precluded. - How to handle the scenario when a search space overlaps in time with an actually transmitted SSB other than the SSB used for corresponding Msg1 transmission - If a search space overlaps in time with UL part, the search space in that particular slot is invalid

With Alt 2, UE has to monitor a different search space for RMSI and Msg 2/3/4. Alt 1 allows UE to keep the same search space to receive RMSI and Msg 2/3/4 grant. Besides, alt 1 gives network the flexibility to use different search spaces for Msg 2/3/4 grant within a slot corresponding to different SSBs.

Observation 2: Alt 1 gives network the flexibility to use different search spaces for Msg 2/3/4 grant within a slot corresponding to different SSBs.

Observation 3: When slots containing search spaces for RMSI and Msg 2/3/4 grants overlap, alt 2 forces UE to monitor a different search space symbols within the slot for RMSI and Msg 2/3/4. Alt1 allows UE to monitor the same search space symbols within the slot to receive RMSI and Msg 2/3/4 grant.

Note that, all these alternatives may lead to a scenario where Msg 2/3/4 search space gets FDMed with a set of SSBs that are not QCLed with the Msg 2/3/4.

Figure: Time domain overlapping between search space of Msg2 and an SSB that UE did not select during corresponding Msg1 transmission

The above figure shows this issue in more details. Assume that Gnb is transmitting two SSBs. SSB locations and RACH occasions are located at the beginning and end of RACH configuration period respectively. Also assume that backoff period is zero. That means, UE can retransmit Msg1 at the end of the RAR window.

UE selects SSB2 and its corresponding RACH occasion to transmit Msg1. The RAR window is overlapped with SSB 1 and SSB 2 locations. Since the DMRS of Msg2 grant will be QCLed with SSB2, UE generates its RX beam to receive Msg2 grant whose DMRS is QCLed with SSB 2. However, this prohibits UE to track SSB 1 before its next Msg1 retransmission opportunity. The same thing happens during every RAR window. UE cannot track the other SSBs that it did not select initially during its entire retransmission period.

Observation 4: If the search space of Msg 2/3/4 overlaps with an SSB that is not QCLed with the DMRS of Msg 2/3/4, UE may not be able to track that SSB during RACH retransmission period.

This can be solved by ensuring that time domain overlapping between search space of Msg 2/3/4 grant can only overlap with an SSB that the UE selected for corresponding Msg1 transmission. If a search space overlaps in time with an actually transmitted SSB other than the SSB used for corresponding Msg1 transmission, search space in that particular slot becomes invalid.

Figure: Avoiding time domain overlapping between search space of Msg2 and a non-QCLed SSB

Proposal 3:

Default search space of Msg 2/3/4 uses the first symbol index within a slot from RMSI search space, configured in MIB, but applies it in every slot

- In case there are multiple search spaces sets in a slot, (corresponding to different SSBs), a UE only monitors the search space set associated with SSB used for corresponding Msg1 transmission

- If a search space overlaps in time with an actually transmitted SSB other than the SSB used for corresponding Msg1 transmission or with UL part, the search space in that particular slot becomes invalid.

# 4. Dedicated RACH occasions for CFRA

RAN2 has agreed that network can configure PRACH configuration indices differently between contention based random access and contention free random access. To accommodate this, RAN2 has included prach-ConfigurationIndex in RACH-ConfigGeneric.

Scenario 1 and scenario 2 of Figure 2 correspond to CBRA and CFRA RACH occasion patterns and show how these can be configured differently.

Figure 2: Use of different prach-ConfigurationIndex for CBRA and CFRA

However, network may not configure UE CFRA resources for all of its actually transmitted SSBs. Hence, the prach-ConfigurationIndex that’s configured for CFRA, does not need to hold RACH occasions for all actually transmitted SSBs. Network should be able to configure the set of actually transmitted SSBs differently for CBRA and CFRA so that the UE can find RACH resources more frequently for initial CFRA Msg1 transmission and subsequent retransmission. 

A comparison between scenario 2 in Figure 2 and scenario 3 in figure 3 explain this issue. Consider a scenario where the neighbour cell transmits SSB index #1-#4 but UE detects only SSB index 3 and 4. Each SSB is mapped to one RACH occasion and all RACH occasions are time division multiplexed. For handover, Network configures dedicated CFRA preambles, corresponding to SSB index 3 and 4, and a prach-ConfigurationIndex that is different from the prach-ConfigurationIndex configured for CBRA. Although UE is allowed to transmit CFRA corresponding to SSB #3 and #4, the opportunity to transmit CFRA corresponding to SSB #3 and #4 arise after every four RACH occasions. This happens because the mapping from SSB to RACH occasions in this dedicated prach-ConfigurationIndex, which is different from the CBRA prach-ConfigurationIndex, still uses the fact that four SSBs get actually transmitted.

However, UE has not been configured CFRA preambles corresponding to SSB #1 and #2. Hence, this dedicated RACH configuration pattern does not need to hold RACH occasions corresponding to SSB #1 and #2 and increase RACH latency.

Network can solve this issue and provide more frequent opportunities for RACH occasions corresponding SSB #3 and #4 simply by informing UE, for CFRA purposes, that only two SSBs (#3 and #4) are actually transmitted. In that case, the RACH occasions corresponding to SSB #3 and #4 will appear in every other RACH occasion. This is shown in Figure 3 and it will reduce handover latency further.

Figure 3

Proposal 4: For CFRA, NR allows network to configure a different set of actually transmitted SSBs to UEs.

- This can be achieved by including ssb-PositionsInBurst IE within RACH-configGeneric IE.

# 5. Decoding P-RNTI PDSCH during RACH procedure

In the offline summary [2] it was noted that the decoding capability of the baseline UE should be maintained in the IDLE state similar to as in the RRC_CONNECTED state, where at most 2 TBs may be handled simultaneously. Further clarification to the proposal in [2] was needed to handle the prioritization of these TBs in IDLE state, namely SI-RNTI, P-RNTI, and RA-RNTI/TC-RNTI. The following proposal provides this prioritization.

Proposal 5: In Rel-15, UE is not required to decode paging PDSCH during RACH procedure.

# 6. CRs regarding SSB to RACH mapping and RACH power control

38.331 shows numberOfRA-PreamblesGroupA to range from 1 to 64. However, RAN1 made the following agreement regarding this parameter in RAN1 92 and 38.331 should reflect RAN1’s agreement.

Agreements: - ....................... - Regarding the ffs value for parameter numberOfRA-PreamblesGroupA - For SSB-per-rach-occasion {1/8, 1/4, ½, 1, 2}: numberOfRA-PreamblesGroupA is selected from the set of values 4*N, with N=1, …, floor(16/max(1, SSB-per-rach-occasion)) - For SSB-per-rach-occasion {4, 8, 16}: numberOfRA-PreamblesGroupA is selected from the set of values N, with N=1, …, floor(64/SSB-per-rach-occasion) - Note: This denotes the number of GroupARachPreambles per SSB in one RACH occasion.

Proposal 6: Adopt the following CR for 38.331 and send an LS to RAN2.

CHANGE START

-- ASN1START

-- TAG-RACH-CONFIG-COMMON-START

RACH-ConfigCommon ::= 				SEQUENCE {

rach-ConfigGeneric			RACH-ConfigGeneric,

totalNumberOfRA-Preambles			INTEGER (1..63)																		OPTIONAL,	-- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}																														OPTIONAL,	-- Need M

groupBconfigured 					SEQUENCE {

ra-Msg3SizeGroupA					ENUMERATED { b56, b144, b208, b256, b282, b480, b640,

b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB			ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA			INTEGER (1..64)

ssb-perRACH-OccasionAndnumberOfRA-PreamblesGroupAPerSSBPerRACH-Occasion	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}

}																														OPTIONAL,	-- Need R

ra-ContentionResolutionTimer			ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB						RSRP-Range																		OPTIONAL,	-- Need R

rsrp-ThresholdSSB-SUL					RSRP-Range																		OPTIONAL,	-- Cond SUL

prach-RootSequenceIndex					CHOICE {

l839									INTEGER (0..837),

l139									INTEGER (0..137)

},

msg1-SubcarrierSpacing					SubcarrierSpacing,

restrictedSetConfig						ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoding					ENUMERATED {enabled}															OPTIONAL,	-- Need R

...

}

CHANGE END

# 7. TBS determination for Msg 2/3/4

During the last meeting, RAN1 made the following agreement:

Agreements: - For PDSCH scheduled by P-RNTI (working assumption), and RA-RNTI, scaling factor is applied in the calculation of Ninfo of TBS determination - (Working assumption) values of scaling factor: 1, ½, ¼ - FFS the possibility of adding another value, e.g., 1/8 or 1/16

The RAR payload per is 56 per UE. Since RAR does not have HARQ, it is preferable to receive the RAR in the very first transmission to reduce latency. The RAR payload can be as low as 56 bits if only one UE is signalled.

Agreements: - The number of bits for RAR is 56 for a UE. Send LS to RAN2 – R1-1803435, which is approved by updating “if need” to “if needed”, and the final LS is in R1-1803475

Below is a simulation result of RAR reception performance showing with current NR spec with just one transmission. The SNR required to attain a 1% BLER for the no CFO error case is > 0dB. Even with 4 retransmissions the SNR attainable is -5dB. The performance is even further degraded when CFO error in initial access is taken into consideration.

Notice the limitation in coverage is not due to lack of sufficient REs in the initial BWP of a slot. The loss is largely because for a small payload of 56 bits, only a certain number of REs based on the lowest code rate can be transmitted. Even if we pad zeros to the TBS to span more REs the performance may not improve due to limitation on lowest code rate.  To improve coverage to reach cell edge target of -6dB, we propose to change the TBS determination rule such that RAR can be decoded at a lower SNR. Our proposal is to apply a scaling factor to the rate determined by the signalled MCS to attain a lower SNR. Some of the reserved fields can be repurposed to signal the scaling factor. A similar issue occurs for Msg3 transmission also. Even though HARQ is enabled for Msg3, from a latency improvement it will be beneficial to apply repetition. The TBS determination procedure can be modified to enhance performance. Based on the above discussions we have the following proposals

Proposal 7: For a PDSCH scheduled by a DCI whose CRC is scrambled by RA-RNTI, a scaling factor can be applied to N_info in TBS calculation (Subclause 5.1.3.2 in 38.214). The scaling factor can be signalled via a subset of the reserved fields. Recommended scaling factors to be signalled are {1, 1/2, 1/4, 1/8}.

Proposal 8: For Msg3 Transmission scheduled by the UL grant carried by RAR, a scaling factor can be applied to N_info in TBS calculation (Subclause 5.1.3.2 in 38.214). The scaling factor can be signalled via a subset of the reserved fields in the UL grant. Recommended scaling factors to be signalled are {1, 1/2,1/4, 1/8}.

# 7. Conclusion

Observation 1: When SSB and RMSI are time division multiplexed, UE assumes the periodicity of RMSI to be 20 ms. The duration of RAR window, on the other hand, can be at most 10 ms. Hence, the search space of RMSI corresponding to the SSB that UE selected may not be located within the RAR window.

Observation 2: Alt 1 gives network the flexibility to use different search spaces for Msg 2/3/4 grant within a slot corresponding to different SSBs.

Observation 3: When slots containing search spaces for RMSI and Msg 2/3/4 grants overlap, alt 2 forces UE to monitor a different search space symbols for RMSI and Msg 2/3/4. Al1 allows UE to monitor the same search space symbols to receive RMSI and Msg 2/3/4 grant.

Observation 4: If the search space of Msg 2/3/4 overlaps with an SSB that is not QCLed with the DMRS of Msg 2/3/4, UE may not be able to track that SSB during RACH retransmission period.

Proposal 1:

Confirm the working assumption of the following agreement.

- For PDCCH ordered CFRA, the minimum timing gap between PDCCH order reception and Msg1 transmission is

  - N2+BWPSwitching+ Delay,

    - If BWP switching is not required, BWPSwitching=0; otherwise, BWPSwitching is up to RAN4

    - N2 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1

    - N2 is dependent on the SCS of Msg1

      - Working assumption: For Msg1 with 1.25 kHz or 5 kHz SCS, calculation of N2 uses 15 kHz SCS.

    - Delay includes at least MAC layer delay in initializing PRACH

      - Value of Delay is FFS (to be decided in RAN1) Delay = 250 us for FR2, Delay = 500 us for FR1

Proposal 2:

Confirm the following working assumption.

For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where

- N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1

- N2 also refers to the value determined in control session with UE capability #1

- L2 refers to the MAC processing latency and is equal to 0.5 msec.

- TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS

- Note: Spec captures the value k for different SCS

- UE is not expected that UL transmissions in the same slot shall apply different TA values.

- If this happens, it is up to UE’s implementation.

•	FFS: the reference SCS of slot n+k and the time duration of (k-1) slots when UL and DL have different SCSs.

Proposal 3:

Default search space of Msg 2/3/4 uses the first symbol index within a slot from RMSI search space, configured in MIB, but applies it in every slot

- In case there are multiple search spaces sets in a slot, (corresponding to different SSBs), a UE only monitors the search space set associated with SSB used for corresponding Msg1 transmission

- If a search space overlaps in time with an actually transmited SSB other than the SSB used for corresponding Msg1 transmission or with UL part, the search space in that particular slot becomes invalid.

Proposal 4: For CFRA, NR considers allowing network to configure a different set of actually transmitted SSBs to UEs.

- This can be achieved by including ssb-PositionsInBurst IE within RACH-configGeneric IE.

Proposal 5: In Rel-15, UE is not required to decode paging PDSCH during RACH procedure.

Proposal 6: Adopt the following CR for 38.331 and send an LS to RAN2.

CHANGE START

-- ASN1START

-- TAG-RACH-CONFIG-COMMON-START

RACH-ConfigCommon ::= 				SEQUENCE {

rach-ConfigGeneric			RACH-ConfigGeneric,

totalNumberOfRA-Preambles			INTEGER (1..63)																		OPTIONAL,	-- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}																														OPTIONAL,	-- Need M

groupBconfigured 					SEQUENCE {

ra-Msg3SizeGroupA					ENUMERATED { b56, b144, b208, b256, b282, b480, b640,

b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB			ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA			INTEGER (1..64)

ssb-perRACH-OccasionAndnumberOfRA-PreamblesGroupAPerSSBPerRACH-Occasion	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}

}																														OPTIONAL,	-- Need R

ra-ContentionResolutionTimer			ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB						RSRP-Range																		OPTIONAL,	-- Need R

rsrp-ThresholdSSB-SUL					RSRP-Range																		OPTIONAL,	-- Cond SUL

prach-RootSequenceIndex					CHOICE {

l839									INTEGER (0..837),

l139									INTEGER (0..137)

},

msg1-SubcarrierSpacing					SubcarrierSpacing,

restrictedSetConfig						ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoding					ENUMERATED {enabled}															OPTIONAL,	-- Need R

...

}

CHANGE END

Proposal 7: For a PDSCH scheduled by a DCI whose CRC is scrambled by RA-RNTI, a scaling factor can be applied to N_info in TBS calculation (Subclause 5.1.3.2 in 38.214). The scaling factor can be signalled via a subset of the reserved fields. Recommended scaling factors to be signalled are {1, 1/2, 1/4, 1/8}.

Proposal 8: For Msg3 Transmission scheduled by the UL grant carried by RAR, a scaling factor can be applied to N_info in TBS calculation (Subclause 5.1.3.2 in 38.214). The scaling factor can be signalled via a subset of the reserved fields in the UL grant. Recommended scaling factors to be signalled are {1, 1/2,1/4, 1/8}.

# 8. References

1. 3GPP TSG Chairman’s Notes RAN1 92bis.

2. R2-1806391, “Miscellaneous EN-DC Corrections” to 38.331.

3. 3GPP TSG Chairman’s Notes RAN1 92.