3GPP TSG RAN WG1 Meeting #93	R1-1807370

Busan, Korea, May 21th – May 25th, 2018

Agenda item:	7.1.3.5

Source: 	Qualcomm Incorporated

Title: 	Remaining issues on PDSCH rate matching

Document for:	Discussion/Decision

# Introduction

In this contribution we discuss remaining open issues on PDCCH structure as follows:

- Spec Clarification for PDSCH Rate-Matching

# Spec Clarification for PDSCH Rate-Matching

In Section 5.1.4.1 of the 38.214 spec [1], there is below text for PDSCH resource mapping with RB symbol level granularity.

====================================Spec Text Start======================================

A configured group rateMatchPatternGroup1 or rateMatchPatternGroup2 contains a list of RB symbol level resource set indices forming a union of resource-sets not available for PDSCH dynamically if corresponding bit in the PDCCH with a scheduling DCI. The REs corresponding to the union of configured RB-symbol level resource-sets that are not included in neither of two groups are not available for PDSCH.

====================================Spec Text End=======================================

The multiple negations shown in the above highlighted sentence is very confusing. It would be much clearer if it is changed to “The REs corresponding to the union of configured RB-symbol level resource-sets that are included in either of two groups are not available for PDSCH.”

We have the below text proposal for this change.

======================================TP Start=========================================

A configured group rateMatchPatternGroup1 or rateMatchPatternGroup2 contains a list of RB symbol level resource set indices forming a union of resource-sets not available for PDSCH dynamically if corresponding bit is in the PDCCH with a scheduling DCI. The REs corresponding to the union of configured RB-symbol level resource-sets that are included in either of two groups are not available for PDSCH.

======================================TP End==========================================

Proposal 1: Adopt the below TP to clarify the spec in Section 5.1.4.1 of the 38.214.

# Conclusions

In this contribution we have discussed remaining open issues on PDCCH structure and have made the following proposal:

Proposal 1: Adopt the below TP to clarify the spec in Section 5.1.4.1 of the 38.214.

======================================TP Start=========================================

A configured group rateMatchPatternGroup1 or rateMatchPatternGroup2 contains a list of RB symbol level resource set indices forming a union of resource-sets not available for PDSCH dynamically if corresponding bit is in the PDCCH with a scheduling DCI. The REs corresponding to the union of configured RB-symbol level resource-sets that are included in either of two groups are not available for PDSCH.

======================================TP End==========================================

# References

- R1-1805796, CR to 38.214 capturing the RAN1#92bis meeting agreements.