3GPP TSG-RAN WG1 Meeting #93 	R1-1807414

# Busan, South Korea, 21st – 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.201 | CR | 0026 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of V2X Phase 2 enhancements into 36.201 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eV2X-Core |  |  |  |  |  |  | Date: |  | 2018-04-04 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of V2X Phase 2 enhancements into 36.201. |  |  |  |  |  |  |  |  |
| Summary of change: |  | V2X Phase 2 enhancements are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | V2X Phase 2 enhancements would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 4.2.2	Physical channels and modulation

The physical channels defined in the downlink are:

-	the Physical Downlink Shared Channel (PDSCH),

-	the Physical Multicast Channel (PMCH),

-	the Physical Downlink Control Channel (PDCCH),

-	the Enhanced Physical Downlink Control Channel (EPDCCH),

-	the MTC Physical Downlink Control Channel (MPDCCH),

-	the Relay Physical Downlink Control Channel (R-PDCCH),

-	the Short Physical Downlink Control Channel (SPDCCH),

-	the Physical Broadcast Channel (PBCH),

-	the Physical Control Format Indicator Channel (PCFICH),

-	the Physical Hybrid ARQ Indicator Channel (PHICH),

-	the Narrowband Physical Broadcast Channel (NPBCH),

-	the Narrowband Physical Downlink Control Channel (NPDCCH),

-	and the Narrowband Physical Downlink Shared Channel (NPDSCH).

The physical channels defined in the uplink are:

-	the Physical Random Access Channel (PRACH),

-	the Physical Uplink Shared Channel (PUSCH),

-	the Physical Uplink Control Channel (PUCCH),

-	the Short Physical Uplink Control Channel (SPUCCH),

-	the Narrowband Physical Random Access Channel (NPRACH),

-	and the Narrowband Physical Uplink Shared Channel (NPUSCH).

The physical channels defined in the sidelink are:

-	the Physical Sidelink Broadcast Channel (PSBCH),

-	the Physical Sidelink Control Channel (PSCCH),

-	the Physical Sidelink Discovery Channel (PSDCH),

-	and the Physical Sidelink Shared Channel (PSSCH).

In addition, signals are defined as reference signals, primary and secondary synchronization signals, and discovery signals.

The modulation schemes supported are:

-	in the uplink, QPSK, 16QAM, 64QAM and 256QAM, and π/2 BPSK and π/4 QPSK for single sub-carrier NB-IoT operation,

-	in the downlink, QPSK, 16QAM, 64QAM and 256QAM,

-	in the sidelink, QPSK, 16QAM and 64QAM.