| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Need of FDD/TDD differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Enh. to LTE operation in unlicensed spectrum (Agreed in R1-1805791) | 1-1 | AUL PUSCH | 1) Configuration of AUL PUSCH 2) Activation/release of AUL PUSCH 3) AUL PUSCH transmission (incl. Possible retransmissions) | UL CA, eLAA | Yes | Network cannot utilize AUL PUSCH transmissions on LAA Scell for such UE | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 1-2 | LAA PUSCH Mode 1 | 1) Configuration for Mode 1 2) PUSCH transmission according to Mode 1 dependent on LBT outcome | UL CA, eLAA | Yes | Network cannot utilize Mode1 PUSCH transmissions on LAA Scell for such UE | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 1-3 | LAA PUSCH Mode 2 | 1) Configuration for Mode 2 2) PUSCH transmission according to Mode 2 dependent on dynamic indication in the DCI | UL CA, eLAA | Yes | Network cannot utilize Mode2 PUSCH transmissions on LAA Scell for such UE | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 1-4 | LAA PUSCH Mode 3 | 1) Configuration for Mode 3 2) PUSCH transmission according to Mode 3 dependent on dynamic indication in the DCI | UL CA, eLAA | Yes | Network cannot utilize Mode3 PUSCH transmissions on LAA Scell for such UE | No need |  |  | RAN1 | Optional with capability signaling |  |
| 2. Further enhancement to multi-point operation in LTE (FeCoMP) | 2-1 | Non coherent joint transmission | 1. Support of QCL type C 2. Separate PDSCH RE mapping indication for different CWs 3. Support of new DMRS port to MIMO layer mapping |  | Yes | UE can’t receive NC-JT transmission | No |  |  | RAN1 | Optional with capability signaling |  |
|  | 2-2 |  | 1. CSI feedback for NC-JT using CRI based reporting | 2-1 | Yes | UE can’t provide CSI information assuming NC-JT from coordinating TPs | No |  |  | RAN1 | Optional with capability signaling |  |
| 3. Enhanced LTE support for aerial vehicles | 3-1 | UL Power Control | 1. Support of UE-specific alpha |  | Yes | Potential UL inter-cell interference from aerial UEs | No |  |  | RAN1 | Optional with capability signaling |  |
|  | 3-2 |  | 1. Support of extended range for P0 parameter |  | Yes | Potential UL inter-cell interference from aerial UEs | No |  |  | RAN1 | Optional with capability signaling |  |
| 4. eV2X | 4-1 | Support of eV2X |  | UE-SL-C? | Yes | eV2X is not supported | No |  |  |  | Mandatory capability is per UE |  |
|  | 4-2 | Support of carrier aggregation |  | 4-1 | Yes | No Rel-15 carrier operation | No |  |  |  | Mandatory if 4-1 capability |  |
|  | 4-3 | Support of 64-QAM |  | 4-1 | Yes | NO 64-QAM support. Use of puncturing instead of rate matching for the last PSSCH symbol | No |  |  |  | Optional |  |
|  | 4-4 | Support of transmit diversity |  | 4-1 | Yes | single antenna reception is assumed by the UE | No |  |  |  | Optional |  |
|  | 4-5 | Support of resource pool sharing between mode-3 and mode-4 Ues |  | 4-1 | Yes, for mode-3 UE scheduling | No resource pool sharing | No |  |  |  | Mandatory if 4-1 capability |  |
|  | 4-6 | Support of low T2 min |  | 4-1 | Yes | Low latency V2X applications cannot be guaranteed | No |  |  |  | Mandatory capability is per UE |  |
| 5. Shortened TTI and PT | 5-1 | Short processing time operation | 1) Configuration of short processing time 2) Support of n+3 processing time between UL DCI and PUSCH transmission 3) Support of n+3 processing time between PDSCH and DL HARQ feedback transmission in PUCCH. * UE capability is reported per band combination. For each band combination the maximum number of supported CCs for short processing time are reported. The reported number of carriers apply to all the FS-type(s) supported. The supported FS-type(s) are also reported per band combination.. * The maximum number of CSI processes for aperiodic CSI is also reported for each band combination |  | yes | Network cannot utilize short processing time | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-2 | EPDCCH with SPT | Support of EPDCCH and short processing time on different serving cells. | 5-1 | yes | Network cannot utilize EPDCCH and SPT on different carriers | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-3 | sTTI operation | 1) Configuration of sTTI related parameters (e.g. sTTI length) 2) sDCI monitoring in PDCCH and CRS based SPDCCH 3) Transmission of SPUCCH in formats 1 and 4 4) Transmission of sublsot/slot based PUSCH on a single antenna port 5) Decoding of CRS based sublsot/slot based PDSCH * different sTTI capabilities for different UL/DL sTTI length combinations and for each band of the reported band combinations. * UE capability is reported on skipping subframe processing to start processing slot/subslot PDSCH/PUSCH * UE capability is reported on the maximum number of blind decodes in USS in one subframe for CCs configured with sTTI operation, including both legacy DCI formats and sDCI * UE capability is reported on the supported processing timeline sets for subslot operation |  | yes | Network cannot schedule slot/subslot data transmission | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-4 | DMRS based SPDCCH | Support of sDCI monitoring in DMRS based SPDCCH | 5-3 | yes | Network cannot utilize DMRS based SPDCCH | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-5 | SPDCCH monitoring on SPDCCH resource sets with different RS types | Support of monitoring sDCI in SPDCCH resource sets with different RS types in a slot/subslot | 5-3 and 5-4 | yes | Network cannot utilize SPDCCH resource sets with different RS types | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-6 | TM8 for slot/subslot PDSCH | 1) configuration of TM8 for slot PDSCH 2) Decoding of TM8 based slot PDSCH. | 5-3 | yes | Network cannot utilize TM8 based slot PDSCH | Only for TDD |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-7 | TM9 and TM10 for slot/subslot PDSCH | 1) configuration of TM9/TM10 for slot/subslot PDSCH 2) Decoding of TM9-10 based slot/subslot PDSCH. | 5-3 | yes | Network cannot utilize TM9/10 based subslot/slot PDSCH | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-8 | Tx Diversity for TM9/10 based slot/subslot PDSCH | 1) configuration of TM9/TM10 for slot/subslot PDSCH 2) Decoding of Tx diversity based subslot/slot PDSCH | 5-3 | yes | Network cannot utilize Tx diversity fallback for TM9 or TM10 | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-9 | SPUCCH format 3 | Support of transmission of SPUCCH in format 3 | 5-3 | yes | Network cannot utilize SPUCCH format 3 | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-10 | Tx Diversity for SPUCCH | SPUCCH format 1/1a/1b/3 transmission with Tx Diversity | 5-3 | yes | Network cannot utilize Tx diversity for SPUCCH format 1/1a/1b/3 | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-11 | TM2 for slot/subslot PUSCH | 1) configuration of TM2 for slot/subslot PUSCH 2) Transmission of sublsot/slot based PUSCH * UE capability is reported on supported maximum number of layers for slot/subslot PUSCH | 5-3 | yes | Network cannot utilize UL TM2 for sulot/subslot PUSCH | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-12 | Simultaneous reception of slot/subslot and subframe PDSCH | Support of simultaneous reception of subframe and slot/subslot PDSCH. | 5-3 | yes | Network cannot utilize simultanenous reception of subframe and slot / subslot PDSCH | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-13 | Simultaneous transmission of PUSCH of different lengths across carriers | Support of simultaneously transmission of PUSCH with different lengths across different carriers * capability reported per band/band combination | 5-3 | yes | Network cannot utilize simultanenous transmission of subslot/slot PUSCH of different lengths in different carriers across carriers | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-14 | A-CSI for sTTI | Support of aperiodic CSI feedback for sTTI * UE capability is reported on the maximum number of CSI processes supported on a CC within a band with slot/sublot PDSCH TM10 for a bandwidth class with a single CC * UE capability is reported on the maximum number of CSI processes supported on a single CC and on all CCs within a band with PDSCH TM10 for a bandwidth class with multiple CCs * UE capability is reported on the maximum number of CSI processes to be updated across CCs | 5-3 | yes | Network cannot utilize CSI on subslot/slot PUSCH | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-15 | EPDCCH with sTTI | Support of EPDCCH and sTTI on different serving cells. | 5-3 | yes | Network cannot utilize EPDCCH and sTTI on different carrier | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-16 | SRS triggers on sDCI | Support of SRS triggers in DCI formats 7.x for FS2 | 5-3 | yes | Network cannot utilize SRS triggers in DCI 7.x | Only for TDD |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-17 | SPS for sTTI | 1) Configuration of SPS with sTTI 2) Monitoring of sDCI for SPS activation and deactivation, 3) Decoding of SPS subslot/slot based PDSCH 4) Transmission of SPS slot/subslot based PUSCH | 5-3 | yes | Network cannot utilize SPS for sTTI | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-18 | L1 based SPDCCH reuse | 1) Configuration of L1 based SPDCCH reuse 2) Rate-matching of subslot/slot based PDSCH around SPDCCH resources according to configured L1 based SPDCCH reuse method | 5-3 | yes | Network cannot utilize L1 indication of rate matching method of sublsot/slot based PDSCH around SPDCCH resources | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 5-19 | CS for SPS | 1) Configuration of cyclic shift in SPS for 1ms TTI UL SPS operation. 2) Transmission of DMRS for SPS PUSCH using the configured cyclic shift | SPS support for 1ms TTI | yes | SPS using different configured CS values not possible | No need |  |  | RAN1 | Optional with capability signaling |  |
| 6. LTE_HRLLC | 6-1 | Subframe PDSCH repetitions | 1) Configuration of subframe PDSCH repetition 2) Operation incl. reception of subframe PDSCH repetition | Semi-static CFI | Yes | Network cannot utilize subframe PDSCH repetitions for such UE | No need |  |  | RAN1 | Optional with capability signalling |  |
|  | 6-2 | Slot PDSCH repetitions | 1) Configuration of slot PDSCH repetition 2) Operation incl. reception of slot PDSCH repetition | 1) Rel-15 slot PDSCH operation 2) semi-static CFI | Yes | Network cannot utilize slot PDSCH repetitions for such UE | No need |  | FFS if this UE capability is dependent of the UL TTI length or the number of supported CCs | RAN1 | Optional with capability signalling |  |
|  | 6-3 | Subslot PDSCH repetitions | 1) Configuration of subslot PDSCH repetition 2) Operation incl. reception of subframe PDSCH repetition | 1) Rel-15 subslot PDSCH operation 2) semi-static CFI | Yes | Network cannot utilize subslot PDSCH repetitions for such UE | No need |  | FFS if this UE capability is dependent of the UL TTI length or the number of supported CCs | RAN1 | Optional with capability signalling |  |
|  | 6-4 | Semi-static CFI | 1) Configuration of CFI for subframe/slot/subslot operation 2) Operation based on configured CFI for subframe/slot/subslot operation |  | Yes | Network cannot utilize semi-static CFI for such UE | No need |  |  | RAN1 | Optional with capability signalling |  |
|  | 6-5 | Subframe PUSCH SPS repetition | 1) Configuration of SPS repetition for subframe PUSCH 2) Operation incl. transmission of subframe SPS UL repetition |  | Yes | Network cannot utilize subframe PUSCH SPS repetition | No need |  | This feature group is conditional to the specification at least one UL SPS repetition scheme in LTE_HRLLC WI. FFS if this UE capability is dependent of the SPS repetition scheme if several schemes are specified | RAN1 | Optional with capability signalling |  |
|  | 6-6 | Slot PUSCH SPS repetition | 1) Configuration of SPS repetition for slot PUSCH 2) Operation incl. transmission of SPS repetition of slot PUSCH | 1) Rel-15 slot PUSCH operation 2) SPS for slot PUSCH | Yes | Network cannot utilize slot PUSCH SPS repetition | No need |  | This feature group is conditional to the specification at least one UL SPS repeittion scheme in LTE_HRLLC WI. FFS if this UE capability is dependent of the SPS repetition scheme if several schemes are specified | RAN1 | Optional with capability signalling |  |
|  | 6-7 | Subslot PUSCH UL SPS repetition | 1) Configuration of SPS repetition for subslot PUSCH 2) Operation incl. transmission of SPS repetition of subslot PUSCH | 1) Rel-15 subslot PUSCH operation 2) SPS for subslot PUSCH | Yes | Network cannot utilize subslot PUSCH SPS repetition | No need |  | This feature group is conditional to the specification at least one UL SPS repeittion scheme in LTE_HRLLC WI. FFS if this UE capability is dependent of the SPS repetition scheme if several schemes are specified | RAN1 | Optional with capability signalling |  |
| 7. High capacity stationary wireless link and introduction of 1024 QAM | 7-1 | Introduction of 1024 QAM for PDSCH | Support of 1024 QAM for PDSCH | NA | Yes | No support of 1024 QAM for PDSCH | Yes |  | RAN1 already agreed that it is optional with capability signaling per BoBC | RAN1 | Optional with capability signaling |  |
|  | 7-2 | DL DMRS overhead reduction | Support of OCC4 for rank 3 and 4 transmission | NA | Yes | NO support of OCC4 for rank 3 and 4 transmission | No |  | UE needs to support TM9/10 | RAN1 | Optional with capability signaling |  |
| 8. Further enhancements of NB-IoT | 8-1 | Wake-up signal | 1. Support of WUS with wake-up time of {X or Y or ….} [milliseconds] before the first associated PO |  | Yes | WUS not sent early enough for UE to wake up before first associated PO. | FDD only |  |  | RAN1 | Optional with capability signaling |  |
|  | 8-2 | Early data transmission | 1. Support of UL data transmission in Msg3 |  | Yes. | From idle mode, UL data cannot be transmitted until after Msg4. | FDD only |  | Implied capability and request signaling will be via NPRACH partitioning. | RAN2 | Leave to RAN2 |  |
|  | 8-3 | Physical layer SR | 1. Support of SR with HARQ-ACK 2. Support of SR without HARQ-ACK |  | Yes | Legacy RACH procedure must be used to request UL resources in connected mode. | FDD only |  |  | RAN1 | Optional with capability signaling |  |
|  | 8-4 | NPRACH range enhancement | 1. Support of FDD NPRACH format 2 |  | Yes | UE range is ambiguous beyond 40 km radius from eNB. | FDD only |  | Capability signaling needed as otherwise NPDCCH order cannot be issued for NPRACH format 2 resources. | RAN1 | Optional with capability signaling |  |
|  | 8-5 | NPRACH reliability enhancement | 1. Support of scrambling NPRACH formats 0/1. 2. Support of scrambling NPRACH format 2, if format 2 is supported. | 4-4 for NPRACH format 2 | No | False alarm rate under inter-cell interference is higher. | FDD only |  |  | RAN1 | Optional without capability signaling |  |
|  | 8-6 | System acquisition time reduction | 1. Support for additional repetitions of SIB1-NB |  | No | UE time to acquire SIB1-NB is not reduced. | FDD only |  |  | RAN1 | Optional without capability signaling |  |
|  | 8-7 | Measurement accuracy enhancement based on NSSS | 1. Support for NSSS-based RRM measurements. |  | Yes | Accuracy of UE measurements may not be improved. | FDD only |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 8-8 | Measurement accuracy enhancement based on NPBCH | 1. Support for NPBCH-based RRM measurements |  | Yes | Accuracy of UE measurements is not improved. | FDD only |  |  | RAN1 | Optional without capability signaling |  |
|  | 8-9 | TDD NB-IoT | 1. FDD NB-IoT Rel-13 features. |  | Yes | Only FDD bands can be used for the UE. | No |  |  | RAN1 | Same as FDD |  |
|  | 8-10 | UE category NB2 for TDD | 1. Support for maximum UL TBS of 2536 bits. 2. Support for maximum DL TBS of 2536 bits |  |  | Reduced peak rates UL and DL | No |  | Separate capability signaling is not needed as indicated capability is connected to UE category NB2 | RAN1 | N.A. (This feature is connected with UE category NB2) |  |
|  | 8-11 | 2 UL/DL HARQ processes for TDD | 1. Two HARQ processes in UL 2. Two HARQ processes in DL | 4-10 |  | Reduced peak rates UL and DL | Yes |  |  | RAN1 | Optional with capability signaling |  |
|  | 8-12 | Non-anchor carrier paging for TDD | 1. Non-anchor carrier paging support |  | Yes, delivered from MME | Less potential for load balancing in the cell | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | 8-13 | Non-anchor carrier RACH for TDD | 1. Non-anchor carrier RACH support |  | Yes | Less potential for load balancing in the cell | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | [8-14] | [OTDOA positioning using NPRS for TDD] | [1. RSTD measurement using original NPRS] |  | [No] | [No support for DL positioning method based on OTDOA] | [No] |  | [LPP/LPPa are assumed to provide signaling to indicate support. For TDD, this may be replaced by 4-19] | [RAN1] | [Optional] |  |
|  | 8-15 | OTDOA positioning using PRS for TDD | 1. RSTD measurement using PRS |  | No | No support for DL positioning method based on OTDOA | No |  | LPP/LPPa are assumed to provide signaling to indicate support | RAN1 | Optional |  |
|  | 8-16 | Positioning techniques other for TDD | Positioning techniques other than OTDOA |  | No | No support for positioning techniques other than OTDOA. | No |  | LPP/LPPa are assumed to provide signaling to indicate component capabilities: OTDOA, A-GNSS, E-CID, TBS, Sensor, WLAN, Bluetooth | RAN1 | Optional |  |
|  | 8-17 | DL interference randomization for TDD | 1. NPDCCH interference randomization 2. NPDSCH interference randomization |  | No | DL performance may be degraded in the presence of inter-cell interference | No |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 8-18 | Enhanced RACH power control for TDD | 1. Enhanced RACH power control for NPRACH transmission 2. Enhanced RACH power control for Msg3 transmission |  | No | RACH procedure performance is degraded | No |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 8-19 | Enhanced NPRS for TDD | 1. RSTD measurement using enhanced NPRS | [4-14] | No | RSTD accuracy may reduce in inter-cell interference | No |  | LPP/LPPa are assumed to provide signaling to indicate support | RAN1 | Optional |  |
|  | 8-20 | RLC-UM | 1. Support of RLC unacknowledged mode |  | Yes | More RLC ARQ signaling has to be exchanged in cases where it is not needed. | FDD only |  |  | RAN2 | Leave to RAN2 |  |
|  | 8-21 | Standalone mode extensions | 1. Support for multi-carrier operation with standalone anchor/non-anchor and in-band/guard-band carriers. |  | Yes | Standalone NB-IoT carriers cannot be configured together with in-band/guard-band NB-IoT carriers | FDD only |  |  | RAN2 | Leave to RAN2 |  |
|  | 8-22 | Access barring enhancement | 1. Support for CE-level based access barring |  | No | Less flexibility in the usage of the available resources during high load and thus in resolving congestion | FDD only |  |  | RAN2 | Leave to RAN2 |  |
|  | 8-23 | PHR feedback | 1 Support of extended NB-IoT power headroom report range and finer granularity |  | yes | Network cannot adjust properly the UL resources | FDD Only |  |  | RAN2 | Leave to RAN2 |  |
| 9. LTE_eMTC4 | 9-1 | Resynchronization signal for BL/CE UE | 1. Resynchronization signal for BL/CE UE | None | No | Resynchronization time impact for BL/CE UE | No need |  |  | RAN1 | Optional without capability signaling |  |
|  | 9-2 | Wake-up signal for BL/CE UE | 1. Wake-up signal for BL/CE UE | ce-ModeA-r13 | Yes | UE power efficiency impact for BL/CE UE | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-3 | UL HARQ-ACK feedback in CE mode A/B | 1. Early termination of MPDCCH monitoring by positive UL HARQ-ACK feedback in DCI (for HD-FDD, FD-FDD and TDD) in CE mode A/B 2. Early termination of PUSCH transmission by positive UL HARQ-ACK feedback in DCI (for FD-FDD and TDD) in CE mode A/B 3. Early termination of PUSCH transmission by new UL grant in DCI (for FD-FDD and TDD) in CE mode A/B | ce-ModeA-r13 | Yes | UE power efficiency impact in CE mode A/B | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-4 | CRS muting for BL UE | 1. CRS muting for BL UE | Cat-M1/M2 | Yes | DL spectral efficiency impact for LTE cell | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-5 | DL 64QAM support in CE mode A | 1. DL 64QAM support in CE mode A | ce-ModeA-r13, 9-6 | Yes | DL spectral efficiency impact in CE mode A | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-6 | Alternative CQI table in CE mode A | 1. Alternative CQI table in CE mode A | ce-ModeA-r13 | Yes | DL spectral efficiency impact in CE mode A | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-7 | PUSCH sub-PRB allocation in CE mode A | 1. PUSCH sub-PRB allocation in CE mode A | ce-ModeA-r13 | Yes | UL spectral efficiency impact and UE power efficiency impact in CE mode A | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-8 | PUSCH sub-PRB allocation in CE mode B | 1. PUSCH sub-PRB allocation in CE mode B | ce-ModeA-r13 | Yes | UL spectral efficiency impact and UE power efficiency impact in CE mode B | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-9 | Flexible PDSCH starting PRB in CE mode A | 1. Flexible PDSCH starting PRB in CE mode A | ce-ModeA-r13 | Yes | DL spectral efficiency impact for LTE cell | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-10 | Flexible PDSCH starting PRB in CE mode B | 1. Flexible PDSCH starting PRB in CE mode B | ce-ModeA-r13 | Yes | DL spectral efficiency impact for LTE cell | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-11 | Flexible PUSCH starting PRB in CE mode A | 1. Flexible PUSCH starting PRB in CE mode A | ce-ModeA-r13 | Yes | UL spectral efficiency impact for LTE cell | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-12 | Flexible PUSCH starting PRB in CE mode B | 1. Flexible PUSCH starting PRB in CE mode B | ce-ModeA-r13 | Yes | UL spectral efficiency impact for LTE cell | No need |  |  | RAN1 | Optional with capability signaling |  |
|  | 9-13 | Early data transmission in CE mode A | 1. Early data transmission in CE mode A | ce-ModeA-r13 | Up to RAN2 | Latency and UE power efficiency impact for BL/CE UE accessing CE mode A | No need |  |  | RAN2 | Up to RAN2 |  |
|  | 9-14 | Early data transmission in CE mode B | 1. Early data transmission in CE mode B | ce-ModeB-r13 | Up to RAN2 | Latency and UE power efficiency impact for BL/CE UE accessing CE mode B | No need |  |  | RAN2 | Up to RAN2 |  |
|  | 9-15 | EARFCN provisioning | 1. EARFCN provisioning | Up to RAN2 | Up to RAN2 | Cell search performance impact | Up to RAN2 |  |  | RAN2 | Up to RAN2 |  |
|  | 9-16 | Reduced MIB acquisition time for BL/CE UE | 1. Reduced MIB acquisition time for BL/CE UE | Up to RAN4 | Up to RAN4 | MIB acquisition time impact for BL/CE UE | Up to RAN4 |  |  | RAN4 | Up to RAN4 |  |
|  | 9-17 | Reduced SIB acquisition time for BL/CE UE | 1. Reduced SIB acquisition time for BL/CE | Up to RAN2 | Up to RAN2 | SIB acquisition time impact for BL/CE UE | Up to RAN2 |  |  | RAN2 | Up to RAN2 |  |
|  | 9-18 | Relaxed monitoring for cell reselection for BL/CE UE | 1. Relaxed monitoring for cell reselection for BL/CE UE | Up to RAN2 | Up to RAN2 | UE power consumption impact for BL/CE UE | Up to RAN2 |  |  | RAN2 | Up to RAN2 |  |
|  | 9-19 | Improved access/load control for idle mode BL/CE UE | 1. Improved access/load control for idle mode BL/CE UE | Up to RAN2 | Up to RAN2 | No improved access/load control for idle mode BL/CE UE | Up to RAN2 |  |  | RAN2 | Up to RAN2 |  |
|  | 9-20 | Support for higher velocity for BL/CE UE | 1. Support for higher velocity for BL/CE UE | Up to RAN4 | Up to RAN4 | No support for higher velocity for BL/CE UE | Up to RAN4 |  |  | RAN4 | Up to RAN4 |  |
|  | 9-21 | Lower UE power class for BL UE | 1. Lower UE power class (14 dBm) for BL UE | Up to RAN4 | Up to RAN4 | No support for 14-dBm power class for BL UE | Up to RAN4 |  |  | RAN4 | Up to RAN4 |  |
|  | 9-22 | New gaps for dense PRS configuration | 1. New gaps for dense PRS configuration | Up to RAN4 | Up to RAN4 | No new gaps for dense PRB configuration | Up to RAN4 |  |  | RAN4 | Up to RAN4 |  |