3GPP TSG RAN WG1 Meeting #93		R1-1807571

Busan, Korea, May 21st – 25th, 2018

Agenda item:	6.2.9

Source: 	Qualcomm Incorporated, [Intel Corporation]

Title: 	Conclusion for modulation enhancements

Document for:	Discussion/Decision

# Background

Table 7.1.7.1-1: Modulation and TBS index table for PDSCH

| MCS Index | Modulation Order | Modulation Order | TBS Index |
| --- | --- | --- | --- |
| 0 | 2 | 2 | 0 |
| 1 | 2 | 2 | 1 |
| 2 | 2 | 2 | 2 |
| 3 | 2 | 2 | 3 |
| 4 | 2 | 2 | 4 |
| 5 | 2 | 4 | 5 |
| 6 | 2 | 4 | 6 |
| 7 | 2 | 4 | 7 |
| 8 | 2 | 4 | 8 |
| 9 | 2 | 4 | 9 |
| 10 | 4 | 6 | 9 |
| 11 | 4 | 6 | 10 |
| 12 | 4 | 6 | 11 |
| 13 | 4 | 6 | 12 |
| 14 | 4 | 6 | 13 |
| 15 | 4 | 6 | 14 |
| 16 | 4 | 6 | 15 |
| 17 | 6 | 6 | 15 |
| 18 | 6 | 6 | 16 |
| 19 | 6 | 6 | 17 |
| 20 | 6 | 6 | 18 |
| 21 | 6 | 6 | 19 |
| 22 | 6 | 6 | 20 |
| 23 | 6 | 6 | 21 |
| 24 | 6 | 6 | 22 |
| 25 | 6 | 6 | 23 |
| 26 | 6 | 6 | 24 |
| 27 | 6 | 6 | 25 |
| 28 | 6 | 6 | 26/26A |
| 29 | 2 | 2 | reserved |
| 30 | 4 | 4 |  |
| 31 | 6 | 6 |  |

Table 7.1.7.1-1A. Modulation and TBS index table 2 for PDSCH

| MCS Index | Modulation Order | Modulation Order | TBS Index |
| --- | --- | --- | --- |
| 0 | 2 | 2 | 0 |
| 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 4 |
| 3 | 2 | 4 | 6 |
| 4 | 2 | 4 | 8 |
| 5 | 4 | 6 | 10 |
| 6 | 4 | 6 | 11 |
| 7 | 4 | 6 | 12 |
| 8 | 4 | 6 | 13 |
| 9 | 4 | 6 | 14 |
| 10 | 4 | 8 | 15 |
| 11 | 6 | 8 | 16 |
| 12 | 6 | 8 | 17 |
| 13 | 6 | 8 | 18 |
| 14 | 6 | 8 | 19 |
| 15 | 6 | 8 | 20 |
| 16 | 6 | 8 | 21 |
| 17 | 6 | 8 | 22 |
| 18 | 6 | 8 | 23 |
| 19 | 6 | 8 | 24 |
| 20 | 8 | 8 | 25 |
| 21 | 8 | 8 | 27 |
| 22 | 8 | 8 | 28 |
| 23 | 8 | 8 | 29 |
| 24 | 8 | 8 | 30 |
| 25 | 8 | 8 | 31 |
| 26 | 8 | 8 | 32 |
| 27 | 8 | 8 | 33/33A/33B |
| 28 | 2 | 2 | reserved |
| 29 | 4 | 4 |  |
| 30 | 6 | 6 |  |
| 31 | 8 | 8 |  |

Table 7.1.7.1-1B. Modulation and TBS index table 3 for PDSCH

| MCS Index | Modulation Order | Modulation Order | TBS Index |
| --- | --- | --- | --- |
| 0 | 2 | 2 | 0 |
| 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 4 |
| 3 | 2 | 4 | 6 |
| 4 | 2 | 4 | 8 |
| 5 | 4 | 6 | 11 |
| 6 | 4 | 6 | 13 |
| 7 | 4 | 8 | 15 |
| 8 | 6 | 8 | 16 |
| 9 | 6 | 8 | 18 |
| 10 | 6 | 8 | 20 |
| 11 | 6 | 8 | 21 |
| 12 | 6 | 8 | 22 |
| 13 | 6 | 8 | 23 |
| 14 | 6 | 8 | 24 |
| 15 | 8 | 8 | 25 |
| 16 | 8 | 8 | 27 |
| 17 | 8 | 8 | 28 |
| 18 | 8 | 8 | 29 |
| 19 | 8 | 8 | 30 |
| 20 | 8 | 8 | 31 |
| 21 | 8 | 8 | 32 |
| 22 | 8 | 8 | 33/33A/33B |
| 23 | 10 | 10 | 34A |
| 24 | 10 | 10 | 35 |
| 25 | 10 | 10 | 36 |
| 26 | 10 | 10 | 37A/37 |
| 27 | 2 | 2 | reserved |
| 28 | 4 | 4 |  |
| 29 | 6 | 6 |  |
| 30 | 8 | 8 |  |
| 31 | 10 | 10 |  |

# New table

Yellow: Entries for legacy 64-QAM table (except 26)

Blue: New Entries for legacy 256-QAM table

Green: Entries for 1024QAM

Proposal: For the solution to modulation enhancements, introduce a 6-bit MCS table as below:

- For the entries with “Scaling = NO”, the UE follows legacy procedure to determine the TBS.

- For the entries with “Scaling = YES”, the UE selects the TBS by scaling the number of allocated PRBs by a factor α, where α is RRC configured (similar to scaling for special subframe)

- Email discussion to decide the contents for rows 42-58, and whether entries 56-58 have scaling applied or not, and agree to CR.

| MCS Index | Modulation Order | Modulation Order | TBS Index | Scaling |
| --- | --- | --- | --- | --- |
| 0 | 2 | 2 | 0 | No |
| 1 | 2 | 2 | 1 | No |
| 2 | 2 | 2 | 2 | No |
| 3 | 2 | 2 | 3 | No |
| 4 | 2 | 2 | 4 | No |
| 5 | 2 | 4 | 5 | No |
| 6 | 2 | 4 | 6 | No |
| 7 | 2 | 4 | 7 | No |
| 8 | 2 | 4 | 8 | No |
| 9 | 2 | 4 | 9 | No |
| 10 | 4 | 6 | 9 | No |
| 11 | 4 | 6 | 10 | No |
| 12 | 4 | 6 | 11 | No |
| 13 | 4 | 6 | 12 | No |
| 14 | 4 | 6 | 13 | No |
| 15 | 4 | 6 | 14 | No |
| 16 | 4 | 6 | 15 | No |
| 17 | 6 | 6 | 15 | No |
| 18 | 6 | 6 | 16 | No |
| 19 | 6 | 6 | 17 | No |
| 20 | 6 | 6 | 18 | No |
| 21 | 6 | 6 | 19 | No |
| 22 | 6 | 6 | 20 | No |
| 23 | 6 | 6 | 21 | No |
| 24 | 6 | 6 | 22 | No |
| 25 | 6 | 6 | 23 | No |
| 26 | 6 | 6 | 24 | No |
| 27 | 6 | 6 | 25 | No |
| 28 | 8 | 8 | 25 | No |
| 29 | 8 | 8 | 27 | No |
| 30 | 8 | 8 | 28 | No |
| 31 | 8 | 8 | 29 | No |
| 32 | 8 | 8 | 30 | No |
| 33 | 8 | 8 | 31 | No |
| 34 | 8 | 8 | 32 | No |
| 35 | 8 | 8 | 33A | No |
| 36 | 8 | 8 | 33/33B | No |
| 37 | 10 | 10 | 34A | No |
| 38 | 10 | 10 | 35 | No |
| 39 | 10 | 10 | 36 | No |
| 40 | 10 | 10 | 37A | No |
| 41 | 10 | 10 | 37 | No |
| 42 |  |  |  | Yes |
| 43 |  |  |  | Yes |
| 44 |  |  |  | Yes |
| 45 |  |  |  | Yes |
| 46 |  |  |  | Yes |
| 47 |  |  |  | Yes |
| 48 |  |  |  | Yes |
| 49 |  |  |  | Yes |
| 50 |  |  |  | Yes |
| 51 |  |  |  | Yes |
| 52 |  |  |  | Yes |
| 53 |  |  |  | Yes |
| 54 |  |  |  | Yes |
| 55 |  |  |  | Yes |
| 56 |  |  |  | No/Yes |
| 57 |  |  |  | No/Yes |
| 58 |  |  |  | No/Yes |
| 59 | 2 | 2 | Reserved |  |
| 60 | 4 | 4 |  |  |
| 61 | 6 | 6 |  |  |
| 62 | 8 | 8 |  |  |
| 63 | 10 | 10 |  |  |