3GPP TSG RAN WG1 Meeting #93	   R1-1807583

Busan, South Korea, 21st -25th may, 2018

Agenda Item:	6.2.7

Source:	Ericsson

Title:	RAN1 decisions for WI Ultra Reliable Low Latency Communication for LTE (LTE_HRLLC) – per topic

Document for:	Information

Ultra Reliable Low Latency Communication for LTE -Collection of RAN1 agreements sorted per topic

This contribution lists RAN1 agreements made for Rel-15 WI Ultra Reliable Low Latency Communication for LTE (WI code: LTE_HRLLC, WID in RP-172845) sorted by topic. Since the purpose of this contribution is to serve as input to RAN1 CR drafting, this contribution does not capture observations and other informative notes, only agreements, working assumptions and conclusions with potential RAN1 specification impacts. The only exception is the annex that collects agreements made on target, simulation assumptions, evaluation methodology and identified candidate technical solutions. Please note that - Some agreements have been combined or edited for better readability. - Agreements, working assumptions and FFSs that have been made obsolete by later agreements are not included. - This document is just an interpretation of the RAN1 agreements listed in R1-1807582, i.e. this document does in no way override the official RAN1 minutes.

# 1	RRC parameters

The RRC configurations listed in this clause will to a large extent be overlapping with information provided in clauses 2 and 3, considering that the RRC parameter information is difficult to completely separate from the associated agreement.

An optional CFI configuration per TTI length is supported through UE and serving cell specific semi-static configuration. - If CFI is semi-statically configured for TTIs of different lengths, the UE does not expect the configured CFI values to be different. - The semi-static CFI value could be configured separately for MBSFN and non-MBSFN subframes over each cell.

The following RRC parameters are introduced for blind/HARQ-less PDSCH repetitions. - Blind/HARQ-less PDSCH repetitions are enabled by RRC configuration. - The RV cycling sequence applied for PDSCH repetition is configured by higher layer signalling and selected between {0, 0, 0, 0} or {0, 2, 3, 1}. - The number of HARQ processes for PDSCH repetition applicable when k > 1 can be configured. The value range is 1 to the maximum number of DL HARQ processes. - The x MSB of the 5-bit MCS field for PDSCH repetitions and k>1 are set to 0. The value of x is configurable and the value range is {0,1}

The following RRC parameters are introduced for UL SPS repetition. - The total number of UL transmissions K obtained with the UL SPS repetition scheme is RRC configured and can take any value from {2, 3, 4, 6} that satisfies K <= P. - RV sequence is configurable and can take one of the following values - Value 1: {0, 0, 0, 0, 0, 0} - Value 2: {0, 2, 3, 1, 0, 2} - Value 3: {0, 3, 0, 3, 0, 3} - The Cyclic shift for UL DMRS is configured by RRC (3-bit CS). - For UL SPS repetition with subslot operation, the TBS scaling factor of subslot PUSCH transmitted using an UL SPS configuration is configurable per UL SPS configuration and can take a value from {1/6, 1/12}

# 2	UE capabilities

Supporting repetition based PDSCH reception is a UE capability.

Separate UE capability is defined for the support of UL SPS repetitions for subframe, slot and subslot.

# 3	LTE URLLC technical solutions

## 3.1	DL control channel for LTE URLLC operation

| Agreement | Potential spec impact |
| --- | --- |
| Support an optional, CFI configuration per TTI length through UE and serving cell specific semi-static configuration. - If CFI is semi-statically configured for TTIs of different lengths, the UE does not expect the configured CFI values to be different. - The semi-static CFI value could be configured separately for MBSFN and non-MBSFN subframes over each cell. | 36.213, 36.211 |
| When a UE is configured with a semi-static CFI for a given TTI length, the UE is not expected to decode PCFICH for that TTI length. | 36.213 |

## 3.2	PDSCH repetitions

| Agreement | Potential spec impact |
| --- | --- |
| The work item supports blind/HARQ-less repetition for PDSCH in different TTIs. | - |
| Blind/HARQ-less PDSCH repetitions are enabled by RRC configuration. There is a field in the DCI that indicates the number of PDSCH transmissions k associated with the DCI, where k >= 1. - If configured for slot or subslot PDSCH, a single DCI format containing this field is used to schedule slot or subslot PDSCH - If configured for subframe PDSCH, the UE (as in legacy) continues to monitor for DCI Format 1A and the DL TM specific DCI format. The field is included only in DCI format 1A on USS | 36.212, 36.213 |
| PDCCH indicates number of PDSCH transmissions associated with the PDCCH. PDCCH may or may not be transmitted with a PDSCH repetition. PDSCH transmissions can be soft combined after a PDCCH is successfully received - The number of transmissions, k, is the number of PDSCH transmissions associated with the PDCCH starting with the current TTI and is indicated in a 2-bit DCI field as follows. The 2-bit field is amended to the PDSCH related DCI (i.e. subframe PDSCH DCI and DCI 7-1x). DCI bit field; Value of k for subframe, slot and subslot. 00; 1. 01; 2. 10; 3. 11; a value configured over RRC from {4, 6}. Note: PDCCH here refers to the DL assignment which may be via PDCCH/SPDCCH/EPDCCH on a slot/subslot/subframe as applicable. | 36.212, 36.213 |
| The UE shall discard any PDSCH assignment for a (s)TTI in a serving cell with CRC scrambled with C-RNTI, if PDSCH by means of blind repetition is being received in that (s)TTI in the same serving cell. | 36.213 |
| For HARQ for repeated PDSCH transmissions, the UE shall report HARQ feedback with the timing given by the last PDSCH repetition. | 36.213 |
| For slot/subslot PDSCH repetition, the rate-matching around SPDCCH resources for the PDSCH transmissions within a repetition window follows the rate-matching around SPDCCH resources for the first PDSCH transmission of the repetition window | 36.211 or 36.213 |
| - For TDD, if a UE is scheduled with K repetitions for slot PDSCH, the UE assumes the PDSCH to be present in the next available K DL slots - UL slots are not counted in the repetition count - For TDD, if a UE is scheduled with K repetitions for subframe PDSCH, the UE assumes the PDSCH to be present in the next available K DL subframes or flexible subframes - UL subframes are not counted in the repetition count - In case different DL transmission modes are configured for MBSFN and non-MBSFN subframes for slot or subslot PDSCH, a UE is only expecting PDSCH repetitions up to the boundary between MBSFN and non-MBSFN subframes in case of a larger indicated K (larger than the number of remaining TTIs of the same subframe type). - For TM10 blind/HARQ-less PDSCH repetition, the UE is to assume the same PQI to be applied to all PDSCH repetitions. | 36.213 |
| Slot & subslot PDSCH repetition across subframe boundaries is supported - For subslot PDSCH, in case subslot#0 cannot contain PDSCH (i.e CFI=1 or 2 / PDCCH length of 2 or 3) then subslot#0 is not counted as valid transmission/repetition and the next repetition resumes from subslot#1 | 36.213 |
| PDSCH repetitions associated with a single DL assignment assume the same RB allocation. | 36.213 |
| For slot/subslot PDSCH repetition and DMRS based PDSCH, the UE may assume that the same precoder is kept between PDSCH repetitions. For TDD, the UE cannot assume coherent DMRS channel estimation across a UL/DL switching point. - Note: this allows the UE to do coherent channel estimation filtering. | 36.213 |
| For subslot PDSCH repetitions DMRS sharing is not supported for blind/HARQ-less PDSCH repetition with k > 1, i.e., DMRS is present in each subslot. - For k > 1, the value of DMRS position field is fixed by specification to 0 | 36.213 |
| For slot/subslot PDSCH, with k > 1, the maximum transmission rank is 2 for reception of PDSCH. | 36.213 |
| For PDSCH repetitions, different RV can be used in different PDSCH transmissions within the repetition window. (i.e., RV cycling is supported). The RV cycling sequence applied for PDSCH repetition when PDSCH repetition is configured is configured by higher layer signalling and selected between {0, 0, 0, 0} or {0, 2, 3, 1}. - The RV field in the DCI scheduling a repetition sequence of k PDSCH transmissions identifies the starting RV in the cycling sequence that is used for the first transmission of PDSCH within the repetition window. | 36.213 |
| The number of HARQ processes for PDSCH repetition applicable when k > 1 can be configured. The value range is 1 to the maximum number of DL HARQ processes. | 36.213 |
| The x MSB of the 5-bit MCS field for PDSCH repetitions and k>1 are set to 0. The value of x is configurable and the value range is {0,1} | 36.213 |

## 3.3	UL SPS repetitions

| Agreement | Potential spec impact |
| --- | --- |
| For LTE URLLC operation, at least an UL SPS repetition configuration is supported where a UE can start the initial transmission of a TB at any (s)TTI |  |
| The following UL SPS repetition scheme with SPS periodicity P is supported with N SPS configurations on a serving cell for subframe/slot/subslot operation, where N<=6. The following is defined per SPS configuration. - The total number of UL transmissions K obtained with the UL SPS repetition scheme is RRC configured and can take any value from {2, 3, 4, 6} that satisfies K <= P. - The initial transmission of a TB shall start at the first transmission occasion of the K transmissions. Single HARQ process/TB per transmission window. - RV sequence is configurable and can take one of the following values - Value 1: {0, 0, 0, 0, 0, 0} - Value 2: {0, 2, 3, 1, 0, 2} - Value 3: {0, 3, 0, 3, 0, 3} - The Cyclic shift for UL DMRS is configured by RRC (3-bit CS). - When N > 1 is configured for UL SPS, the 3 LSB of the bit field for HARQ process ID in the SPS activation/deactivation DCI indicate which SPS configuration is activated/deactivated. - For subslot operation, the TBS scaling factor of subslot PUSCH transmitted using an UL SPS configuration is configurable and can take a value from {1/6, 1/12} independently of the number of PUSCH data symbols. - The UE shall not transmit PUSCH simultaneously using different UL SPS configurations on a serving cell. | 36.211, if any 36.213 |
| For UL SPS repetition with subframe/slot/subslot operation with multiple UL SPS configurations on a serving cell for a given TTI length: - In one TTI only one UL SPS configuration can be activated - The initial transmission of a TB for different UL SPS configurations is on different TTIs. - Different HARQ process IDs are used for different UL SPS configurations. - Each UL SPS configuration is activated by individual SPS activation DCI. - The UE is not expected to receive more than one activation DCI in a TTI. | 36.213, if any |
| UL SPS repetition across subframe boundary is supported |  |
| If the UE is configured with UL SPS and the configured number of SPS PUSCH transmission k>1, simultaneous transmission of PUSCH and PUCCH is not configured. | 36.213, if any |
| In case of collision between non-SPS PUSCH and a SPS PUSCH (that is part of a SPS repetition window) of the same TTI length in the same subframe/slot/subslot on a given carrier, the UE shall transmit non-SPS PUSCH transmission and drop the colliding SPS PUSCH transmission. - Note: Not necessarily captured in RAN1 specifications | 36.213, if any |
| In case of collision between (S)PUCCH and a SPS PUSCH (that is part of a SPS repetition window) of the same TTI length in the same subframe/slot/subslot on a given carrier, - the UE shall transmit SPS PUSCH transmission and drop/stop the (S)PUCCH transmission - HARQ-ACK of (S)PUCCH is transmitted via the SPS PUSCH that collides with (S)PUCCH. - CSI of (S)PUCCH, if any, is dropped. | 36.213, if any |
| - In case of collision between SPS subframe-PUSCH (that is part of a SPS repetition window) and a non-SPS slot/subslot-PUSCH in the same subframe on a given carrier, the UE shall transmit non-SPS slot/subslot-PUSCH transmission and drop the colliding subframe SPS PUSCH transmission. - In case of collision between SPS subframe-PUSCH (that is part of a SPS repetition window) and a slot/subslot-PUCCH in the same subframe on a given carrier, the UE shall transmit slot/subslot-PUCCH transmission and drop the colliding subframe SPS PUSCH transmission. | 36.213, if any |
| For each UL subslot SPS repetition configuration, the UL DMRS pattern for UL SPS is reused. | 36.211, if any |
| From RAN1 perspective to guarantee the number of repetitions, on a serving cell an ongoing UL SPS repetition transmission should not be interrupted by another UL SPS repetition configuration having new data arriving (i.e. UE should wait with a SPS transmission of another TB till having finished the repetitions of an ongoing UL SPS repetition transmission) . - Note: This may not have RAN1 specification impact. |  |

## 4	Annex

## 4.1	Target requirements and other design constraints

URLLC for LTE should target the requirement defined by ITU, i.e., 10-5 error probability in transmitting a layer 2 PDU of 32 bytes within 1 ms. Additional less stringent requirements can be considered.

In addition to (10-5, 1ms, 32 bytes packet), URLLC for LTE should target the requirement of 10-4 error probability in transmitting a layer 2 PDU of 32 bytes within 10 ms.

Table 4.1-1: Agreed LTE URLLC targets

| Latency bound | Error probability | Layer 2 PDU size | Comment |
| --- | --- | --- | --- |
| 1 ms | 1e-5 | 32 B | IMT-2020 (ITU 5G target) |
| 10 ms | 1e-4 | 32 B |  |

Conclusion:

It is expected that UEs can have URLLC traffic only and can have both eMBB and URLLC traffic.

## 4.2	Evaluation methodology and metrics

For LTE URLLC evaluation, reliability is used as metric. The reliability definition from NR in 3GPP TR 38.802 is reused.

- Definition: Reliability is defined as the success probability R of transmitting X bits within L seconds, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface, at a certain channel quality Q (e.g., coverage-edge).

  - Spectral efficiency should be considered.

  - The latency bound L includes transmission latency, processing latency, retransmission latency and queuing/scheduling latency (including scheduling request and grant reception if any).

- Evaluation method:

  - Use Link level simulation based on ITU methodology (i.e. a step-wise approach)

    - The fulfilment of the reliability target is verified in link level simulations at a reference SINR, i.e. Q, resulting from system level simulations.

      - FFS:

        - The simulation assumptions to derive the reference SINR, i.e. Q

        - The reference SINR is calibrated among companies

  - Other link level simulation methodologies not focussing on the ITU requirement are not precluded.

    - FFS details

  - The error probability should be provided for a range of SNR

NOTE: The ITU evaluation methodology for reliability is defined in section 7.1.5 of “Guidelines for evaluation of radio interface technologies for IMT-2020” from ITU Radiocommunication Study Groups.

The reliability of each channel are evaluated independently by link level simulation. The overall reliability is computed analytically based on the reliability obtained in link level simulation (companies report their details in analysis). When repetitions/retransmissions are used, the performance for PDSCH/PUSCH with repetitions/retransmissions is evaluated in a single simulation where correlations in the channel for the different repetitions is modelled.

### 4.2.1	Reference SINR, i.e. Q

To derive the reference SINR, the following principles are used:

The SINR from system level evaluations is based on long-term SINR at a given position in the network and excludes fast fading component. The SINR collected in system level simulations is the one at the antenna connector reference point (no combination of antenna ports are considered). Since the SINR is derived at the antenna connector without combination between ports, it is assumed that the assumption on number of antenna elements for the UE does not have an impact to the derived SINR point.

The following Q-values (5th percentile SINR) is adopted for URLLC for LTE:

|  | Q [dB] |
| --- | --- |
| DL SINR | -2.6 |
| PUSCH | 2.5 |
| Subframe-PUCCH | -4.1 |
| Slot-PUCCH | -2.8 |
| Subslot-PUCCH | -1.7 |

Only the Q-values derived for the macro scenario are used for link-level evaluations

## 4.3	System-level simulation assumptions

The channel model used for system level evaluations is the one described in 3GPP TR 38.901 (also called model B in ITU Eval document).

The full channel model in “ITU IMT2020 Eval“/38.901 is adopted for system level simulations, where the magnitude squared of the channel coefficients over time and frequency are averaged (to reflect long-term SINR) to determine the average path gain for each link

### 4.3.1	Macro scenario

The following simulation settings for Urban Macro-URLLC are used:

Table 4.3.1-1: System level simulation assumptions Urban Macro-URLLC

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 700 MHz |
| Inter-site distance | 500 m |
| UE power class | 23 dBm |
| Minimum UE power | -40 dBm |
| BS antenna height | 25 m |
| Channel model | 3GPP TR 38.901 (also called model B in ITU Eval document). |
| Percentage of high loss and low loss building type | 100% low loss |
| Inter-site interference modelling | Explicitly modelled |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB (Note: This does not have any implications on the demodulation requirements that will be set) |
| BS antenna element gain | 8 dBi |
| UE antenna element gain | 0 dBi |
| Thermal noise level | -174 dBm/Hz |
| UE antenna height | 1.5 m |
| Traffic model | Full buffer (Note: it is for SINR CDF distribution derivation) |
| Number of URLLC UEs/TRxP | 10 for SINR CDF distribution derivation |
| URLLC UE location | 80% outdoor, 20% indoor |
| Total transmit power per TRxP | 49 dBm for 20 MHz bandwidth 46 dBm for 10 MHz bandwidth |
| System bandwidth | 20 MHz |
| UL PUSCH power control parameters | α=1.0, P0,PUSCH=-106 (suggested value for UL SINR CDF distribution derivation and calibration) Other values are not precluded. If other values are used, it shall be reported. |
| UL PUCCH power control parameters | P0, subframe-PUCCH = -116 P0, slot-SPUCCH = -113 P0, subslot-SPUCCH = -108 (suggested value for UL SINR CDF distribution derivation and calibration) |
| Bandwidth allocation | PUSCH: The system bandwidth on the UL is equally split between the number of UEs simulated. Each UE in each TTI/sTTI will be allocated 10 RBs (assuming 10 UE per sector and 100 RB system bandwidth) in a proportional fair fashion. Note: This does not impact the RB allocations assumed for the link level simulations Note: The Q value used was derived based on an average of interference over time with the nodes in the network using a proportional fair scheduler. PUCCH: 1 RB (To get a full load SINR for PUCCH, the same mutual interferers as for PUSCH are assumed but on a bandwidth of 1 RB) |
| Number of antenna elements per TRxP | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ |
| Number of TXRU per TRxP | 2TXRU, =(Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| Tilt | Electrical down-tilt (no mechanical tilt, reference is the horizontal plane) for system level evaluation is 8 degrees |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) |
| Wrapping around method | Geographical distance based wrapping |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |

### 4.3.2	Indoor-hotspot

The following simulation settings for Indoor hotspot are used.

Common parameters with the macro scenario are highlighted in light green.

NOTE: The indoor hotspot scenario was found to have less challenging SINR values than the macro scenario and hence only the Q-values derived from the macro scenario are investigated on link level.

Table 4.3.2-1: System level simulation assumptions Indoor Hotspot-URLLC

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 2 GHz |
| Inter-site distance | 40 m |
| UE power class | 23 dBm |
| Minimum UE power | -40 dBm |
| BS antenna height | 3 m |
| Channel model | 3GPP TR 38.901 (also called model B in ITU Eval document). |
| Inter-site interference modeling | Explicitly modelled |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB (Note: This does not have any implications on the demodulation requirements that will be set) |
| BS antenna element gain | 5 dBi |
| UE antenna element gain | 0 dBi |
| Number of UE antenna elements | 1 Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) 0° polarization NOTE: For the purpose of Q derivation |
| Thermal noise level | -174 dBm/Hz |
| UE antenna height | 1.5m |
| Traffic model | Full buffer |
| Number of URLLC UEs/TRxP | 10 for SINR CDF distribution derivation |
| URLLC UE location and speed | 100% indoor, 3 km/h |
| Total transmit power per TRxP | 24 dBm for 20 MHz bandwidth 21 dBm for 10 MHz bandwidth |
| System bandwidth | 20 MHz |
| UL PUSCH power control parameters | α=1.0, P0, PUSCH=-106dBm |
| UL PUCCH power control parameters | P0, subframe-PUCCH = -116 P0, slot-SPUCCH = -113 P0, subslot-SPUCCH = -108 |
| Bandwidth allocation | PUSCH: Equal bandwidth PUCCH: 1 RB (To get a full load SINR for PUCCH, the same mutual interferers as for PUSCH are assumed but on a bandwidth of 1 RB) |
| Number of antenna elements per TRxP | (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ |
| Number of TXRU per TRxP | (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) |
| Tilt | Mechanical: 110 degrees, GCS Electrical: 90 degrees, LCS |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) |
| Wrapping around method | Not used |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |
| Beam forming | Ideal |
| TRxP number per site | 3 |
| TRxP boresight | 30/150/270 degrees |

## 4.4	Link-level simulation assumptions

The following link simulation assumptions are used

Table 4.4-1: Link simulations

| Parameter | Value |  |
| --- | --- | --- |
|  | Urban Macro–URLLC | Indoor Hotspot-eMBB |
| Carrier frequency for evaluation | 700 MHz | 2 GHz |
| Channel model | NLOS: TDL-C in TR 38.901 LOS: TDL-E in TR 38.901 | NLOS: TDL-A in TR 38.901 LOS: TDL-D in TR 38.901 |
| Delay spread scaling parameter | LOS: 93ns NLOS: 363ns | LOS: 20ns NLOS: 39ns |
| UE speed | 3km/h, 30km/h | 3km/h |
| Packet size | 32 bytes at Layer 2 PDU as a baseline for the 1 ms latency bound 32 and 100 bytes at Layer 2 PDU as a baseline for the 10 ms latency bound |  |
| BS TX antenna configuration | 2 TX ports |  |
| BS RX antenna configuration | 2/4 Rx ports |  |
| UE TX antenna configuration | 1TX port as baseline |  |
| UE RX antenna configuration | 2/4 RX ports for the 2 GHz case |  |
| Modulation and coding rate | IMCS ={0,3} (see 3GPP TS 36.213, table 7.1.7.1-1 and table 8.6.1-1) The use of other MCSs is not precluded |  |
| System bandwidth | 20 MHz |  |
| Resource allocation | Companies report Up to 20 MHz |  |
| Transmission mode for PDSCH | TM2 as baseline. |  |
| DL control payload in simulation for PDCCH/SPDCCH | Companies report their assumptions. |  |
| UL control payload in simulation for PUCCH/SPUCCH | A single carrier (using a single TTI length in each direction), single codeword for PDSCH is assumed as the baseline |  |
| PUCCH formats used as a baseline in the evaluations | - PUCCH format 1a - Slot-sPUCCH format 1b - Subslot-sPUCCH format 1b |  |
| Processing time line | Companies report their assumptions. |  |
| SINR range | A range including 5th percentile downlink/uplink SINR in system level simulation |  |
| Link adaptation for PDSCH | Disabled as baseline. Companies report if link adaptation is used. |  |
| Latency bound | 1ms, 10ms Companies report delay assumptions according to table 5.3-2 |  |
| Sub-carrier spacing | 15kHz |  |
| TTI length | Subslot (2 or 3 symbols per TTI), slot (7 symbols per TTI, 0.5ms), 1ms TTI (14 symbols per TTI, 1ms) Other values are not precluded (companies report if other value is used) |  |
| Number of UEs | 1 UE (other UE numbers are not precluded) |  |
| Channel estimation | Practical |  |
| Receiver type | MMSE |  |

Table 4.4-2: Latency analysis for URLCC

| Step | Description | Value |
| --- | --- | --- |
| 1.0 | Scheduling request and scheduling for uplink transmission |  |
| 1.1 | Transmitter Processing Delay (eNB for DL; UE for UL) |  |
| 1.2 | Frame Alignment |  |
| 1.3 | Data channel transmission duration (note) |  |
| 1.4 | Receiver Processing Delay |  |
| 1.5 | HARQ Retransmission |  |
|  | Total one-way delay [ms] |  |
| Note: This includes the potential blind/HARQ-less repetitions |  |  |

## 4.5	Identified LTE URLLC candidate technical solutions

Agreement

Changes to the frame structure and TTI length are not considered in this work item.

One or more of the following solutions are needed for DL control for URLLC operation - A reduced size/compact DCI is needed for URLLC operation - Study the design of reduced size/compact DCI. The studies should at least include the use of - reduced resource allocation - reduced MCS signaling overhead - Use of multiple PDCCH/sPDCCH for URLLC operation - Study whether the multiple PDCCHs are repetitions that need to be combined to decode a single PDCCH - Study whether the DCI contents need to be the same across the multiple PDCCH assignments - Use aggregation level greater than 8 - Study PDCCH candidate aggregation or definition of a single candidate with a higher aggregation level Study the effect of false alarm rate on the URLLC performance. Candidate techniques to solve the issues (if identified) may (beside others) include using larger CRC size as well as using (a-priory) known information field content.

There is no need to consider solutions that enable transmission by the eNB of a single PDCCH/sPDCCH candidate over more than 16 CCEs/sCCEs.

One or more of the following solutions for DL data are needed for URLLC operation - blind/HARQ-less PDSCH repetition in different TTIs - Consider the following variants - Variant 1: dynamic indication of the PDSCH repetition factor in DCI - Variant 2: semi-static configuration of the PDSCH repetition factor over RRC - Variant 3: independent PDSCH assignment for each PDSCH transmission - Variant 4: combination of semi-static and dynamic indication (combination of variants 1 and 2) - Study if and how PDSCH repetition can be combined with TTI level FH. - URLLC PDSCH MCS design/operation - Consider the need for URLLC PDSCH MCS design considering other candidate techniques such as blind/HARQ-less PDSCH repetition as well as the compact DCI design. - Study the following aspects - the MCS range for URLLC operation e.g. support of lower MCS, maximum supported MCS - Combination of MCS with other information e.g. RV, number of repetitions - URLLC related CQI enhancements - Consider CQI definition - lower target BLER(s) for URLLC CQI/CSI - the CQI relation with lower PDSCH MCS and/or PDSCH repetition. - Consider CQI reporting enhancements

Study PUSCH repetition (on TTI level) as one key UL SPS enhancement for URLLC and study further how to realize it. The studies should at least include indication of the repetition factor in the activation DCI, higher layer configuration of the repetition factor and combining PUSCH repetition with TTI level FH.

One or more of the following solutions for UL data are needed for URLLC operation - blind/HARQ-less PUSCH repetition for scheduled PUSCH in different TTIs - Consider the following variants - Variant 1: dynamic indication of the PUSCH repetition factor in DCI - Variant 2: semi-static configuration of the PUSCH repetition factor over RRC - Variant 3: independent PUSCH grant for each PUSCH transmission - Variant 4: combination of semi-static and dynamic indication (combination of variants 1 and 2) - Study if and how PUSCH repetition can be combined with TTI level FH. - URLLC MCS design/operation - Consider the need for URLLC MCS design considering other candidate techniques such as blind/HARQ-less PUSCH repetition as well as the compact DCI design. - Study the following aspects - the MCS range for URLLC operation e.g. support of lower MCS, maximum supported MCS - Combination of MCS with other information e.g. RV, number of repetitions - PUSCH TPC enhancements - Study the following aspects - separate TPC loops - separate TPC parameters - sub-band specific TPC - UL preemption techniques

The following are studied for UL control for URLLC operation - TPC enhancements - Different TPC parameters (e.g., P0) - sTTI length dependent power boosting - HARQ-ACK state dependent power boosting and constellation mapping - PUCCH repetition techniques. - The need for PUCCH enhancements for URLLC - Modifications to the beta-offset for UCI on PUSCH