3GPP TSG RAN WG1#93	R1-1807664

Busan, Korea, May 21st – 25th, 2018

Agenda item:	7.1.2.3.7

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Feature lead summary on QCL

Document for:	Discussion

# Introduction

Previous meetings agreements:

Agreements: (RAN1#89 Hangzhou) - PDSCH DMRS ports in a PDSCH DMRS group per [bundled PRB] in CC are implicitly assumed QCLed w.r.t average gain, delay spread, Doppler spread, Doppler shift, and average delay parameters, spatial Rx parameters). - PTRS port and PDSCH DMRS port can be assumed QCL - w.r.t average gain, delay spread, Doppler spread, Doppler shift, and average delay parameters, spatial Rx parameters (e.g. PTRS and PDSCH DMRS sharing the same beam) - w.r.t Doppler spread, Doppler shift (e.g. PTRS and PDSCH DMRS sharing the same RF chain) - FFS impact due to configurable association between PTRS port and PDSCH DM-RS port (if supported) - CSI-RS ports within a CSI-RS resource have at least two types of QCL assumptions - QCL w.r.t average gain, delay spread, Doppler spread, Doppler shift, and average delay parameters, spatial Rx parameters - Not QCL’ed (e.g. for beam selection based on beamformed CSI-RS codebook) - FFS whether some parameters can still be QCL’ed Agreements: (RAN1#AH2 Qingdao) - For QCL, NR supports: - At least one or two DM-RS antenna port groups per PDSCH - FFS other number of groups - QCL assumption across carriers and bandwidth parts for DL - FFS details for indication, the applicable RS(s), the applicable QCL parameters, and configurability - FFS whether or not to have UE assisted management Agreements: (RAN1#90 Prague) - Support the QCL indication of DM-RS for PDSCH via DCI signaling: - The N-bit indicator field in the agreed WF R1-1714885 is extended to support: - Each state refers to one or two RS sets, which indicates a QCL relationship for one or two DMRS port group (s), respectively - Each RS set refers to one or more RS(s) which are QCLed with DM-RS ports within corresponding DM-RS group - Note: The RSs within a RS set may be of different types - If there are more than one RS per RS set, each of them may be associated with different QCL parameters, e.g. one RS may be associated with spatial QCL while another RS may be associated with other QCL parameters, etc - Configuration of RS set for each state can be done via higher layer signaling - E.g., RRC/RRC + MAC CE - FFS the timing when the QCL is applied relative to the time of the QCL indication Agreements: (RAN1#90 Prague) - FFS the indicator state referred in below text may or may not be the indicator state defined in R1-1714885 - Following ways of configuring QCL relations between a reference RS and a targeted RS are supported - If configured, at least spatial QCL relation between SS Block (ref) and at least P/SP CSI-RS is signaled - FFS signaling details, e.g., by RRC, etc. - FFS: AP CSI-RS for targeted RS - If configured, at least spatial QCL relation between a P/SP CSI-RS resource (ref) and another (different) at least P/SP CSI-RS resource is signaled - FFS signaling details, e.g., by RRC, etc. - FFS: AP CSI-RS for targeted RS - FFS : Other candidate relations Agreements: (RAN1#90 Prague) - For single CC/BWP and single TRP, at least the following is supported: - Antenna port(s) of NZP CSI-RS can be QCL-ed with antenna port(s) of other RS wrt to the following parameters: - TRS w.r.t . {average delay, delay spread, Doppler shift, Doppler spread} and one of SSblock w.r.t {spatial Rx parameters} or NZP CSI-RS w.r.t. {spatial Rx parameters) - FFS Antenna ports of DM-RS can be QCL-ed with SSblock w.r.t. to the following parameters {average delay, delay spread, Doppler shift, Doppler spread, spatial Rx parameters} for PDSCH demodulation before TRS configuration - FFS for PDCCH - FFS the case where antenna port(s) of NZP CSI-RS can be QCL-ed with antenna port(s) of other RS: - SS block: w.r.t .{average delay, FFS delay spread, Doppler shift, FFS Doppler spread, spatial Rx parameters} - Especially regarding whether this case must be supported if there is potential absence of TRS or not - FFS Antenna ports of DM-RS can be QCL-ed with NZP CSI-RS w.r.t. to the following parameters {average delay, delay spread, Doppler shift, Doppler spread, spatial Rx parameters} - FFS Antenna ports of DM-RS can be QCL-ed with TRS w.r.t. to the following parameters {average delay, delay spread, Doppler shift, Doppler spread, FFS spatial Rx parameters} - Other cases are FFS - Companies are encouraged to perform analysis and evaluations to finalize the supported case(s) Agreement: (RAN1#AH3 Nagoya) - The source and target RSs of a QCL configuration can be in the same CC or in different CCs by configuration - Above is supported at least for CCs with same numerology in the same band from UE perspective - FFS: Whether all or part of QCL parameters are derived from the reference CC - FFS: Specification details on restriction on using this configuration (e.g.: based on UE capability, UE report) Agreement: (RAN1#AH3 Nagoya) - RS combinations holding QCL assumptions: - Before RRC configuration of TRS and CSI-RS for both below and above 6GHz - SSB DMRS for PDSCH w.r.t Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters (spatial RX parameters are used only for above 6GHz) - FFS whether restriction on PDSCH scheduling - SSB DMRS for PDCCH w.r.t Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters (spatial RX parameters are used only for above 6GHz) - After RRC configuration of TRS and CSI-RS for below 6 GHz for single TRP - SSB (can be from a different CC) + TRS + CSI-RS for CSI acquisition + DMRS for PDSCH - SSB TRS: [Doppler shift, average delay] - Type A: - TRS CSI-RS for CSI acquisition: Doppler shift, Doppler spread, average delay, delay spread - TRS/CSI-RS DMRS: Doppler shift, Doppler spread, average delay, delay spread - FFS: Type B: - TRS CSI-RS for CSI acquisition: Doppler shift, Doppler spread - CSI-RS DMRS: Doppler shift, Doppler spread, average delay, delay spread - Study whether the widebeam TRS can be QCLed referenced for a narrow beam CSI-RS - Study whether widebeam CSI-RS can be QCLed with a narrow beam DMRS - After RRC for above 6 GHz - RS combinations holding QCL assumptions TBD. Agreement: (RAN1#90 Prague) QCL after RRC for below 6GHz SSB TRS: Doppler shift, average delay Support type B Agreement: (RAN1#90 Prague) The following P/SP/AP CSI-RS signaling options are supported: QCL parameter; Reference RS; Target RS; Signalling mode; Reference RS and Target RS should belong to the same CC/BWP or not. Spatial; SSB; P CSI-RS; RRC; Can be on different CCs/BWPs. Spatial; SSB; SP CSI-RS; SP CSI-RS activation signal; Can be on different CCs/BWPs. Spatial; P CSI-RS; Another P CSI-RS; RRC; Can be on different CCs/BWPs. Spatial; SSB or P/SP CSI-RS; AP CSI-RS; RRC or RRC+MAC CE for configuration, indication with DCI; Can be on different CCs/BWPs. Working Assumption R1-1719069 Summary of offline discussions on QCL Nokia, Nokia Shanghai Bell (RAN1#90 Prague) After RRC for above 6 GHz - SSB [TRS - if is supported for above 6GHz] w.r.t average delay, Doppler shift, spatial RX parameters - SSB CSI-RS for BM w.r.t. [average delay, Doppler shift], spatial RX parameters - SSB CSI-RS for CSI w.r.t. [average delay, Doppler shift], spatial RX parameters - SSB DMRS for PDCCH (before TRS is configured) w.r.t. [average delay, Doppler shift, delay spread, Doppler spread], spatial RX parameters - SSB DMRS for PDSCH w.r.t. [average delay, Doppler shift, delay spread, Doppler spread], spatial RX parameters - CSI-RS for BM DMRS for PDCCH w.r.t. [average delay, Doppler shift, delay spread, Doppler spread], spatial RX parameters - CSI-RS for BM DMRS for PDSCH w.r.t. [average delay, Doppler shift, delay spread, Doppler spread], spatial RX parameters - CSI-RS for CSI DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread, [spatial RX parameters] - CSI-RS for BM CSI-RS for CSI w.r.t. spatial RX parameters - Agreement: (RAN1#91 Reno) - RS combinations holding QCL assumptions after RRC for above 6 GHz, for one CC: - SSB TRS w.r.t average delay, Doppler shift, spatial RX parameters - TRS CSI-RS for BM w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation - TRS CSI-RS for CSI w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation - TRS DMRS for PDCCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation - TRS DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation - SSB CSI-RS for BM w.r.t. average delay, Doppler shift, spatial RX parameters - SSB CSI-RS for CSI w.r.t., spatial RX parameters - SSB DMRS for PDCCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters - SSB DMRS for PDSCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters. - CSI-RS for BM DMRS for PDCCH w.r.t., spatial RX parameters. - CSI-RS for BM DMRS for PDSCH w.r.t., spatial RX parameters - CSI-RS for CSI DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; Note: QCL parameters may not be derived directly from CSI-RS for CSI - CSI-RS for BM CSI-RS for TRS/BM/CSI w.r.t. spatial RX parameters Agreement: (RAN1#91 Reno) QCL types A: Doppler shift, Doppler spread, average delay, delay spread QCL types B: Doppler shift, Doppler spread QCL types C: average delay, Doppler shift QCL types D: Spatial Rx QCL linkage for below 6GHz before RRC; signalling. SSB DMRS for PDSCH w.r.t Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters (spatial RX parameters are used only for above 6GHz). SSB DMRS for PDCCH w.r.t Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters (spatial RX parameters are used only for above 6GHz). QCL linkage for below 6GHz after RRC; signalling. SSB TRS: Doppler shift, average delay; QCL type: C. TRS CSI-RS for CSI acquisition: Doppler shift, Doppler spread, average delay, delay spread; QCL type: A. TRS DMRS: Doppler shift, Doppler spread, average delay, delay spread; QCL type: A. TRS CSI-RS for CSI acquisition: Doppler shift, Doppler spread; QCL type: B. CSI-RS DMRS: Doppler shift, Doppler spread, average delay, delay spread; QCL type: A. QCL linkage for above 6GHz after RRC; signalling. SSB TRS w.r.t average delay, Doppler shift, spatial RX parameters; QCL type: C + D. TRS CSI-RS for BM w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A. TRS CSI-RS for CSI w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A. TRS DMRS for PDCCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A. TRS DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A. SSB CSI-RS for BM w.r.t. average delay, Doppler shift, spatial RX parameters; QCL type: C+D. SSB CSI-RS for CSI w.r.t, spatial RX parameters; QCL type: D. SSB DMRS for PDCCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; QCL type: A+D. SSB DMRS for PDSCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; QCL type: A+D. CSI-RS for BM DMRS for PDCCH w.r.t. spatial RX parameters; QCL type: D. CSI-RS for BM DMRS for PDSCH w.r.t., spatial RX parameters; QCL type: D. CSI-RS for CSI DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; Note: QCL parameters may not be derived directly from CSI-RS for CSI; QCL type: A+D. CSI-RS for BM CSI-RS for TRS/BM/CSI w.r.t. spatial RX parameters; QCL type: D. - If the QCL target RS is periodic CSI-RS or TRS, - QCL-Info IE is placed in CSI-RS-Resource-Config and refers to the TCI-State index - TCI-State structure for one RS set is either: - DL Reference RS1¦ QCL_type1, DL Reference RS2¦ QCL_type2 - Note that there is no overlap of QCL_type1 and QCL_type2 - DL Reference RS1¦ QCL_type1 - Note that QCL_type1 and QCL_type2 are selected from {QCL types A, QCL types B, QCL types C, QCL types D} - Note that if DL Reference RS1 and DL Reference RS2 are the same, both QCL types apply Agreement: (RAN1#91 Reno – BM session) Mechanism to indication of source QCL for a resource: P-CSI-RS – through RRC configuration FFS: If the spatial QCL can be configured through a reference to a configured TCI state SP-CSI-RS – configuring the resource(s) through RRC, activation/deactivation through MAC-CE; The QCL for SP- CSI-RS is indicated in the same MAC-CE message that activates the SP- CSI-RS. The QCL is provided through an association with one of the M candidate TCI states AP-CSI-RS – Through DCI (AP-CSI-report-triggering state indication) For each AP-CSI-RS resource associated with each triggering state, QCL configuration is provided through an association with one of the M candidate TCI states by RRC FFS: Value of M FFS: TCI association on NZP-CSI-RS/ZP-CSI-RS based IMR Agreement: (RAN1#91 Reno – BM session) For the number of TCI states and mapping to DCI bits, N is 3 bits. Note: The number of TCI states supported by a UE depends on its capability Agreement: (RAN1#91 Reno – BM session) The state Is-TCI-Present is configured on a per-CORESET basis For beam management with beam indication, on all CORESETs configured with Is-TCI-Present=false, the TCI state used for PDCCH is reused for PDSCH reception Agreement: (RAN1#91 Reno – BM session) A candidate set of DL RSs are configured using RRC mechanism Each state of M TCI states is RRC configured with a downlink RS set used as a QCL reference, and MAC-CE is used to select up to 2^N TCI states out of M for PDSCH QCL indication The same set of M TCI states are reused for CORESET K TCI states are configured per CORESET When K>1, MAC CE can indicate which one TCI state to use for control channel QCL indication When K=1, no additional MAC CE signaling is necessary Agreement: (RAN1#91 Reno – BM session) When the scheduling offset is <=k, the PDSCH uses QCL assumption that is based on a default TCI state (e.g. the first state of the 2^N states used for PDSCH QCL indication)

In RAN1#93 meeting a total of 9 contributions [2-10], have been submitted on the QCL agenda item. Most of the documents contain text proposals, summarized in the following sections.

Initial Prioritization of Proposals for Offline Discussion:

    - Issues 5.1-7

    - TBD

# Text proposals for 38.331

Issue 2.1: clarification in 38.331: RAN1 confirms that as a reference RS ID for TCI configuration, TRS should be represented by a CSI-RS resource ID representing a CSI-RS resource in a CSI-RS resource set configured with TRS-Info. The IE CSI-RS-for-tracking which is part of the TCI-State can be removed as the following TP for 38.331 shown:

#### TCI-State ----------------------------begin text change------------------------ QCL-Info ::= SEQUENCE { -- The carrier which the RS is located in. If the field is absent, it applies to the serving cell in which the TCI-State is configured cell ServCellIndex OPTIONAL, -- Need R -- The DL BWP which the RS is located in. bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated -- Cond NZP-CSI-RS-Indicated: mandatory if csi-rs or csi-RS-for-tracking is included, absent otherwise referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb SSB-Index, -- A set of CSI-RS resources for tracking csi-RS-for-tracking NZP-CSI-RS-ResourceSetId }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD}, ... } -- TAG-TCI-STATE-STOP -- ASN1STOP

Companies views:

| Company | Comment |
| --- | --- |
| Ericsson | This change seems unnecessary. In 38.331, the field csi-RS-for-tracking is given by NZP-CSI-RS-ResourceSetId. In 38.214, CSI-RS for tracking is described as a set of CSI-RS resources configured with trs-Info, and this is what NZP-CSI-RS-ResourceSetId refers to. Hence there does not appear to be ambiguity. |
| Qualcomm | We also think that this change is unnecessary. |

# Text proposals for 38.211

Issue 3.1: During the previous meeting, a working assumption was made on the assumption of DM-RS ports (see the text below).

Proposal: Confirm the working assumption on DM-RS antenna ports

---- Unchanged parts omitted----

4.4.1 Antenna ports

An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed.

        - For DM-RS associated with PDSCH the channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are within the same scheduled PDSCH, same slot and in the same PRG as described in 3GPP TS 38.214 Section 5.1.2.3.

---- Unchanged parts omitted----

Companies views:

| Company | Comment |
| --- | --- |
| Confirming the working assumption: | ZTE, Ericsson, Lenovo, Motorola, Nokia, Qualcomm |
| Not-confirming the working assumption |  |
| Revisit the word “same” with “scheduled slot(s)” | Samsung |

Issue 3.2: UE default assumption on QCL between ports when signaling not present (text proposal below)

Proposal: Add the below correction on QCL assumption between ports when signaling not present

---- Unchanged parts omitted, section 7.2 of 38.211---- ## 7.2 Physical resources The frame structure and physical resources the UE shall assume when receiving downlink transmissions are defined in Clause 4. The following antenna ports are defined for the downlink: - Antenna ports starting with 1000 for PDSCH - Antenna ports starting with 2000 for PDCCH - Antenna ports starting with 3000 for channel-state information reference signals - Antenna ports starting with 4000 for SS/PBCH block transmission A UE shall not assume that two antenna ports are quasi co-located with respect to any QCL type unless specified otherwise. The UE may assume that antenna ports within a CSI-RS resource are QCL with QCL Type A, Type D (when applicable) and average gain ---- Unchanged parts omitted----

---- Unchanged parts omitted, section 7.4.1.5.3 of 38.211----

The UE shall assume that CSI-RS is transmitted in a candidate slot only if all OFDM symbols of that slot corresponding to the configured CSI-RS resource are classified as 'downlink'.

The UE shall assume that CSI-RS ports within one resources are QCL-ed w.r.t average gain, delay spread, Doppler spread, Doppler shift, and average delay parameters, spatial Rx parameters.

---- Unchanged parts omitted----

Companies views:

| Company | Comment |
| --- | --- |
| Huawei | A UE shall not assume that two antenna ports are quasi co-located with respect to any QCL type unless specified otherwise. |
| Intel | a default assumption for QCL of antenna port is not defined. It is therefore proposed to introduce default QCL assumption which states that antenna ports of reference signals are not QCL with any parameters unless specified otherwise. |
| Ericsson | OK with Huawei proposal In addition, add this sentence: The UE may assume that antenna ports within a CSI-RS resource are QCL with QCL Type A, Type D (when applicable) and average gain In addition, consider adding QCL Type E as “average gain” for consistency and allow the editors to update specifications accordingly |
| Nokia | “average gain” has a single occurrence in the spec, this is why so far there is no type-E, but this could be added. |
| Qualcomm | We prefer to treat this default assumption on the QCL of antenna ports after some common understanding and progress has been made with respect to issue 5.2 |

# Text proposals for 38.213

Issue 4.1: various changes in 38.213

Proposal: Agree on the below changes:

---- Unchanged parts omitted, section 10.1 of 38.213---- The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the control resource set configured by pdcch-ConfigSIB1 in MasterInformationBlock and for corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain and ‘QCL-typeA’ and ‘QCL-typeD’, , when applicable. The value for the DM-RS scrambling sequence initialization is the cell ID. A subcarrier spacing is provided by higher layer parameter subCarrierSpacingCommon in MasterInformationBlock. ---- Unchanged parts omitted ----

Issue 4.2: Applicable QCL indication within the gap of TCI activation/deactivation via MAC CE

---- Unchanged parts omitted, section 10.1 of 38.213----

If a UE has received initial configuration of more than one TCI states by higher layer parameter TCI-StatesPDCCH containing more than one TCI states but has not received a MAC CE activation command for one of the TCI states, the UE assumes that the DM-RS antenna port associated with PDCCH reception is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure with respect to delay spread, Doppler spread, Doppler shift, average delay, and spatial Rx parameters, when applicable. After a UE receives higher layer re-configuration of TCI states, the UE may assume that the activated TCI state(s) configured before the re-configuration are remained activated until reception and application of next activation/deactivation command.

If a UE has received a configuration of more than one TCI states by higher layer parameter TCI-StatesPDCCH containing more than one TCI states but has not received a MAC CE activation for one of the TCI states, the UE uses the previous known configuration for the reception of PDCCH.

---- Unchanged parts omitted ----

Companies views:

| Company | Comment |
| --- | --- |
| Ericsson | Issue 4-2 is being treated in the beam management agenda item, so should not be considered here. Issue 4-1 is ok in principle but please use the agreed QCL Types A-D instead. |
| Qualcomm | OK with 4-1, indeed 4-2 is discussed in BM AI |

# Text proposals for 38.214

Issue 5.1: TP in Section 5.1 of 38.214 for the determination of QCL parameters and rate-matching behavior when unicast PDSCH and configured CORESET are frequency division multiplex over the same OFDM symbol. (Qualcomm, R1-1807351)

---- Unchanged parts omitted ----

When receiving PDSCH conveying Msg4 of Random Access Procedure the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block the UE selected for RACH association and transmission, as defined in Subclause [TBD] of [6, TS 38.213], with respect to [spatial RX parameters].

A priority rule shall be defined when UE cannot simultaneously receive both PDCCH and PDSCH using the same spatial filter.

When unicast PDSCH is to be received by the UE in the same OFDM symbol containing a search space associated with the CORESET quasi-colocation parameters associated with the PDSCH takes precedence and the UE shall receive the PDSCH with its associated quasi co-location parameters. The UE is not expected to monitor this CORESET for PDCCH. Additionally, UE may assume that no PDCCH is transmitted in the REs indicated to receive unicast PDSCH conveyed by a PDCCH.

---- Unchanged parts omitted ----

Companies views:

| Company | Comment |
| --- | --- |
| Ericsson | Replace [spatial RX parameters] with “QCL Type D when applicable”, “ The sentence “A priority rule shall be defined when UE cannot simultaneously receive both PDCCH and PDSCH using the same spatial filter.” does not need to be captured in specifications. The last paragraph should instead be discussed in the simultaneous reception agenda, however we support that PDCCH is prioritized over PDSCH reception in case QCL Type D is applicable and the source RS for PDCCH and PDSCH are different with respect to QCL Type D |
| Nokia | Same views as E/// above. For the second paragraph, we also think that I think CORESET may need to have precedence over PDSCH because UE needs to be able to receive PDCCH for DL, UL grants, common control, etc. |

Issue 5.2: Various corrections in section 5.1.5 of 38.214

Proposal: agree below changes

---- Unchanged parts omitted ---- 5.1.5 Antenna ports quasi co-location The UE may assume the DM-RS antenna ports associated with a PDSCH are quasi co-located with respect QCL Type A, Type D (when applicable) and average gain. The UE can be configured with a list of up to M TCI-State configurations within higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability. ---- Unchanged parts omitted ---- ---- Unchanged parts omitted ---- The UE receives an activation command [10, TS 38.321] used to map up to 8 TCI states to the codepoints of the DCI field ‘Transmission Configuration Indication’. When the HARQ-ACK corresponding to the PDSCH carrying the activation command is transmitted in slot n, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from slot n+3Nslotsubframe,µ. After a UE receives higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the SSB determined in the initial access procedure with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial Rx parameters, where applicable. ---- Unchanged parts omitted ----

5.1.5 Antenna ports quasi co-location

The UE may assume the DM-RS antenna ports in one DM-RS port group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial Rx parameters, when applicable.

The UE can be configured with a list of up to M TCI-State configurations within higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability. Each configured TCI state includes one or two RS set s TCI-RS-SetConfig1, or two RS sets TCI-RS-SetConfig1 and TCI-RS-SetConfig2. Each TCI-State contains parameters for configuring thequasi co-location relationship between one or two downlink reference signals and the corresponding DM-RS port group of the PDSCH. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, there is no overlap between the two QCL types , regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location type corresponding to each DL RS given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values:

-	'QCL-TypeA': {Doppler shift, Doppler spread, average delay, delay spread}

-	'QCL-TypeB': {Doppler shift, Doppler spread}

-	'QCL-TypeC': { Doppler shift, average delay}

-	'QCL-TypeD': {Spatial Rx parameter}

The UE receives an activation command [10, TS 38.321] used to map up to 8 TCI states to the codepoints of the DCI field ‘Transmission Configuration Indication’. When the HARQ-ACK corresponding to the PDSCH carrying the activation command is transmitted in slot n, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from slot n+3Nslotsubframe,µ.After a UE receives initial higher layer configuration of TCI states and before reception and application of the activation command, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the SSB determined in the initial access procedure with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial Rx parameters, where applicable. After a UE receives higher layer re-configuration of TCI states, the UE may assume that the activated TCI state(s) configured before the re-configuration are remained activated until reception and application of next activation/deactivation command.

If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 of the PDCCH transmitted on the CORESET. If tci-PresentInDCI is set as 'disabled' for the CORESET scheduling the PDSCH or the PDSCH is scheduled by a DCI format 1_0, for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state for the PDSCH is identical to the current TCI state applied for the CORESET used for the PDCCH transmission, where the current TCI state is the state at the beginning of the slot carrying the PDSCH.

If the tci-PresentInDCI is set as 'enabled', when the PDSCH is scheduled by DCI format 1_1, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the RS(s) in the RS set with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold Threshold-Sched-Offset, where the threshold is based on reported UE capability [12, TS 38.331]. For both the cases when tci-PresentInDCI = 'enabled' and tci-PresentInDCI = 'disabled', if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold Threshold-Sched-Offset, or if PDSCH is semi-persistently scheduled, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located based on the TCI state used for PDCCH quasi co-location indication of the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are configured for the UE. If all configured TCI states do not contain 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

---- Unchanged parts omitted ----

Companies views:

| Company | Comment |
| --- | --- |
| Ericsson | All mention of TCI-RS-SetConfig’s should be removed; it is not in-line with 38.331 (no parameter called this). Agree with OPPO to remove this phrase. Disagree with Huawei for adapting a TCI state to handle to 2 DMRS port groups. Multi-TRP was downscoped by RAN Plenary, and this incomplete feature should be removed from the specs. The word “initial” should be removed from the 3rd paragraph. There is no distinction between an initial configuration and a re-configuration. Agree with CATT to remove the word “spatial” in the 3rd paragraph. The issue of “After a UE receives higher layer re-configuration …” in the 3rd paragraph is being discussed in the beam management agenda item, so should not be considered here. The first sentence should be modified as The UE may assume the DM-RS antenna ports associated with a PDSCH are quasi co-located with respect QCL Type A, Type D (when applicable) and average gain. |
| Qualcomm | We also agree with Ericsson that the changes of two TCI-RS-SetConfig are neither inline with 38.331, not the downscoping from the plenary. We also thing that the first sentence should change to: The UE may assume the DM-RS antenna ports associated with a PDSCH are quasi co-located with respect QCL Type A, Type D (when applicable) and average gain. |

Issue 5.3: Corrections on QCL rules between RS types

Proposal 5.3.1: add the missed QCL that for a CSI-RS for CSI acquisition, a previous agreement on having SSB as a QCL-TypeD.

---- Unchanged parts omitted, section 5.1.5 of 38.214---- A UE should expect only the following qcl-Type configurations in the higher layer parameter TCI-State: - If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect - 'QCL-TypeC' or {QCL-TypeC' and QCL-TypeD'} configurations with SS/PBCH block, or - ’QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition , the UE should only expect - 'QCL-TypeA' or 'QCL-TypeB' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info or - 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - 'QCL-TypeD' with SS/PBCH block. ---- Unchanged parts omitted ----

Proposal 5.3.2: agree on below QCL rules

If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect the following two possible configurations of the higher layer parameter TCI-State: Valid TCI state Configuration; DL RS 1; qcl-Type1; DL RS 2 (if configured); qcl-Type2 (if configured). 1; SS/PBCH Block; QCL-TypeC; SS/PBCH Block; QCL-TypeD. 2; SS/PBCH Block; QCL-TypeC; CSI-RS (BM); QCL-TypeD. If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, the UE should only expect the following three possible configurations of the higher layer parameter TCI-State: Valid TCI state Configuration; DL RS 1; qcl-Type1; DL RS 2 (if configured); qcl-Type2 (if configured). 1; TRS; QCL-TypeA; SS/PBCH Block; QCL-TypeD. 2; TRS; QCL-TypeA; CSI-RS (BM); QCL-TypeD. 3; TRS; QCL-TypeB. If a CSI-RS resource in a NZP-CSI-RS-ResourceSet is configured with higher layer parameter repetition, the UE should only expect the following three configurations of the higher layer parameter TCI-State: Valid TCI state Configuration; DL RS 1; qcl-Type1; DL RS 2 (if configured); qcl-Type2 (if configured). 1; TRS; QCL-TypeA; SS/PBCH Block; QCL-TypeD. 2; TRS; QCL-TypeA; CSI-RS (BM); QCL-TypeD. 3; SS/PBCH Block; QCL-TypeC; SS/PBCH Block; QCL-TypeD. For the DM-RS of PDCCH scheduling the PDSCH, the UE should only expect the following three configurations of the higher layer parameter TCI-State while the fourth configuration is valid as default, before TRS is configured: Valid TCI state Configuration; DL RS 1; qcl-Type1; DL RS 2 (if configured); qcl-Type2 (if configured). 1; TRS; QCL-TypeA; SS/PBCH Block; QCL-TypeD. 2; TRS; QCL-TypeA; CSI-RS (BM); QCL-TypeD. 3; CSI-RS (CSI); QCL-TypeA. 4*; SS/PBCH Block*; QCL-TypeA; SS/PBCH Block*; QCL-TypeD. * Before TRS configured. Note: this is not a TCI state, rather a valid QCL assumption For the DM-RS of PDSCH, the UE should only expect the three following configurations of the higher layer parameter TCI-State while the fourth is valid by default, before TRS is configured: Valid TCI state Configuration; DL RS 1; qcl-Type1; DL RS 2 (if configured); qcl-Type2 (if configured). 1; TRS; QCL-TypeA; SS/PBCH Block; QCL-TypeD. 2; TRS; QCL-TypeA; CSI-RS (BM); QCL-TypeD. 3; CSI-RS (CSI); QCL-TypeA; CSI-RS (CSI)**; QCL-TypeD**. 4*; SS/PBCH Block*; QCL-TypeA; SS/PBCH Block*; QCL-TypeD. * Before TRS configured. Note: this is not a TCI state, rather a valid QCL assumption ** Note: QCL-TypeD parameter may not be derived directly from CSI-RS (CSI)

---- Unchanged parts omitted, section 5.1.5 of 38.214----

A UE should expect only the following qcl-Type configurations in the higher layer parameter TCI-State:

-	If a periodic CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect

-	'QCL-TypeC' or {QCL-TypeC' and QCL-TypeD'} configurations with SS/PBCH block, or

-	’QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

-	If an aperiodic CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect

-	’QCL-TypeA' or {QCL-TypeA' and QCL-TypeD'} with a periodic CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.

-	If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition , the UE should only expect

-	'QCL-TypeA' or 'QCL-TypeB' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info or

-	 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'QCL-TypeD' with SS/PBCH block.

-	If a CSI-RS resource in a NZP-CSI-RS-ResourceSet is configured with higher layer parameter repetition, the UE should only expect

-	'QCL-TypeA' configuration with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or

-	{'QCL-TypeC' and 'QCL-TypeD'} configurations with SS/PBCH block, or

-	'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

-	For the DM-RS of CORESET scheduling the PDSCH, the UE should only expect

-	 'QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and QCL-TypeD' with either SSB/PBCH block or a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	For the DM-RS of PDSCH, the UE should only expect

-	’QCL-TypeA' or {QCL-TypeA' and QCL-TypeD'} configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, or

-	’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and QCL-TypeD' with either SSB/PBCH block or a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	{QCL-TypeA' and QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with a CSI-RS resource in a NZP-CSI-RS-ResourceSet with higher layer parameter trs-Info

-	{QCL-TypeA' and QCL-TypeD'} configuration with CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition

---- Unchanged parts omitted ----

Intel’s proposals on QCL rules:

QCL signalling without TRS configuration

---- Unchanged parts omitted ----

-	For the DM-RS of CORESET scheduling the PDSCH, the UE should only expect the following TCI-State configuration:

-	qcl-Type1 with 'QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, and, if applicable, qcl-Type2 with 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. or

-	'QCL-TypeA' and 'QCL-TypeD' configuration with SS/PBCH block if UE is not configured with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or

-	'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

-	For the DM-RS of PDSCH, the UE should only expect the following TCI-State configurations:

-	qcl-Type1 with ’QCL-TypeA' and, if applicable, qcl-Type2 with ‘QCL-TypeD’ configuration with a the CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, or

-	qcl-Type1 with ’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, and, if applicable, qcl-Type2 with ‘QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition or

-	{QCL-TypeA' and QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with a CSI-RS resource in a NZP-CSI-RS-ResourceSet with higher layer parameter trs-Info or QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	qcl-Type1 with {QCL-TypeA' and, if applicable, with qcl-Type2 with QCL-TypeD'} configuration with the CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition.

---- Unchanged parts omitted ----

TRS configuration without SSB

---- Unchanged parts omitted ----

-	If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect the following TCI-State configurations:

-	qcl-Type1 with 'QCL-TypeC' and, if applicable, qcl-Type2 with or {QCL-TypeC' and QCL-TypeD'} configurations with the SS/PBCH block, or

-	qcl-Type1 with 'QCL-TypeC' with SS/PBCH block and, if applicable, qcl-Type2 with ’QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

---- Unchanged parts omitted ----

Partial QCL information for CSI-RS for CSI acquisition

---- Unchanged parts omitted ----

-	If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition , the UE should only expect the following TCI-State configuration:

-	qcl-Type1 with 'QCL-TypeA' or 'QCL-TypeB' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, if applicable, with qcl-Type2 with 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.or

-	 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

---- Unchanged parts omitted ----

Ericsson’s QCL linkages

The following diagram illustrates the QCL linkages in the above agreements for both below and above 6 GHz:

The links colored in red are proposed to be added based on extensive discussion in the beam management where SSB is assumed as a spatial (QCL-TypeD) reference for PDCCH/PDSCH DMRS and CSI-RS

In 38.214, it is necessary to capture what combinations of DL RSs and QCL types can be configured within a TCI state. This can be done for each target RS separately, as in 38.214 today. For each target RS, one can look at all incoming links. The above/below 6 GHz distinction can be made by the optional configuration of the 2nd DL RS and qcl-Type2.

## Valid TCI states for a TRS configuration

If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect the following two possible configurations of the higher layer parameter TCI-State:

| Valid TCI state Configuration | DL RS 1 | qcl-Type1 | DL RS 2 (if configured) | qcl-Type2 (if configured) |
| --- | --- | --- | --- | --- |
| 1 | SS/PBCH Block | QCL-TypeC | SS/PBCH Block | QCL-TypeD |
| 2 | SS/PBCH Block | QCL-TypeC | CSI-RS (BM) | QCL-TypeD |

## Valid TCI states for a CSI-RS for CSI configuration

If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, the UE should only expect the following three possible configurations of the higher layer parameter TCI-State:

| Valid TCI state Configuration | DL RS 1 | qcl-Type1 | DL RS 2 (if configured) | qcl-Type2 (if configured) |
| --- | --- | --- | --- | --- |
| 1 | TRS | QCL-TypeA | SS/PBCH Block | QCL-TypeD |
| 2 | TRS | QCL-TypeA | CSI-RS (BM) | QCL-TypeD |
| 3 | TRS | QCL-TypeB |  |  |

## Valid TCI states for a CSI-RS for BM

If a CSI-RS resource in a NZP-CSI-RS-ResourceSet is configured with higher layer parameter repetition, the UE should only expect the following three configurations of the higher layer parameter TCI-State:

| Valid TCI state Configuration | DL RS 1 | qcl-Type1 | DL RS 2 (if configured) | qcl-Type2 (if configured) |
| --- | --- | --- | --- | --- |
| 1 | TRS | QCL-TypeA | SS/PBCH Block | QCL-TypeD |
| 2 | TRS | QCL-TypeA | CSI-RS (BM) | QCL-TypeD |
| 3 | SS/PBCH Block | QCL-TypeC | SS/PBCH Block | QCL-TypeD |

## Valid QCL assumptions for PDCCH DMRS

For the DM-RS of PDCCH scheduling the PDSCH, the UE should only expect the following three configurations of the higher layer parameter TCI-State while the fourth configuration is valid as default, before TRS is configured:

| Valid TCI state Configuration | DL RS 1 | qcl-Type1 | DL RS 2 (if configured) | qcl-Type2 (if configured) |
| --- | --- | --- | --- | --- |
| 1 | TRS | QCL-TypeA | SS/PBCH Block | QCL-TypeD |
| 2 | TRS | QCL-TypeA | CSI-RS (BM) | QCL-TypeD |
| 3 | CSI-RS (CSI) | QCL-TypeA |  |  |
| 4* | SS/PBCH Block* | QCL-TypeA | SS/PBCH Block* | QCL-TypeD |

* Before TRS configured. Note: this is not a TCI state, rather a valid QCL assumption

## Valid QCL assumptions for PDSCH DMRS

For the DM-RS of PDSCH, the UE should only expect the three following configurations of the higher layer parameter TCI-State while the fourth is valid by default, before TRS is configured:

| Valid TCI state Configuration | DL RS 1 | qcl-Type1 | DL RS 2 (if configured) | qcl-Type2 (if configured) |
| --- | --- | --- | --- | --- |
| 1 | TRS | QCL-TypeA | SS/PBCH Block | QCL-TypeD |
| 2 | TRS | QCL-TypeA | CSI-RS (BM) | QCL-TypeD |
| 3 | CSI-RS (CSI) | QCL-TypeA | CSI-RS (CSI)** | QCL-TypeD** |
| 4* | SS/PBCH Block* | QCL-TypeA | SS/PBCH Block* | QCL-TypeD |

* Before TRS configured. Note: this is not a TCI state, rather a valid QCL assumption

** Note: QCL-TypeD parameter may not be derived directly from CSI-RS (CSI)

Companies views:

| Company | Comment |
| --- | --- |
| Ericsson | Agree with OPPO that for a CSI-RS for CSI acquisition, a previous agreement on having SSB as a QCL-TypeD reference is missing. For PDCCH and PDSCH DMRS, a QCL-TypeD link to SSB is missing and should be added. After RRC configuration, it is a common configuration for PDCCH and PDSCH DMRS to have TRS as a QCL-TypeA reference and SSB as a QCL-TypeD reference. Our interpretation of the sentences that say “not configured with a TRS” is that this condition exists prior to RRC configuration of TRS. Intel may be on the right track by listing possible combinations for qcl_type1 and qcl_type2 rather than having them split into separate cases with the word ‘or’ in every paragraph. Another alternative is for each RS type (CSI-RS for tracking, CSI-RS for BM, CSI-RS for CSI, PDCCH DMRS, and PDSCH DMRS), we could list what is allowed for each QCL type. Something like the following: A UE should expect only the following configurations in qcl-Type1 and qcl-Type2 in the higher layer parameter TCI-state”: If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect - 'QCL-TypeC' with SS/PBCH block - ‘ QCL-TypeD' with SS/PBCH block, or a “CSI-RS(BM)” If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, the UE should only expect - 'QCL-TypeA' with “TRS” - ‘ QCL-TypeC' with SS/PBCH block QCL Type D with SS/PBCH block or a “CSI-RS(BM)” And so on for the other reference signals … |

Issue 5.4: CSI-RS for tracking

---- Unchanged parts omitted, section 5.1.6.1.1 of 38.214----

A UE does not expect to be configured with a CSI-ReportConfig that is linked to a CSI-ResourceConfig containing an NZP-CSI-RS-ResourceSet configured with trs-Info and with the CSI-ReportConfig configured with the higher layer parameter timeRestrictionForChannelMeasurements set to ‘configured’. For each resource configured with resourceType set to 'periodic', the UE can measure the channel based on the received CSI-RS across slots.

---- Unchanged parts omitted----

Companies views:

| Company | Comment |
| --- | --- |
| Ericsson | No need to capture this is spec. If the opposite, i.e a restriction, is not captured then it is allowed by UE implementation. |

Issue 5.5: CSI-RS for L1-RSRP computation

---- Unchanged parts omitted, section 5.1.6.1.2 of 38.214----

If a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the antenna ports of CSI-RS resources, described in Subclause 5.2.2.3.1, within the NZP-CSI-RS-ResourceSet are  same, where the CSI-RS resources in the NZP-CSI-RS-ResourceSet are transmitted in different OFDM symbols. If repetition is set to 'off', the UE shall not assume that the antenna ports of CSI-RS resources within the NZP-CSI-RS-ResourceSet are same. The UE is not expected to receive different periodicity in periodicityAndOffset and different nrofPorts in every CSI-RS resource within the set configured with layer parameter repetition. For each resource configured with resourceType set to 'periodic' or ‘Semi-persistent’, the UE can derive the measurement for computing L1-RSRP based on the received CSI-RS across slots. The UE may assume that all CSI-RS resource transmissions with the same CSI-RS resource configuration identity for a given CC and BWP is quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

---- Unchanged parts omitted----

Companies views for above changes:

| Company | Comment |
| --- | --- |
| Ericsson | Disagree with forcing the ports of different CSI-RS resources to be the same within a set configured with repetition. This is too heavy a restriction for beam management where strict coherency across resources should not be required, e.g., transmitting resources from different antenna panels. Note that the proposal in Issue 5.7 offers an alternative to the same port restriction. |
| Qualcomm | We also disagree with this change. The agreement of Repetition “ON” was that the “same downling spatial domain transmission filter” is used, so this should not be updated. |

Issue 5.6: QCL assumption within one CSI-RS resource (text proposal below)

Companies views:

| Company | Comment |
| --- | --- |
| Huawei | A UE may assume the CSI-RS antenna ports of a CSI-RS resource configuration are quasi co-located with respect to the large scale parameters including average gain, Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx parameters, when applicable. |
| Ericsson | Wasn’t this agreed last meeting already? |
| Qualcomm | We also think that this was agreed previous meeting, if it is not captured in the spec it should be captured. |

Issue 5.7: NZP CSI-RS configuration

Proposal: agree below clarification on repetition for NZP CSI-RS

---- Unchanged parts omitted, 38.214---- #### 5.2.2.3 Reference signal (CSI-RS) ##### 5.2.2.3.1 NZP CSI-RS The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource for each CSI-RS resource configuration: - … repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and if repetition is “on”, defines whether then CSI-RS resources are QCL with Type D as described in Subclause 5.1.6.1.2. and . Repetition can be configured only when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP' or ‘none'. ---- Unchanged parts omitted----

---- Unchanged parts omitted, 38.214----

#### 5.2.2.3	Reference signal (CSI-RS)

##### 5.2.2.3.1	NZP CSI-RS

The UE can be configured with one or more NZP CSI-RS resource set configuration(s) as indicated by the higher layer parameters CSI-ResourceConfig, and NZP-CSI-RS-ResourceSet. Each NZP CSI-RS resource set consists of K≥1 NZP CSI-RS resource(s).

A UE may assume the CSI-RS antenna ports of a CSI-RS resource configuration are quasi co-located with respect to the large scale parameters including average gain, Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx parameters, when applicable.

The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource for each CSI-RS resource configuration:

-	…

-	repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and defines whether CSI-RS resources are QCL-ed a repetition in conjunction with spatial domain transmission filter is on/off at gNB-side as described in Subclause 5.1.6.1.2. and can be configured only when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP' or ‘none'.

---- Unchanged parts omitted----

Companies views for above changes:

| Company | Comment |
| --- | --- |
| Ericsson | On the first change, it has been discussed as Issue 3.2 already. Agree in principle with the second edit in case of Type D parameter, we should use the adopted definition of QCL types. Hence we suggest the TP: repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and if repetition is “on”, defines whether then CSI-RS resources are QCL with Type D as described in Subclause 5.1.6.1.2. and . Repetition can be configured only when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP' or ‘none'. |

# Other proposals

Issue 6.1: proposal from VIVO (R1-1803627):

For channel/RS that are QCL’ed with aperiodic signals

- If UE does not receive any aperiodic signal within an X window, the periodic RS associated with the aperiodic RS could be used as the QCL source for the channel/RS.

- X is the same as the periodicity of the periodic signals;

- The QCL relationship between the channel/RS and the periodic RS is the same as the configured QCL relationship between channel/RS and the aperiodic RS.

Companies views:

| Company | Comment |
| --- | --- |
| Ericsson | Not needed to specify, UE implementation. |

Issue 6.2: proposal from Ericsson (R1-1806229):

Proposal 2: When the optional field qcl-InfoPeriodicCSI-RS is not present, the UE may assume that consecutive instances of the NZP CSI-RS resource are QCL with respect to all applicable QCL types.

Companies views:

| Company | Comment |
| --- | --- |

Issue 6.3: proposals from Lenovo, Motorola (R1-1806342):

Proposal 2: For CSI acquisition CSI-RS, CSI-RS resources in different occasions can be considered QCLed when the CSI-RS resource is not associated with ReportQuantity configured with timeRestrictionForChannelMeasurements or timeRestrictionForInterferenceMeasurements.

Proposal 3: For beam management CSI-RS, no QCL is assumed among CSI-RS resource in different occasions.

Proposal 4: For CSI-RS used as TRS, QCL is assumed among all CSI-RS resources within a resource set.

Companies views:

| Company | Comment |
| --- | --- |

Issue 6.4: proposal from Intel (R1-1806515):

Proposal: clarify that QCL-Type D configuration should always provided to the UE in conjunction with QCL-Type A configuration, so that UE has all QCL parameter available for processing.

Companies views:

| Company | Comment |
| --- | --- |
| Ericsson | Agree |
| Qualcomm | I assume Intel’s intention is to say that Type-D is provided “when it is applicable” |

Issue 6.5: proposal from Qualcomm (R1-1807351):

Proposal 3: In FR2, for any NZP CSI-RS resource set with CSI-RS Repetition ‘ON’, if there is a symbol-level overlap of a scheduled PDSCH with any symbol of a CSI-RS resource in the set, PDSCH is symbol-level rate-matched on the overlapping OFDM symbols.

- Note: UE is not expected to be configured with a PDSCH which results in collision between the symbols carrying DMRS and this CSI-RS resource.

- Note: PTRS is symbol-level punctured on the OFDM symbols transmitting this CSI-RS resource.

In FR2, for a NZP CSI-RS resource set with CSI-RS-Repetition ‘OFF’, or when CSI-RS-Repetition is not configured, if QCL type D is different between a scheduled PDSCH and a CSI-RS resource in the set, then PDSCH is symbol-level rate-matched on the overlapping OFDM symbols, otherwise PDSCH is RE-level rate-matched.

- Note: UE does not expect to be configured with a PDSCH which results in any DMRS to occupy the same OFDM symbol with a CSI-RS resource with different QCL Type D.

- Note: PTRS is symbol-level punctured on the OFDM symbols transmitting with a CSI-RS resource with different QCL Type D

Proposal 4.1: If only one SRS resource is configured, SRI is assumed to be implicitly present and pointing to that SRS resource, for the purpose of determining PUSCH beam.

Proposal 4.2: For PUSCH without associated SRI, i.e., (a) when SRS is not configured, or (b) PUSCH is scheduled with DCI format 0_0 and Nsrs>1 SRS resources are configured, the transmit beam to be used is that determined by the PUCCH-SpatialRelationInfo associated with a PUCCH-Resource of fixed PUCCH-ResourceIndex

- Index = 0 or 127 (Note: there are maximum 128 PUCCH-Resources)

- In case of CA/DC, this PUCCH-Resource is associated with the PUCCH-PCell or PUCCH-PSCell if it is in the same frequency band (FR1 or FR2) as the PUSCH. Otherwise, the PUCCH-Resource is associated with a fixed CC in the same frequency band as the PUSCH.

Proposal 4.3: After RACH procedure, until RRC configuration and MAC-CE activation allows default PUSCH beams to be based on PUCCH resources, the beams identified during RACH procedure serve as default. These are the beam of SSB identified by the UE for DL, and the beam of the PRACH or msg3 transmission that led to completion of RACH procedure for UL.

Proposal 4.4: For PDSCH or PUSCH with slot-repetition, all slots share the same QCL/beam. PUSCH beam determination is independent of whether PUSCH carries UCI, SCH data, or both.

Proposal 5.1: A Transition Window shall be specified in the BWP switch, wherein during the Transition window the QCL assumptions for reception of DL Control and Data and transmission of UL control and Data in the new BWP are based on the QCL assumptions derived from the RSs in the old BWP.

Proposal 5.2: A simple rule can be specified to interpret the TCI states for Control/Data during the Transition Window. Transition window can be defined to have a length spanning the start of reception in the new BWP to the first RS transmission occasion in the new BWP.

Proposal 7: If UE Feature 2-4 TCI states for PDSCH is 1, then the UE is expected to support only single beam operation where both the PDCCH and PDSCH share the same TCI state, and the UE is not expected to be configured with TCI-PresentInDCI=Enabled.

Companies views:

| Company | Comment |
| --- | --- |

# References

- 3GPP, RAN1#92, Chairman’s Notes

- R1-1805836	Remaining issues on QCL	ZTE

- R1-1805961	Remaining issues and TP for QCL assumptions	Huawei, HiSilicon

- R1-1806054	Remaining issues on QCL	vivo

- R1-1806229	Remaining issues on QCL	Ericsson

- R1-1806287	Remaining issues on QCL	CATT

- R1-1806342	Remaining issues on QCL	Lenovo, Motorola Mobility

- R1-1806515	On the remaining issues of QCL	Intel Corporation

- R1-1806727	Remaining issues on QCL	Samsung

- R1-1806844	Text Proposal for QCL	OPPO

- R1-1807351	Remaining details on QCL	Qualcomm Incorporated

Appendix – Proposals

Proposals from different companies related to this topic are listed in the following:

ZTE, Sanechips (R1-1805836)

Proposal 1: RAN1 confirms that as a reference RS ID for TCI configuration, TRS should be represented by a CSI-RS resource ID representing a CSI-RS resource in a CSI-RS resource set configured with TRS-Info. The IE CSI-RS-for-tracking which is part of the TCI-State can be removed as the following TP for 38.331 shown:

#### – TCI-State ----------------------------begin text change------------------------ QCL-Info ::= SEQUENCE { -- The carrier which the RS is located in. If the field is absent, it applies to the serving cell in which the TCI-State is configured cell ServCellIndex OPTIONAL, -- Need R -- The DL BWP which the RS is located in. bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated -- Cond NZP-CSI-RS-Indicated: mandatory if csi-rs or csi-RS-for-tracking is included, absent otherwise referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb SSB-Index, -- A set of CSI-RS resources for tracking csi-RS-for-tracking NZP-CSI-RS-ResourceSetId }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD}, ... } -- TAG-TCI-STATE-STOP -- ASN1STOP ---------------------------- end text change------------------------

Proposal 2: Confirming the working assumption on ‘same’ slot in Rel-15.

Proposal 3: For each periodic/semi-persistent CSI-RS resource for BM, inference of the channel among symbols transmitted on the same antenna port can be maintained across slots.

TP 1：Capture the following description in section 5.1.6.1.2 of 38.214[3]:

If a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the antenna ports of CSI-RS resources, described in Subclause 5.2.2.3.1, within the NZP-CSI-RS-ResourceSet are same, where the CSI-RS resources in the NZP-CSI-RS-ResourceSet are transmitted in different OFDM symbols. If repetition is set to 'off', the UE shall not assume that the antenna ports of CSI-RS resources within the NZP-CSI-RS-ResourceSet are same. The UE is not expected to receive different periodicity in periodicityAndOffset and different nrofPorts in every CSI-RS resource within the set configured with layer parameter repetition. For each resource configured with resourceType set to 'periodic' or ‘Semi-persistent’, the UE can drive the measurement for computing L1-RSRP based on the received CSI-RS across slots.

Proposal 4: For the periodic CSI-RS resource for tracking, inference of the channel among symbols transmitted on the same antenna port can be maintained across slots.

TP 2：Capture the following description in section 5.1.6.1.1 of 38.214[3]:

A UE does not expect to be configured with a CSI-ReportConfig that is linked to a CSI-ResourceConfig containing an NZP-CSI-RS-ResourceSet configured with trs-Info and with the CSI-ReportConfig configured with the higher layer parameter timeRestrictionForChannelMeasurements set to ‘configured’. For each resource configured with resourceType set to 'periodic', the UE can measure the channel based on the received CSI-RS across slots.

TP 3：Capture the following description in section 5.1.5 of 38.214[3]:

----------------------------begin text change------------------------ - If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition , the UE should only expect - 'QCL-TypeA' or 'QCL-TypeB' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info or - 'QCL-TypeD' with SS/PBCH block or, - 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - If a CSI-RS resource in a NZP-CSI-RS-ResourceSet is configured with higher layer parameter repetition, the UE should only expect - 'QCL-TypeA' configuration with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - {'QCL-TypeC' and 'QCL-TypeD'} configurations with SS/PBCH block, or - 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - For the DM-RS of CORESET scheduling the PDSCH, the UE should only expect - 'QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and QCL-TypeD' with either SSB/PBCH block or a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - {'QCL-TypeA' and 'QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - For the DM-RS of PDSCH, the UE should only expect - ’QCL-TypeA' or {QCL-TypeA' and QCL-TypeD'} configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, or - ’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and QCL-TypeD' with either SSB/PBCH block or a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - {QCL-TypeA' and QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with a CSI-RS resource in a NZP-CSI-RS-ResourceSet with higher layer parameter trs-Info ----------------------------End text change------------------------

TP 4: Capture the following description in section 5.1.5 of 38.214[3].

- The UE receives an activation command [10, TS 38.321] used to map up to 8 TCI states to the codepoints of the DCI field ‘Transmission Configuration Indication’. When the HARQ-ACK corresponding to the PDSCH carrying the activation command is transmitted in slot n, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from slot n+3Nslotsubframe,µ.After a UE receives initial higher layer configuration of TCI states and before reception and application of the activation command, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the SSB determined in the initial access procedure with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial Rx parameters, where applicable. After a UE receives higher layer re-configuration of TCI states, the UE may assume that the activated TCI state(s) configured before the re-configuration are remained activated until reception and application of next activation/deactivation command.

TP 5: Capture the following description in section 10.1 of 38.213 [7].

- If a UE has received initial configuration of more than one TCI states by higher layer parameter TCI-StatesPDCCH containing more than one TCI states but has not received a MAC CE activation command for one of the TCI states, the UE assumes that the DM-RS antenna port associated with PDCCH reception is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure with respect to delay spread, Doppler spread, Doppler shift, average delay, and spatial Rx parameters, when applicable. After a UE receives higher layer re-configuration of TCI states, the UE may assume that the activated TCI state(s) configured before the re-configuration are remained activated until reception and application of next activation/deactivation command.

TP 6: Capture the following description in section 7.4.1.5.3 of 38.211[8]

The UE shall assume that CSI-RS is transmitted in a candidate slot only if all OFDM symbols of that slot corresponding to the configured CSI-RS resource are classified as 'downlink'. The UE shall assume that CSI-RS ports within one resources are QCL-ed w.r.t average gain, delay spread, Doppler spread, Doppler shift, and average delay parameters, spatial Rx parameters.

TP 7: Capture the following description in section 10.1 of 38.213[7].

- The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the control resource set configured by pdcch-ConfigSIB1 in MasterInformationBlock and for corresponding PDSCH receptions, and the DM-RS antenna port associated with a corresponding SS/PBCH reception are quasi co-located with respect to average gain, delay spread, Doppler spread, Doppler shift, average delay, and spatial Rx parameters, when applicable. The value for the DM-RS scrambling sequence initialization is the cell ID. A subcarrier spacing is provided by higher layer parameter subCarrierSpacingCommon in MasterInformationBlock.

Huawei, HiSilicon (R1-1805961)

Proposal 1: A UE shall not assume that two antenna ports are quasi co-located with respect to any QCL type unless specified otherwise.

Text proposal for the latest version of TS:

<Unchanged parts are omitted>

## 7.2	Physical resources

The frame structure and physical resources the UE shall assume when receiving downlink transmissions are defined in Clause 4.

The following antenna ports are defined for the downlink:

-	Antenna ports starting with 1000 for PDSCH

-	Antenna ports starting with 2000 for PDCCH

-	Antenna ports starting with 3000 for channel-state information reference signals

-	Antenna ports starting with 4000 for SS/PBCH block transmission

A UE shall not assume that two antenna ports are quasi co-located with respect to any QCL type unless specified otherwise.

<Unchanged parts are omitted>

Proposal 2: The agreement of QCL relationship for DMRS ports should be captured.

Text proposal for the latest version of TS:

<Unchanged parts are omitted>

### 5.1.5	Antenna ports quasi co-location

The UE may assume the DM-RS antenna ports in one DM-RS port group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial Rx parameters, when applicable.

The UE can be configured with a list of up to M TCI-State configurations within higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability. Each configured TCI state includes one RS set TCI-RS-SetConfig. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS port group of the PDSCH. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location type corresponding to each DL RS given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values:

<Unchanged parts are omitted>

Text proposal for the latest version of TS:

<Unchanged parts are omitted>

### 5.1.5	Antenna ports quasi co-location

The UE can be configured with a list of up to M TCI-State configurations within higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability. Each configured TCI state includes one  RS set TCI-RS-SetConfig1, or two RS sets TCI-RS-SetConfig1 and TCI-RS-SetConfig2. Each TCI-State contains parameters for configuring thequasi co-location relationship between one or two downlink reference signals and the corresponding DM-RS port group of the PDSCH. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location type corresponding to each DL RS given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values:

<Unchanged parts are omitted>

Proposal 3: A UE may assume the CSI-RS antenna ports of a CSI-RS resource configuration are quasi co-located with respect to the large scale parameters including average gain, Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx parameters, when applicable.

Text proposal of TS 38.214:

<Unchanged parts are omitted>

#### 5.2.2.3	Reference signal (CSI-RS)

##### 5.2.2.3.1	NZP CSI-RS

The UE can be configured with one or more NZP CSI-RS resource set configuration(s) as indicated by the higher layer parameters CSI-ResourceConfig, and NZP-CSI-RS-ResourceSet. Each NZP CSI-RS resource set consists of K≥1 NZP CSI-RS resource(s).

A UE may assume the CSI-RS antenna ports of a CSI-RS resource configuration are quasi co-located with respect to the large scale parameters including average gain, Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx parameters, when applicable.

The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource for each CSI-RS resource configuration:

<Unchanged parts are omitted>

Proposal 4: NR should capture the following QCL linkage into specification:  SSB  CSI-RS for CSI w.r.t. ‘QCL-type D’.

Proposal 5: NR should capture missed QCL linkage for PDSCH, e.g. TRS for type A and CSI-RS for BM for type D QCL relationships.

Text proposal of TS 38.214:

<Unchanged parts are omitted>

A UE should expect only the following qcl-Type configurations in the higher layer parameter TCI-State:

-	If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect

-	'QCL-TypeC' or {QCL-TypeC' and QCL-TypeD'} configurations with SS/PBCH block, or

-	’QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

-	If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition , the UE should only expect

-	'QCL-TypeA' or 'QCL-TypeB' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info or

-	 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'QCL-TypeD' with SS/PBCH block.

-	If a CSI-RS resource in a NZP-CSI-RS-ResourceSet is configured with higher layer parameter repetition, the UE should only expect

-	'QCL-TypeA' configuration with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or

-	{'QCL-TypeC' and 'QCL-TypeD'} configurations with SS/PBCH block, or

-	'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

-	For the DM-RS of CORESET scheduling the PDSCH, the UE should only expect

-	 'QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or

-	{'QCL-TypeA' and 'QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or

-	'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

-	For the DM-RS of PDSCH, the UE should only expect

-	’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, or

-	’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or

-	{QCL-TypeA' and QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with a CSI-RS resource in a NZP-CSI-RS-ResourceSet with higher layer parameter trs-Info or QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	{QCL-TypeA' and QCL-TypeD'} configuration with CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, or

-	{QCL-TypeA' and QCL-TypeD'} configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition respectively.

<Unchanged parts are omitted>

VIVO (R1-1803627)

Proposal 1: For channel/RS that are QCL’ed with aperiodic signals

- If UE does not receive any aperiodic signal within an X window, the periodic RS associated with the aperiodic RS could be used as the QCL source for the channel/RS.

- X is the same as the periodicity of the periodic signals;

- The QCL relationship between the channel/RS and the periodic RS is the same as the configured QCL relationship between channel/RS and the aperiodic RS.

Ericsson (R1-1806229)

- Confirm working assumption on antenna port definition

Text proposal for TS38.211

----------------------------begin text change------------------------

4.4.1 Antenna ports

An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed.

-	For DM-RS associated with PDSCH the channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are within the same scheduled PDSCH, same slot and in the same PRG as described in 3GPP TS 38.214 Section 5.1.2.3.

---------------------------- end text change------------------------

When the optional field qcl-InfoPeriodicCSI-RS is not present, the UE may assume that consecutive instances of the NZP CSI-RS resource are QCL with respect to all applicable QCL types.

We are also open to discuss other assumptions the UE may make when the optional field is not present.

CATT (R1-1806287)

Proposal 1: To capture the agreement in RAN1 #91 more precisely, modify the description of relationship between the two reference RSs in section 5.1.5 of 214.

Proposal 2: To capture the agreement on supporting up to 2 DMRS groups, modify the description in section 5.15 of 214.

Proposal 3: To capture the agreement on QCL linkage between SSB and CSI-RS for CSI, modify the description in section 5.15 of 214.

Based on discussion above, we propose to modify the text in section 5.1.5 of 214 as shown below.

-------------------------------------------------------------Start of TP to 38.214---------------------------------------------------- ### 5.1.5 Antenna ports quasi co-location The UE can be configured with a list of up to M TCI-State configurations within higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability. Each configured TCI state includes one or two RS sets TCI-RS-SetConfig. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS port group of the PDSCH. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, there is no overlap between the two QCL types, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location type corresponding to each DL RS given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values: - 'QCL-TypeA': {Doppler shift, Doppler spread, average delay, delay spread} - 'QCL-TypeB': {Doppler shift, Doppler spread} - 'QCL-TypeC': { Doppler shift, average delay} - 'QCL-TypeD': {Spatial Rx parameter} The UE receives an activation command [10, TS 38.321] used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. When the HARQ-ACK corresponding to the PDSCH carrying the activation command is transmitted in slot n, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from slot n+3Nslotsubframe,µ.After a UE receives [initial] higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the SSB determined in the initial access procedure with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial Rx parameters, where applicable. If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 of the PDCCH transmitted on the CORESET. If tci-PresentInDCI is set as 'disabled' for the CORESET scheduling the PDSCH or the PDSCH is scheduled by a DCI format 1_0, for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the CORESET used for the PDCCH transmission. If the tci-PresentInDCI is set as 'enabled', when the PDSCH is scheduled by DCI format 1_1, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the RS(s) in the RS set with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold Threshold-Sched-Offset, where the threshold is based on reported UE capability [12, TS 38.331]. For both the cases when tci-PresentInDCI = 'enabled' and tci-PresentInDCI = 'disabled', if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold Threshold-Sched-Offset, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located based on the TCI state used for PDCCH quasi co-location indication of the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are configured for the UE. If all configured TCI states do not contain 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. A UE should expect only the following qcl-Type configurations in the higher layer parameter TCI-State: - If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect - 'QCL-TypeC' or {QCL-TypeC' and QCL-TypeD'} configurations with SS/PBCH block, or - ’QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition , the UE should only expect - 'QCL-TypeA' or 'QCL-TypeB' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info or - 'QCL-TypeD' with a SS/PBCH block or CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - If a CSI-RS resource in a NZP-CSI-RS-ResourceSet is configured with higher layer parameter repetition, the UE should only expect - 'QCL-TypeA' configuration with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - {'QCL-TypeC' and 'QCL-TypeD'} configurations with SS/PBCH block, or - 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - For the DM-RS of CORESET scheduling the PDSCH, the UE should only expect - 'QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - {'QCL-TypeA' and 'QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - For the DM-RS of PDSCH, the UE should only expect - ’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, or - ’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - {QCL-TypeA' and QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with a CSI-RS resource in a NZP-CSI-RS-ResourceSet with higher layer parameter trs-Info or QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - {QCL-TypeA' and QCL-TypeD'} configuration with CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition. --------------------------------------------------------------END of TP to 38.214--------------------------------------------------

Lenovo, Motorola (R1-1806342)

Proposal 1: Confirm the working assumption in last meeting. The DMRS QCL should be restricted to the same slot.

Proposal 2: For CSI acquisition CSI-RS, CSI-RS resources in different occasions can be considered QCLed when the CSI-RS resource is not associated with ReportQuantity configured with timeRestrictionForChannelMeasurements or timeRestrictionForInterferenceMeasurements.

Proposal 3: For beam management CSI-RS, no QCL is assumed among CSI-RS resource in different occasions.

Proposal 4: For CSI-RS used as TRS, QCL is assumed among all CSI-RS resources within a resource set.

Intel (R1-1806515)

Issue 1: clarify that QCL-Type D configuration should always provided to the UE in conjunction with QCL-Type A configuration, so that UE has all QCL parameter available for processing.

Issue 2: QCL signalling without TRS configuration: Certain cases defined for QCL assumes no TRS configuration, which contradicts to the current wording that UE should expect TRS configuration. Therefore, these cases should be considered as error and should be removed from the specification,

- For the DM-RS of CORESET scheduling the PDSCH, the UE should only expect the following TCI-State configuration: - qcl-Type1 with 'QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, and, if applicable, qcl-Type2 with 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. or - 'QCL-TypeA' and 'QCL-TypeD' configuration with SS/PBCH block if UE is not configured with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - For the DM-RS of PDSCH, the UE should only expect the following TCI-State configurations: - qcl-Type1 with ’QCL-TypeA' and, if applicable, qcl-Type2 with ‘QCL-TypeD’ configuration with a the CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, or - qcl-Type1 with ’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, and, if applicable, qcl-Type2 with ‘QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition or - {QCL-TypeA' and QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with a CSI-RS resource in a NZP-CSI-RS-ResourceSet with higher layer parameter trs-Info or QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - qcl-Type1 with {QCL-TypeA' and, if applicable, with qcl-Type2 with QCL-TypeD'} configuration with the CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition.

Issue 3: TRS configuration without SSB

- If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect the following TCI-State configurations: - qcl-Type1 with 'QCL-TypeC' and, if applicable, qcl-Type2 with or {QCL-TypeC' and QCL-TypeD'} configurations with the SS/PBCH block, or - qcl-Type1 with 'QCL-TypeC' with SS/PBCH block and, if applicable, qcl-Type2 with ’QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

Issue 4: Partial QCL information for CSI-RS for CSI acquisition

- If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition , the UE should only expect the following TCI-State configuration: - qcl-Type1 with 'QCL-TypeA' or 'QCL-TypeB' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, if applicable, with qcl-Type2 with 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.or - 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

Issue 5: QCL assumption for the same CSI-RS resource over time

##### 5.1.6.1.2 CSI-RS for L1-RSRP computation If a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the CSI-RS resources, described in Subclause 5.2.2.3.1, within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter, where the CSI-RS resources in the NZP-CSI-RS-ResourceSet are transmitted in different OFDM symbols. The UE is not expected to receive different periodicity in periodicityAndOffset and different nrofPorts in every CSI-RS resource within the set. If repetition is set to 'off', the UE shall not assume that the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter. The UE may assume that all CSI-RS resource transmissions with the same CSI-RS resource configuration identity for a given CC and BWP is quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

Issue 6: QCL assumption across CSI-RS resources

##### 5.1.6.1.2 CSI-RS for L1-RSRP computation If a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the CSI-RS resources, described in Subclause 5.2.2.3.1, within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filterQCL-ed wrt to spatial Rx parameters, where the CSI-RS resources in the NZP-CSI-RS-ResourceSet are transmitted in different OFDM symbols. The UE is not expected to receive different periodicity in periodicityAndOffset and different nrofPorts in every CSI-RS resource within the set. If repetition is set to 'off', the UE shall not assume that the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter filterQCL-ed wrt to spatial Rx parameters. #### … ##### 5.2.2.3.1 NZP CSI-RS The UE can be configured with one or more NZP CSI-RS resource set configuration(s) as indicated by the higher layer parameters CSI-ResourceConfig, and NZP-CSI-RS-ResourceSet. Each NZP CSI-RS resource set consists of K≥1 NZP CSI-RS resource(s). The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource for each CSI-RS resource configuration: - … - repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and defines whether CSI-RS resources are QCL-ed a repetition in conjunction with spatial domain transmission filter is on/off at gNB-side as described in Subclause 5.1.6.1.2. and can be configured only when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP' or ‘none'.

Issue 7: Lack of default QCL assumptions

According to current NR specification, a default assumption for QCL of antenna port is not defined. It is therefore propose to introduce default QCL assumption which states that antenna ports of reference signals are not QCL with any parameters unless specified otherwise.

Samsung (R1-1806727)

Proposals:

- For slot aggregation case, same slot in antenna port definition on DMRS port should be revisited.

### 4.4.1	Antenna ports

An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed. For DM-RS associated with PDSCH the channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are within the same scheduled PDSCH, the same scheduled slot(s), and in the same PRG as described in clause 5.1.2.3 of [6, TS 38.214].

Two antenna ports are said to be quasi co-located if the large-scale properties of the channel over which a symbol on one antenna port is conveyed can be inferred from the channel over which a symbol on the other antenna port is conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

OPPO (R1-1806844)

Suggestion 1: In Section 5.1.5 of TS 38.214, remove the RS set TCI-RS-SetConfig to align with TS 38.331.

Suggestion 2: In Section 5.1.5 of TS 38.214, capture the above-mentioned QCL assumption highlighted by BLUE.

| SSB CSI-RS for CSI w.r.t, spatial RX parameters | QCL type: D |
| --- | --- |

Suggestion 3: In Section 5.1.5 of TS 38.214, remove the corresponding description on the QCL assumptions highlighted by YELLOW.

Suggestion 4: In Section 5.1.5 of TS 38.214, adopt the following text proposal to capture the newly introduced QCL assumption between periodic TRS and aperiodic TRS:

-	If an aperiodic CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect

-	’QCL-TypeA' or {QCL-TypeA' and QCL-TypeD'} with a periodic CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.

Suggestion 5: In Section 5.1.5 of TS 38.214, adopt the following modification to capture that the current description about TRS is only for periodic TRS:

-	If a periodic CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect

-	'QCL-TypeC' or {QCL-TypeC' and QCL-TypeD'} configurations with SS/PBCH block, or

-	’QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

Suggestion 6: In Section 10.1 of TS 38.213, fix the description that the DMRS port of Type0-PDCCH/Type0A-PDCCH/Type2-PDCCH common search space is QCLed with the associated SS/PBCH block.

## TS38.214

Based on the discussions, we make the following text proposal to capture the above-mentioned suggestions and some minor corrections for Section 5.1.5 of TS 38.214

/******************** Start of Text Proposal for Section 5.1.5 of TS 38.214*********************/

- 5.1.5	Antenna ports quasi co-location

The UE can be configured with a list of up to M TCI-State configurations within higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS port group of the PDSCH. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location type corresponding to each DL RS given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values:

-	'QCL-TypeA': {Doppler shift, Doppler spread, average delay, delay spread}

-	'QCL-TypeB': {Doppler shift, Doppler spread}

-	'QCL-TypeC': { Doppler shift, average delay}

-	'QCL-TypeD': {Spatial Rx parameter}

The UE receives an activation command [10, TS 38.321] used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. When the HARQ-ACK corresponding to the PDSCH carrying the activation command is transmitted in slot n, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from slot n+3Nslotsubframe,µ.After a UE receives initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are spatially quasi co-located with the SSB determined in the initial access procedure with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial Rx parameters, where applicable.

If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 of the PDCCH transmitted on the CORESET. If tci-PresentInDCI is set as 'disabled' for the CORESET scheduling the PDSCH or the PDSCH is scheduled by a DCI format 1_0, for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the CORESET used for the PDCCH transmission.

If the tci-PresentInDCI is set as 'enabled', when the PDSCH is scheduled by DCI format 1_1, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the RS(s) in the RS set with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold Threshold-Sched-Offset, where the threshold is based on reported UE capability [12, TS 38.331]. For both the cases when tci-PresentInDCI = 'enabled' and tci-PresentInDCI = 'disabled', if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold Threshold-Sched-Offset, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located based on the TCI state used for PDCCH quasi co-location indication of the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are configured for the UE. If all configured TCI states do not contain 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

A UE should expect only the following qcl-Type configurations in the higher layer parameter TCI-State:

-	If a periodic CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect

-	'QCL-TypeC' or {QCL-TypeC' and QCL-TypeD'} configurations with SS/PBCH block, or

-	’QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

-	If an aperiodic CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect

-	’QCL-TypeA' or {QCL-TypeA' and QCL-TypeD'} with a periodic CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.

-	If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition , the UE should only expect

-	'QCL-TypeA' or 'QCL-TypeB' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info or

-	 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. or

-     ‘QCL-TypeD’ configuration with SS/PBCH block

-	If a CSI-RS resource in a NZP-CSI-RS-ResourceSet is configured with higher layer parameter repetition, the UE should only expect

-	'QCL-TypeA' configuration with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or

-	{'QCL-TypeC' and 'QCL-TypeD'} configurations with SS/PBCH block, or

-	'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

-	For the DM-RS of CORESET scheduling the PDSCH, the UE should only expect

-	 'QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or

-	'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

-	For the DM-RS of PDSCH, the UE should only expect

-	’QCL-TypeA' or {QCL-TypeA' and QCL-TypeD'} configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, or

-	’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or

-	' QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

/******************** End of Text Proposal for Section 5.1.5 of TS 38.214*********************/

## TS38.213

Based on the discussions, we make the following text proposal to capture the above-mentioned suggestion for Section 10.1 of TS 38.213

/******************** Start of Text Proposal for Section 10.1 of TS 38.213*********************/

<Text omitted>

If a UE is not provided higher layer parameter pagingSearchSpace for Type2-PDCCH common search space set, the association between PDCCH monitoring occasions for Type2-PDCCH common search space set and the SS/PBCH block index are same as the association of PDCCH monitoring occasions for Type0-PDCCH common search space set as described in Subclause 13. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level are given in Table 10.1-1.

If a UE is not provided higher layer parameter ra-SearchSpace for Type1-PDCCH common search space set, the control resource set and the association between PDCCH monitoring occasions for Type1-PDCCH common search space set and the SS/PBCH block index are same as the control resource set and the association of PDCCH monitoring occasions for Type0-PDCCH common search space set as described in Subclause 13. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level are given in Table 10.1-1.

The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the control resource set configured by pdcch-ConfigSIB1 in MasterInformationBlock and for corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to delay spread, Doppler spread, Doppler shift, average delay, and spatial Rx parameters. The value for the DM-RS scrambling sequence initialization is the cell ID. A subcarrier spacing is provided by higher layer parameter subCarrierSpacingCommon in MasterInformationBlock.

If a UE is configured with one or more downlink bandwidth parts (BWPs), as described in Subclause 12, the UE can be configured with PDCCH-ConfigCommon and PDCCH-Config for each configured DL BWP on the primary cell, other than the initial active DL BWP, as described in Subclause 12.

<Text omitted>

/******************** Start of Text Proposal for Section 10.1 of TS 38.213*********************/

Qualcomm (R1-1807351)

Proposal 1:  Support the following priority rule TP in Section 5.1 of 38.214 for the determination of QCL parameters and rate-matching behaviour when unicast PDSCH and configured CORESET are frequency division multiplex over the same OFDM symbol. Support the following TP in Section 5.1 of 214

--------------------------------------------------------- Beginning of change -------------------------------------------------------------

When receiving PDSCH conveying Msg4 of Random Access Procedure the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block the UE selected for RACH association and transmission, as defined in Subclause [TBD] of [6, TS 38.213], with respect to [spatial RX parameters].

A priority rule shall be defined when UE cannot simultaneously receive both PDCCH and PDSCH using the same spatial filter.

When unicast PDSCH is to be received by the UE in the same OFDM symbol containing a search space associated with the CORESET quasi-colocation parameters associated with the PDSCH takes precedence and the UE shall receive the PDSCH with its associated quasi co-location parameters. The UE is not expected to monitor this CORESET for PDCCH. Additionally, UE may assume that no PDCCH is transmitted in the REs indicated to receive unicast PDSCH conveyed by a PDCCH.

------------------------------------------------------------- End of change ----------------------------------------------------------------

Proposal 2.1: Adopting TCI state/QCLed RS of SPS PDSCH DMRS from the active TCI state of a suitable CORESET is supported.

Proposal 2.2: Support Alt-1. I.e., TCI state of each SPS PDSCH transmission is the default PDSCH TCI state associated with that transmission time.

--------------------------------------------------------- Beginning of change [38.214, 5.1.5]-------------------------------------------

For both the cases when TCI-PresentInDCI = 'Enabled' and TCI-PresentInDCI = 'Disabled', if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold Threshold-Sched-Offset, or if PDSCH is semi-persistently scheduled, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located based on the TCI state used for PDCCH quasi co-location indication of the lowest CORESET-ID in the latest slot in which one or more CORESETs are configured for the UE.

------------------------------------------------------------- End of change ----------------------------------------------------------------

Proposal 2.3: When TCI state/QCLed RS of PDSCH is adopted from the active TCI state of a CORESET, it is the current active TCI state of the CORESET. Here, ‘current’ refers to the time of the beginning of PDSCH-carrying slot.

--------------------------------------------------------- Beginning of change -------------------------------------------------------------

If a UE is configured with the higher layer parameter TCI-PresentInDCI that is set as 'Enabled' for the CORESET scheduling the PDSCH, the UE assumes that the TCI field is present in the DL DCI of the PDCCH transmitted on the CORESET. If TCI-PresentInDCI is set as 'Disabled' for the CORESET scheduling the PDSCH or the PDSCH is scheduled by a DCI format 1_0, for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state for the PDSCH is identical to the current TCI state applied for the CORESET used for the PDCCH transmission, where the current TCI state is the state at the beginning of the slot carrying the PDSCH.

If the TCI-PresentinDCI is set as 'Enabled', the UE shall use the TCI-States according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the RS(s) in the RS set with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold Threshold-Sched-Offset, where the threshold is based on UE capability. For both the cases when TCI-PresentInDCI = 'Enabled' and TCI-PresentInDCI = 'Disabled', if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold Threshold-Sched-Offset, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located based on the current TCI state used for PDCCH quasi co-location indication of the lowest CORESET-ID in the latest slot in which one or more CORESETs are configured for the UE. If all configured TCI states do not contain ‘QCL-TypeD’, the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

------------------------------------------------------------- End of change ----------------------------------------------------------------

Proposal 3: In FR2, for any NZP CSI-RS resource set with CSI-RS Repetition ‘ON’, if there is a symbol-level overlap of a scheduled PDSCH with any symbol of a CSI-RS resource in the set, PDSCH is symbol-level rate-matched on the overlapping OFDM symbols.

- Note: UE is not expected to be configured with a PDSCH which results in collision between the symbols carrying DMRS and this CSI-RS resource.

- Note: PTRS is symbol-level punctured on the OFDM symbols transmitting this CSI-RS resource.

In FR2, for a NZP CSI-RS resource set with CSI-RS-Repetition ‘OFF’, or when CSI-RS-Repetition is not configured, if QCL type D is different between a scheduled PDSCH and a CSI-RS resource in the set, then PDSCH is symbol-level rate-matched on the overlapping OFDM symbols, otherwise PDSCH is RE-level rate-matched.

- Note: UE does not expect to be configured with a PDSCH which results in any DMRS to occupy the same OFDM symbol with a CSI-RS resource with different QCL Type D.

- Note: PTRS is symbol-level punctured on the OFDM symbols transmitting with a CSI-RS resource with different QCL Type D

Proposal 4.1: If only one SRS resource is configured, SRI is assumed to be implicitly present and pointing to that SRS resource, for the purpose of determining PUSCH beam.

Proposal 4.2: For PUSCH without associated SRI, i.e., (a) when SRS is not configured, or (b) PUSCH is scheduled with DCI format 0_0 and Nsrs>1 SRS resources are configured, the transmit beam to be used is that determined by the PUCCH-SpatialRelationInfo associated with a PUCCH-Resource of fixed PUCCH-ResourceIndex

- Index = 0 or 127 (Note: there are maximum 128 PUCCH-Resources)

- In case of CA/DC, this PUCCH-Resource is associated with the PUCCH-PCell or PUCCH-PSCell if it is in the same frequency band (FR1 or FR2) as the PUSCH. Otherwise, the PUCCH-Resource is associated with a fixed CC in the same frequency band as the PUSCH.

Proposal 4.3: After RACH procedure, until RRC configuration and MAC-CE activation allows default PUSCH beams to be based on PUCCH resources, the beams identified during RACH procedure serve as default. These are the beam of SSB identified by the UE for DL, and the beam of the PRACH or msg3 transmission that led to completion of RACH procedure for UL.

Proposal 4.4: For PDSCH or PUSCH with slot-repetition, all slots share the same QCL/beam. PUSCH beam determination is independent of whether PUSCH carries UCI, SCH data, or both.

Proposal 5.1: A Transition Window shall be specified in the BWP switch, wherein during the Transition window the QCL assumptions for reception of DL Control and Data and transmission of UL control and Data in the new BWP are based on the QCL assumptions derived from the RSs in the old BWP.

Proposal 5.2: A simple rule can be specified to interpret the TCI states for Control/Data during the Transition Window. Transition window can be defined to have a length spanning the start of reception in the new BWP to the first RS transmission occasion in the new BWP.

Proposal 6:  Support the following TP in section 10.1 of 38.213

If a UE has received initial configuration of more than one TCI states by higher layer parameter TCI-StatesPDCCH containing more than one TCI states but has not received a MAC CE activation for one of the TCI states, the UE assumes that the DM-RS antenna port associated with PDCCH reception is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure with respect to delay spread, Doppler spread, Doppler shift, average delay, and spatial Rx parameters, when applicable

If a UE has received a configuration of more than one TCI states by higher layer parameter TCI-StatesPDCCH containing more than one TCI states but has not received a MAC CE activation for one of the TCI states, the UE uses the previous known configuration for the reception of PDCCH.

Proposal 7: If UE Feature 2-4 TCI states for PDSCH is 1, then the UE is expected to support only single beam operation where both the PDCCH and PDSCH share the same TCI state, and the UE is not expected to be configured with TCI-PresentInDCI=Enabled.