3GPP TSG RAN WG1 Meeting #93   			           	            R1-1807836

- Busan, Korea, May 21st – 25th, 2018

______________________________________________________________________Agenda item: 7.5.2

Source: LG Electronics

Title: Offline discussions on working assumptions in channel model

Document for: Discussion and decision

- Introduction

This contribution summarizes offline discussions on the working assumptions regarding the channel model of eV2X evaluation methodology.

- Offline discussion summary

Agreements: - The first proposal in Section 3 of R1-1807755 is agreed, with the except that the detailed table is agreed as a working assumption - If no additional concensus can be reached by Friday, the working assumption automatically becomes agreements

Proposal: The following tables are used for LOS/NLOSv probability in highway and urban scenarios respectively:

Highway (probability vs. distance d)

| LOS (d<=475m) | a | b | c |
| --- | --- | --- | --- |
|  | 2.1013*10-6 | -0.002 | 1.0193 |
|  | P(LOS)= min(1,ad^2+bd+c) |  |  |
| LOS (d>475m) | PLOS= max⁡{0, 0.54-0.001*(d-475)} |  |  |
| NLOSv | P(NLOSv)=1-P(LOS) |  |  |

Urban (probability vs. distance d)

| LOS | PLOS=min{1, 1.05*exp(-0.0114*d)} |
| --- | --- |
| NLOSv | P(NLOSv)=1-P(LOS) |

Agreements: - The proposal in Section 6 of R1-1807755 is taken as a working assumption - If no additional concensus can be reached by Friday, the working assumption automatically becomes agreements

Proposal: The working assumption is confirmed.