3GPP TSG RAN WG1 Meeting #93			R1-1807843

Busan, Korea, May 21st – 25th, 2018

Title:	[Draft] LS on NR CSI-RS

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Huawei, HiSilicon, [RAN1]

To:	RAN2

Cc:	-

Contact Person:

Name: Xi Zhang

E-mail Address: panda.zhang at huawei dot com

1. Overall Description

RAN1 would like inform RAN2 about the new agreements made on NR CSI-RS, so that RAN2 can update the corresponding parts in RAN2 specs accordingly.

The changes are summarized as follows.

- For bandwidth configuration for CSI-RS

  - Reference point for startingRB is CRB#0 on the common resource block grid.

  - Value range of nrofRBs is from 24 to maxNrofPhysicalResourceBlocks + 1.

- For indication of time domain location for CSI-RS

  - Value range of firstOFDMSymbolInTimeDomain2 is from 2 to 12.

2. New agreements on NR CSI-RS made in RAN1#93

Agreement (Impact to RAN2 specifications)

Text proposal #3 for Section 5.2.2.3.1 in TS38.214

< Unchanged parts are omitted >

The bandwidth and initial CRB index of a CSI-RS resource within a BWP, as defined in Subclause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min24, NBWPsize.

< Unchanged parts are omitted >

Agreement (Impact to RAN2 specifications)

Change the value range of nrofRBs in CSI-RS bandwidth configuration from 24 to maxNrofPhysicalResourceBlocks + 1 and include as part of an LS to RAN2. Ask RAN2 to update the relevant description.

Agreement

Suggest RAN2 to change the value range of firstOFDMSymbolInTimeDomain2 as from 2 to 12 and include as part of an LS to RAN2.

3. Actions:

RAN1 respectfully ask RAN2 to take above agreements into account and update the corresponding parts in RAN2 specs accordingly.

4. Date of Next TSG-RAN1 Meetings:

TSG-RAN WG1 Meeting #94	20 – 24 August 2018	Gothenburg, Sweden