3GPP TSG RAN WG1 Meeting 93		 R1-1807973

Busan, Korea, May 21st – 25th, 2018

Agenda item:	7.1.5

Source: 	Qualcomm Incorporated

Title: 	Summary of email discussion [93-NR-01] LS on intra-band EN-DC MPR, A-MPR

Document for:	Discussion/Decision

# Introduction

This contribution is the summary of email discussion [93-NR-01], LS on intra-band EN-DC MPR, A-MPR.

The subject of the email discussion is to answer the following:

- whether any response is needed to the RAN4 LS (R1-1807805) and

- if needed, the response LS details

The RAN4 LS [2] “LS on RAN4 agreement on intraband EN-DC A-MPR” informed RAN1 about the RAN4 agreements on MPR/A-MPR.

The contents of the LS are copied below.

---------------------------------------------------------------------------------------------------------------------------------------

1. Overall Description:

RAN4 has been studying the impact of intra-band EN-DC intermods and reverse intermods on the ability of UEs to meet emission requirements. Power reduction techniques including MPR and A-MPR can be used to minimize the impact of these IMDs and RIMDs. Over the last several meetings, RAN4 has been working on MPR and A-MPR with the assumption of equal PSD between LTE and NR, and equal back-off for E-UTRA and NR, which may help to minimize the amount of MPR and/or A-MPR that is required. Recently an issue has been raised by a single company [1] in RAN4 that there may be a potential contradiction between the RAN1 design for power sharing and the RAN4 assumption for MPR and A-MPR.  While typically MPR and A-MPR decision are left to RAN4, RAN4 would like to inform RAN1 of this assumption that has been made for MPR and A-MPR, and ensure that it is consistent with the RAN1 power control architecture.

2. Actions:

To RAN1:

RAN4 respectfully requests RAN1 to consider whether the RAN4 definition of MPR and A-MPR where the calculation of transmission power for LTE may take into consideration NR transmission and vice versa is consistent with RAN1 power control design. This is a similar approach that has been taken in E-UTRA intraband CA.

---------------------------------------------------------------------------------------------------------------------------------

Also copied below the relevant RAN1 agreement on EN-DC power control:

----------------------------------------------------------------------------------------------------------------------------------

R1-1715185, Way forward on power sharing for LTE-NR NSA operation, Ericsson, CATT, Samsung, Mediatek, Nokia, NSB

- Agreements:

  - At least for LTE-NR NSA operation

    - Maximum allowed power values for LTE (P_LTE) and NR (P_NR) are set separately

      - i.e., when UE is configured for NR, P_LTE can be configured up to P_cmax and  P_NR can be configured up to P_cmax.

      - e.g. P_LTE + P_NR > P_cmax or P_LTE + P_NR = P_cmax

    - Signaling details for P_LTE, P_NR are left to RAN2, RAN4.

    - Note: ‘P_cmax’ is a limit that is similar to ‘The configured maximum UE output power’ that was specified for LTE.

    - Note: The network will still have flexibility to prioritize or reserve certain NR transmission power depending on network implementation

    - All UEs are mandated to handle P_LTE + P_NR = P_cmax while handling of P_LTE + P_NR > P_cmax depends on UE capability

    - At least, when DL/UL LTE sTTI/reduced UE processing time based operation is not configured for the UE, if total transmit power exceeds P_cmax when there is simultaneous NR and LTE UL tx,

      - For NR, UE scales down/drops NR transmission and NR power scaling details are left to UE implementation (note: it is not intended to have RAN4 test from RAN1 perspective)

        - If there are two or more UL carriers, the power scaling or tx dropping can be performed for each of the UL carriers separately or jointly up to UE implementation

      - For LTE, no change in power control procedure

    - FFS the case when DL/UL LTE sTTI/reduced UE processing time based operation is configured for the UE

    - The following is FFS

      - The case when P_NR is configured such that P_NR < P_cmax, and UE can use power up to P_cmax in NR when it knows that there will be no UL transmission in LTE by semi-static configuration (e.g., measurement gap, DL/UL configuration)

--------------------------------------------------------------------------------------------------------------------------------------

# Discussion

Companies are asked to add their views in the Table below on the following:

- whether any response is needed to the RAN4 LS (R1-1807805) and

- if needed, the response LS details

| Company | Response needed Yes / No | If Yes, proposed response |
| --- | --- | --- |
| Nokia | Preferably, not a necessarily | RAN1 thanks RAN4 of the LS and understands the concern on the definition of MPR and A-MPR definition relation to the functional description defined by RAN1. However, RAN1 considers the two to operate at different levels, one being the principal functionality and another being about how the requirements provide relaxation to the defined requirement. Given this, RAN1 did not agree on a necessity to redefine the functional specification. |
| Samsung | Yes | The RAN4 definition of MPR/A-MPR is not consistent with the RAN1 design for dynamic power control sharing in EN-DC (Type 1 UEs) or with the related LS previously provided to RAN4 from RAN1. It can require either (a) dropped transmissions to the gNB/SCG (without the gNB knowing – also, the RAN4 motivation for equal PSDs will not be achieved), or (b) stricter requirements for the LTE modem processing timelines, or (c) scheduling timelines for NR that are same or slower than the ones for LTE. None of the above is desirable. |
| Qualcomm | Yes | State that the RAN4 definition of MPR/A-MPR is not consistent with the RAN1 design. Any solution should not change the LTE processing time requirements. Should avoid solutions that only work when NR processing is made as slow as LTE. |
| Panasonic | Yes | On RAN4 LS "RAN4 has been working on MPR and A-MPR with the assumption of equal PSD between LTE and NR, and equal back-off for E-UTRA and NR, which may help to minimize the amount of MPR and/or A-MPR that is required", our understanding is the worst condition of MPR and A-MPR is equal PSD between LTE and NR. Therefore, if MPR and A-MPR are defined in such worst condition (maximum allowed MPR and A-MPR condition) but actual UE transmission of LTE and NR are not equal PSD (i.e. one of transmission of PSD is less than the condition to obtain MPR and A-MPR), there is no inconsistency with RAN1 agreement. On the other hand, if MPR and A-MPR are defined in such condition and UE's transmission is only allowed when two transmissions of PSD are equal, we think it is not consistent with RAN1 agreement. On the other hand, LS action part text says "whether the RAN4 definition of MPR and A-MPR where the calculation of transmission power for LTE may take into consideration NR transmission". This part of the text would not be consistent with RAN1 agreement. To ask the relation how to define MRP/A-MPR and final transmission allowed power (whether to support only same PSD or not) would be useful. In addition, to inform RAN1 agreement should be useful. |
| CATT | Yes. | RAN1 thanks RAN4 for the LS on intra-band EN-DC A-MPR setting. RAN4’s assumption of equal PSD between LTE and NR, and equal back-off for E-UTRA and NR in minimizing the amount of required MPR and/or A-MPR is against the UL power control framework and the independently UL power setting for LTE and NR for EN-DC. The principle of the power control in LTE and NR is to compensate the path loss and to adjust to the interference in meeting the target SINR in LTE and NR carrier independently. The assumption of equal PSD between LTE and NR would require the power setting of UL transmission either NR or LTE to relinquish its open power control loop and to accommodate the output power of the power control from the other technology. The equal PSD and back-off between LTE and NR also requires communication between LTE and NR processing at the UE and coordination in scheduling between LTE and NR at the base station. RAN1 would sincerely request RAN4 to change the UE assumption for intra-band EN-DC and study the IMD and RIMD when the UL transmission power are set independently based on the respective power control formula in LTE and NR. |
| T-Mobile USA | Perhaps. RAN4 is not asking for a response or action | RAN1 thanks RAN4 for the LS explaining the assumptions that they have used to determine MPR/A-MPR. These assumptions assume a worst case operating point. RAN1 designs are based on a model and assumptions in line with actual operating conditions. It may not be necessary to make any changes to RAN1 power control or RAN4 assumptions. However, RAN1 will continue to discuss this LS to determine if any changes need to take place. |
| LG | Yes | We have a similar question by Panasonic. If RAN4’s definition is applied for worst case A-MPR/MPR, there may be no conflict between RAN1 and RAN4 decisions. Yet, if that’s the case, LTE side will perform equal power backoff based on worst case A-MPR/MPR which may be worse than actual backoff. The gap between worst and actual A-MPR/MPR can be utilized by NR side. If this is the case, some clarification may be necessary. We also agree that LTE processing requirement should not be changed by RAN4 decision. In our view, asking clarification on A-MPR/MPR condition (e.g., worst A-MPR/MPR assuming worst conditions or actual A-MPR/MPR) and informing RAN1’s understanding regarding LTE/NR processing time would be necessary. |
| OPPO | Yes | RAN4 determined the MPR/A-MPR requirements for the worse case of equal PSD, whereas RAN1 specified a practical power control mechanism. In this sense, the MPR/A-MPR requirements are valid for all practical power control mechanisms including the one specified by RAN1. Thus there is no conflict between RAN1 and RAN4 on this issue. Given the above reasons, no change is necessary from the perspective of RAN1 and RAN4. |
| Ericsson | Yes | - It is RAN1 understanding that A-MPR/MPR definitions for EN-DC power sharing case will be handled by RAN4. - Regarding the following in RAN4 LS - “… assumption of equal PSD between LTE and NR…”, - RAN1 understanding is that according to both RAN1 and RAN4 specifications, there is no restriction to maintain equal PSD between LTE and NR (i.e., there can be e.g. PUSCH/PUCCH on NR and e.g. PUCCH/PUCCH on LTE transmitted simultaneously with different PSD), and that this assumption was made in RAN4 only for the purpose of deriving A-MPR values in the specification. - RAN1 requests RAN4 to confirm if this understanding is correct. - Regarding the following in RAN4 LS - “…where the calculation of transmission power for LTE may take into consideration NR transmission…”, - RAN1 did not assume this requirement for the UE as part of EN-DC power sharing framework. - RAN1 does not see a need to make RAN1 specification changes to take into account this difference in assumption between RAN1 and RAN4. |
| InterDigital | Yes | - The RAN4 decision to assume equal PSD for EN DC MPR/A-MPR derivations was motivated to support coexistence operation, as it was the case for intra-band LTE CA. Therefore, given NSA operation, it seems reasonable that RAN4 decided to proceed with the worst case PSD assumption for MPR/A-MPR calculations. - If power sharing is needed, an equal PSD assumption for MPR/A-MPR derivation only leads to an equal power reduction for LTE and NR transmissions, but it does not mandate actual transmissions with equal PSD. Therefore, we don’t see any contradiction and/or an impact in RAN1 specification. |
| MediaTek | Yes | RAN1 confirm that the requirement “calculation of transmission power for LTE may take into consideration NR transmission” is not consistent with RAN1 power control design where LTE modem is not required to know NR configuration. NR can apply much shorter scheduling delay than LTE. If LTE MRP/A-MPR needs to base on NR configuration, LTE modem will be forced to comply with NR timing requirement. There will introduce significant change impact to LTE side of an EN-DC UE. RAN1 understand that equal PSD assumption between LTE and NR is a possible consequence regarding the channel similarity under intra-band setting and implies the minimal UE power back-off which can be useful for system performance. On the other hand, to avoid NR timing requirement imposed on LTE modem of an EN-DC UE, RAN1 will add the following note in RAN1 spec related to EN-DC power control: UE doesn’t expect NR UL scheduling or trigger delay is smaller than 4 ms whenever LTE MPR/A-MPR related to NR configuration is required to apply. RAN1 respectfully request RAN4 to include the same constraint in the related RAN4 test requirements. |
| Huawei, HiSilicon | Yes | The RAN4 definition of MPR/A-MPR for intra-band EN-DC provided in the related LS is not consistent with the RAN1 design for dynamic power control sharing in EN-DC. RAN1 should inform RAN4 that the relevant RAN1 agreements on dynamic power sharing. |
| Sprint | Yes | General comments first, followed by proposed response to RAN4: We don’t believe that there is a conflict between the RAN1 and RAN4 agreements related to equal PSD, but maybe some misunderstanding. RAN4 has been working under the assumption of equal PSD for simulations and measurements, but it is not a RAN4 requirement that equal PSD always be maintained. For intra-band EN-DC, equal PSD is a likely scenario as a natural consequence of similar channel conditions. RAN4 has agreed to equal A-MPR in order to avoid extremely inefficient applications of A-MPR and to bound the required simulations and A-MPR conditions. We understand that the LTE timeline would need to be reduced or NR timeline increased so that LTE A-MPR can be calculated based on knowledge of NR uplink allocations. Given this is 2018, and technology has advanced in the last decade, we think that we should not dismiss out of hand more aggressive LTE timelines that enable EN-DC operation without crippling NR performance. But we understand companies would not like to mandate these for initial devices. We would hope that future EN-DC implementations could optionally include a more aggressive LTE timeline, at least for A-MPR calculation based on knowledge of NR uplink. Sprint believes that one problem with power sharing and A-MPR always favoring LTE is that uplink split bearers are not currently mandatory. If a bearer is only on NR, then applying A-MPR to the NR bearer only would be problematic. In order to maintain uplink performance while applying all power to LTE and/or all A-MPR to NR, split-bearer is required. Regardless of the outcome of the A-MPR issue, because of the issue of power sharing favoring LTE, RAN1 should send an LS to RAN2 requesting that RAN2 make uplink split bearers mandatory for EN-DC. Sprint agrees with the note proposed by MediaTek, with some slight modifications. When the NR scheduling or trigger delay is at least 4 ms or the minimum required for the UE implementation, the LTE modem can use the actual NR allocation for calculating LTE A-MPR. In the case where the NR scheduling or trigger delay is less than 4 ms, or less than the UE LTE modem can handle (if it can handle less than 4 ms), the UE can use the worst case NR allocation assumptions for calculating the LTE A-MPR, as is done for A-MPR for Type 2 UEs. The NR side of a Type 1 UE will always know the LTE allocation 4 ms in advance so the NR side can use the actual LTE allocation to calculate NR A-MPR. Proposed response to RAN4: RAN1 confirm that the requirement “calculation of transmission power for LTE may take into consideration NR transmission” is not consistent with RAN1 power control design where LTE modem is not required to know NR configuration. NR can apply much shorter scheduling delay than LTE. If LTE MRP/A-MPR needs to be based on NR configuration, LTE modem will be forced to comply with NR timing requirement. This may introduce a significant impact to LTE side of an EN-DC UE. It should be optional for a UE to comply with improved LTE timing and not mandatory, so capability signalling will likely be needed. RAN1 understands that equal PSD assumption between LTE and NR is a likely consequence regarding the channel similarity under intra-band setting and implies the minimal UE power back-off which can be useful for system performance. On the other hand, to avoid NR timing requirement imposed on LTE modem of an EN-DC UE, RAN1 will add the following note in RAN1 spec related to EN-DC power control: If the NR UL scheduling trigger is 4 ms, or greater than the minimum scheduling delay that the UE requires, the LTE modem can use the NR allocations to determine LTE A-MPR. If the NR scheduling trigger is less than required by the UE, the UE shall make worst case assumptions for the NR allocations for calculating LTE A-MPR, as is done for a Type 2 UE. The NR modem can use the LTE allocations for determining NR A-MPR in a Type 1 UE. RAN1 respectfully request RAN4 to include these same constraints in the related RAN4 test requirements. |
| Motorola Mobility, Lenovo | Yes | RAN4 assumption for LTE MPR/A-MPR depending on NR allocation is not consistent with the agreed RAN1 power control framework, which prohibits any modification of LTE transmission power in the EN-DC operation and is also not consistent with the generally faster timeline of NR compared to LTE. One suggestion in order to align RAN4 definitions with RAN1 framework (in line with the proposal by Sprint) is to have LTE MPR/A-MPR definition based on a worst case allocation for NR, but NR MPR/A-MPR definition based on the actual allocation for LTE. |
| Intel | No | If we need to send an LS reply to RAN4, the following is out input: RAN1 thanks RAN4’s LS on checking potential inconsistency between RAN1 and RAN4 definition of UL power control in EN-DC. As for Type 1 UE (dynamic power sharing capable UE), due to more flexible UE processing time of NR than LTE, there are some cases that RAN1 needs to take care of (e.g. when UE has already sent LTE UL before getting NR UL grant) for intra-band EN-DC, where RAN1 is working on. As for Type 2 UE (dynamic power sharing incapable UE), there is no issue with assuming the current RAN4 assumption of equal MPR/A-MPR is applied for Type 1 UE only since UE can drop or scale down NR UL by UE implementation according to the current specification. Thus, RAN1 would like to confirm there is no discrepancy between RAN1 and RAN4 assumptions while some further RAN1 works are required. <Background comments> First off, as for type 2 UE (dynamic power sharing incapable UE) assuming equal PSD is applied for type 1 UE only, we don’t think there is any issue due to advanced UE processing time line of NR since UE can drop or scale down UL NR transmission. As for type 1 UE (dynamic power sharing capable UE), we agree that there are some problematic cases as shown by Qualcomm due to different UE processing time. It doesn’t necessarily mean UE should be able to realize future scheduling grant of NR. It is not surprise at all as the different UE processing time issue has been well handled in RAN1 so far (e.g. UCI multiplexing in UL CA or non-SUL/SUL), which needs to be handled by RAN1 anyway. We hear the concern that RAN4’s current assumption may not be aligned with RAN1’s design philosophies (e.g. no look ahead, LTE part does not need to know NR part transmission, etc.). However, it is not really RAN1’s explicit agreement but it is indeed RAN1’s design assumptions for UL power control of EN-DC. Therefore, we do not agree to mention that the RAN4 assumption is inconsistent with RAN1 agreement especially given that MPR/A-MPR is a Black Box to RAN1 where it has been taken care of by RAN4 so far. Any potential issue should be able to be handled by RAN1; otherwise, such feedback of potential discrepancy may be able to cause the RAN4 working progress to four months back, which is based on equal PSD for intra-band EN-DC. In addition, we don’t think RAN4 would revisit the current RAN4’s assumption before they receive LS from RAN1 (thus, with the potential LS reply by RAN1, RAN4 might start re-discuss it in October). We believe such delay of RAN4 progress should be avoided especially by such LS from RAN1. In this sense, we believe it should be handled by RAN1 not by giving such negative impression. We also think the proposed solution of RP-180679 is a good starting point with some clarifications of the cases (e.g. applied SCSs between LTE and NR, SRS symbol location, etc.) while we don’t touch upon the current RAN4 assumption, which should be concluded in RAN1#94. Finally, regardless of this email discussion, we also like to second the phase coherency issue raised by Qualcomm which should be resolved by RAN1 for intra-band EN-DC with 1PA structure. We believe the proposed solution of RP-180679 can be a baseline to address the problem as well. |

# Summary

There were three topics commented on. These are summarized below

1)  Should RAN1 send a LS to RAN4 in response to R1-1807805?

- Yes:

  - Samsung, Qualcomm, Panasonic, CATT, LG, OPPO, Ericsson, InterDigital, MediaTek, Huawei, HiSilicon, Sprint, Motorola Mobility, Lenovo

- No:

  - Intel

- No strong view:

  - Nokia, T-Mobile USA

Recommendation: Send a response LS to RAN4

2)  Is it the understanding that the equal PSD assumption mentioned in the RAN4 LS means mandatory equal PSD operation?

- Clarification is necessary:

  - Panasonic, LG

- Yes, and this is not aligned with the RAN1 assumption:

  - CATT

- Yes, and this is aligned with the RAN1 assumption:

  - --

- No, and this is aligned with the RAN1 assumption:

  - OPPO, Ericsson (but clarification is necessary), InterDigital, Sprint, Intel, MediaTek, T-Mobile USA

- No input:

  - Nokia, Samsung, Qualcomm, Huawei, HiSilicon (but question on whether equal PSD is worst case assumption), Motorola Mobility, Lenovo

Recommendation: Include the following in the LS: RAN1’s view is that LTE UL and NR UL in intra-band EN-DC will not operate with equal PSD in general. It is RAN1’s understanding that this in itself would not be in conflict with RAN4’s equal PSD assumption because the MPR/A-MPR values defined by RAN4 would be applied irrespective of the PSD delta between LTE and NR. RAN1 would like to ask RAN4 to kindly clarify whether this is the correct interpretation of the RAN4 agreements.

3)  Is the RAN4 assumption “the calculation of transmission power for LTE may take into consideration NR transmission” consistent with RAN1 power control design?

- Yes:

  - Nokia, T-Mobile USA, Intel

- Qualified Yes: (if NR scheduling could meet 4 ms, or LTE modem could meet shorter scheduling timeline)

  - Sprint

- No:

  - Samsung, Qualcomm, Panasonic, LG, Ericsson, MediaTek, Huawei, HiSilicon, Motorola Mobility, Lenovo

- No input:

  - CATT, OPPO, InterDigital

Recommendation: Regarding the RAN1 action in the RAN4 LS: “RAN4 respectfully requests RAN1 to consider whether the RAN4 definition of MPR and A-MPR where the calculation of transmission power for LTE may take into consideration NR transmission and vice versa is consistent with RAN1 power control design.”, inform RAN4 that the RAN4 definition is not consistent with the RAN1 power control design.  The basis for the RAN1 power control agreements was that LTE processing times in EN-DC are the same as in LTE CA and LTE DC, while NR processing times are faster and therefore the calculation of transmission power for LTE cannot consider the presence or RB allocation of simultaneous NR transmission(s). This assumption holds irrespective of whether the UE is dynamic power sharing capable.  RAN1 is not planning to change this assumption; therefore, RAN1 should ask RAN4 to reconsider their intra-band EN-DC MPR/A-MPR definition to make it consistent with the RAN1 power control definition.

# Conclusions

The following is a proposed draft LS response

------------------------------------------------------------------------------------------------------------------

RAN1 would like to thank RAN4 for their LS R1-1807805, “LS on RAN4 agreement on intraband EN-DC A-MPR”. In response, RAN1 would like to provide the following answers:

Regarding the following information included in the RAN4 LS: “Over the last several meetings, RAN4 has been working on MPR and A-MPR with the assumption of equal PSD between LTE and NR…”,

RAN1 would like to clarify the following:

RAN1’s view is that LTE UL and NR UL in intra-band EN-DC will not operate with equal PSD in general. It is RAN1’s understanding that this in itself would not be in conflict with RAN4’s equal PSD assumption because the MPR/A-MPR values defined by RAN4 would be applied irrespective of the PSD delta between LTE and NR. RAN1 would like to ask RAN4 to kindly clarify whether this is the correct interpretation of the RAN4 agreements.

Regarding the RAN1 action in the RAN4 LS: “RAN4 respectfully requests RAN1 to consider whether the RAN4 definition of MPR and A-MPR where the calculation of transmission power for LTE may take into consideration NR transmission and vice versa is consistent with RAN1 power control design.”

RAN1 would like to provide the following information:

The RAN1 majority view is that the RAN4 definition is not consistent with the RAN1 power control design while three companies in RAN1 think that the RAN4 definition is not inconsistent with RAN1 power control design. The basic design principle for the RAN1 power control agreements was that LTE processing times in EN-DC are the same as in LTE CA and LTE DC, while NR processing times are faster and therefore either the calculation of transmission power for LTE cannot take into account the RB allocation(s) of simultaneous NR transmission(s), or the calculation of transmission power for LTE cannot take into account both the presence and RB allocation(s) of simultaneous NR transmission(s), at least in the following cases:

- when the NR grant is less than 4ms in advance

- when the overlapping NR transmission incudes HARQ ACK/NAK corresponding to DL grant(s) less than 4ms in advance

- when either NR uses PUSCH mapping Type B or LTE and NR use different numerologies, or both, so multiple NR transmissions overlap with a single LTE subframe and those NR transmissions cannot be scheduled in the same DL control monitoring occasion 4ms earlier (in which case, in order to meet the LTE 4ms processing time, some NR grants may have to be even more than 4ms in advance).

This assumption holds whether or not the UE is dynamic power sharing capable. RAN1 is not planning to change this assumption and the RAN1 preference is to have the same assumptions between RAN1 and RAN4 for the EN-DC power control framework. RAN1 would like to understand the feasibility of aligning the power control framework from the RAN4 perspective.

RAN1 would like to note that the term “4ms in advance” in the above description means information sent at approximately the same time as the applicable LTE grant for the overlapping LTE transmission.

Some companies expressed views that while the calculation of transmission power for LTE cannot take into account the RB allocation(s) of simultaneous NR transmission(s) in the cases mentioned above, the calculation may be able to take into account the presence of NR transmissions in some of those cases. In the view of some other companies, this operation would not be feasible.

It is the view of RAN1 that a solution that makes NR as slow as LTE (LTE without shortened TTI or processing time), or makes NR even slower than LTE, would have severe effects on latency. At the same time, RAN1 doesn’t intend to preclude discussing partial solutions that rely on making NR as slow as LTE or making NR even slower than LTE. In addition, some companies in RAN1 expressed views that a solution that makes NR as slow as LTE, or makes NR even slower than LTE, would have negative effects on throughput from the user perspective and by that also reduce the system capacity for the NR side. However, in other companies’ view, this could be offset by lower MPR/A-MPR. RAN1 has not concluded on the corresponding impacts on throughput, and system capacity.

In addition, RAN1 would kindly request some information regarding the following in the RAN4 LS “with the assumption of equal PSD between LTE and NR, and equal back-off for E-UTRA and NR, which may help to minimize the amount of MPR and/or A-MPR that is required”. It is the view of some companies in RAN1 that equal back-off based on equal PSD assumption is the worst-case assumption in the sense that it might allow the least total combined output power among all possible assumptions; therefore, it is unclear why it is RAN4’s view that this would help minimize the amount of MPR/A-MPR required.

Action:  RAN1 would like to ask RAN4 to clarify the above questions and to take the above information into account in their further work.

--------------------------------------------------------------------------------------------------------------------

# References

- R4-1807927, “Power sharing and MPR/A-MPR for EN-DC”, Qualcomm

- R1-1807805, “LS on RAN4 agreement on intraband EN-DC A-MPR”, RAN4

- R1-1807865, “Proposals on LTE-NR coexistence”, Qualcomm

- RP-181304, “Discrepancy between RAN1 and RAN4 power control decisions”, Qualcomm