3GPP TSG RAN WG1 Meeting #93                                              R1-1805992

Busan, Korea, 21st – 25th May 2018

Agenda item:            6.2.9

Source:	MediaTek Inc.

Title:	Discussion on Modulation Enhancement

Document for:          Discussion

# 1	Introduction

In RAN1#92b, five solutions [1] are discussed:

- RAN1 to consider one of the following solutions to this issue in Rel-15 under TEI-15:

  - Solution 1: Select modulation scheme based on TBS and number of REs used for rate matching.

  - Solution 2: Introduce “Modulation overriding” field in DCI to change the modulation scheme.

  - Solution 3: Introduce an alternative table for 256QAM by RRC configuration.

  - Solution 4: Extend the MCS field in DCI to 6 bits, and introduce overlapped entries with different modulation scheme.

    - Study the possibility of a single MCS table covering from QPSK to 1024QAM.

  - Solution 5: NRB scaling for TBS selection (similar to TBS selection for special subframes)

In this paper, we discuss the Solutions 5 and 4.

# 2     Issues with PRB scaling for HARQ operations

In RAN1#92bis, issues were discussed with solution 5 as described in Figure 1 and 2 respectively.

- Issue 1: Re-transmission with TBS entries removed from Table

- Issue 2: Re-transmission with missed initial transmission DCI

Figure 1: Re-transmission with TBS entries removed from Table

Figure 2: Re-transmission with missed initial transmission DCI

To ensure correct HARQ operations with Solution 5, it is necessary to have the following:

- Issue 1: TBS of retransmission is same as initial PDSCH TBS.

- Issue 2: PDSCH associated with DCI of retransmission shall be self-decodable because UE may miss PDCCH associated with initial transmission.

Issues 1 and 2 can be solved by disabling PRB scaling for HARQ re-transmission with one-bit PRB scaling enable / disable flag in DCI. When disabling PRB scaling, TBS size determination is directly based on scheduled PRB in DCI. All entries in TBS determination become available. For retransmission with disabling PRB scaling, as shown in Fig. 3, TBS can be same as initial transmission TBS by proper configuration and there is no ambiguity on TBS determination even if initial PDCCH is missed.

Figure 1: example of retransmission DCI with same TBS as one in initial DCI

Solution 5 can be applied for all MCS indices. There is additional one bit to indicate enabling/disabling PRB scaling.

- If disabling PRB scaling, scaling factor is 1.

- If enabling PRB scaling, the multi-step procedure outlined in [3] is used for consistent PDSCH performance, where a scaling factor is determined based on a range of code rate values assuming a given CFI value and RS overhead. An example of enabling PRB scaling is shown in the appendix.

Proposal 1: Enable /Disable PRB scaling for HARQ re-transmission by DCI indication.

# 3     Comparison of Solution 4 and Solution 5

Both solutions 4 and 5 requires an additional bit in DCI.

- Solution 5 requires one-bit enabling / disabling of PRB scaling indication in DCI, as discussed in previous section.

- Solution 4 requires an additional bit for MCS indication with modulation change depending on CFI and CSI-RS overhead [2].

Observation 1: Solution 4 and Solution 5 requires an additional bit in DCI.

CFI and RS overhead summarized in Table 1, with further details given in the appendix.

- In solution 5 with CFI=3 and 24 CRS-RS, the required SNR difference is only 0.2dB compare to CFI=1. As the required SNR is approximately the same as the reference required SNR, there is consistent PDSCH performance and no change for the CQI tracking and reporting in the UE-OLLA procedure is expected.

- In Solution 4 with change of modulation from 16QAM to 64QAM, using same example as above the required SNR difference is only 6.5 dB. Using legacy table, the required SNR difference is 7.9 dB.

Observation 2: Solution 5 with PRB scaling provides more consistent PDSCH performance than Solution 4.

Note that although the code rate reduces to 0.61 with change of the modulation to 64QAM in Solution 4,  larger signal power are needed to maintain same Euclidean distance.  Table 2 shows the additional power in dB for modulation changes.

Observation 3: The PRB scaling solution can maintain consistent code rate under difference overhead scenarios. Moreover there is no additional power need because of using same modulation order.

|  | CFI = 1, Reference | CFI = 3, 24 CSl-RS ports |  |  |
| --- | --- | --- | --- | --- |
|  |  | Legacy | Change mod. Order | PRB scaling |
| Code rate | 0.58 | 0.91 | 0.61 | 0.57 |
| Required SNR (dB) | 18.2 | 26.1 (+7.9*) | 24.7 (+6.5*) | 18.0 (-0.2*) |
| Note*: Required SNR difference |  |  |  |  |

Table 1: summary of simulation results

|  | Additional required power in dB |
| --- | --- |
| From QPSK to 16QAM | 20*log10(sqrt(10/2)) = 7.0dB |
| From 16QAM to 64QAM | 20*log10(sqrt(42/10)) = 6.2dB |
| From 64QAMv to 256QAM | 20*log10(sqrt(170/42)) = 6.1dB |

Table 2: Addition required power to maintain same Euclidean distance

Proposal 2: Consider Solution 5 and Solution 4 to minimize impact on CQI tracking and reporting for modulation enhancement.

Solutions 4 and 5 have no impact on the CQI tracking and reporting. The same CQI table is used and there is no new reporting mechanisms required. Solution 4 requires larger required SNR difference when there is change of modulation. This means the SNR difference between MCS indices is not consistent. This complicates the MCS testing for UE to determine the adequate CQI indices to report. Solution 5 has the advantage of maintaining consistent PDSCH performance with no SNR difference. For these reason, we prefer Solution 5 from UE implementation viewpoint

Proposal 3: Adopt Solution 5 to maintain consistent PDSCH performance for MCS test in the UE.

# 5   Conclusions

Based on the discussion in section 2, we have the following observation and proposals:

Proposal 1: Enable /Disable PRB scaling for HARQ re-transmission by DCI indication.

Observation 1: Solution 4 and Solution 5 requires an additional bit in DCI.

Observation 2: Solution 5 with PRB scaling provides more consistent PDSCH performance than Solution 4.

Observation 3: The PRB scaling solution can maintain consistent code rate under difference overhead scenarios. Moreover there is no additional power need because of using same modulation order.

Proposal 2: Consider Solution 5 and Solution 4 to minimize impact on CQI tracking and reporting for modulation enhancement.

Proposal 3: Adopt Solution 5 to maintain consistent PDSCH performance for MCS test in the UE.

# 6	Appendix

|  | Legacy |  | Change modulation order |  | PRB scaling |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Case 1 | Case 2 | Case 1 | Case 3 | Case 1 | Case 4 |
| DL cell bandwidth | 10MHz |  |  |  |  |  |
| CRS port number | 2-CRS port |  |  |  |  |  |
| DMRS port number | 2-DMRS port |  |  |  |  |  |
| TM mode | TM9 |  |  |  |  |  |
| Precoder | Random# |  |  |  |  |  |
| PDSCH PRB number | 100 |  |  |  |  |  |
| MCS | 16 (64QAM table) 16QAM | 16 (64QAM table) 16QAM | 16 (64QAM table) 16QAM | 17 (64QAM table) 64QAM | 16 (64QAM table) 16QAM | 16 (64QAM table) 16QAM |
| ITBS | 15 | 15 | 15 | 15 | 15 | 15 |
| CFI | 1 (from PCFICH) | 3(from PCFICH) | 1 (from PCFICH) | 3(from PCFICH) | 1 (from PCFICH) | 3(from PCFICH) |
| CSI-RS port number | N.A. | 24-CSI-RS port | N.A. | 24-CSI-RS port | N.A. | 24-CSI-RS port |
| N_RB scaling factor | N.A. | N.A. | N.A. | N.A. | 1 | 0.625[Note1] |
| TBS (bits) | 30576 | 30576 | 30576 | 30576 | 30576 | 19080[Note1] |
| TBS+CRCs (bits) | 30720 | 30720 | 30720 | 30720 | 30720 | 19200 |
| RE NUM | 13200 | 8400 | 13200 | 8400 | 13200 | 8400 |
| Coderate | 0.58 | 0.91 | 0.58 | 0.61 | 0.58 | 0.57 |

Table 4: simulation parameters

Note 1:  PRB scaling for TBS selection. NRB scaling factor, α, is as follows (further details in [3])

if (r < (4.5/8 = 0.5625)),          α= 4/8 = 0.5;

else if (r < (5.5/8 = 0.6875)),   α  = 5/8 = 0.625;

else if (r < (6.5/8 = 0.8125)),   α  = 6/8 = 0.75;

else if (r < (7.5/8 = 0.9375)),   α  = 7/8= 0.825;

else                                         ,   α  = 8/8 =1;

Example of enabling PRB scaling:

- Step 1: All_RE = 100 * (12* 14 -12(CFI =1) - 12 (CRS port)-12 DMRS) = 13200 REs

- Step 2: Avail_RE = 100*(12*14- 36 (CFI from PCFICH) - 12 (CRS port) - 24 (CSI-RS port)-12(DMRS)) = 8400 REs

- Step 3: r = 8400/13200 = 0.63

- Step 4: NRB scaling factor α = 0.625, NPRB for TBS is 100*0.625=62PRBs.

- Step 5: TBS = 19080 when ITBS = 15 and NPRB = 62

Figure 2: TM9 PDSCH performance

# 6   Reference

[1] R1-1721268, ``WF on modulation enhancements,’’ Qualcomm, Intel, Verizon, KDDI, Samsung

[2] R1-1804938, Discussion on modulation enhancements, Qualcomm, RAN1#92bis, April 2018

[3] R1-1804135, ``Discussion on Modulation Enhancement,’’ Mediatek Inc., RAN1#92bis, April 2018