3GPP TSG-RAN WG1 Meeting #93	 R1-1806018

Busan, Korea, May 21st – 25th, 2018	Revision of R1-1804765

Agenda Item:	7.2.2

Source:	Ericsson

Title:	Precoder Update for SPS UL

Document for:	Information

# Introduction

The ITU target for URLLC reliability in IMT 2020 has been set to 10-5 error within 1ms for a 32B packet [1]. To reach the latency requirement configured grant has been identified as a key feature. In this contribution we discuss uplink precoding as a feature to improve robustness in combination with configured grant.

# Discussion

NR supports both codebook-based and non-codebook-based UL precoding where the codebook-based relies on the precoder being indicated in the uplink grant as in LTE. UL precoding can provide considerable improvements as illustrated in Figure 1.

Figure 1: Effective UL SINR with and without UL precoding. Simulation parameters according to Table 1 in Section 5. Precoding is limited to rank 1 LTE Rel-10 precoders on 4 antenna ports. For “No Precoding” transmissions are performed on a single antenna element.

Precoding can give significantly improved robustness to URLLC.

NR supports two types of configured grant, Type 1 and Type 2. Type 2 configured grant is dynamically activated and deactivated using L1 signalling. Using UL precoding in combination with configured grant the Type 2 is likely the most effective way since updating the precoder of the configured grant for Type 2 only requires L1 signalling. Precoder update of the configured grant can be achieved by a DCI indicating activation of the configured grant where the “activation” DCI comprises the updated precoder.

Precoder update for configured grant Type 2 can be achieved using configured grant “activation” DCI.

However, to benefit from precoding, the precoder needs to be frequently updated using an “activation” DCI. Furthermore, the UE shall confirm Type 2 “activation” DCI by triggering a configured grant confirmation [2]. The confirmation from the UE is a triggered MAC control element that will be transmitted on the configured grant resource. Frequently updated precoders for configured grant Type 2 may lead to significant “confirmation” signalling on the configured grant resource. Since the configured grant may be a shared resource among many UE to increase capacity this “confirmation” signalling is likely undesirable.

Precoder update for configured grant Type 2 using configured grant “activation” DCI leads to undesirable control signalling on configured grant resource especially when it is a shared resource.

It may hence be desirable to avoid “confirmation” signalling on the configured grant resource when only the precoder is updated for the configured grant Type 2. But to avoid error propagation (wrong precoder used due to missed updating DCI) some sort of confirmation is needed.

In scenarios where UL precoding is desirable to be used, also DL precoding can be expected to be used. This holds true also in scenarios where the UE has no DL data traffic since uplink grants are carried by PDCCH that also may need to be pre-coded. Hence, DL CSI including DL precoding information may be needed for PDCCH precoding.

Since, DL CSI can be a-periodically triggered using a UL grant resulting in a PUSCH transmission, a pre-coded PUSCH transmission comprising an a-periodic CSI report is clearly a confirmation that the UE received the UL grant. Such CSI reports are likely not desirable to be carried on the configured grant resource either but rather on a separate resource. Hence, a UE supporting UL precoding configured with configured grant Type 2 may likely receive other UL grants that could update the precoder of the configured grant and where the transmissions according to the other grants may serve as confirmations of configured grant precoder update. We make the following observation.

Precoder for configured grant Type 2 can be updated from other (dynamic) UL grants wherein the transmissions according to the other grants may serve as confirmation of updated precoder for the configured grant. Pre-coded UL transmission comprising a DL CSI report is an example of such transmissions.

# Conclusion

In this contribution we made the following observations:

- Precoding can give significantly improved robustness to URLLC.

Precoder update for configured grant Type 2 can be achieved using configured grant “activation” DCI.

Precoder update for configured grant Type 2 using configured grant “activation” DCI leads to undesirable control signalling on configured grant resource especially when it is a shared resource.

Precoder for configured grant Type 2 can be updated from other (dynamic) UL grants wherein the transmissions according to the other grants may serve as confirmation of updated precoder for the configured grant. Pre-coded UL transmission comprising a DL CSI report is an example of such transmissions.

# References

- ITU-R M.[IMT-2020-TECH PERF REQ] (WP5D-TD-0300)

- TS 38.321, Medium Access Control (MAC) protocol specification, V15.0.0

# Appendix

Table 1: System simulation parameters

| Numerology | 7000 TTIs/sec, 30 kHz SCS, 4 OFDM/TTI |
| --- | --- |
| Duplex | FDD |
| Propagation | ITU UMa Model B |
| Carrier frequency/bandwidth | 700 MHz, 20 MHz (50 PRBs) |
| Deployment | Hexagonal Grid, One 3-sector site |
| BS TX power | 46 dBm |
| UE TX power | 23 dBm |
| ISD | 500 m |
| BS antenna configuration | 4 TX/RX, 2x1 X-pol antenna array, separation = 0.5 lambda, height= 25 m |
| UE antenna configuration | 1x2 X-pol, separation = 0.5 lambda No precoding: 1 TX Precoding: 4 TX, LTE Rel-10 rank 1 precoders |
| PDCCH | 2 symbols (symbol #0-#1), 6 REGs/CCE, ¼ DMRS overhead |
| PDSCH | 2 symbols (symbol #2-#3), DMRS: 6 RE per PRB |
| Short PUCCH | 1 symbol (symbol #3) |
| PUSCH | 2 symbols (symbol #0-#1), not frequency-multiplexed with PUCCH, DMRS: In symbol #0 |
| Traffic model | Periodic packets of size 32 B on application level. TBS comprises overhead for PDCP, RLC UM and MAC |
| UE distribution | No eMBB traffic 80% outdoor: 30 km/h 20% indoor: 3 km/h 30 UEs in 3 sectors, i.e. on average 10 UE per sector |
| Mobility | Multi-path only (no macro mobility) |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| CSI | A-periodic CSI targeting every 5th TTI |
| SRS | Cell-specific SRS resource: every 5th TTI, UE SRS period: every 10th TTI, PUSCH rate-matched around SRS |
| Impairments | Ideal channel estimation |