3GPP TSG RAN WG1 Meeting #93														R1-1806239

Busan, Korea, May 21th – 25th, 2018

Agenda Item:	7.1.7

Source:	Ericsson

Title:	Discussion on UE capability list

Document for:	Discussion, Decision

# Introduction

In this contribution we provide our view related to the UE feature list that was discussed in RAN1 and RAN plenary. We have provided our input in the column ‘Ericsson view RAN1#93 Busan’. Note also that we have additional views related to the incoming RAN2 LS (R2-1806507) in the contribution R1-1806240. Those views are not reflected below, but should be considered in the discussion.

# Input on UE feature list

| Features | # | Feature group |  |  |  | Components |  | Prerequisite feature groups (listed in this sheet only) |  |  |  | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) |  | Consequences if the feature is not supported by the UE |  | Type (see R2-1712078) | Need of FDD/TDD differentiation |  | Need of FR1/FR2 differentiation | RAN5 implication | Note |  | Responsible WG | RAN WG recommendation |  | TSG-RAN decision | Ericsson view RAN1#93 Busan |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL |  |  |  | 1) CP-OFDM for DL 2) CP -OFDM for UL |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  |  |  | RAN1 | Mandatory without capability signaling |  | Mandatory without capability signaling |  |
|  | 0-2 | DFT-S-OFDM waveform for UL |  |  |  | Transform precoding for single-layer PUSCH |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  |  |  | RAN1 | Mandatory without capability signaling |  | Mandatory without capability signaling |  |
|  | 0-3 | DL modulation scheme |  |  |  | 1) QPSK modulation 2) 16QAM modulation 3) 64QAM modulation for FR1 |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  | RAN4 will check 64QAM modulation for FR2 |  |  | Mandatory without capability signaling |  | Mandatory without capability signaling |  |
|  | 0-4 | UL modulation scheme |  |  |  | 1) QPSK modulation 2) 16QAM modulation |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  |  |  |  | Mandatory without capability signaling |  | Mandatory without capability signaling |  |
|  | 0-5 | pi/2-BPSK for PUSCH |  |  |  | pi/2-BPSK for PUSCH |  | 0-2 |  |  |  | Yes |  | pi/2-BPSK for PUSCH is not possible |  | [Type 3] RAN4 to decide | RAN4 to decide |  | RAN4 to decide |  | RAN4 will discuss if it is per band, common for all bands or FR1/2 |  | RAN4 |  |  |  |  |
|  | 0-6 | 64QAM for PUSCH |  |  |  | 64QAM for PUSCH |  |  |  |  |  | Yes |  | 64QAM for PUSCH is not possible |  | [Type 3] RAN4 to decide | RAN4 to decide |  | RAN4 to decide |  | RAN4 will discuss if it is per band or common for all bands |  | RAN4 |  |  |  |  |
|  | 0-7 | 256QAM for PDSCH |  |  |  | 256QAM for PDSCH |  |  |  |  |  | Yes |  | 256QAM for PDSCH is not possible |  | [Type 3] RAN4 to decide | RAN4 to decide |  | RAN4 to decide |  | RAN4 will discuss if it is per band or common for all bands |  | RAN4 |  |  |  |  |
|  | 0-8 | 256QAM for PUSCH |  |  |  | 256QAM for PUSCH |  |  |  |  |  | Yes |  | 256QAM for PUSCH is not possible |  | [Type 3] RAN4 to decide | RAN4 to decide |  | RAN4 to decide |  | RAN4 will discuss if it is per band or common for all bands |  | RAN4 |  |  |  |  |
|  | 0-9 | Subcarrier spacings and FFT size in conjunction with supportable BW with normal CP |  |  |  | 1) 15kHz 2) 30 kHz 3) 60 kHz 4) 120 kHz |  |  |  |  |  | Yes |  |  |  | Type 4 [Type 3] RAN4 to decide | RAN4 to decide |  | RAN4 to decide |  | It is up to RAN4 decision Baseband processing (memory) in CA combination related as well as RF (SCS support is per band between sub6 and mmWave) |  | RAN4 |  |  |  |  |
|  | 0-10 | Extended CP |  |  |  | Extended CP |  | 0-9 (component 3; SCS60) |  |  |  | Yes |  |  |  | Type 1 | N.A. |  | N.A. |  |  |  | RAN1 | Optional with capability signaling |  | Optional with capability signaling |  |
|  | 0-11 | pi/2-BPSK for PUCCH format 3/4 |  |  |  | pi/2-BPSK for PUCCH format 3/4 |  | 0-2 |  |  |  | Yes |  | pi/2-BPSK for PUCCH format 3/4 is not possible |  | Type 3 RAN4 to decide | RAN4 to decide |  | RAN4 to decide |  | RAN4 will discuss if it is per band, common for all bands, or FR1/2 |  | RAN4 |  |  |  |  |
|  | 0-12 | Non-contiguous UL PRB CP-OFDM per CC |  |  |  | When VRB-to-PRB mapping is used, PRB is after interleaving |  | 0-1 |  |  |  | Yes |  |  |  | RAN4 to decide | RAN4 to decide |  | RAN4 to decide |  | It is up to RAN4 to decide |  | RAN4 |  |  |  |  |
|  | 0-14 | [1-symbol GP for 120KHz SCS in unpaired spectrum] |  |  |  | 1) Slot formats with 1-symbol GP(s) for 120KHz SCS in unpaired spectrum |  |  |  |  |  | Yes |  |  |  | Type 4 | Applicable only to TDD |  | Applicable only to FR2 |  | RAN4 to check whether this feature is included in their list |  | RAN4 |  |  |  |  |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures |  |  |  | 1) RACH preamble format 2) SS block based RRM measurement [3) RMSI/broadcast OSI reception] [4] Paging] |  |  |  |  |  | [Yes, or No] |  |  |  | N.A. | No need |  | No need |  | RAN2 to check components 3 and 4 for SA and NSA applicability |  | RAN1 | Mandatory without capability signaling |  | Mandatory without capability signaling |  |
|  | 1-1a | [On demand based system information] |  |  |  |  |  |  |  |  |  | [Yes, or No] |  |  |  |  | [Yes] |  | [No need] |  | RAN2 to check |  | RAN2 |  |  |  |  |
|  | 1-3 | SS block based SINR measurement (SS-SINR) |  |  |  | 1) SS-SINR measurement |  | 1-1 |  |  |  | Yes |  | Not support SS-SINR measurement |  | Type 4 | No need |  | Yes |  |  |  | RAN1 |  |  | Optional with capability signaling |  |
|  | 1-4 | SS block based RLM |  |  |  | 1) SS block based RLM |  | 1-1 |  |  |  | Yes |  | Not support SS block based RLM |  | Type 4 | No need |  | No need |  |  |  | RAN1 |  |  | Mandatory with capability signaling which shall be set to ‘1’ |  |
|  | 1-5 | CSI-RS based RRM measurement with associated SS-block |  |  |  | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement |  | 1-1, CSI-RS |  |  |  | Yes |  | Not support CSI-RSRP and CSI-RSRQ measurement |  | Type 4 | No need |  | Yes |  |  |  | RAN1 |  |  | Optional with capability signaling Note: This does not discourage RAN4 to complete their work Note: there is expectation that RAN4 will complete the corresponding RRM measurement |  |
|  | 1-5a | CSI-RS based RRM measurement without associated SS-block |  |  |  | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement 3) There is SS-block in the target frequency on which the RRM measurement is performed |  | 1-1, CSI-RS |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  | RAN1 |  |  | Optional with capability signaling Note: This does not discourage RAN4 to complete their work Note: there is expectation that RAN4 will complete the corresponding RRM measurement |  |
|  | 1-6 | CSI-RS based RS-SINR measurement |  |  |  | 1) CSI-SINR measurement |  | 1-1 1-5 |  |  |  | Yes |  | Not support CSI-SINR measurement |  | Type 4 | No need |  | Yes |  |  |  | RAN1 | Optional with capability signaling |  |  |  |
|  | 1-7 | CSI-RS based RLM |  |  |  | 1) CSI-RS based RLM |  | 1-1, CSI-RS |  |  |  | Yes |  | Not support CSI-RS based RLM |  | Type 4 | No need |  | Yes |  |  |  | RAN1 |  |  | Mandatory with capability signaling |  |
|  | 1-8 | RLM based on a mix of SS block and CSI-RS signals |  |  |  |  |  | 1-4 and 1-7 |  |  |  | Yes |  | UE does not support RLM based on a mix of SS block and CSI-RS signals |  | Type 4 | No need |  | No need |  |  |  | RAN1 | [Mandatory /optional with capability signaling] |  |  | Optional with UE capability signaling |
|  | 1-9 | CSI-RS based contention free RA for HO |  |  |  |  |  | 1-1 CSI-RS 1-5 or 1-5a |  |  |  | Yes |  | UE does not support CSI-RS based contention free RA for HO |  | Type 4 | No need |  | No need |  |  |  | RAN1 | Optional with capability signaling |  |  |  |
|  | 1-10 | Support of Scell without SS/PBCH block |  |  |  | 1) Support Scell without SS/PBCH block |  | 1-1 |  |  |  | Yes |  | Not support Scell without SS/PBCH |  | Type 3 | N.A. |  | N.A. |  | Component 1) Whether or not UE is able to use SS/PBCH block from other Cells for time/frequency synchronization of Scell without SS/PBCH block |  | RAN1 | Mandatory with capability signaling for intra-band CA This feature is not supported for inter band CA |  | Mandatory with capability signaling for intra-band CA FFS: whether this feature is supported for inter band CA |  |
|  | 1-11 | Support of CSI-RS RRM measurement for Scell without SS/PBCH block |  |  |  |  |  | 1-10 |  |  |  | Yes |  |  |  | Type 3 | N. A. |  | N.A. |  | RAN4 to check |  |  |  |  | Optional with capability signaling |  |
|  | 1-12 | E-UTRA RS-SINR measurement |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | [No need] |  | RAN2 to decide FR1/FR2 differentiation |  |  | Optional with capability signaling |  |  |  |
| 2. MIMO | 2 -1 | Basic PDSCH reception |  |  |  | 1. Data RE mapping 2. Single layer transmission |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  |  |  | RAN1 | Mandatory without capability signaling |  | Mandatory without capability signaling |  |
|  | 2-2 | PDSCH beam switching |  |  |  | 1. Time duration (definition follows section 5.1.5 in TS 38.214), Xi, to determine and apply spatial QCL information for corresponding PDSCH reception Note: candidate value will be decided after feature is completed. (note: this may not needed) Time duration is defined counting from end of last symbol of PDCCH to beginning of the first symbol of PDSCH Xi is the number of OFDM symbols, i is the index of SCS, l=1,2, corresponding to 60,120 kHz SCS. |  | 2-1 |  |  |  | Yes |  |  |  | Type-3 | No need |  | Applicable only for FR2 |  |  | RAN1 |  | Candidate value set for X1 is {7, 14, 28}, Candidate value set for X2, {14, 28} |  |  | Mandatory with UE capability signaling Support 14 symbols for both X1 and X2. Also 0 symbols should be a possible value. |
|  | 2-3 | PDSCH MIMO layers |  |  |  | 1. Supported maximal number of MIMO layers |  | 2-1 |  |  |  | Yes |  | Only one layer is supported |  | Type 3 | N.A. |  | N.A. |  |  |  | RAN1 | Candidate values: {1,2,4,8} FFS on the minimal layers for different band or band combination. |  |  | Mandatory with UE capability signaling At least 2 layers are supported. The value 1 should be removed since it is supported under 2-1, the basic capability. |
|  | 2-4 | TCI states for PDSCH |  |  |  | 1. Support number of active TCI states per CC 2. maximum number of configured TCI states, |  | 2-1 |  |  |  | Yes |  | Only one TCI state can be supported |  | Type 1 | N.A. |  | N.A. |  |  |  |  | Component-1: Candidate value set: {1, 2, 4, 8 } Component-2: candidate value set: {4, 8, 16, 32, 64} FFS: mandatory value |  |  | Mandatory with UE capability signaling Component 1: 8 active TCI states are supported. (reason: Too few active states leads to excessive MAC-CE signaling.) Component 2: A value of 64 states is supported by all UEs and is not a capability. |
|  | 2-5 | Basic downlink DMRS for scheduling type A |  |  |  | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol 3. Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port. |  | 2-1 |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  | conditioned to whether PDSCH scheduling type A is supported |  |  | Mandatory without UE capability(condition to scheduling capability) |  |  | Mandatory without UE capability |
|  | 2-6 | Basic downlink DMRS for scheduling type B |  |  |  | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  | conditioned to whether PDSCH scheduling type B is supported |  |  | Mandatory without UE capability (condition to scheduling capability) |  |  | Mandatory without UE capability signaling |
|  | 2-6 | Support 1+2 DMRS (downlink) |  |  |  | 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port |  | 2-6 |  |  |  | Yes |  | 1 FL + 2 additional DMRS for more than one port is not supported |  | Type 4 | No need |  | Yes |  |  |  |  | Mandatory with UE capability signaling |  | Mandatory with capability signaling | The reference should be both 2-5 (Type A) and not 2-6 |
|  | 2-7 | Supported 2 symbols front-loaded DMRS(downlink) |  |  |  | 1. Support 2 symbols FL-DMRS |  | 2-5 |  |  |  | Yes |  | 2 FL DMRS is not supported |  | Type 4 | No need |  | Yes |  |  |  |  |  |  |  | Mandatory without UE capability signaling (reason: 2 symbol FL avoids DMRS PAPR problem with rank 3 and 4 SU-MIMO) |
|  | 2-8 | Supported 2 symbols front-loaded +2 symbols additional DMRS(downlink) |  |  |  | 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS |  | 2-5 |  |  |  | Yes |  | 2 FL DMRS + one additional 2-symbols DMRS is not supported |  | Type 4 | No need |  | Yes |  |  |  |  |  |  |  | Mandatory without UE capability signaling (reason: 2 symbol DL avoids DMRS PAPR problem with rank 3 and 4 SU-MIMO) |
|  | 2-9 | Support 1+3 DMRS symbols(downlink) |  |  |  | 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols |  | 2-5 |  |  |  | Yes |  | 1 symbol FL DMRS and 3 additional DMRS symbols is not supported |  | Type 4 | No need |  | Yes |  |  |  |  |  |  |  | Optional with UE capability signaling. |
|  | 2-10 | Support DMRS type (downlink) |  |  |  | Support DMRS {type 1, type 2} |  |  |  |  |  | Yes |  | Only the mandatory DMRS type(s) are supported |  | Type 4 | No need |  | Yes |  |  |  |  | RAN1 will further discuss which Type will be mandatory or both type will be mandatory |  |  | Type 1 is mandatory without UE capability signaling Type 2 is mandatory with UE capability signaling. (reason for type 2 mandatory: type 2 required for all UEs to support for the high order (12 layers) MU-MIMO to be useful in reality) |
|  | 2-11 | Downlink dynamic PRB bundling (downlink) |  |  |  | 1. Support dynamic PRB bundling indication via DCI Note: Support of semi-static PRB bundling—mandatory |  | 2-1 |  |  |  | Yes |  | No support of dynamic PRB bundling |  | Type 4 | No need |  | No need |  |  |  |  | Optional |  | Optional with capability signaling | Optional with UE capability signalling |
|  | 2-12 | Basic PUSCH transmission |  |  |  | Data RE mapping Single layer (single Tx) transmission |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  |  |  |  | Mandatory without UE capability |  | Mandatory without UE capability |  |
|  | 2-13 | PUSCH codebook coherency subset |  |  |  | Proposal: 1. Supported codebook coherency subset type: Candidate value set: {non-coherent, partial/non-coherent, full/partial/non-coherent} |  | 2-12 |  |  |  | Yes |  | Only non-coherent codebook subset is supported |  | Type 1 | N.A. |  | N.A. |  |  |  |  | Candidate values: {non-coherent, partial-non-coherent, full-coherent} |  |  | Feature is optional with UE capability signaling. Candidate values: Non-coherent should be mandatory for UEs capable of >1 layer PUSCH |
|  | 2-14 | Codebook based PUSCH MIMO transmission |  |  |  | 1. Supported codebook based PUSCH MIMO with maximal number of supported layers |  | 2-13 |  |  |  | Yes |  | Uplink codebook based MIMO (with >1 Tx port) transmission is not supported |  | Type 3 | N.A. |  | N.A. |  |  |  |  | Candidate value: {no-codebook based MIMO, 1, 2, 4} |  |  | Feature is optional with UE capability signaling. Candidate values: >1 layer should be optional |
|  | 2-15 | non-codebook based PUSCH transmission |  |  |  | 1. Maximal number of supported layers (non-codebook transmission scheme): |  | 2-12 |  |  |  | Yes |  | No support of non-codebook based PUSCH transmission |  | Type 3 | N.A. |  | N.A. |  |  |  |  | Component-1 candidate values: {“No non-codebook based MIMO”, 1, 2, 4} |  |  | Feature is optional with UE capability signaling. Candidate values: at least 1 layer is supported. |
|  | 2-15a | Association between CSI-RS and SRS |  |  |  | Support association between NZP-CSI-RS and SRS resource set via RRC parameter “SRS-AssocCSIRS” |  | 2-15 |  |  |  | Yes |  | Association between CSI-RS and SRS is not supported |  | Type 3 | N.A. |  | N.A. |  |  |  |  | Optional |  | Optional with capability signaling | Optional with UE capability signaling Clarify description to … via RRC parameter “SRS-AssocCSIRS” and/or AperiodicSRS-ResourceTrigger”’ to align with 38.214 6.1.1.2 |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A |  |  |  | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbols 3. Support 1 symbol FL DMRS and 2 additional DMRS symbols |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  | conditioned to whether PUSCH scheduling type A is supported |  |  | Mandatory without UE capability |  | Mandatory without capability signaling |  |
|  | 2-16a | Basic uplink DMRS for scheduling type B |  |  |  | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  | conditioned to whether PUSCH scheduling type B is supported |  |  | Mandatory without UE capability |  | Mandatory without capability signaling |  |
|  | 2-16b | Support 1+2 DMRS (uplink) |  |  |  | 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port |  | 2-6 |  |  |  | Yes |  | 1+2 DMRS for more than one port is not supported |  | Type 4 | No need |  | Yes |  |  |  |  | Mandatory with UE capability signaling |  | Mandatory with capability signaling | Reference should be 2-16 and 2-16a. Applies to Type A and B |
|  | 2-17 | Support DMRS type (uplink) |  |  |  | Support DMRS {type 1, type 2 } |  | 2-16 |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  |  | At least type-1 is mandatory. FFS on type-2 is mandatory or optional |  |  | Type 1 is mandatory without UE capability signaling Type 2 is mandatory with UE capability signaling. (reason: type 2 required for all UEs to support for the high order (12 layers) MU-MIMO to be useful in reality) |
|  | 2-18 | Supported 2 symbols front-loaded DMRS (uplink) |  |  |  | 1. Support 2 symbols FL-DMRS |  | 2-16 |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  |  | FFS on whether it’s mandatory or optional |  |  | Mandatory without UE capability signaling (reason: 2 symbol DL avoids DMRS PAPR problem with rank 3 and 4 SU-MIMO) |
|  | 2-18a | Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink) |  |  |  | 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS |  | 2-16 |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  |  | FFS on whether it’s mandatory or optional |  |  | Mandatory without UE capability signaling (reason: 2 symbol DL avoids DMRS PAPR problem with rank 3 and 4 SU-MIMO). This is both for Type A and Type B when supported. (reference 2-16a as well) |
|  | 2-19 | Support 1+3 uplink DMRS symbols(uplink) |  |  |  | 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols |  | 2-16 |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  |  | FFS on whether it’s mandatory or optional |  |  | Optional with UE capability signaling. This is both for Type A and Type B when supported. (reference 2-16a as well) |
|  | 2-20 | Beam correspondence |  |  |  | 1. Support Beam correspondence |  |  |  |  |  | Yes |  | Beam correspondence is not supported |  | Type 1 | No need |  | N.A. |  | Note: Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports |  |  | [Mandatory at least for FR2] |  |  | Mandatory without UE capability signaling |
|  | 2-21 | Periodic beam report |  |  |  | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot |  |  |  |  |  | Yes |  | No support of periodic L1-RSRP report |  | Type 1 | No need |  | N.A. |  |  |  |  | Mandatory with UE capability at least for FR2 FFS: for FR1 |  |  | Mandatory with UE capability at least for FR2 |
|  | 2-22 | Aperiodic beam report |  |  |  | 1. Support report on PUSCH |  |  |  |  |  | Yes |  | No support of aperiodic L1-RSRP report |  | Type 1 | No need |  | Yes |  |  |  |  | Mandatory with UE capability at least for FR2 |  |  |  |
|  | 2-23 | Semi-persistent beam report on PUCCH |  |  |  | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot(or piggybacked on a PUSCH) 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot(or piggybacked on a PUSCH) |  |  |  |  |  | Yes |  | No support of PUCCH based SP L1-RSRP report |  | Type 1 | N.A. |  | Yes |  |  |  |  | Optional |  | Optional with capability signaling | Agree with RAN WG recommendation |
|  | 2-23a | Semi-persistent beam report on PUSCH |  |  |  | 1. Support report on PUSCH |  |  |  |  |  | Yes |  | No support of PUSCH based SP L1-RSRP report |  | Type 1 | N.A. |  | Yes |  |  |  |  | Optional |  | Optional with capability signaling | Agree with RAN WG recommendation |
|  | 2-24 | SSB/CSI-RS for beam measurement |  |  |  | 1. The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_1 2. The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 3. Supported density of CSI-RS |  | 2-21, 2-22 or 2-23, 2-23a |  |  |  | Yes |  | RSRP measurement is not supported |  | Type 1 | No need |  | Yes |  |  |  |  | Component-1, candidate value set for MB_1 is {8, 16, 32, 64} Support MB_1 =8 is mandatory for at least for >6Ghz bands Component-2, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-3: candidate value set: {“1 only”, “3 only”, “both 1 and 3”} At least density of CSI-RS =3 is mandatory at least for FR2 |  |  | Mandatory with UE capability signaling. Component-1: Smallest value is 16 Component-2: Smallest value is 16 Component-3: Both 1 and 3 are mandatory. |
|  | 2-25 | Beam reporting timing |  |  |  | 1. The number of symbols, Xi, between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report is at least RBi, where i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. |  | 2-24 |  |  |  | Yes |  | Beam reporting time capability is not known by gNB |  | Type 1 | N.A. |  | N.A. |  |  | RAN1/4 |  | Candidate value set for X1 is {2, 4, [8]} Candidate value set for X2 is {4,8, [14]} Candidate value set for X3 is {7 or 8,14 or 15, 28} RAN1 will further decide between 7 and 8 and between 14 and 15. Candidate value set for X4, {14, 28, [42]} |  |  | Mandatory with UE capability signaling for FR2. X1: Support 2 X2: Support 4 X3: Support 7 X4: Support 14 |
|  | 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" |  |  |  | 1. Support Rx beam switching procedure using CSI-RS resource repetition “"ON" 2. Recommended CSI-RS resource repetition number per resource set, |  |  |  |  |  | Yes |  | Rx beam switching is not supported |  | Type 1 | N.A. |  | N.A. |  |  |  |  | Support Rx beam switching is mandatory for bands at least > 6GHz Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} |  |  | Optional with UE capability signaling |
|  | 2-27 | Beam switching |  |  |  | 1. Maximum number of Tx + Rx beam changes a UE can conduct during a slot across the whole band CC BB_Total,. This number is defined as per SCS Note: it is assumed that spec enable the possibility to restrict the same beam across intra-band CCs |  | 2-24 |  |  |  | Yes. |  | No resetriction on the maximum number of Tx+Rx beam change for a slot |  | Type 1 | N.A. |  | N.A |  |  |  |  | Candidate value set: {4, 7, 14} |  |  | Mandatory with UE capability signaling for FR2 Prefer large as possible: 7, 14 |
|  | 2-28 | A-CSI-RS beam switching timing |  |  |  | 1. Minimum time between the DCI triggering of AP-CSI-RS and aperiodic CSI-RS transmission shall be at least KBi symbols. (Symbols measured from last symbol containing the indication to first symbol of CSI-RS), where i is the index of SCS, l=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. |  | 2-27 |  |  |  | Yes |  | No recommendation on the desired beam switching timing |  | Type1 | N.A. |  | N.A. |  | [Note: any value larger than 56 is not supported in RRC configuration now. ] |  |  | Only applicable to FR2 Candidate values: Alt.1: {14, 26, 28, [42], [336]} Alt.2: {“14-14”, “26-26”, “28-28”, “26-280”, “28-336” } the first threshold corresponding to the case when UE doesn’t activate antenna panel, and the second threshold corresponding to the case when UE actives antenna panel |  |  | Mandatory with UE capability signaling at least for FR2 Candidate values should be aligned with PDSCH beam switching time (see 2-2 above). Prefer 14. 0 should be one option; add switching time of 0. |
|  | 2-29 | Non-group based beam reporting |  |  |  | 1. Support of non-group based RSRP reporting with N_max RSRP values reported |  | 2-24 |  |  |  | Yes |  | Non-group based beam reporting is not supported |  | Type1 | N.A |  | N.A. |  |  |  |  | candidate value set is {1, 2,4} |  |  | Optional with UE capability signaling. Clarify that 1 is always mandatory. |
|  | 2-29a | Group based beam reporting |  |  |  | 1. Support of beam group RSRP reporting for group of 2 beams |  |  |  |  |  | Yes |  | Group based beam reporting is not supported |  | Type1 | N.A. |  | N.A. |  |  |  |  | Optional |  | Optional with capability signaling | Optional with UE capability signaling. |
|  | 2-30 | Uplink beam management |  |  |  | 1 Support of SRS based beam management 2. Supported max number of SRS resource per set (SRS set use is configured as for beam management). 3. Supported max number of SRS resource sets (SRS set use is configured as for beam management). |  |  |  |  |  | Yes |  | Uplink beam management is not supported |  | Type1 | N.A. |  | N.A. |  |  |  |  | Component-2, candidate value set is {8, 16, 32} Component-3, candidate value set is {from 1 to 8} |  |  | Optional with UE capability signaling. Component 3: Don’t need such high granularity. Number of sets corresponds to number of antenna panels. |
|  | 2-31 | Beam failure recovery |  |  |  | 1. Maximal number of CSI-RS resources across all CCs for UE to monitor PDCCH quality 2. Maximal number of different SSBs across all CCs for UE to monitor PDCCH quality 3. Maximal number of different CSI-RS and/or SSB resources across all CCs for new beam identifications. 4. COREST recoveryControlResourceSetId if configured |  |  |  |  |  | Yes |  | Beam failure recovery is not supported |  | Type 1 | N.A. |  | N.A. |  | Component 4) is in addition to component 1) of 3-1 |  |  | Component-1 candidate value set: {from 1 to 64} Component-2 candidate: {from 1 to 64} Component-3: Candidate value set is: {from 1 to 256} [UE is mandate to support at least 64.] |  |  | Optional with UE capability signaling. Granularity for Component 1, 2, and 3 should be reduced |
|  | 2-32 | Basic CSI feedback |  |  |  | 1. Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory) 2. 2Tx codebook for FR1 and FR2 3. 4Tx codebook for FR1 4. 8Tx codebook for FR1 when configured as wideband CSI report 5. p-CSI on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) 6. p-CSI report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) 7. a-CSI on PUSCH (at least Z value >= 14 symbols, detail processing time to be discussed separately) further check a-CSI on p-CSI-RS and/or SP-CSI-RS from component-7 |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  |  |  |  | Mandatory without UE capability |  | Mandatory without capability signaling |  |
|  | 2-32a | Semi-persistent CSI report on PUCCH |  |  |  | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot(or piggybacked on a PUSCH) s 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) |  |  |  |  |  | Yes |  | SP-CSI on PUCCH is not supported |  | Type 4 | No |  | No |  |  |  | RAN1 | Optional |  | Optional with capability signaling |  |
|  | 2-32b | Semi-persistent CSI report on PUSCH |  |  |  | 1. Support report on PUSCH |  |  |  |  |  | Yes |  | SP-CSI on PUSCH is not supported |  | Type 4 | No |  | No |  |  |  | RAN1 | Optional |  | Optional with capability signaling |  |
|  | 2-33 | CSI-RS and CSI-IM reception for CSI feedback |  |  |  | 1. Supported max # of configured NZP-CSI-RS resources per CC, 2. Supported max # of ports across all configured NZP-CSI-RS resources per CC 3. Supported max # of configured CSI-IM resources per CC |  | 2-32 |  |  |  | Yes |  | CSI acquisition is not supported |  | Type 3 | N.A. |  | N.A. |  | Note: all the candidate values are the range of capability signaling which doesn’t determine whether UE is mandatory to support all the signaling values. |  |  | Component-1 candidate values: {from 1 to 32} Component-2 candidate values: , further down-select between: Alt.1: {from 4 to 64}, Alt.2: {from 4 to 256}, Alt.3: {from 4 to 256} Component-3: candidate values: {1,2,4,8,16,32} |  |  | Component-1: Smallest value is 32. Component-2: Smallest value is 64. Also propose granularity of 16, ie. {64, 80, 96, …, 256} Component-3: Smallest value is 8. |
|  | 2-33a | PDSCH RE-mapping Note: this FG will be moved to 5-x family |  |  |  | 1. Supported max # of RE mapping patterns, each pattern can be described as a RS resource (including NZP/ZP CSI-RS and CRS) or a bitmap configured in 5-26/27 per OFDM symbol and per CC |  |  |  |  |  | Yes |  | PDSCH RE mapping is not supported |  | Type 4 | No need |  | Yes |  |  |  |  | candidate values: {10, 20} for FR1 {6, 20} for FR2 |  |  |  |
|  | 2-34 | NZP-CSI-RS based interference measurement |  |  |  | 1. Support NZP-CSI-RS based interference measurement FFS whether to add the number of NZP-CSI-RS port for interference measurement |  | 2-33 |  |  |  | Yes |  | NZP-CSI-RS based interference measurement is not supported |  | Type 4 | No need |  | No need |  |  |  |  | [Optional] |  |  | Optional with UE capability signaling |
|  | 2-35 | CSI report framework |  |  |  | 1. Maximum number of periodic CSI report setting per BWP 2. Maximum number of aperiodic CSI report setting per BWP 3. Maximum number of semi-persistent CSI report setting per BWP 4. Support of advanced CSI process table (the Minimum duration Z, Z’ for processing a CSI) [Conditioned to the completion of advanced CSI process] 5. UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. 6. Supported CSI computation type FFS: whether X should also count the SRS precoder derivation in case of reciprocity based SRS Tx |  | 2-32 |  |  |  | Yes |  | CSI report is not supported |  | Type 3 | N.A. |  | N.A. |  | NOTE: Other MIMO capability other than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update |  |  | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4: signaling is a bitmap of size 8 Component-5: candidate values: {from 5 to 32} Componet-6: Candidate values: {Type-A, Type-B} |  |  | Mandatory with capability signaling Component-1: Smallest value is 4 Component-2: Smallest value is 4 Component-3: Smallest value is 4 Thus, component 1-3 can be removed. (reason: The number of CSI report settings does not drive complexity significantly, while the number of CSI reports does.) Component 4: Can be optional Component-5: Smallest value is 5 Component 6: Support one of the two mandatory |
|  | 2-36 | Type I single panel codebook |  |  |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Supported Codebook Mode(s) |  | 2-35 |  |  |  | Yes |  | No additional Type I codebook configurations other than the basic CSI feedback (2-32) is supported |  | Type 3 | No need |  |  |  | Note: simultaneously doesn’t mean in the same slot |  |  | Component-1: the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS basednterferencee measurement) is: {from 2 to 256} Component-2 candidate values: {Mode-1, Mode-2} Down-select: Alt.1 Mode-1 as mandatory Alt.2: Both Mode-1 and Mode-2 are mandatory (in this case, this component is not needed) |  |  | Optional with UE capability signaling. Component-1: Smallest value of max # of Tx port in one resource is 32. Smallest value of max # of resources is 32. Smallest value of total # of ports is 128. Component-2: Mode-2 is optional |
|  | 2-37 | Support Semi-open loop CSI |  |  |  | 1. Support Semi-open loop CSI report |  | 2-35 |  |  |  | Yes |  | Semi-Open loop CSI report is not supported |  | Type 4 | No need |  | Yes |  |  |  |  | FFS |  |  | Optional with UE capability signaling |
|  | 2-38 | CSI report without PMI |  |  |  | Support CSI report without PMI |  | 2-35 |  |  |  | Yes |  | CSI report without PMI is not supported |  | Type 4 | No need |  | Yes |  |  |  |  | FFS |  |  | Mandatory with UE capability signaling. Should be required for CSI-RS resources of up to 8 ports. (reason: Provides low complexity CSI in TDD systems.) |
|  | 2-39 | CSI report with CRI |  |  |  | Support CSI report with CRI |  | 2-35 |  |  |  | Yes |  | CSI report with CRI is not supported |  | Type 4 | No need |  | Yes |  |  |  |  | FFS |  |  | Mandatory with UE capability signaling. |
|  | 2-39a | CSI report without CQI |  |  |  | Support CSI report without CQI |  | 2-35 |  |  |  | Yes |  | CSI report wihout CQI is not supported |  | Type 4 | No need |  | Yes |  |  |  |  | FFS |  |  | Optional with UE capability signaling. |
|  | 2-40 | Type I multi-panel codebook |  |  |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Supported Codebook Mode(s): 3. Supported number of panels, Ng, |  | 2-35 |  |  |  | Yes |  | multi-panel Type I codebook is not supported |  | Type 3 | N.A. |  | N.A. |  | Note: simultaneously doesn’t mean in the same slot |  |  | Optional Component-1: the candidate values for the max # of Tx port in one resource is {8, 16, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: Component-2 candidate values: {Mode-1, Mode-2, both} Component-3: Candidate value: {2,4} |  |  | Optional with UE capability signaling. Component 2: Mode-2 can be an optional value. |
|  | 2-41 | Type II codebook |  |  |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Parameter “Lx” (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3. Support amplitude scaling type 4. Support amplitude subset restriction level |  | 2-35 |  |  |  | Yes |  | Type II codebook is not supported |  | Type 3 | N.A. |  | N.A. |  | Note: simultaneously doesn’t mean in the same slot |  |  | Component-1: the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4, candidate value set: {no restriction, subset restriction}, “no restriction” is mandatory and RAN1 hasn’t decide whether “subset restriction” is mandatory or not, if it’s mandatory then this component is not needed. |  |  | Mandatory with UE capability signaling, at least for L=2 beams (Component-2) and Wideband amplitude scaling (Component-3) Component 1-1: Such fine granularity is not needed, it should be enough with {16, 32} Component 1-2: Again, the granularity is too fine. Type II CSI does not support CRI reporting, so max 1 CSI-RS resource can be configured per report setting. This component relates to how many CCs a UE can report Type IICSI for simultaneously. So {1,2,4,8,16} should be enough Component 1-3: Granularity is to fine. {16:16:256} should be enough Component 2: L=2 mandatory Component 3: ‘wideband’ mandatory Component 4; “subset restriction” optional |
|  | 2-42 | Support Type II SP-CSI feedback on long PUCCH |  |  |  | 1.Support type II SP-CSI feedback part-1 on PUCCH formats over 4 – 14 OFDM symbols once per slot |  | 2-41 |  |  |  | Yes |  | Type II SP-CSI feedback on long PUCCH is not supported |  | Type 4 | No need |  | No need |  |  |  |  | [Optional] |  |  | Optional with UE capability signaling |
|  | 2-43 | Type II codebook with port selection |  |  |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Parameter “Lx” (number of selected ports) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3. Support amplitude scaling type |  |  |  |  |  | Yes |  | Type II codebook with port selection is not supported |  | Type 3 | N.A. |  | N.A. |  | Note: simultaneously doesn’t mean in the same slot |  |  | Component-1: the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values set for “Lx” is {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} |  |  | Optional with UE capability signaling |
|  | 2-44 | Basic DL PTRS |  |  |  | Support 1 port of PTRS |  |  |  |  |  | Yes |  | DL PTRS is not supported |  | Type 4 | N.A. |  | Yes |  |  |  |  | Mandatory with UE capability signaling for FR2 Optional for FR1 |  |  | Mandatory with UE capability signaling for FR2 Optional with UE capability signaling for FR1 |
|  | 2-45 | Downlink 2Tx PTRS |  |  |  | 1. Supported 2 ports of PTRS |  | 2-44 |  |  |  | Yes |  | 2 ports of PTRS is not supported |  | Type 1 | N.A. |  | N.A |  |  |  |  | Optional |  | Optional with capability signaling |  |
|  | 2-46 | Downlink PTRS density recommendation |  |  |  | 1. Preferred threshold sets, TSi for determine PTRS density, candidate value range is the same as that of downlink PTRS RRC configuration. i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. |  | 2-44 |  |  |  | Yes |  | Threshold recommendation is not supported |  | Type 1 | N.A. |  | N.A. |  |  |  |  | Optional For each TSi, it composes of two values each selected from {1..276} for frequency density, and three values each selected from {0..29} for time density |  |  | Optional with UE capability signaling |
|  | 2-47 | Basic UL PTRS |  |  |  | Support 1 port of PTRS |  |  |  |  |  | Yes |  | Uplink 1 port PTRS is not supported |  | Type 4 | N.A. |  | Yes |  |  |  |  | Mandatory with UE capability signaling for FR2 [Optional for FR1] |  |  | Mandatory with UE capability signaling for FR1 and FR2 |
|  | 2-48 | Uplink PTRS |  |  |  | 1. Supported 2 ports of PTRS |  | 2-47 |  |  |  | Yes |  | Uplink 2 ports PTRS is not supported |  | Type 1 | N.A. |  | N.A. |  |  |  |  | Optional |  |  | Optional with capability signalling |
|  | 2-49 | Uplink PTRS density recommendation |  |  |  | 1. Preferred threshold sets, TSi , for determine PTRS density, candidate value range is the same as that of uplink PTRS RRC configuration. i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. |  | 2-47 |  |  |  | Yes |  | Threshold recommendation is not supported |  | Type 1 | N.A. |  | N.A. |  |  |  |  | Optional For each TSi, it composes of two values each selected from {1..276} for frequency density, and three values each selected from {0..29} for time density, and five values each selected from {1..276} for sample density |  |  | Optional with capability signalling |
|  | 2-50 | Basic TRS |  |  |  | 1. Support of TRS (mandatory) 2. All the periodicity are supported. FFS: TRS bandwidth configuration = [Atl.1 BWP; Alt.2: min(52,BWP) Alt.3: Both] |  |  |  |  |  |  |  |  |  | N.A. | N.A. |  | N.A. |  |  |  |  | Mandatory without UE capability signaling |  | Mandatory without capability signaling | Alt.3: All bandwiths must be supports by all UEs, otherwise a TRS cannot be shared by all UEs in the cell and the NW must configure two TRS (and ZP CSI-RS for protection) |
|  | 2-51 | TRS (CSI-RS for tracking) |  |  |  | 1. Support TRS BW 2. TRS burst length (X), 3. Max # of TRS resource sets (per CC) UE is able to track simultaneously 4. Max # of TRS resource sets configured to UE per CC 5. Max # of TRS resource sets configured to UE across CCs |  | 2-50 |  |  |  | Yes |  | TRS is not supported |  | Type 1 | N.A. |  | N.A. |  |  |  |  | Component-1: candidate values set: {BWP, min(52,BWP), both} Component-2: candidate values {1,2} Component-3: Candidate value set: {1 to 8} Component-4: Candidate value set: {1 to 64} Component-5: Candidate value set: {1 to 128} |  |  | Remove component 1, since all BW must be supported by the UE anyway, see comment in 2-50. Reduction of the number of values needed for component 4 and 5. Support for the full contiguous range {1..64} leads to device fragmentation and very difficult to operate system… Component t 3, 4 1n 5 refer to the max number of periodic TRS resource sets. |
|  | 2-51b | Aperiodic TRS |  |  |  | - Support of aperiodic TRS |  | 2-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling. New field due to agreement in RAN1#92bis For each configured periodic TRS resource set the UE can be configured with one aperiodic TRS resource set. For each periodic TRS resource set (per CC) UE is able to track simultaneously, UE is able to track one aperiodic TRS resource set. Simutaneously. |
|  | 2-52 | Basic SRS |  |  |  | 1. Support 1 port SRS transmission 2. Support periodic/aperiodic SRS transmission 3. Support SRS Frequency intra/inter-slot hopping within BWP 4. At least one SRS resource per CC for aperiodic and periodic separately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Mandatory without UE capability |  | Mandatory without capability signaling |  |
|  | 2-53 | SRS resources |  |  |  | 1. Maximum number of aperiodic SRS resources (configured to UE) per BWP 2. Maximum number of aperiodic SRS resources (configured to UE) per BWP per slot 3. Maximum number of periodic SRS resources (configured to UE) per BWP 4. Maximum number of periodic SRS resources (configured to UE) per BWP per slot 5. Maximum number of semi-persistent SRS resources (configured to UE) per BWP 6. Maximum number of semi-persistent SRS resources (configured to UE) per BWP per slot 7. Maximum number of SRS port per resource |  | 2-52 |  |  |  | Yes |  | No more than one periodic and one aperiodic SRS resources per CC are supported and no SP-SRS is supported |  | Type 3 | N.A. |  | N.A. |  |  |  |  | Component-1: candidate value: {from 1 , 2, 4, 8, 16} Component-2 candidate value: {1,2,3,4,5,6} Component-3: candidate value: {from 1 , 2, 4, 8, 16} Component-4 candidate value: {1,2,3,4,5, 6} Component-5: candidate value: {from 0, 1 , 2, 4, 8, 16} } Component-6 candidate value: {0,1, 2,3,4,5, 6} Component-7 candidate values: {1, 2, 4} Support SP-SRS is mandatory |  |  | Mandatory with UE capability signaling Components 1-4: Smallest value is 2 Component 5: Smallest value is 1 Component 6: Smallest value is 1 Note: min values of 1 for components 5,6 given are required since SP-SRS is mandatory. Component 7: smallest value is 1. |
|  | 2-54 | SRS transmission |  |  |  | 1. Minimum time interval, N in unit of symbols, between DCI triggering and A-SRS transmission, |  | 2-53 |  |  |  | Yes |  |  |  | Type 1 | No need |  | N.A. |  | Note: there is a minimal timing (42 symbols) between A-CSI-RS reception and updating of A-SRS precoding |  |  | candidate value range is the same as that of N2 plus 42. |  |  | Mandatory with UE capability signaling (since aperiodic SRS is part of basic SRS capability) |
|  | 2-54a | Simultaneous SRS Tx |  |  |  | 1. Maximum number of simultaneous transmitted SRS resources per CC at one symbol, FFS whether to break this FG into different SRS purposes and potentially adding total number of Tx ports |  |  |  |  |  | Yes |  | Only one SRS resource can be transmitted at a given time |  | Type 1 | No need |  | N.A. |  |  |  |  | Candidate Value Set: {1, 2, 3, 4} |  |  | Optional with UE capability signaling |
|  | 2-55 | SRS Tx switch |  |  |  | 1. Support SRS Tx port switch, 2. Report whether the uplink TX switching impact to downlink receiving in a band, |  | 2-53 |  |  |  | Yes |  | SRS Tx Switch is not supported |  | Type 3 | N.A. |  | N.A. |  | Component-2 is agreed with conditioned to RAN4’s decision.— |  | RAN1/4 | Component-1 is a list of TRx pairs, candidates are {1T2R, 1T4R, 2T4R, 1T4R/2T4R} Component-2: Candidate value set:, {yes, no}, |  |  | Optional with UE capability signaling But need to clarify that if Tx antenna switching is not supported, then UE is capable of sounding all antennas simultaneously (T = R). Alternatively, add a UE capability for T = R. |
|  | 2-56 | SRS carrier switch |  |  |  | 1. Report inter-cell switching time capability |  | 2-53 |  |  |  | Yes |  | SRS carrier switch is not supported |  | Type 1 | No need |  | N.A. |  |  |  | RAN1/4 | candidate values set is up to RAN4 |  |  | Optional with UE capability signaling |
|  | 2-57 | Support low latency CSI feedback |  |  |  | Support low latency CSI feedback |  |  |  |  |  | Yes |  | Low latency CSI is not supported |  | Type 3 | N.A. |  | N.A. |  |  |  | RAN1 | [Optional] |  |  | Mandatory with UE capability signaling Clarify that this feature implies supporting the low latency (Z,Z’) values regardless of advanced/normal UE capability |
|  | 2-58 | Support CQI report for BLER=1e-5 |  |  |  | Support the CQI report using the CQI table defined for BLER target of 1e-5 |  |  |  |  |  | Yes |  | CQI table for BLER=1e-5 is not supported |  | Type 4 | N.A. |  | N.A. |  |  |  | RAN1 |  |  |  | Optional with capability signaling New feature added |
| 3.DL control channel and procedure | 3-1 |  | Basic DL control channel |  |  |  | 1) One UE-specific configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - Single TCI state for a CORESET configuration 2) CSS and USS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - For type 1 with dedicated RRC configuration, type 3, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 without dedicated RRC configuration and for type 0, 0A, and 2, the monitoring occasion can be any OFDM symbol(s) of a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DLand one unicast DCI scheduling UL per slot per scheduled CC 6) Processing one of RA-RNTI or SI-RNTI or P-RNTI or C-RNTI in a slot per scheduled CC |  |  |  |  | Yes |  |  |  | N.A. |  | N.A. | N.A |  |  |  |  | Mandatory without capability signaling |  | Mandatory without capability signaling | Discussion point 6) in agenda 7.1.3.3.2. Discuss whether or not there is problem supporting TDD frequencies if only a single UL DCI is able to be sent per slot. We are worried that this will limit scheduling possibility significantly when operating NR on TDD bands. Supporting feature. Supporting feature 3-5a comes with the mandate to configure PDCCHs at different occasions in the slot which is not necessary for this scenario. |
|  | 3-1’ | For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  |  |  |  | Mandatory with capability signaling |  | Mandatory with capability signaling |  |
|  | 3-2 | PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot |  |  |  | - Number of PDCCH blind decodes per slot with a given SCS follows Case 1-2 table for 15kHz SCS - For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Applicable only to FR1 |  |  |  |  | [Optional with capability signaling] |  |  | Mandatory with capability signaling |
|  | 3-3 | More than one CORESET configurations per BWP in addition to CORESET0 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  | Whether/How to differentiate FR1 and FR2 is up to RAN2 |  |  | Optional with capability signaling for FR1 Mandatory with capability signaling for FR2 |  |  |  |
|  | 3-4 | More than one TCI state configurations per CORESET |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 1 | N.A. |  | N.A. |  |  |  |  | Optional with capability signaling |  |  |  |
|  | 3-5 | For type 1 with dedicated RRC configuration, type 3, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  |  |  |  | Optional with capability signaling |  |  |  |
|  | 3-5a | For type 1 with dedicated RRC configuration, type 3, and UE-SS,, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a DCI gap |  |  |  | For type 1 with dedicated RRC configuration, type 3 and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2, with minimum time separation between two unicast DCIs for a same UE as - 2OFDM symbols for 15kHz - 4OFDM symbols for 30kHz - 7OFDM symbols for 60kHz with NCP 14OFDM symbols for 120kHz |  |  |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  |  |  |  | Optional with capability signaling |  |  |  |
|  | 3-6 | Dynamic SFI monitoring and dynamic UL/DL determination |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  | [Optional with capability signaling] |  |  | Optional with capability signalling |
|  | 3-7 | Precoder-granularity of CORESET size |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  | Optional with capability signaling |  |  |  |
| 4.UL control channel and procedure | 4-1 | Basic UL control channel |  |  |  | 1) PUCCH format 0 over 1 OFDM symbols once per slot 2) PUCCH format 0 over 2 OFDM symbols once per slot with frequency hopping as “enabled” 3) PUCCH format 1 over 4 – 14 OFDM symbols once per slot with intra-slot frequency hopping as “enabled” 5) One SR configuration per PUCCH group 6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI 7) Multiplexing of SR and HARQ-ACK on a PUCCH 8) HARQ-ACK piggyback on PUSCH 9) Semi-static beta-offset configuration for HARQ-ACK |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  | RAN4 to check feasibility of frequency hopping for PUCCH formats for FR2 |  |  | Mandatory without capability signaling |  | Mandatory without capability signaling |  |
|  | 4-2 | 2 PUCCH of format 0 or 2 in consecutive symbols |  |  |  | 1) 2 PUCCH format 0/2 in different symbols and once per slot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per slot for SR 3) 2 PUCCH format 2 in different symbols and once per slot for CSI over two consecutive OFDM symbols |  |  |  |  |  | Yes |  |  |  | Type 4 | Yes |  | Yes |  |  |  | RAN1 |  |  |  | Do they strictly need to be in consecutive symbols? For example, if SR/CSI is reported separately from HARQ-ACK they do not need to be in consecutive symbols. Consider breaking the capability into multiple capabilities as all combinations may not be used. |
|  | 4-3 | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as “enabled” |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  | Note: RAN1 to check consistency with 2-32 |  | RAN1 | Mandatory with capability signaling which shall be set to ‘1’ |  |  |  |
|  | 4-4 | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  | Note: RAN1 to check consistency with 2-32 |  | RAN1 | Mandatory with capability signaling which shall be set to ‘1’ |  |  |  |
|  | 4-5 | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  | Note: RAN1 to check consistency with 2-32 |  | RAN1 | Mandatory with capability signaling |  |  |  |
|  | 4-6 | Non-frequency hopping for PUCCH formats 0 and 2 with frequency hopping as “disabled” |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  | RAN1 | [Optional with capability signaling] |  |  | Optional with capability signalling |
|  | 4-7 | Non-frequency hopping for PUCCH format 1, 3, and 4 with frequency hopping as “disabled” |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  | RAN1 | [Optional with capability signaling] |  |  | Optional with capability signalling |
|  | 4-10 | Dynamic HARQ-ACK codebook |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  | RAN1 understanding is that at least one of 4-10 and 4-11 is set to ‘1’ |  | RAN1 | Mandatory with capability signaling |  | Mandatory with capability signaling |  |
|  | 4-11 | Semi-static HARQ-ACK codebook |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  | RAN1 | Mandatory with capability signaling |  | Mandatory with capability signaling |  |
|  | 4-12 | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  | Applicable to UE supporting more than 4 layers |  | RAN1 | Mandatory with capability signaling |  |  |  |
|  | 4-13 | More than one SR configurations per PUCCH group |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  | RAN2 to check |  | RAN1 and RAN2 | Optional with capability signaling |  |  |  |
|  | 4-19 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or piggybacked on a PUSCH) |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  | RAN1 | [Optional with capability signaling] |  |  | Optional with capability signaling. Given it was some confusion at last RAN1 meeting what the bullet means. Our understanding based on the text is that this relates to SR/HARQ-ACK/periodic CSI being multiplexed on PUCCH. If that is not the common understanding, we would need to reassess our view. The text would need to be clarified and this specific feature described here would need to be captured clearly. |
|  | 4-20 | UCI code-block segmentation |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  | RAN1 | Mandatory with capability signaling |  | Mandatory with capability signaling |  |
|  | 4-21 | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  | RAN1 |  |  |  | Optional with capability signaling |
|  | 4-22 | 1 long PUCCH format and 1 short PUCCH format in the same slot |  |  |  | 1) 1 long PUCCH format and 1 short PUCCH format in the same slot |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  | RAN1 |  |  |  | Optional with capability signaling |
|  | 4-22a | 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 4-23 | Repetitions for PUCCH format 1, 3,and 4 over multiple slots with K =, 2, 4, 8 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  | RAN1 | [Mandatory with capability signaling] |  |  | Optional with capability signaling |
|  | 4-24 | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  | RAN1 |  |  |  | Mandatory with capability signaling |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation |  |  | 1) Frequency-domain resource allocation - RA Type 0 only and Type 1 only for PDSCH without interleaving - RA Type 1 for PUSCH without interleaving 2) Time-domain resource allocation - 1-14 OFDM symbols for PUSCH once per slot - Starting symbol, and duration are determined by using the DCI - PDSCH mapping type A with 7-14 OFDM symbols - PUSCH mapping type A and type B - For type 1 without dedicated RRC configuration and for type 0, 0A, and 2, PDSCH mapping type A with {4-14} OFDM symbols and type B with {2, 4, 7} OFDM symbols 3) TBS determination 4) Nominal UE processing time for N1 and N2 (Capability #1) 5) HARQ process operation with configurable number of DL HARQ processes of up to 16 6) Cell specific RRC configured UL/DL assignment 7) Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment 8) Intra-slot frequency-hopping for PUSCH scheduled by Type 1 before RRC connection |  |  |  |  |  | Yes |  |  |  |  | N.A. | N.A. |  | N.A. |  | Note: If UE is configured with more than 8 HARQ processes, RAN4 continue to discuss the impact of 16 HARQ processes |  | RAN1 | Mandatory without capability signaling | Mandatory without capability signaling |  | Clarity is needed in terms of what of behavior the UE has when no UL/DL configuration pattern is configured and behavior when flexible resources are configured. In our understanding UL/DL configuration pattern is not mandated to be configured by the network in RRC signaling. This by both RAN1 agreements and ASN.1 implementation. Further the UE should support the case that flexible resources are signaled by RRC and then follow the DCI messages indicating the direction. |
|  | 5-1a | UE specific RRC configure UL/DL assignment |  |  |  | Dynamic UL/DL determination based on L1 scheduling DCI with cell-specific and UE specific RRC configured UL/DL assignment |  |  |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  | RAN1 needs to check component |  | RAN1 |  |  |  | Optional with capability signaling |
|  | 5-2 | RA Type 0 for PUSCH |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-3 | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-4 | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-6 | PDSCH mapping type A with less than 7 OFDM symbols |  |  |  | For type 1 with dedicated RRC configuration and USS, PDSCH mapping type A with less than 7 OFDM symbols |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  | Mandatory with capability signaling which shall be set to “1” |  |  | Mandatory with capability signaling |
|  | 5-6a | PDSCH mapping type B |  |  |  |  |  |  |  |  |  |  |  |  |  | Type 4 | No need |  | No need |  |  |  |  | Mandatory with capability signaling |  | Mandatory with capability signaling |  |
|  | 5-7 | Interleaving for VRB-to-PRB mapping for PDSCH |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-8 | Interleaving for VRB-to-PRB mapping for PUSCH |  |  |  |  |  | 0-12 |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | To be deleted |
|  | 5-9 | Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 before RRC connection |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  |  | Mandatory with capability signaling |  | Mandatory with capability signaling |  |
|  | 5-10 | Inter-slot frequency hopping for PUSCH |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-11 | Up to 2 unicast PDSCHs per slot for different TBs |  |  |  | - PDSCH(s) for Msg. 4 is included |  |  |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  | Up to 2 unicast PDSCHs per slot in FDM is not supported This capability is necessary for each SCS |  |  |  |  |  | Optional with capability signaling |
|  | 5-11a | Up to 7 unicast PDSCHs per slot for different TBs |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  | Up to 7 unicast PDSCHs per slot in FDM is not supported This capability is necessary for each SCS |  |  |  |  |  | Optional with capability signaling |
|  | 5-12 | Up to 2 PUSCHs per slot for different TBs |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  | Up to 2 unicast PUSCHs per slot in FDM is not supported This capability is necessary for each SCS |  |  |  |  |  | Optional with capability signaling. Does not per say require capability 5-3a, could be multiple UL DCI from the same PDCCH |
|  | 5-12a | Up to 7 PUSCHs per slot for different TBs |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  | Up to 7 unicast PUSCHs per slot in FDM is not supported This capability is necessary for each SCS |  |  |  |  |  | Optional with capability signaling. Does not per say require capability 5-3a, could be multiple UL DCI from the same PDCCH |
|  | 5-13 | Type 1 configured PUSCH repetitions within a slot |  |  |  | 1) K = 2, 4, 8 times repetitions with RV sequences |  | 5-19 |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-14 | Type 1 configured PUSCH repetitions over multiple slots |  |  |  | 1) K = 2, 4, 8 times repetitions with RV sequences |  | 5-19 |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-15 | Type 2 configured PUSCH repetitions within a slot |  |  |  | 1) K = 2, 4, 8 times repetitions with RV sequences |  | 5-20 |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-16 | Type 2 configured PUSCH repetitions over multiple slots |  |  |  | 1) K = 2, 4, 8 times repetitions with RV sequences |  | 5-20 |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-17 | PUSCH repetitions over multiple slots |  |  |  | 1) K = 2, 4, 8 times repetitions |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-17a | PDSCH repetitions over multiple slots |  |  |  | 1) K = 2, 4, 8 times repetitions |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  |  |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-18 | DL SPS |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-19 | Type 1 Configured UL grant |  |  |  | 1) K = 1 |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-20 | Type 2 Configured UL grant |  |  |  | 1) K = 1 |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-21 | Pre-emption indication for DL |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-22 | CBG-based re-transmission for DL using CBGTI |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-23 | CBGFI for CBG-based re-transmission for DL |  |  |  |  |  | 5-22 |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-24 | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-25 | CBG-based re-transmission for UL using CBGTI |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-26 | Semi-static rate-matching resource set configuration for DL |  |  |  | 1) Bitmap 1/2/3 |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  | Mandatory with capability signaling |  |
|  | 5-27 | Dynamic rate-matching resource set configuration for DL |  |  |  | 1) Bitmap 1/2/3 |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 5-28 | Rate-matching around LTE CRS |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Mandatory with capability signaling |
|  | 5-25 5-29 | LBRM for PUSCH |  |  |  | Limited buffer rate matching in UL |  |  |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  |  |  |  |  | Mandatory with capability signaling Numbering is strange so we changed it to 5-29 |
|  | 5-30 | Scheduling PDSCH and PUSCH with the MCS table of high reliability |  |  |  | Scheduling PDSCH and PUSCH with the 64QAM MCS table which contains MCS entries with spectral efficiency lower than 2*120/1024 |  |  |  |  |  | Yes |  | High reliability transmission is not achieved for cell edge UEs |  | Type 4 | N.A. |  | N.A. |  |  |  | RAN1 |  |  |  | Optional with capability signaling New feature added |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation |  |  |  | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 2) RRC reconfiguration of any parameters related to BWP |  |  |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  | This feature should be mandatory for at least BWPs which is the same as the set of specified channel BW RAN4 may discuss other BW requirements. UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP |  |  | [Mandatory without capability signaling] |  |  | Mandatory with capability signaling |
|  | 6-2 | Type A BWP adaptation with same numerology |  |  |  | 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of each BWP includes BW of the same initial DL BWP if there is an initial DL BWP in a carrier |  | 6-1 |  |  |  | Yes |  | Type A BWP adaptation with same numerology is not possible |  | Type 1 | N.A |  | N.A. |  |  |  |  |  |  |  | Optional with capability signaling. Proposal 5) in yellow is okay |
|  | 6-3 | Type B BWP adaptation with same numerology |  |  |  | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier |  | 6-1 |  |  |  | Yes |  | Type B BWP adaptation with same numerology is not possible |  | Type 1 | N.A. |  | N.A. |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 6-4 | BWP adaptation with different numerologies |  |  |  | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time |  | 6-1 |  |  |  | Yes |  | BWP adaptation with different numerologies is not possible |  | Type 1 | N.A. |  | N.A. |  |  |  |  | Optional with capability signaling |  | Optional with capability signaling |  |
|  | 6-5 | Basic DL NR-NR CA operation |  |  |  | 1) Up to16 DL carriers 2) Same numerology across carrier for data/control channel [at a given time] |  |  |  |  |  | Yes |  |  |  | N.A. | N. A. |  | N.A. |  | This is conditioned on the support of DL CA band combination(s). The band combination definition is up to RAN4. |  |  |  |  |  | Optional with capability signaling. |
|  | 6-5a | PDCCH blind detection capability for CA |  |  |  | - More than 4 DL CCs - Reporting value is one of integer from 4 to 16 |  | 6-5 |  |  |  | Yes |  |  |  | Type 4 | No need |  | Yes |  |  |  |  |  |  |  | Mandatory with capability signaling if the UE supports more than 4 DL carriers |
|  | 6-6 | Basic UL NR-NR CA operation |  |  |  | 1) Up to16 UL carriers 2) Same numerology across carrier for data/control channel [at a given time] 3) One PUCCH group 4) Single TAG |  | [6-5] |  |  |  | Yes |  |  |  | N.A. | N.A. |  | N.A. |  | This is conditioned on the support of UL CA band combination(s). The band combination definition is up to RAN4. |  |  |  |  |  | Optional with capability signaling. |
|  | 6-7 | Two PUCCH group |  |  |  | 1) Same numerology across carriers for data/control channel [at a given time] |  | 6-5, 6-6 |  |  |  |  |  |  |  | Type 3 | N.A. |  | N.A. |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 6-8 | Different numerology across PUCCH groups |  |  |  |  |  | 6-5, 6-7 |  |  |  |  |  |  |  | Type 3 | N.A. |  | N.A. |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 6-9 | Different numerologies across carriers within the same PUCCH group |  |  |  | 1) Same numerology between DL and UL per carrier for data/control channel at a given time |  | 6-5 |  |  |  |  |  |  |  | Type 3 | N.A. |  | N.A. |  |  |  |  |  |  |  | Mandatory with capability signaling for all band combinations that include at least one band that is applicable for an LTE Band |
|  | 6-10 | Cross carrier scheduling |  |  |  | 1) Cross carrier scheduling with CIF |  | 6-5, 6-6 |  |  |  | Yes |  | Cross carrier scheduling is not possible |  | Type 3 | N.A. |  | N.A. |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 6-11 | Number of supported TAGs |  |  |  | Need of multiple capability question about the resolution here |  |  |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 6-12 | Support 2 simultaneous UL transmissions for problematic cases |  |  |  | Delete [1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell] [2) Case 2: Release 15 LTE-FDD HARQ timing] [3) UL offset for Case 1 based HARQ feedback] |  |  |  |  |  | Yes |  | 2 simultaneous UL transmissions are not supported for problematic cases |  | Type 2 |  |  |  |  | RAN2/4 to decide This is a UE feature for LTE for a LTE/NR dual connectivity UE |  | RAN2/4 |  |  |  | Comment “Delete …” should we do something about it? |
|  | 6-13 | Case 1 Single Tx UL LTE-NR DC |  |  |  | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-Pcell 2) HARQ subframe offset |  |  |  |  |  | Yes |  |  |  | Type 2 | Yes |  | Yes |  | This is a UE feature for LTE for a LTE/NR dual connectivity UE |  |  |  |  |  | Optional with capability signaling. |
|  | 6-15 | 7.5kHz UL raster shift |  |  |  | 7.5kHz UL raster shift |  |  |  |  |  | Yes |  |  |  |  | [No need] |  | [No need] |  |  |  | RAN4 |  |  |  |  |
|  | 6-16 | Supplemental uplink |  |  |  | Initial access and RRC connected operation on SUL carrier (incl 7.5kHz configurable shift) 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL 2) Supplemental uplink with same numerology between SUL and non SUL carriers |  | 6-15 |  |  |  | Yes |  | The UE will not be able to access or operate on a SUL carrier UE will not be able to perform the RACH/PUSCH/PUCCH/SRS operation in a band combination including SUL |  | N.A. | N.A. |  | N.A. |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  |  |  |  |  | Optional. |
|  | 6-17 | Supplemental uplink with different numerologies between SUL and non SUL carriers |  |  |  | Numerology other than that of associated DL |  | 6-16 |  |  |  | Yes |  | The UE will not be able to access or operate on a SUL carrier |  | Type 3 | N.A. |  | N.A. |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  |  |  |  |  | Optional with capability signaling. |
|  | 6-18 | Supplemental uplink with dynamic switch |  |  |  | DCI based selection of PUSCH carrier |  | 6-16 |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  |  |  |  |  | Optional with capability signaling. |
|  | 6-19 | Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS/PRACH on the other UL carrier in the same cell |  |  |  |  |  | 6-16 |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  |  |  |  |  |  |  | Mandatory with capability signaling. |
|  | 6-20 | Simultaneous reception and transmission on different carriers for each band combination |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Type 3 | N.A. |  | N.A. |  | RAN4 to check if there is duplicate future in their capability or not. If not, this capability is to be defined |  |  |  |  |  |  |
|  | 6-21 | DL search space sharing for CA |  |  |  |  |  |  |  |  |  |  |  |  |  | Type 3 | N.A. |  | N.A. |  |  |  |  |  |  |  | Optional with capability signaling. |
|  | 6-22 | UL search space sharing for CA |  |  |  |  |  |  |  |  |  |  |  |  |  | Type 3 | N.A. |  | N.A. |  |  |  |  |  |  |  | Optional with capability signaling. |
| 7. Channel coding | 7-1 | Channel coding |  |  |  | 1) LDPC encoding and associated functions for data on DL and UL 2) Polar encoding and associated functions for PBCH, DCI, and UCI 3) Coding for very small blocks |  |  |  |  |  | Yes |  | UE will not be able to transmit or receive data or control |  | N.A. | No need |  |  |  |  |  | RAN1 | Mandatory without capability signaling |  | Mandatory without capability signaling |  |
| 8. UL TPC | 8-1 |  |  | Dynamic power sharing for LTE-NR DC |  |  | When total transmission power exceeds Pcmax, UE scales NR transmission power. |  | EN-DC | Yes |  |  |  |  |  | Type 2 | Yes |  | Yes |  |  |  | RAN |  |  |  |  |
|  | 8-2 |  |  | Operation A with single UL Tx case 1 |  |  |  |  | EN-DC | Yes |  |  |  |  |  | Type 2 | Yes |  | Yes |  |  |  |  | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, optional for UEs supporting dynamic power sharing |  | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, optional for UEs supporting dynamic power sharing |  |
|  | 8-2 |  |  | Basic power control operation |  |  | 1) Accumulated power control mode for closed loop 2) 1 TPC command loop for PUSCH, PUCCH respectively 3) One or multiple DL RS configured for pathloss estimation 4) One or multiple p0-alpha values configured for open loop PC 5) PUSCH power control 6) PUCCH power control 7) PRACH power control 8) SRS power control 10) PHR |  |  | Yes |  |  |  |  |  | Type 4 | No need |  | No need |  |  |  |  |  |  |  | Mandatory with capability signaling, capability set = mandatory |
|  | 8-3 |  |  | TPC-PUSCH-RNTI |  |  | Specific group DCI message for TPC commands for PUSCH |  |  | Yes |  |  |  |  |  | Type 4 | No need |  | Yes |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 8-4 |  |  | TPC-PUCCH-RNTI |  |  | Specific group DCI message for TPC commands for PUCCH |  |  | Yes |  |  |  |  |  | Type 4 | No need |  | Yes |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 8-5 |  |  | TPC-SRS-RNTI |  |  | Specific group DCI message for TPC commands for SRS |  |  | Yes |  |  |  |  |  | Type 4 | No need |  | Yes |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 8-6 |  |  | Absolute TPC command mode |  |  | Absolute TPC command mode |  |  | Yes |  |  |  |  |  | Type 4 | No need |  | Yes |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 8-7 |  |  | UL power control with 2 PUSCH closed loops |  |  | Two different TPC loops |  |  | Yes |  |  |  |  |  | Type 4 | Yes |  | Yes |  |  |  |  | Mandatory with capability signaling |  | Mandatory with capability signaling |  |
|  | 8-8 |  |  | UL power control with 2 PUCCH closed loops |  |  | Two different TPC loops |  |  | Yes |  |  |  |  |  | Type 4 | Yes |  | Yes |  |  |  |  | Mandatory with capability signaling |  | Mandatory with capability signaling |  |