3GPP TSG RAN WG1 Meeting # 93                                                                               R1-1806310

Busan, Korea, May 21st –25th, 2018

Source:	CATT

Title:	Remaining evaluation assumptions

Agenda Item:	7.4.5

Document for:	Discussion and Decision

Introduction

System level simulation assumptions were discussed in RAN1#92bis with a couple of agreements.

Agreements:

- The traffic model below is used for NOMA evaluations in mMTC scenario:

  - Packet arrival per UE: Poisson arrival with arrival rate λ;

  - Packet size: 20~200 bytes Pareto + higher layer protocol overhead of [29] bytes, as defined in TR 45.820 to be the starting point

    - Other packet sizes are not precluded.

- The traffic model for NOMA evaluations in URLLC scenario is to be decided in May meeting.

- The traffic model for NOMA evaluations in eMBB scenario is to be decided in May meeting.

In this contribution, we give our evaluation results of coupling loss based on the agreed assumptions and also share our views on traffic models for NOMA evaluation in URLLC and eMBB scenarios.

Discussion

Coupling loss

The CDF of coupling loss based on the assumptions in Annex is shown below. The percentage of UEs with coupling loss larger than 144dB is 0.3% with BS antenna downtilt of 92 degree. Based on the results, It is proposed confirm the UE distribution for mMTC as 20% outdoor UEs and 80% indoor UEs.

Figure 1: CDF of coupling loss

Proposal 1: It is proposed to confirm the UE distribution for mMTC to be 20% outdoor UEs and 80% indoor UEs.

Traffic model

- URLLC

For URLLC service, there are various use cases including e.g. factory automation and smart power grid etc. Different use cases may have different traffic models and requirements of latency and/or reliability. A couple of use cases have been identified in TS22.261. There is an ongoing email discussion on use cases for Rel-16 URLLC on RAN draft email reflector.

Since the traffic model is essential for system evaluation, it is proposed to start with the assumption in Rel-15 URLLC, i.e. fixed packet size of 32 bytes and consider other traffic later if there is consensus on Rel-16 URLLC use cases.

Proposal 2: The traffic model below is used for NOMA evaluations in URLLC scenario as a starting point:

  - Packet arrival per UE: Poisson arrival with arrival rate λ;

  - Packet size: 32 bytes + higher layer protocol overhead of [29] bytes

- eMBB

In addition to full buffer traffic, the eMBB service can also be non-full buffer and the packet size can be relatively small. One of the eMBB small data traffics can be background traffic. The packet size distribution of light background in [1] is copied below.

Figure 2: UL packet size distribution of light background traffic

It is proposed to start with light background traffic and the traffic model can be discussed based on the CDF of UL packet size in Figure 3.

Proposal 3: Aperiodic traffic with packet size derived from light background traffic in 33.822 is used as a starting point for NOMA evaluations in eMBB scenario.

Conclusion

In this contribution, we gave our evaluation results of coupling loss and shared our views on traffic models for NOMA evaluation in URLLC and eMBB scenarios with following proposals.

Proposal 1: It is proposed to confirm the UE distribution for mMTC to be 20% outdoor UEs and 80% indoor UEs.

Proposal 2: The traffic model below is used for NOMA evaluations in URLLC scenario as a starting point:

  - Packet arrival per UE: Poisson arrival with arrival rate λ;

  - Packet size: 32 bytes + higher layer protocol overhead of [29] bytes

Proposal 3: Aperiodic traffic with packet size derived from light background traffic in 33.822 is used as a starting point for NOMA evaluations in eMBB scenario.

References

- TR36.822, LTE Radio Access Network (RAN) enhancements for diverse data applications, V11.0.0

- Annex

- Evaluation assumptions for couple loss

| Parameters | Values or assumptions |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 1732m |
| Carrier frequency | 700MHz |
| Simulation bandwidth | 6RB |
| Number of UEs per cell | 40 |
| Channel model | 3D UMa |
| UE Tx power | 23 dBm |
| BS antenna configurations | 2Rx |
|  | (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), 2 TXRU |
|  | dH = dV = 0.5λ; |
|  | BS antenna downtilt: 92 degree |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8dB |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 1Tx |
| UE antenna height | Follow the modelling of TR 36.873 |
| UE antenna gain | 0dBi |
| UE distribution | 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); |