3GPP TSG RAN WG1 Meeting #93																	R1-1806480

Busan, Korea, May 21st – 25th, 2018

Source: 	Intel Corporation

Title:	Remaining issues of 1024QAM support in LTE

Agenda item:	    6.2.4.1

Document for:	Discussion and Decision

# Introduction

In RAN1#92 meeting, a work item 1024QAM on support for LTE has been completed [1]. However some of the technical details relevant to 1024QAM are still missing in current specifications. In particular, the values for physical layer parameter corresponding to newly agreed UE categories and determination of soft buffer size for the rate matching purposes in case of 1024QAM has not been defined yet. In this contribution, we propose details addressing the above issues. In addition, we discuss the benefits of supporting alternative TBS for 1024QAM, which was designed in the work item without considering any target overhead assumption.

# Discussion

Physical layer parameters for DL categories

In [92b-LTE-05] email discussion the following agreement was reached regarding support of new DL Cateogiry.

Agreements: • Introduce Rel-15 DL Categories with the following data rates with optional support of 1024QAM – Cat 22: ~2.4-2.5Gbps – (20 layer 1024 or 24 layer 256QAM) – Cat 23: ~2.75-2.8Gbps – (22 layer 1024 or 28 layer 256QAM) – Cat 24: ~3Gbps – (24 layer 1024 or 30 layer 256QAM) – Cat 25: ~3.25 – 3.2 Gbps – (26 layer 1024QAM or 32 layer 256QAM) – Cat 26: ~3.4 – 3.5Gbps – (28 layer 1024QAM or 34 layer 256QAM) – Cat 20 (legacy): Enable use of 1024QAM for this category. – Other UE categories for 1024QAM are not precluded to be defined during Rel-15 and later releases • Details (including soft buffer size/legacy categories signaled/rate matching) to be decided no later than during RAN1#93.

The physical layer parameters for the proposed DL Categories are provided in the table below.

| UE DL Category | Maximum number of DL-SCH transport block bits received within a TTI (Note 1) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| DL Category 22 (2.4-2.5Gbps) | 2349504 - 2516400 | [299856 (8 layers, 64QAM) 391656 (8 layers, 256QAM) 502624 (8 layers, 1024QAM)] 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM, if alternativeTBS-Index-r14 is not supported) 201936 (4 layers, 256QAM, if alternativeTBS-Index-r14 is supported) 251640 (4 layers, 1024QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM, if alternativeTBS-Index-r14 is not supported) 100752 (2 layers, 256QAM, if alternativeTBS-Index-r14 is supported) 125808 (2 layers, 1024QAM) | 29233152 | 2 or 4 [or 8] |
| DL Category 23 (2.7-2.8Gbps) | 2697920 - 2827104 | [299856 (8 layers, 64QAM) 391656 (8 layers, 256QAM) 502624 (8 layers, 1024QAM)] 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM, if alternativeTBS-Index-r14 is not supported) 201936 (4 layers, 256QAM, if alternativeTBS-Index-r14 is supported) 251640 (4 layers, 1024QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM, if alternativeTBS-Index-r14 is not supported) 100752 (2 layers, 256QAM, if alternativeTBS-Index-r14 is supported) 125808 (2 layers, 1024QAM) | 34105344 | 2 or 4 [or 8] |
| DL Category 24 (3.2Gbps) | 3132672 – 3316544 | [299856 (8 layers, 64QAM) 391656 (8 layers, 256QAM) 502624 (8 layers, 1024QAM)] 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM, if alternativeTBS-Index-r14 is not supported) 201936 (4 layers, 256QAM, if alternativeTBS-Index-r14 is supported) 251640 (4 layers, 1024QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM, if alternativeTBS-Index-r14 is not supported) 100752 (2 layers, 256QAM, if alternativeTBS-Index-r14 is supported) 125808 (2 layers, 1024QAM) | 38977536 | 2 or 4 [or 8] |
| DL Category 25 (3Gbps) | 2936880 - 3029040 | [299856 (8 layers, 64QAM) 391656 (8 layers, 256QAM) 502624 (8 layers, 1024QAM)] 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM, if alternativeTBS-Index-r14 is not supported) 201936 (4 layers, 256QAM, if alternativeTBS-Index-r14 is supported) 251640 (4 layers, 1024QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM, if alternativeTBS-Index-r14 is not supported) 100752 (2 layers, 256QAM, if alternativeTBS-Index-r14 is supported) 125808 (2 layers, 1024QAM) | [36602208] | 2 or 4 [or 8] |
| DL Category 26 (3.5Gbps) | 3522624 - 3634848 | [299856 (8 layers, 64QAM) 391656 (8 layers, 256QAM) 502624 (8 layers, 1024QAM)] 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM, if alternativeTBS-Index-r14 is not supported) 201936 (4 layers, 256QAM, if alternativeTBS-Index-r14 is supported) 251640 (4 layers, 1024QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM, if alternativeTBS-Index-r14 is not supported) 100752 (2 layers, 256QAM, if alternativeTBS-Index-r14 is supported) 125808 (2 layers, 1024QAM) | [42275520] | 2 or 4 [or 8] |

It should be noted that the baseband and RF capabilities of the newly defined DL categories would be substantially increased comparing to the existing one. Therefore, it is also proposed to introduce enhanced UL categories supporting maximum of 8 MIMO layers and 256QAM to balance DL and UL throughput capabilities of the UE.

Proposal

- Introduce new UL categories to be used with new DL categories

  - The data rate of new UL categories should be derived assuming 256QAM support with 4-8 MIMO layers

The example of uplink physical layer parameters corresponding to the proposed UL categories are provided below.

| UE UL Category | Maximum number of UL-SCH transport block bits transmitted within a TTI | Maximum number of bits of an UL-SCH transport block transmitted within a TTI | Support for 64QAM in UL | Support for 256QAM in UL |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| UL Category 22 | 422112 | 105528 | Yes | Yes |
| UL Category 23 | 527640 | 105528 | Yes | Yes |
| UL Category 24 | 633168 | 105528 | Yes | Yes |
| UL Category 25 | 738696 | 105528 | Yes | Yes |
| UL Category 26 | 844224 | 105528 | Yes | Yes |
| NOTE 1: The UE supports "Maximum number of UL-SCH transport block bits transmitted within a TTI" and "Maximum number of bits of an UL-SCH transport block transmitted within a TTI" of 2984 bits if the UE indicates support of ce-PUSCH-NB-MaxTBS-r14. Otherwise the UE supports 1000 bits. |  |  |  |  |

The possible combinations of DL-UL Category pairings are proposed in the table below.

| UE DL Category | UE UL Category |
| --- | --- |
| DL Category 22 | UL Category 26, 25, 24, 23, 22, 21, 20, 18, 16, 15, 13, 7, 5, 3 |
| DL Category 23 | UL Category 26, 25, 24, 23, 22, 21, 20, 18, 16, 15, 13, 7, 5, 3 |
| DL Category 24 | UL Category 26, 25, 24, 23, 22, 21, 20, 18, 16, 15, 13, 7, 5, 3 |
| DL Category 25 | UL Category 26, 25, 24, 23, 22, 21, 20, 18, 16, 15, 13, 7, 5, 3 |
| DL Category 26 | UL Category 26, 25, 24, 23, 22, 21, 20, 18, 16, 15, 13, 7, 5, 3 |

To address backwards compatibility issue for new DL categories with previous LTE Releases, the pairing with UE DL Category and UE Category are provided below.

| UE DL Category | UE DL Category | UE Category |
| --- | --- | --- |
| DL Category 22 | DL Category 20, 19, 16, 12 | UE Category 12, 10, 7, 4 |
| DL Category 23 | DL Category 20, 19, 16, 12 | UE Category 12, 10, 7, 4 |
| DL Category 24 | DL Category 20, 19, 16, 12 | UE Category 12, 10, 7, 4 |
| DL Category 25 | DL Category 20, 19, 16, 12 | UE Category 12, 10, 7, 4 |
| DL Category 26 | DL Category 20, 19, 16, 12 | UE Category 12, 10, 7, 4 |

Rate matching for 1024QAM

The procedure of selecting soft buffer size for the rate matching purpose should be also updated taking into account increased TBS for 1024QAM. In particular, the selection of the soft buffer size for the rate matching purpose can be based on the DL category indicated for backward compatibility purpose with updated scaling factor Kc. The proposed text to TS 36.212 [3] is provided below.

If the UE signals ue-CategoryDL-v14xy indicating UE category M2, Nsoft is the total number of soft channel bits according to the UE category indicated by ue-CategoryDL-v14xy. Otherwise, if the UE signals ue-CategoryDL-v1310 indicating UE category M1, Nsoft is the total number of soft channel bits according to the UE category indicated by ue-CategoryDL-v1310. Otherwise, if the UE signals ue-CategoryDL-v13xy, and is configured with transmission mode 9 or transmission mode 10 and the higher layer parameter maxLayersMIMO-r10 is configured to eightLayers, Nsoft is the total number of soft channel bits according to the UE category indicated by ue-CategoryDL-v13xy. Otherwise, if the UE signals ue-CategoryDL-r12 indicating UE category 0, or if the UE signals ue-CategoryDL-r12 indicating UE category 14 and is configured by higher layers with altCQI-Table-r12 for the DL cell, Nsoft is the total number of soft channel bits according to the UE category indicated by ue-CategoryDL-r12. Otherwise, if the UE signals ue-Category-v11a0, and is configured by higher layers with altCQI-Table-r12 for the DL cell, Nsoft is the total number of soft channel bits according to the UE category indicated by ue-Category-v11a0. Otherwise, if the UE signals ue-Category-v1020, and is configured with transmission mode 9 or transmission mode 10, or is configured with transmission mode 3 or transmission mode 4 and the higher layer parameter maxLayersMIMO-r10 is configured to fourLayers, for the DL cell, Nsoft is the total number of soft channel bits [4] according to the UE category indicated by ue-Category-v1020 [6]. Otherwise, Nsoft is the total number of soft channel bits [4] according to the UE category indicated by ue-Category (without suffix) [6]. If Nsoft = 35982720 or 47431680, KC= 5, elseif Nsoft = 303562752, KC= 32, elseif Nsoft = 14616576, if the UE is configured by higher layers with altCQI-Table-r12, KC = 3/2 else KC = 2 end if. elseif Nsoft = 19488768, if the UE is configured by higher layers with altCQI-Table-1024QAM-r15, KC = 8/5 elseif the UE is configured by higher layers with altCQI-Table-r12, KC = 2 else KC = 8/3 end if. elseif Nsoft = 7308288 and the UE is configured by higher layers with altCQI-Table-r12, if the UE is capable of supporting no more than a maximum of two spatial layers for the DL cell in the transmission mode configured for the UE, or if the configured maximum number of layers indicated by the maxLayersMIMO-r10 field is no more than two, KC = 3 else KC = 3/2 end if. elseif Nsoft = 3654144 and the UE is capable of supporting no more than a maximum of two spatial layers for the DL cell, or if the configured maximum number of layers indicated by the maxLayersMIMO-r10 field is no more than two, KC = 2 else KC = 1 End if.

Alternative TBS

Rel-15 LTE specificfies altenrtive value for the maximum TBS with 1024QAM. It should be noted that the altetnative TBS for 1024QAM was desinged without consdering any target overhead assumption. At the same time the alternative ITBS (26A) for 64QAM and ITBS (33A) for 256QAM were defined in RAN1 #82bis meeting with specific overhead assumption which enables efficient support of that modulation orders in transmission mode 9 and 10.

Agreements: - In Rel-12, introduce new ITBS (26A) for 64QAM and ITBS (33A) for 256QAM and the corresponding highest TBS values assuming 120 REs/PRB-pair for the PDSCH and targeting max. code rate of 0.931 - Target max. code rate of 0.931 can be revisited to 0.93 if any issue is identified for multi-layer transmissions - Configure new ITBS for 64QAM and 256QAM by higher layer signaling - When configured, the corresponding TBS values should be applied to all the subframes for the UE scheduled by DCI formats 2C or 2D. - Introduce a new UE capability signaling to support the new maximum TBS - Capability signaling applies to 64QAM and 256QAM if UE supports UE capability of 256QAM. - RAN1 considers this feature to be optional.

Figure 1: Illustration of the code rate for 1024QAM with alternative TBS under overhead assumption defined for alternative TBS for 64QAM and 256QAM

Figure 1 shows the coding rate for the alternative TBS for 1024QAM under overhead assumption defined for alternative TBS for 64QAM and 256QAM (i.e. 120 REs per RB). It can be seen that the selected alterative TBS for 1024QAM are not usable for such assumption due to very high coding rate. Therefore, it is proposed to redesign alternative TBS values for 1024QAM taking into account the same overhead assumption as was used for 64QAM and 256QAM.

# Conclusions

In this contribution we address the reaming issues of supporting 1024QAM in Rel-15. In particular details of supporting new DL categories with 1024QAM with corresponding physical layer parameters and determination of soft buffer size for the rate matching purposes in case of 1024QAM are discussed. Based on the discussion it is also proposed to define new UL Categories with data rates of 400Mbps, 500Mbps, 600Mbps, 700Mbs and 800Mbps to be supported with newly defined DL categories. In addition, the benefits of supporting alternative TBS for 1024QAM (which was designed in the work item without considering any target overhead assumption) are also evaluated showing little benefits.

# References

- RP-171738, Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE

- TS 36.306, v.15.0.0.

- TS 36.212 v.15.0.0.